3GPP TSG-RAN2#123			R2-2308935

Toulouse, 21st – 25th August 2023

Agenda item:	7.2.2 Sidelink Positioning

Title:	On the support of UE-only SL positioning in in-coverage and partial coverage scenarios

Source:	Philips International B.V.

Document for:	Discussion

# 1. Introduction

Section 5.11 of TS 23.586 [1] provides a basis for UE-only support of SL positioning when a suitable LMF is unavailable, “e.g. the LMF in the serving network does not support SL Positioning.”

In this contribution, we argue that, even if the LMF is available and does support SL positioning, there may be reasons to prefer the use of UE-only positioning. We give some examples of this for the SL-MO-LR case and propose some guidelines on considering the use of the SL positioning server in PC and IC scenarios.

# 2. Discussion

## 2.1 Support of UE-only operation in in-coverage and partial coverage scenarios

### 2.1.1 LMF unavailable

For the SL-MO-LR case, where the LMF is involved, the UE originating the positioning request may trigger a positioning service request with the AMF. The AMF will refuse this if no suitable LMF is available. As the excerpt quoted from [1] above, this may be because there is no LMF that supports SL positioning. Other possibilities include the lack of an LMF with the available capacity to handle the request. Thus, even in a nominally in-coverage scenario, the originating UE may need to invoke UE-only SL positioning.

As per the conclusions in clause 8.4 of TS 23.700-86 [2] even if the LMF accepts the request, it can still decide that Target UE or Anchor UE should execute the result calculation.

Proposal 1: RAN2 should define the procedures to allow use of UE-only SL positioning if the LMF is unavailable and to allow the LMF to hand-off some positioning functions (e.g., result calculations) to the Target UE or Anchor UE.

### 2.1.2 Preference to use UE-only positioning over LMF-based positioning

In some in-coverage and partial coverage scenarios, it may be expedient to support the use of a SL positioning server in preference to the LMF. In a vehicular scenario, in which a vehicle, operating as Target UE, wants to assess its location with respect to vehicles around it, all respondents to the positioning operation can be Anchor UEs that, like the Target UE, are highly mobile. While all UEs are likely to be in-coverage, a requirement to turn to a fixed network LMF would seem to risk adding significant latency to the determination of location, particularly given the amount of location traffic. This may make the end result unusable. In such a situation, it would seem more appropriate to manage the location operation locally by allowing the Target UE (or an Anchor UE of its choice) to provide SL positioning server functionality.

Observation 1: There can be cases in in-coverage scenarios where it is more expedient to use a SL position server even when the LMF is available.

In the example above the latency is an important factor. Hence in such low-latency scenarios, the Target UE should be able to request that the LMF allows it to use a Positioning Server in preference to the LMF. The LMF can indicate its decision as above and the subsequent SL Positioning session can take place using an SL Positioning Server.

Proposal 2: The Target UE should be able to initiate an UE-only SL positioning operation even if it is in coverage and a suitable LMF is available.

For this to work, in-coverage UEs should be able to request or indicate a preference to the LMF to use UE-only operation.

Proposal 3: In-coverage UEs should be permitted to request or indicate a preference to the LMF to use UE-only operation.

### 2.1.3 Discovery of LMF before SL Positioning Server Selection

A Target UE that is out-of-coverage will initiate a UE-only SL positioning operation and attempt to discover Anchor UEs and Positioning Server UEs. If an Anchor UE is in coverage and has access to an LMF that supports Ranging/Sidelink positioning (i.e. Anchor UE is served by a suitable LMF), then it needs to be able to indicate this to the Target UE as soon as possible, so that the Target UE may, if it chooses, abandon the UE-only operation and engage the services of the LMF or vice-versa. Similarly, the Target UE may select another Anchor UE.

Proposal 4: During SL Positioning Discovery, the Anchor UE needs to be able to indicate that it is in coverage and has access to an LMF that supports Ranging/Sidelink positioning or not.

### 2.1.4 Discovery of LMF after Positioning Server selection

The decision to use UE-only sidelink positioning is based on whether the UEs involved are out-of-coverage/not served by a suitable LMF. However, the coverage situation may change/fluctuate, in particular in partial coverage scenarios and/or when the UEs are moving. It could be that some of the UEs involved in UE-only sidelink positioning procedure with a SL positioning server UE get in coverage during the procedure. What should happen in this scenario? Since the Target UE has engaged the services of the SL positioning server, any attempt to hand over the positioning operation to the LMF results in some complexity because the SL positioning server has to hand over some of the information it has acquired to the LMF and the Target UE and other Anchor UEs have to be notified that they should now communicate with the LMF.

Observation 2: Handover of a positioning operation from a SL positioning server to the LMF could require significant signalling.

It would seem cleaner to allow the SL positioning server to complete the positioning operation.

Proposal 5: Handover of an ongoing positioning operation from a SL positioning server to the LMF shall not be supported. A SL positioning server shall be able to complete an ongoing positioning operation.

# 3. Conclusion

In this contribution, we make the following observations:

Observation 1: There can be cases in in-coverage scenarios where it is more expedient to use a SL position server even when the LMF is available.

Observation 2: Handover of a positioning operation from a SL positioning server to the LMF could require significant signalling.

Accordingly, based on these observations, we make the following proposals:

Proposal 1: RAN2 should define the procedures to allow use of UE-only SL positioning if the LMF is unavailable and to allow the LMF to hand-off some positioning functions (e.g., result calculations) to the Target UE or Anchor UE.

Proposal 2: The Target UE should be able to initiate an UE-only SL positioning operation even if it is in coverage and a suitable LMF is available.

Proposal 3: In-coverage UEs should be permitted to request or indicate a preference to the LMF to use UE-only operation.

Proposal 4: During SL Positioning Discovery, the Anchor UE needs to be able to indicate that it is in coverage and has access to an LMF that supports Ranging/Sidelink positioning or not.

Proposal 5: Handover of an ongoing positioning operation from a SL positioning server to the LMF shall not be supported. A SL positioning server shall be able to complete an ongoing positioning operation.

# 4. References

[1]	3GPP TS 23.586: “Architectural Enhancements to support Ranging based services and Sidelink Positioning” V18.0.0

[2]	3GPP TS 23.700-86: “Study on Architecture Enhancement to support Ranging based services and sidelink positioning” V18.0.0