3GPP TSG-RAN WG2 #123	R2-2308936

Toulouse France August 21~25 2023

Agenda Item:	7.20.3

Source:	CEWiT

Title:	Discussion on MAC CE design for M-TRP

Document for:	Discussion and Decision

# Introduction

The enhancements to unified TCI framework extension for MTRP was discussed in RAN1 and agreements were made relevant to this extension. In this contribution, we discuss the MAC CE design for unified TCI framework for M-TRP based on the RAN1 agreements.

RAN2#122 [1] made the following agreements on unified TCI states for M-TRP cases:

for multi-DCI M-TRP operation

RAN2 confirm the following working assumption as an agreement:

- Revise the legacy unified TCI state activation/deactivation MAC CE by adding a “CORESET Pool ID” field to support mDCI based mTRP operation.

single DCI M-TRP operation

For sDCI based mTRP operation using unified TCI state framework, introduce the new MAC CE, with the following high level design principles:

    - If the signaling type of the unified TCI state configuration is configured by RRC (i.e. either joint DL/UL TCI state or separate DL/UL TCI state), it applies to both TRP (i.e., as configured by RRC for both TRPs).

- The following information can be indicated by the MAC CE (for joint DL/UL TCI mode):

    - if the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs,

    - if the indicated TCI codepoint consists of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s)

# Discussion

In Rel-16 NR, if a UE is configured for M-TRP operation, BS configures the UE with up to 128 TCI states using RRC signaling. Then UE is signaled with a MAC-CE that consists of up to eight code points where each code point maps to one or two TCI states out of the configured TCI states. BS signals one out of the eight code points (from MAC CE) using a DCI to the UE for activating the corresponding TCI states. In Rel-17 NR, with the introduction of unified TCI, UE can be configured with up to 128 unified TCI states for joint/DL TCI state indication and 64 TCI states for UL TCI state indication. Further, the BS signals a MAC CE to the UE that consists of up to eight code points where each code point maps to either one joint TCI state in case of a UE configured with joint unified TCI operation or one DL TCI state and one UL TCI state in case of a UE is configured with separate unified TCI operation.

When a M-TRP architecture is combined with unified TCI framework, BS needs to signal a TCI state common for data/channel/RS for each TRP in case of both separate and joint TCI indication to the UE. In order to signal the Common TCI state to a UE using DCI for each TRP, for both separate TCI and joint TCI operation, codepoints should map to multiple DL/UL TCI states in MAC CE structure. Therefore, the MAC CE structure needs to be further studied and modified for indicating multiple DL/UL TCI states to support unified TCI framework for M-TRP transmission.

For example, as shown in Fig.1 if joint TCI indication is configured through RRC, one codepoint in MAC CE maps to up to two TCI states corresponding to two TRPs respectively and if separate TCI indication is configured through RRC, one codepoint in MAC CE maps to up to four TCI states corresponding to two TRPs respectively. Therefore, for separate and joint TCI indication one codepoint in MAC CE can be associated with up to four TCI states and two TCI states corresponding to two TRPs respectively. Thus, RAN2 should consider the new MAC CE supporting up to 4 TCI states per TCI codepoint for separate TCI mode.

Fig1. Beam indication for joint and separate TCI operation

Proposal 1: Support new MAC CE structure, where one codepoint in MAC CE structure can be associated to up to four states for separate TCI mode.

For the separate TCI mode, the code point indicates the full set or subset which are following below:

- One code point can indicate full set such as DL TCI state 1st, UL TCI state 1st, DL TCI state 2nd, UL TCI state 2nd.

- One code point can indicate subset of the 4 indicate TCI states (DL TCI state 1st, UL TCI state 1st, DL TCI state 2nd, UL TCI state 2nd.)

For the above listed points, we need to define the indicators or additional bit field to indicate whether unified TCI state is for DL TCI state or the UL TCI state.

Proposal 2: Support additional bit fields in MAC CE structure to indicate whether unified TCI sate is for DL or UL.

For unified TCI state framework for MTRP, we can design the common MAC CE structure for separate TCI mode and Joint TCI mode. To design such MAC CE structure, we need to study or define additional field to indicate whether octet containing Joint/DL TCI state is present or not, whether octet containing UL TCI state is present or not and UE whether the preceding octet as padding or as DL TCI state.

In MAC CE structure, to identify that which TCI state belongs to which TRP can be done implicitly by the order of the TCI state in code point. To reduce the bits, we can define the default rules such as first TCI state (s) always represents the first TRP and second TCI state (s) always represents the second TRP.

Proposal 3: Support to design the common MAC CE structure for separate and Joint TCI mode with additional bit field to indicate:

- To indicate whether octet containing Joint/DL TCI state is present or not,

- To indicate whether octet containing UL TCI state is present or not,

Proposal 4: Support to define the rules to indicate the association of TCI state (s) to TRP implicitly.

# Conclusion

This contribution we discussed the MAC CE design for unified TCI framework for M-TRP.

Further, we summarize the proposals as follows:

Proposal 1: Support new MAC CE structure, where one codepoint in MAC CE structure can be associated to up to four states for separate TCI mode.

Proposal 2: Support additional bit fields in MAC CE structure to indicate whether unified TCI sate is for DL or UL.

Proposal 3: Support to design the common MAC CE structure for separate and Joint TCI mode with additional bit field to indicate:

- To indicate whether octet containing Joint/DL TCI state is present or not,

- To indicate whether octet containing UL TCI state is present or not,

Proposal 4: Support to define the rules to indicate the association of TCI state (s) to TRP implicitly.

# References

[1]	R2-2306552, “Report from NR MIMO evolution session, RAN2#122.