3GPP TSG-RAN2 Meeting #123	R2-2308942

Toulouse, France, 21 - 25 August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0697 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on gap requirement for inter-RAT LTE measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Nokia (Rapporteur), Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the section 9.3.1.3 and 9.3.2.3 in TS 38.300, it is stated that when the UE only supports per-UE measurement gaps or when the UE supports per-FR measurement gaps and at least one of the NR serving cell is in FR1, a measurement gap configuration is always provided to perform E-UTRA Inter RAT measurements. However, according to TS 38.331, the UE is allowed to report whether measurement gap is required to perform measurement on E-UTRA target band, and if reporting ‘nogap-noncsg’, the UE would perform measurement outside measurement gap. This needs to be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 9.3.1.3 and 9.3.2.3, for inter-RAT EUTRA measurement, specify that whether it is up to the network whether to configure the measurement gap based on UE capabilities, e.g. the needForGapNCSG-InfoEUTRA information (if supported by both UE and Network). Impact Analysis Impacted 5G architecture options: NR SA Impacted functionality: Inter-RAT EUTRA measurement Inter-operability: none as the corresponding mechanisms are already correctly specified in 38.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage-2 description of measurement gaps for inter-RAT mobility is incomplete and does not provide the same level of details as for intra-NR movbility. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.3, 9.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

9.3.1.3	Measurements

Inter RAT measurements in NR for this use case are limited to E-UTRA.

Whether a measurement is non-gap-assisted or gap-assisted depends on the capability of the UE and the current operating frequency:

-	For E-UTRA Inter RAT measurement, if the measurement gap requirement information is reported by the UE, a measurement gap configuration may be provided according to the information. Otherwise, a measurement gap configuration is always provided when:

-	The UE only supports per-UE measurement gaps; or

-	The UE supports per-FR measurement gaps and at least one of the NR serving cells is in FR1.

Next Modified Subclause

9.3.2.3	Measurements

Inter RAT measurements in NR for this use case are limited to E-UTRA.

Whether a measurement is non-gap-assisted or gap-assisted depends on the capability of the UE and the current operating frequency:

-	For E-UTRA Inter RAT measurement, if the measurement gap requirement information is reported by the UE, a measurement gap configuration may be provided according to the information. Otherwise, a measurement gap configuration is always provided when:

-	The UE only supports per-UE measurement gaps; or

-	The UE supports per-FR measurement gaps and at least one of the NR serving cells is in FR1.

End of Changes