3GPP TSG-RAN WG2 Meeting #123	R2-2309011

Toulouse, France, 21– 25 August 2023

Agenda item:	5.1.3.1

Source:	Nokia, Nokia Shanghai Bell

Title:	CSI-RS resource coordination

WID/SID:	NR_NewRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

This is a document detailing offline discussions on CSI-RS resource coordination as detailed below:

CSI-RS resource coord NR-DC R2-2307338 CSI-RS resource coordination in NR-DC Nokia, Nokia Shanghai Bell CR Rel-16 38.331 16.13.0 3990 5 F NR_newRAT-Core, TEI16 R2-2304873 R2-2307339 CSI-RS resource coordination in NR-DC Nokia, Nokia Shanghai Bell CR Rel-17 38.331 17.5.0 3991 4 A NR_newRAT-Core, TEI16 R2-2304874 R2-2307468 Discussion on CSI-RS resource coordination in NR-DC vivo discussion Rel-15 NR_newRAT-Core Moved from 6.1.3.1 Proposal 1: The following capabilities need not to be coordinated: BandNR capabilities: csi-RS-IM-ReceptionForFeedback, maxNumberSSB-CSI-RS-ResourceOneTx in beamManagementSSB-CSI-RS, maxSimultaneousResourceSetsPerCC, maxConfiguredResourceSetsPerCC in csi-RS-ForTracking. Proposal 2: MN only need to indicate the maximum number of resources that SCG is allowed to configure for the band combination currently being used by UE. R2-2308055 Discussion on CSI-RS coordination in NR-DC ZTE Corporation, Sanechips discussion Rel-15 NR_newRAT-Core Proposal 1 MN/SN coordination can be supported for per-UE capability maxNumberResAcrossCC-AcrossFR-r16, i.e. the MN can indicate a restriction value to the SN via INM. Proposal 2 MN/SN coordination can be supported for following UE capabilities, i.e. the MN can indicate separate values for FR1 and FR2 to the SN via INM. activeConfiguredGrant-r16 -> maxNumberConfigsAllCC-r16 sps-r16 -> maxNumberCSI-RS-Resource-r16 beamManagementSSB-CSI-RS -> maxNumberCSI-RS-Resource beamManagementSSB-CSI-RS ->maxNumberAperiodicCSI-RS-Resource csi-RS-ForTracking ->maxConfiguredResourceSetsAllCC maxNumberCSI-RS-BFD maxNumberCSI-RS-SSB-CBD maxNumberSSB-BFD Proposal 3 MN/SN coordination is not supported for UE capabilities that defined as maximum number of resources configured in one slot or simultaneously (e.g. maxNumberSSB-CSI-RS-ResourceOneTx, maxNumberSSB-CSI-RS-ResourceTwoTx). Proposal 4 MN/SN coordination is not supported for per-BC UE capabilities that defined as maximum number of resources configured simultaneously. (e.g. simultaneousCSI-ReportsAllCC, simultaneousSRS-AssocCSI-RS-AllCC, csi-RS-IM-ReceptionForFeedbackPerBandComb-r16). Proposal 5 MN/SN coordination is not supported for UE capabilities that defined as maximum number of resources configured per CC. (e.g. maxSimultaneousResourceSetsPerCC, maxConfiguredResourceSetsPerCC, csi-RS-IM-ReceptionForFeedback). DISCUSSION - Nokia think that SN requested update is missing, and is needed. Ericsson think that SN shall be able to indicte what is used. Huawei agrees with Ericsson. Offline 003 (Nokia) R2-2308056 Introducing CSI-RS coordination in NR-DC ZTE Corporation, Sanechips CR Rel-16 38.331 16.13.0 4222 - F NR_newRAT-Core R2-2308057 Introducing CSI-RS coordination in NR-DC ZTE Corporation, Sanechips CR Rel-17 38.331 17.5.0 4223 - A NR_newRAT-Core

# 2	CSI-RS Resource coordination

The CSI-RS resource discussions offline involved Nokia, Huawei, ZTE and Ericsson. The following principles are proposed:

-	Only per-UE limits are coordinated. Per-CC limits can be inferred from these

-	MN indicates the resource restrictions to SN and SN applies them. As response, SN indicates how many resources it has used. The MN limits are still maintained.

-	SN may request changes to the per-UE limits from MN

-	The CRs are done only from Rel-17 onwards (to avoid issues with earlier Rel-17 additions in INM).

The discussion rapporteur proposes the following to be adopted:

Proposal 1: RAN2 to adopt the following principles for the CSI-RS resource coordination:

1) MN indicates the limits to SN and SN indicates as response how many values it ends up using.

2) SN can request additional resources from MN later on and MN can accept or reject.

3) The resource limitations are done per-SCG. SN decides on how to adopt the per-CC limitations based on per-CC UE capabilities.

4) the CRs are only done from Rel-17 onwards.

There are some FFS, all of which can be decided during the post-meeting email on the detailed CR:

-	Whether to have request per allowed band combination or per SCG

-	How the encoding of SN response to the MN restrictions is done

Hence, a short email discussion seems needed to converge on the CRs, so we propose.

Proposal 2: Agree to the CRs according to P1 principles in short email discussion (ie. CRs are submitted to RAN#101).

# 3	Conclusion

This document has proposed the following:

Proposal 1: RAN2 to adopt the following principles for the CSI-RS resource coordination:

1) MN indicates the limits to SN and SN indicates as response how many values it ends up using.

2) SN can request additional resources from MN later on and MN can accept or reject.

3) The resource limitations are done per-SCG. SN decides on how to adopt the per-CC limitations based on per-CC UE capabilities.

4) the CRs are only done from Rel-17 onwards.

Proposal 2: Agree to the CRs according to P1 principles in short email discussion (ie. CRs are submitted to RAN#101).