3GPP TSG-RAN2 Meeting #123	R2-2309032

# Toulouse, France, Aug 21-25, 2023

Agenda Item:	7.11.3

Release:	Rel-18

Work Item:	NR_MBS_enh-Core

Source: 	Qualcomm Incorporated

Title: 	[Offline 601] Discussion report: Frequency and bandwidth signalling

Document for:	Discussion/Decision

# Introduction

RAN2#119bis-e agreed the following:

In MBSInterestIndication, for a broadcast service that the UE is receiving or is interested to receive, at least the following information can be signalled: broadcast frequency, subcarrier spacing, and bandwidth. FFS details/exact parameters and other information. FFS in which scenarios the UE reports this information (e.g. intra-PLMN case, inter-PLMN case)

Further, RAN2#122 agreed the following:

No additional information is added to MII on top of what has been already agreed.

RAN2#123 discussed on addressing the above highlighted FFS (stage 3 details/exact parameters of the MII contents), and agreed the following:

The SCS in the MII is set to the SCS of the CORESET#0 for the MBS broadcast cell.

Following offline was setup to discuss further on frequency and bandwidth:

[AT123][601][eMBS] Frequency and bandwidth signalling (QCM)

Scope: Discuss with companies to achieve a common understanding on frequency and bandwidth signalling for MII for shared processing

Deadline: Report available for Friday CB session

Deadline for initial feedback (please provide your comments in tables below): Thursday August 25, 7pm Tolouse local time.

# Contact information

| Company | Contact Name | Email |
| --- | --- | --- |
| Qualcomm | Umesh Phuyal | uphuyal@qti.qualcomm.com |
| Huawei, HiSilicon | Xubin | xubin10@huawei.com |
| Xiaomi | Yumin Wu | wuyumin@xiaomi.com |
| ZTE | Tao QI | qi.tao3@zte.com.cn |
| LGE | SangWon Kim | sangwon7.kim@lge.com |
| NEC | Rao | shi_rao@nec.cn |
| Ericsson | Tuomas | Tuomas.tirronen@ericsson.com |
| vivoi | Yitao Mo (Stephen) | yitao.mo@vivo.com |
| CMCC | Xiaoman Liu | liuxiaoman@chinamobile.com |

# Discussion

## Frequency Information in MII for shared processing

How frequency information is provided in USD was discussed in the context of Rel-17 discussion, particularly during offline discussion [AT119-e][605][MBS-R17] Reply LS to SA4 (Qualcomm). See Report in R2-2208884. Following snippet is from the report:

On a quick look it may look like we can simply refer to ARFCN-ValueNR. However, there is some difference between how the LTE and NR indicate frequency/band information. EARFCN in LTE can uniquely identify a particular frequency belonging to a particular band. However, in NR, ARFCN-ValueNR only indicates an absolute frequency without saying anything about the corresponding NR band that the UE may or may not support. And such frequency can belong to more than one bands indicated by FreqBandIndicatorNR. Therefore, for NR, USD should include FreqBandIndicatorNR along with ARFCN-ValueNR so that UE can correctly identify whether the USD includes a frequency of interest. Without knowledge of NR band, the UE cannot decide whether to include a given frequency in the frequency of interest, e.g. when that frequency may be supported in one band and not other if it is not clear to UE whether the service is being provided in the supported band.

Observation 4: In NR, FreqBandIndicatorNR+ ARFCN-ValueNR identifies a frequency in a band.

The above observation was supported by all the companies which responded during the offline. Based on the company inputs (see question 2 in section 4.1.1 in the report), following was included in the LS reply to SA4 (see LS Out in R2-220885):

- In NR, the frequency parameter is coded as combination of FreqBandIndicatorNR and ARFCN-ValueNR as defined in 3GPP TS 38.331 and TS 38.101.

While the above discussion was about encoding ‘frequency’ info in the USD, it seems relevant to the encoding for the MII for shared processing. Considering the above, companies are requested to answer the following question.

Q1: What IE/parameters are used to encode ‘frequency’ in the MII for shared processing sent to the unicast gNB?

- FreqBandIndicatorNR and ARFCN-ValueNR

- ARFCN-ValueNR only

- absoluteFrequencyPointA + offsetToCarrier

- Something else (please specify in comments)

| Company | Answer option a/b/c | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | b) | We think it is enough to follow the current Rel-17 signaling to only indicate ARFCN-ValueNR in the MII. The serving gNB is able to identify which band the absolute frequency is referring to. |
| Xiaomi | a or b | We agree with the above analysis that multiple bands could share the same ARFCN-ValueNR. The UE could supports only one band or all of them. Reporting only ARFCN-ValueNR can also work, since the gNB can remove all band (referring to ARFCN-ValueNR) supported by the UE. However removing all band (referring to ARFCN-ValueNR) supported by the UE is a bit over-kill. |
| ZTE | a) | not sure if for different PLMNs the serving PLMN can do the deduction as Huawei suggested. |
| LGE | a | If UE supports more than one bands that the ARFCN beongs to, the network cannot know which band is used for bradacst reception. The signalling gain which can be achieved by not transmiitng the band indicator would be marginal. |
| NEC | a | The description from rapporteur make sense to us. |
| Ericsson |  | a) defines the frequency unambiguously |
| vivo |  | Based on NR frequency definition, we think a) is the correct understanding. |
| Nokia | a) | Seems option a) provides some flexibility if both can be signalled with the FreqBandIndicatorNR as OPTIONAL field. |
| CMCC | a) | The description from rapporteur make sense to us. |
| Qualcomm | a | As explained by companies above. |

Summary:

Observation 1: On frequency info: Almost all companies (9/10) suggest to use combination of FreqBandIndicatorNR and ARFCN-ValueNR to signal the frequency information in the MII for shared processing.

Proposal 1: Combination of FreqBandIndicatorNR and ARFCN-ValueNR is used to signal the frequency information in the MII for shared processing.

## Bandwidth information in MII for shared processing

Next question is about the bandwidth info to be included in the MII for shared processing. Various options has been raised by companies on this issue. The views have been from carrier BW to CFR BW. However, we should note that

    - Carrier BW from NW point of view may be different form Carrier BW from UE point of view

    - CFR BW may not be available to the UE until later but unicast network gNB may need to take the UE resources/capability reduction for the frequency+BW even before the UE can be enabled to acquire the info about CFR from non-serving gNB.

Companies are requested to provide their view on what info is necessary/sufficient to be indicated in MII

Q2: What IE/parameters are used to encode ‘bandwidth’ in the MII for shared processing sent to the unicast gNB?

- carrierBandwidth only (INTEGER (1..maxNrofPhysicalResourceBlocks))

- carrierBandwidth + offsetToCarrier (as described in SCS-SpecificCarrier)

- locationAndBandwidthBroadcast-r17 from the MBS broadcast cell (frequencyDomainResources of CORESET0 if locationAndBandwidthBroadcast-r17 is not configured)

- CHOICE of a) and c)  e.g. based on availability at the UE

- SEQUENCE of a) and c) – one or both if available at the UE

- Other (please explain)

| Company | Answer option | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | f | We understand the reported bandwidth should be the channel bandwidth actually used by the UE to receive the broadcast from the non-serving cell, which may cover the CFR bandwidth but no larger than the carrier bandwidth of the non-serving cell. For example, the configured CFR is 20MHz and the UE may not support 20MHz but only supports 40MHz. In this case, the UE will use 40MHz channel bandwidth which covers the CFR to receive the broadcast service. We understand 40MHz should be indicated through the MII. In our view, the MII is to help the serving gNB know the shared processing situation within the UE. As we have agreed that the non-serving cell broadcast reception capability will be reported in FSPC level, with such BW indication, the serving gNB can know easily which FSPC could be used for non-serving cell broadcast reception, and help the UE with simultaneous unicast and broadcast reception with proper scheduling. |
| Xiaomi | a | The CFR bandwidth is larger than or full contains the bandwidth of the initial BWP of the non-serving cell. This means that while receiving MBS, the channel bandwidth used by the UE should be the CFR bandwidth. On the other hand, the serving gNB is able to know all the bandwidth reported via the UE capability signaling. Using the CFR bandwidth and the UE capability signaling together, the serving gNB is able to know the remaing capability can be used. If we use the channel bandwidth used by the UE, it is unclear how this could be tested. Additionally, we think that PRB should be used to indicate the bandwidth, as the SCS is always reported together with bandwidth. |
| ZTE | c | or the CFR bandwidth |
| LGE | c | Though the channel bandwidth actually used by the UE is larger than the CFR BW, network can know it based on the CFR BW and UE capability. |
| NEC | c fine to d | With a small comment on c that it is not clear to still keep r17 here, maybe we can remove “r17”. |
| Ericsson | C / ? | Initially we were thinking c) but are open to discuss more whether CFR relation could be used |
| vivo | C | We think the UE should include the CFR bandwidth as it is related to the FFT/IFFT implementation and UE buffer status (which are UE internal resources that are shared between unicast and broadcast ). |
| Nokia | c) |  |
| CMCC | c) | We think CFR related information can be reported. |
| Qualcomm | D or e | At initial thinking, C may look sufficient, but since UE may not have information about the CFR in all cases, it may be beneficial to provide CFR BW if that is already known, if not then provide carrier BW. |

Summary:

Observation 2: On BW info in MII for shared processing, company views are slightly diverse on this question where several companies leaning towards CFR information. Further discussion is needed for convergence.

Proposal 2: On BW information in MII for shared processing: Attempt a bried online discussion. TBD whether to continue via post meeting email or discuss next meeting based on contributions.

# Summary

Observation 1: On frequency info: Almost all companies (9/10) suggest to use combination of FreqBandIndicatorNR and ARFCN-ValueNR to signal the frequency information in the MII for shared processing.

Proposal 1: Combination of FreqBandIndicatorNR and ARFCN-ValueNR is used to signal the frequency information in the MII for shared processing.

Observation 2: On BW info in MII for shared processing, company views are slightly diverse on this question where several companies leaning towards CFR information. Further discussion is needed for convergence.

Proposal 2: On BW information in MII for shared processing: Attempt a bried online discussion. TBD whether to continue via post meeting email or discuss next meeting based on contributions.