3GPP TSG-RAN2 Meeting # 123	R2-2309111

# Toulouse, France, August 21st - 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.351 | CR | 0024 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SRAP for sidelink relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Philips International B.V., Nokia, Samsung, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_Relay-Core |  |  |  |  |  | Date: |  |  | 2023-08-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In clause 5.2.2.2 on egress RLC channel determination for DL data transfer, when Relay UE determines egress PC5 Relay RLC channel by matching bearer mapping of the RB identity based on BEARER ID, for DRB, the DRB identity is BEARER ID plus 1. It should be clarified in the specification. - In clause 5.2.3, need alignment on the text for the DRB identity calculation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 5.2.2.2, clarify for DRB the DRB identity is BEARER ID plus 1. - In clause 5.2.3, clarify on the text in the brackets for the DRB identity calculation at Remote UE passing SRAP SDU to the upper layer entity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Relay UE may determine the wrong DRB identity and match the wrong bearer and wrong egress PC5 Relay RLC channel thereof. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2, 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 0: R2-2307756 Rev 1: - Align the text of DRB identity calculation in clause 5.2.3; - Unmark the Radio Access Network checkbox in the “Proposed change affects” section of the coversheet, as suggested by Apple. |  |  |  |  |  |  |  |  |

Start Change

#### 5.2.2.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	if the SRAP Data PDU is for SRB0 (the BEARER ID field is 0 and the bearer is identified as SRB based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelUu which matches the LCID of the Uu Relay RLC Channel from which the SRAP Data PDU is received):

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC0 as specified in TS 38.331 [3].

-	else if there is an entry in sl-RemoteUE-ToAddModList, whose sl-LocalIdentity included in sl-SRAP-ConfigRelay matches the UE ID field in SRAP Data PDU, which includes an sl-RemoteUE-RB-Identity that matches the SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (For the BEARER ID shared by both SRB and DRB, SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelUu which matches the LCID of the Uu Relay RLC Channel from which the SRAP Data PDU is received, and for DRB, the DRB identity is BEARER ID plus 1):

-	if the SRAP Data PDU is for SRB1 but the corresponding sl-EgressRLC-ChannelPC5 is absent in sl-SRAP-ConfigRelay:

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC1 as specified in TS 38.331 [3].

-	else:

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to sl-EgressRLC-ChannelPC5 configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

Next Change

### 5.2.3	Receiving operation of U2N Remote UE

Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity shall:

-	if the SRAP Data PDU is not for SRB0 (not received from SL-RLC0 as specified in TS 38.331 [3]):

-	if the SRAP Data PDU is received from SL-RLC1 as specified in TS 38.331 [3]:

-	remove the SRAP header of this SRAP Data PDU and deliver the SRAP SDU to PDCP entity of SRB1 by ignoring the UE ID field and BEARER ID field of this SRAP Data PDU;

-	else:

-	remove the SRAP header of this SRAP Data PDU and deliver the SRAP SDU to upper layer entity corresponding to the BEARER ID field of this SRAP Data PDU (For the BEARER ID shared by both SRB and DRB, SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelPC5 which matches LCID of the PC5 Relay RLC Channel from which the SRAP Data PDU is received, and for DRB, the DRB identity is BEARER ID plus 1);

-	else:

-	deliver the SRAP SDU (i.e., same as SRAP PDU for SRB0) to upper layer, i.e., RRC layer entity (TS 38.331 [3]).

End of Change