3GPP TSG-RAN WG2 Meeting #123	R2-2309178

Toulouse, France, August 21-22, 2023

Title:	LS on U2U Agreements

Response to:	-

Release:	Release 18

Work Item:	NR_SL_relay_enh-Core

Source:	InterDigital

To:	TSG SA WG2

Cc:

Contact Person:

#### Name:	Martino Freda

E-mail Address:	martino.freda@interdigital.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 discussed U2U relays in RAN2#123 and made the following agreements:

Agreements:

UE in RRC_CONNECTED state can obtain UE-to-UE relay discovery parameters in dedicated discovery configuration.

For integrated discovery DCA message, no AS criterion is needed for the relay UE to forward the response message to the source Remote UE.

For Model B, the relay UE forwards the solicitation message only if the PC5 RSRP between the relay UE and the source remote UE is above a threshold.

For Model B, no AS criterion is needed for the relay UE to forward the response message to the source Remote UE.

E2E SL-SRB and E2E SL-DRB use different index(es).

Fixed index (i.e., 0/1/2/3) are defined for E2E SL-SRB 0/1/2/3 respectively.

Use specified PC5 RLC Channel configuration on each hop for E2E SL-SRB 0/1/2/3.

The TX Remote UE derives the PDCP and SDAP configuration for e2e SL-DRB and provides the portion of the configuration related to RX to the RX Remote UE using E2E PC5-RRC message (similar to legacy PC5 configuration).

The TX Remote UE derives the first hop configuration (e.g. PC5 relay RLC Channel configuration) for SL-DRB and provides to the relay UE the portion of the configuration related to RX on the first hop (i.e., Rx by the relay UE), using per-hop PC5-RRC message (similar to legacy PC5 configuration).

The two conclusions above do not exclude the derivation involving information from gNB/preconfiguration/specified configuration.

Split PDB is sent to the source (TX) Remote UE from the Relay UE.

It is left to Relay UE implementation on how to split the PDB.

The Relay UE derives the second hop configuration (e.g. PC5 relay RLC Channel configuration) for each SL-DRB.

It is FFS how the Relay UE derives second hop configuration for SL-DRB.

Same as L3 based U2U relay, the QoS split should be per e2e QoS flow, and RAN2 expect that the source UE will inform the Relay UE QoS flow(s) and corresponding QoS profiles.  FFS if this requires AS signalling or can be done in upper layers.

At least PDB is sent from the source UE to the relay UE for splitting.

The source UE sends to the Relay UE all the QoS profiles for the e2e QoS flows.

At least for single-hop relay, use local ID instead of L2 ID as UE ID in SRAP header.

At least for single-hop U2U relay, two local IDs are included in SRAP header to identify source and target Remote UE respectively.  FFS impact on SRAP header.

For single-hop U2U relay, the local ID for a particular UE is the same on both hops.

2. Actions:

To SA WG2

ACTION: 	RAN2 would like to ask SA2 to take the above agreements into account into their work.

3. Date of Next TSG-RAN WG2 Meeting:

RAN2#123-bis-e	from 2023-10-9	to 2023-10-13		Xiamen, CN

RAN2#124	from 2023-11-13	to 2023-11-17		Chicago, US