3GPP TSG-RAN WG2 #123	R2-2309230

Toulouse, France, 21st – 25th August 2023

Agenda Item:	7.24.2

Source:	Ericsson

- Title:			[AT123][408][POS] Rel-18 positioning RRC CR (Ericsson)

Document for:	Discussion, Decision

# Introduction

This is to kick off the email discussion.

- [AT123][408][POS] Rel-18 positioning RRC CR (Ericsson)

Scope: Collect comments on the CR in R2-2308484 and produce an endorsable version.

Intended outcome: Endorsable CR

Deadline: Thursday 2023-08-24 2000 UTC

# 2	Discussion

Please provide your comments on the CR.

| Company Name | Comments |
| --- | --- |
| Intel | - RRC CR and MAC CR need to align the name of validityArea based TAT time; - SRS-PosRRC-Inactive-VA-Config-r18, dash is not needed between Inactive and VA; - The name should be aligned, e.g. VA should be used for all fields or not use for any fields. It would be good to avoid to use VA for some fields, and validityArea for other fields. - srs-PosRRC-InactiveValidityArea-r18 SEQUENCE (SIZE(1..maxNrOfCellIsInVA-r18)) OF PhysCellId, is based on PCI, should we add freq in order to avoid PCI confusion issue? Rapp: Yes cellID has been used instead of PCI. |
| Huawei HiSilicon | 1> if the RRCRelease includes suspendConfig: 2> reset MAC and release the default MAC Cell Group configuration, if any; 2> apply the received suspendConfig except the received nextHopChainingCount; 2> if the sdt-Config is configured: 3> for each of the DRB in the sdt-DRB-List: 4> consider the DRB to be configured for SDT; 3> if sdt-SRB2-Indication is configured: 4> consider the SRB2 to be configured for SDT; 3> for each RLC bearer (except those associated with broadcast MRBs) that is not suspended: 4> re-establish the RLC entity as specified in TS 38.322 [4]; 3> for SRB2 (if it is resumed) and for SRB1: 4> trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5]; 3> if sdt-MAC-PHY-CG-Config is configured: 4> configure the PCell with the configured grant resources for SDT and instruct the MAC entity to start the cg-SDT-TimeAlignmentTimer; 2> if srs-PosRRC-Inactive is configured: 3> apply the configuration and instruct MAC to start the inactivePosSRS-TimeAlignmentTimer; 2> if srs-PosRRC-InactiveValidityArea is configured: 3> apply the configuration and instruct MAC to start the srs-PosConfigValidityAreaTAT; The above chagne has not been agreed yet. For pre-configured SRS, tehre is no need to start the inactive POS TAT immediately after the UE receives the pre-configured SRS configuration Rapp: FFS has been added |
| Huawei, HiSilicon | SRS-PosRRC-InactiveConfig-r17 ::= SEQUENCE { srs-PosConfigNUL-r17 SRS-PosConfig-r17 OPTIONAL, -- Need R srs-PosConfigSUL-r17 SRS-PosConfig-r17 OPTIONAL, -- Need R bwp-NUL-r17 BWP OPTIONAL, -- Need S bwp-SUL-r17 BWP OPTIONAL, -- Need S inactivePosSRS-TimeAlignmentTimer-r17 TimeAlignmentTimer OPTIONAL, -- Need M inactivePosSRS-RSRP-ChangeThreshold-r17 RSRP-ChangeThreshold-r17 OPTIONAL -- Need M } SRS-PosRRC-Inactive-VA-Config-r18 ::= SEQUENCE { srs-PosRRC-InactiveValidityArea-r18 SEQUENCE (SIZE(1..maxNrOfCellIsInVA-r18)) OF PhysCellId, srs-PosPreConfigNUL-r18 SRS-PosConfig-r17 OPTIONAL, -- Need R srs-PosPreConfigSUL-r18 SRS-PosConfig-r17 OPTIONAL, -- Need R bwp-NUL-r18 BWP OPTIONAL, -- Need S bwp-SUL-r18 BWP OPTIONAL, -- Need S inactivePosSRS-TAT-VA-r18 ENUMERATED {ms1280, ms1920, ms2560, ms5120, ms10240, ms20480, ms40960, infinity}, ... } Need to explain that the SRS-POSpreconfig and SRS-POSConfig do not need to be present at the same time. Do we need to configure two bwp-NUL/SUL?? Rapp: Yes, I think so since either one of them will be configured. |
| Qualcomm | SRS-PosRRC-Inactive-VA-Config-r18 ::= SEQUENCE { srs-PosRRC-InactiveValidityArea-r18 SEQUENCE (SIZE(1..maxNrOfCellIsInVA-r18)) OF PhysCellId, srs-PosPreConfigNUL-r18 SRS-PosConfig-r17 OPTIONAL, -- Need R srs-PosPreConfigSUL-r18 SRS-PosConfig-r17 OPTIONAL, -- Need R bwp-NUL-r18 BWP OPTIONAL, -- Need S bwp-SUL-r18 BWP OPTIONAL, -- Need S inactivePosSRS-TAT-VA-r18 ENUMERATED {ms1280, ms1920, ms2560, ms5120, ms10240, ms20480, ms40960, infinity}, ... } Is PCI enough? Shouldn't this be a CGI? This is not necessarily pre-configured SRS. |
| OPPO | - No need to introduce the new abbreviation VA - Clause 5.3.8.3: srs-PosConfigValidityAreaTAT should be inactivePosSRS-TAT-VA 2> if srs-PosRRC-InactiveValidityArea is configured: 3> apply the configuration and instruct MAC to start the srs-PosConfigValidityAreaTAT; - Suggest the following changes of the names to align the naming style of Rel-17 Inactive mode positioning SRS configurations, i.e., - Remove "Config" in SRS-PosRRC-InactiveValidityAreaConfig-r18 - Use ValidityArea instead of VA - Remove "Pre" in srs-PosPreConfigNUL-r18 and srs-PosPreConfigSUL-r18, since "Pre" is also not used for Rel-17 Inactive mode positioning SRS configurations, e.g. srs-PosConfigNUL-r17. SuspendConfig ::= SEQUENCE { (The unrelated text is omitted.) srs-PosRRC-Inactive-r17 SetupRelease { SRS-PosRRC-Inactive-r17 } OPTIONAL, -- Need M (The unrelated text is omitted.) [[ srs-PosRRC-InactiveValidityArea-r18 SetupRelease { SRS-PosRRC-InactiveValidityArea-r18 } OPTIONAL -- Need M ]] } (The unrelated text is omitted.) SRS-PosRRC-Inactive-r17 ::= OCTET STRING (CONTAINING SRS-PosRRC-InactiveConfig-r17) SRS-PosRRC-InactiveValidityArea-r18 ::= OCTET STRING (CONTAINING SRS-PosRRC-InactiveValidityArea-Config-r18) SRS-PosRRC-InactiveConfig-r17 ::= SEQUENCE { srs-PosConfigNUL-r17 SRS-PosConfig-r17 OPTIONAL, -- Need R srs-PosConfigSUL-r17 SRS-PosConfig-r17 OPTIONAL, -- Need R bwp-NUL-r17 BWP OPTIONAL, -- Need S bwp-SUL-r17 BWP OPTIONAL, -- Need S inactivePosSRS-TimeAlignmentTimer-r17 TimeAlignmentTimer OPTIONAL, -- Need M inactivePosSRS-RSRP-ChangeThreshold-r17 RSRP-ChangeThreshold-r17 OPTIONAL -- Need M } SRS-PosRRC-InactiveValidityArea-Config-r18 ::= SEQUENCE { srs-PosRRC-ValidityArea-r18 SEQUENCE (SIZE(1..maxNrOfCellIsInValidityArea-r18)) OF PhysCellId, srs-PosConfigNUL-r18 SRS-PosConfig-r17 OPTIONAL, -- Need R srs-PosConfigSUL-r18 SRS-PosConfig-r17 OPTIONAL, -- Need R bwp-NUL-r18 BWP OPTIONAL, -- Need S bwp-SUL-r18 BWP OPTIONAL, -- Need S inactivePosSRS-TAT-ValidityArea-r18 ENUMERATED {ms1280, ms1920, ms2560, ms5120, ms10240, ms20480, ms40960, infinity}, ... } |
| vivo | 2> if srs-PosRRCInactiveValidityAreaVA-Config is configured: IE names of SRS-PosRRC-InactiveValidityArea are not aligned in the procedure and ASN.1 |
| vivo | inactivePosSRS-ValidityAreaTAT-r18 ENUMERATED {ms1280, ms1920, ms2560, ms5120, ms10240, ms20480, ms40960, infinity}, The value of inactivePosSRS-ValidityAreaTAT should be FFS, may rely on RAN1 LS Rapp: FFS has been added. |
| vivo | Editor’s Note: For preconfigured SRS, there is no need to start the inactivePosSRS-ValidityAreaTAT immediately. But for Periodic SRS the above clause would be needed. Agreement says: “Periodic SRSis supported to be configured with validity area. This agreement does not affect preconfigured SRS.” How to differentiate normal and preconfigured SRS. How/When to start/restart the TAT for preconfiguraton has not been fully discussed. Prefer to remove the EN or refine as ' FFS how/when to start/restart the TAT for pre-configured SRS‘ Rapp: Ok FFS has been updated. |
| vivo | 1> else if cell reselection occurs when srs-PosRRC-InactiveValidityAreaConfig is configured and if the cell is not included in the srs-PosConfigValidityArea: 2> indicate to the lower layer to stop inactivePosSRS-ValidityAreaTAT; 2> release the srs-PosRRC-InactiveValidityAreaConfig. FFS whether to release the configuration upon relection to a cell out of validity area. E.g., the configuration can be used for delta configuration Rapp: FFS has been added |
| vivo | SRS-PosRRC-InactiveValidityAreaConfig-r18 ::= SEQUENCE { srs-PosRRC-InactiveValidityArea-r18 SEQUENCE (SIZE(1..maxNrOfCellIsInVA-r18)) OF CellIdentity srs-PosConfigNUL-r18 SRS-PosConfig-r17 OPTIONAL, -- Need R srs-PosConfigSUL-r18 SRS-PosConfig-r17 OPTIONAL, -- Need R bwp-NUL-r18 BWP OPTIONAL, -- Need S bwp-SUL-r18 BWP OPTIONAL, -- Need S inactivePosSRS-ValidityAreaTAT-r18 ENUMERATED {ms1280, ms1920, ms2560, ms5120, ms10240, ms20480, ms40960, infinity}, ... } Agreement - From RAN1 perspective, it is feasible to configure SRS positioning validity area-specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state. Details can be up to RAN2. - For TA validation, use of area-specific RSRP change threshold is feasible RSRP change threshold is missing. Rapp: added as FFS |
| Lenovo | Cover page: in the meeting dates wrong month “May”. On validity area configuration: - The unique global cell id (CellIdentity = 36 bits) is needed. Phy cell id is not sufficient due to PCI confusion. - As discussed the cells of the validity area can belong to different frequencies, so the frequency information is missing in ASN.1. Name of validity area timer and should be kept align, and the use of abbreviations for VA should also kept align. For example: In 5.3.8, the decription is as following: 2> if srs-PosRRC-InactiveValidityArea is configured: 3> apply the configuration and instruct MAC to start the srs PosConfigValidityAreaTAT; While in other field, the valiidty TAT timer is: inactivePosSRS-TAT-VA Rapp: Name aligned |
| ZTE | Comment based on V02 version: 2> release the srs-PosRRC-Inactive. 1> else if cell reselection occurs when srs-PosRRC-InactiveValidityAreaConfig is configured and if the cell is not included in the srs-PosConfigValidityArea: 2> indicate to the lower layer to stop inactivePosSRS-ValidityAreaTAT; 2> release the srs-PosRRC-InactiveValidityAreaConfig. This ‘release SRS configuration‘ part is not agreed yet Rapp: Added as Editor’s Note |

# Conclusion

In the previous sections we made the following observations:

Based on the discussion in the previous sections we propose the following:

# References

R2-2308484, “Rapporteur CR for Positioning RRC Changes”, Ericsson