# 3GPP TSG RAN2#123 Meeting  	                                          R2-230xxxx

# Toulouse, France, Aug 21st – 25th, 2023

Agenda item:	7.4.2.3

Title:	Report of Offline Discussion [016] for Early TA Acquisition and RACH-Less

Source:	ZTE Corporation

Document for:	Discussion and Decision

Introduction

This contribution is for the offline discussion 016：

CB Friday Offline 016, progress Early TA and RACH-less (ZTE)

Companies participant in the offline discussion pleas fill in the following table:

| Company | Email |
| --- | --- |
| ZTE | Dong.fei@zte.com.cn |
| CATT | Zhangbufang@catt.cn |
| Samsung | anilag@samsung.com |
| Huawei, HiSilicon | Shiyulong5@huawei.com |
| OPPO | youxin@oppo.com |
| Transsion Holdings | junwei.huang@transsion.com |
| Futurewei | jialinzou@futurewei.com |
| Panasonic | Quan.kuang@eu.panasonic.com |
| ITRI | Moumou3@itri.org.tw |
| MediaTek | li-chuan.tseng@mediatek.com |
| vivo | Chenli5g@vivo.com |
| Ericsson | antonino.orsino@ericsson.com |
| Qualcomm | Ozcan Ozturk, oozturk@qti.qualcomm.com |
| LGE | gyeongcheol.lee@lge.com |
| Xiaomi | wuyumin@xiaomi.com |
| NTT DOCOMO | kouki.yamashita.dz@nttdocomo.com |
| Charter Communications | c-phillip.oni@charter.com |
| DENSO | daiki.maemoto.j7w@jp.denso.com |
| Fujitsu | Jia Meiyi, jiameiyi@fujitsu.com |
| Nokia | Endrit Dosti, endrit.dosti@nokia.com |
| Sharp | kyosuke_inoue@sharp.co.jp |
| CMCC | tangxiaoxuan@chinamobile.com |

Discussion

RACH for Early TA Acquisition

RACH Configuration for early TA acquisition

| Contributions | Proposals |
| --- | --- |
| R2-2307610 HuaWei | Proposal 7a: For each candidate, the ASN.1 structure of early RACH configuration includes: - uplinkConfigCommon UplinkConfigCommon; - supplementaryUplinkConfig UplinkConfigCommon; where UplinkConfigCommon includes the BWP-UplinkCommon IE, which at least configures the genericParameters field (with the IE BWP) and rach-ConfigCommon field. |
| R2-2307223 CATT | Proposal 2: To enable UE to decide the frequency resource of the RACH occasion, the following candidate options can be considered. - Option 1: NW provides the absolute frequency of lowest RACH occasion in frequency domain; - Option 2: NW provides the generic parameters of the BWP where the RACH resource is configured and also the IE absoluteFrequencyPointA and offsetToCarrier corresponding to the subcarrierSpacing as indicated within the IE BWP; - Option 3: NW provides the absolute frequency of the lowest BWP in frequency domain where the RACH resource is configured, and the SCS. |

In the contribution R2-2307610 and R2-2307223, it is mentioned how to configure the RRC configuration for early TA acquisition. In general both contributions thinks the RRC configurations for early TA acquisition should be configured out of the container (e.g. UE is not necessary to decode the container for applying the early TA configuration before LTM). In this sense, rapporteur would like to confirm whether all companies are standing on the same page.

Q1: Should RRC configuration related to early TA acquisition be configured out of the container for each LTM candidate configuration?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | Assuming the early RACH configuration is configured in the container, that means, UE have to decode and apply the early RACH related configuration (e.g. most of them are PHY layer parameters) when performing early RACH, after that, UE should release the configuration of candidate cell and reapply the configuration of the serving cell when coming back to the serving cell. The UE behavior are too complex to define, in this sense, we think the early RACH related configuration shall be configured out of the container. |
| CATT | Yes, but | The issue on early RACH configuration is similar like the L1 measurement for LTM or TCI states for LTM configuration, so it is nature to place the early RACH configuration in the same place like the L1 measurement configuration and TCI. Maybe more exacted, we can say “RRC configuration related to early TA acquisition is included in the LTM-Config IE and is a separate configuration, e.g. outside of the LTM candidate configuration”. Besides, to my understanding, HW and CATT propose another question on early RACH configuration, that is, how to configure the early RACH configuration so that UE can correctly decide the RACH resources. That is because, in legacy, the frequency location of the RACH resource is configured/defined relative to the BWP, but now before UE access the candidate cell, there is no BWP there. So we need to further discuss on how to configure the early RACH resource. |
| Samsung | Yes | For detailed signaling prefer to approach indicated by Huawei (R2-2307610) |
| Huawei | Yes, but | It is already agreed somehow. We should work on the detailed configuration of this early RACH, as commented in above. CATT proposal (appreciated) only considers the frequency issue. As in Huawei proposal, reuse the current IE like “BWP-UplinkCommon: {genericParameters BWP;rach-ConfigCommon” would be the sufficient configuration (cover the option 2 in CATT proposal). |
| OPPO | Yes |  |
| Transsion | Yes | Using separate IE outside the container to provides RACH configuration can be refered. The UE can using RACH configuration through candidate cell ID as indicated from lower without applying candidate cell configurations. |
| Futurewei | Yes | The same reason as for LTM measurement configuration. |
| Panasonic | Yes | The same reason as for LTM measurement configuration and TCI state configuration for candidate cells. |
| NEC | Yes | we think it is better to put the early RACH related configuration out of the container. |
| Lenovo | Yes | early TA acquisition should be configured out of the container which can avoid the decoding container before performing cell switching. |
| ITRI | Yes |  |
| MediaTek | Yes | As UE needs such information before LTM cell switch, and we do not mandate UE to read the candidate container before receiving LTM command, it would be clearer to have RRC configuration related to early TA acquisition configured outside the container |
| vivo | Yes | In RAN2#121bis, we have the following agreements: RRC RACH configuration for early TA acquisition (e.g., including whether RAR needs to be received) is specific per target cell and is signalled separately (separate IEs) from the candidate cell configuration (the part that need to be applied at cell switch). We think we have agreed that the RRC configuration for early TA acquisition is out of the container of candidate configuraiton. And UE needs to decode the RRC configuraiton for early TA acquisition upon UE receives the RRC message. |
| Ericsson | Yes |  |
| Qualcomm | Yes | Agree with ZTE |
| LGE | Yes |  |
| Xiaomi | Yes |  |
| NTT DOCOMO | Yes |  |
| Charter Communications | Yes |  |
| DENSO | Yes | Separate IE from candidate configuration is better, and the UE can use RACH configuration for early TA acquisition without applying candidate configuration. |
| Fujitsu | Yes |  |
| Nokia, Nokia Shanghai Bell | Yes | The RRC configuration related to early TA acquisition should be outside the container of the candidate cells, in order to be decodable at the time of the reception of the PDCCH order. In case it is part of the candidate cell configuration, it will require to perform decoding at the time of the reception of the PDCCH order. |
| Sharp | Yes | We have same understanding as CATT for the location of early RACH related configuration. And we are also fine with Huawei’s proposal for the detailed signalling. |

| CMCC | Yes |  |
| --- | --- | --- |

For this issue, there are 22 companies participating in the discussion, and all companies think the early RACH related RRC configuration shall be configured out of the container for LTM candidate cell configuration. And also two companies mentioned that there was one agreement achieved in RAN2#121bis meeting have shown the same meaning. In rapporteur understanding, the agreement in RAN2#121bis meeting has some differences with the current question, in RAN2#121bis meeting, only confirm the RACH configuration shall be put outside of the container, and in this question, we confirm all RRC configuration related to early RACH shall be configured out of the container. In this sense, we suggest to propose:

(24/24)All the RRC configurations related to early RACH are specific per LTM candidate cell and signalled separately from the candidate cell configuration (i.e. LTM Candidate configuration).

RACH modelling for early TA acquisition

| Contribution | Proposals |
| --- | --- |
| R2-2307290 vivo | Proposal 4: RAN2 to confirm there is only one RACH procedure ongoing at any point in time in a MAC entity, no matter whether the RACH procedure is legacy RACH or RACH for early TA acquisition. |
| R2-2307670 Xiaomi | Proposal 1: The early-TA RACH procedure is associated with the MAC entity providing the PDCCH-order triggering the early-TA RACH procedure. Proposal 2: As legacy, a MAC entity does not support parallel RACH procedures. |
| R2-2307180 Lenovo | - Proposal 1: A separate MAC entity is used for Early TA acquisition - Proposal 2: For early TA acquisition while there is an ongoing RA in serving cell, two RACHs are allowed in parallel. FFS: UE capability |
| R2-2308840 ZTE | Proposal 1: The PDCCH ordered early TA acquisition is a kind of PDCCH ordered RACH. Proposal 2: As legacy, the power ramping counter for PDCCH ordered RACH toward a LTM candidate cell only can be reset at each instance of initiation of RACH. Proposal 3: As legacy, in case of preamble transmission, the power ramping counter for PDCCH ordered early RACH can be incremented by 1 only at the instance of preamble transmission. |
| R2-2307209 CATT | Proposal 1: The early TA acquisition should follow the legacy principle, i.e., only one Random Access procedure ongoing at any point in time in a MAC entity. Parallel RACH procedure between serving cell and candidate cells as well as between multiple candidate cells is not supported. |
| R2-2307396 Fujitsu | Proposal 1: One MAC entity is maintained for RACH over serving cell and candidate cell. In case of DC, MAC entity associated with received PDCCH order is used for RACH procedure over candidate cell(s). |

As mentioned in the contribution shown in above, most companies think the RACH for early TA acquisition is the RACH procedure share one MAC entity with legacy RACH procedure in RRC connected mode. And one company think the RACH for early TA acquisition performed in a separate MAC entity from the legacy RACH procedure in RRC connected mode. Rapporteur would like to align the companies’ understanding on the early RA modeling, Therefore, companies are invited to answer the following question:

Q2: The early RACH modelling is confirmed as:

- Option 1: The RACH for early TA acquisition share one MAC entity with legacy RACH procedure in RRC connected mode.

- Option 2: The RACH for early TA acquisition performed in a separate MAC entity from the legacy RACH procedure in RRC connected mode.

| Company | Options | Comments |
| --- | --- | --- |
| ZTE | Option 1 | Early RACH procedure is a kind of the RACH procedure which should share a same entity with other legacy RACH procedure. |
| CATT | Option 1 | In fact, I do not think this is a real RACH procedure e.g., UE just send a signal (preamble) to NW to acquire TA. |
| Samsung | Option 1 | Parallel RA procedures are not needed. |
| Huawei | Option 1 | Which is already endorsed in the MAC running CR. |
| OPPO | Option 1 |  |
| Transsion | Option 1 | There is no need to introduce another separate MAC entity for candidate cell, which masks implementation complicated and specification impacts. |
| Futurewei | Option 1 |  |
| Panasonic | Option 1 |  |
| NEC | Option 1 |  |
| Lenovo | See comments | Regarding early TA RACH, this RACH is performed towards candidate SCG based on the RO configured by candidate SCG. In legacy principle, MAC entity is per cell group(CG). Therefore, a separate MAC entity for Early TA acquisition could be possible. But we can follow majority view if most companies prefer option 1. |
| ITRI | Option 1 |  |
| MediaTek | Option 1 | We do not see the need to have separate MAC entity |
| vivo | Option 1 | Option 2 has larger specification impact, while option 1 is straightforward. |
| Ericsson | Option 1 |  |
| Qualcomm | Option 1 |  |
| LGE | Option 1 with comment | Option 1 would be clear for intra-DU case, but considering that each DU may have different MAC configuration, it would be good to clarify how to work option 1 for inter-DU case. |
| Xiaomi | Option 1 |  |
| NTT DOCOMO | Option 1 |  |
| Charter Communications | Option 1 | Use same MAC CE as the legacy RACH |
| DENSO | Option 1 | We think parallel RA procedures are not need. |
| Fujitsu | Option 1 |  |
| Nokia, Nokia Shanghai Bell | 1 | We think that the “share/not share AC entity” is not very clear wording as there is no sharing of random access procedures. Regarding whether the UE will maintain a separate power ramping counter for LTM, and after the transmission of PRACH towards a candidate cell, the status (value) of the counter can be kept at MAC for later retransmissions even meanwhile a RACH procedure is triggered towards a serving cell. |
| Sharp | Option 1 |  |
| CMCC | Option 1 |  |

For this issue, there are 24 companies participating in the discussion, and two companies think it may have benefits and need discussion for the inter-DU LTM, but both of them also accept and follow majorities. From rapporteur understanding, there is no critical issue can be found so far in case where one MAC entity is shared between early RACH and legacy RACH. In this sense. Rapporteur propose:

(24/24) The early RACH procedure share a same MAC entity with the legacy RACH procedure. (e.g. no extra MAC entity is needed for early RACH)

| Contribution | Proposals |
| --- | --- |
| R2-2307290 vivo | Proposal 6: It is up to UE implementation on whether UE performs UE initiated RACH in serving cell or PDCCH order triggered RACH for early TA acquisition when RACH collision occurs. |
| R2-2307670 OPPO | - RACH towards serving cell is prioritized if there is concurrent RACH for early TA acquisition. - If a new RACH towards serving cell is triggered while another RACH for early TA acquisition is already ongoing in the MAC entity, UE stops the ongoing RACH towards candidate and initiates the new RACH procedure towards serving cell. - If a new RACH for early TA acquisition is triggered while another RACH procedure towards serving cell is already ongoing in the MAC entity, UE keeps the ongoing procedure and ignores the new RACH trigger. - If a new RACH for early TA acquisition is triggered while another RACH for early TA acquisition is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new RACH procedure. |
| R2-2307396 Fujitsu | Proposal 1bis: If Proposal 1 is agreed, RAN2 to discuss the prioritization of RA procedure on serving cell and RA procedure on candidate cell. Proposal 2: It is up to UE implementation whether to start with a new RACH procedure if the new RACH for early TA acquisition is triggered while there is RACH for early TA acquisition ongoing. - Proposal 2bis: RAN2 to discuss how the UE identifies whether a new or same RA procedure on candidate cell is triggered. |
| R2-2308840 ZTE | - The early RACH procedure have a lowest priority than any other legacy RACH procedure. - The collision among early RACH procedures can be avoided by NW implementation. |
| R2-2307209 CATT | Proposal 2: It’s up to UE implementation whether to continue with the ongoing procedure or start with new procedure upon RACH collision between candidate cells or between candidate cell and serving cell. |

Assuming option 1 is preferred, the handling of parallel RACH procedures in one MAC entity are mentioned by above contributions in the list, and it can be mainly categorized into two options in general:

- Option 1: It is up to UE implementations to handle the collision of RACH procedures where the early RACH is getting involved.

- Option 2: The early RACH procedure has a lowest priority than any other legacy RACH procedures.

  - Option 2-1 the collision between early RACH procedure can be handled by UE implementation

  - Option 2-2 the collision between early RACH procedure can be avoided by NW implementation.

The companies support option 1 in Q2 are invited to answer the following question:

Q3: For handling the RACH procedure collision where the early RACH is getting involved, which option is preferred, option 1 or option 2; And if option 2 is preferred, for the case of collision between early RACH procedures, which option is preferred, option 2-1 or option 2-2?

| Company | Options | Comments |
| --- | --- | --- |
| ZTE | Option 2, Option 2-2 | From the NW perspective, the certain UE behavior is always preferred. Considering the legacy RACH procedure is much more critical than early RACH, we prefer to unify the UE behavior when collision with early RACH is occurred. As for the collision case beween two early RACH procedures, we think the NW can handle it the collision will never be happened. |
| CATT | Option 1 | For UE triggered RACH, how NW can know it is conflicted? |
| Samsung | Option 1 | Prefer to leave to UE implementation as in legacy |
| Huawei | Option 1 | It can be the baseline. |
| OPPO | Option 1 or option 2-1 | For NW triggered RACH, we believe NW can avoid the collision since both the early sync to candidate cell and the RACH to serving cell are triggered by source serving cell. While we understand the issue is mainly about the collision between early sync and UE triggered RACH, which can not be avoided by NW. The early RACH procedure has a lower priority than any other RACH towards serving cell and we can leave it to UE implementation on collision handling. |
| Transsion | Option 2, Option 2-1 | UE keeps connection with servig cell has higher priority than raely TA acquisition. Once there is a collision in RACH to serving ccll and requeset to candidate cell UE should initial RACH to serving first. According to current specification, there are many reasons to initial RACH procedure, for instance the SR has reach muximun retransmissions, the NW may not exactly nows the current UE situration and to avoid RACH collision, for such case, the process for RACH collision can be handled by UE implementation. |
| Futurewei | Option 1 | Prefer Option 1. If needed, can consider setting priority rule for UE to handle. It is also under discussion in RAN1. |
| Panasonic | Option 1 | Prefer to leave to UE implementation as in legacy. RAN1 is also discussing the same topic. |
| NEC | Option 1 | UE implementation is sufficient |
| Lenovo | Option 2 | If option 2 can be agreed, it means not ‘UE implementation’ because it will impact UE behaviour. In addition, if we go for option 2-2 and network implementation can be used to avoid collision, the option2 is not needed. |
| ITRI | Option 1 | The same view as CATT. Regarding the UE-triggered RACH, the network cannot have knowledge. Therefore, a preference is to leave it to UE implementation as in the legacy approach. |
| MediaTek | Option 1 | Early RACH is a one-shot procedure since UE does not monitor RAR. We believe that the probability of RACH collision is very low, and UE can handle it. |
| Vivo | Option 1 with comments | For the collision between early TA acquisition based RACH procedure, it could be avoided by NW implementation. For the collision between early TA acquisition based RACH procedure and legacy RACH, it is up to UE implementation to determine to perform which RACH procedure. However, this question implies that UE can determine whether a early TA acquisition procedure collides with a legacy RACH. We wonder whether this is the case. For example, after UE sent preamble for a early TA acquisition procedure, a legacy RACH is triggered. Let’s assume the UE prioritizes the early TA acquisition procedure. Now UE has different actions based on whether the early TA acquisition procedure is finished(i.e. whether the RACH collision occurs). Action1: if the early TA acquisition procedure is finished, UE should start the legacy RACH; Action2: if the early TA acquisition procedure is ongoing, UE should continue the ongoing early TA acquisition procedure; But, the UE has no idea whether the early TA acquisition procedure is ongoing(i.e. whether a PDCCH order with retx indication will be received), the UE can not determine which action to take. Hence, we propose RAN2 to discuss for UE how to determine a early TA acquisition procedure is finished or not(or how to determine a early TA acquisition procedure collide between a legacy RACH procedure if the early TA acquisition procedure is performed before the legacy RACH procedure). |
| Ericsson | Option 1 but see comment | In current MAC spec we have the following NOTE: NOTE 1: If a new Random Access procedure is triggered while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure (e.g. for SI request). We think same should apply to LTM in this case. |
| Qualcomm | Option 1 | NW may not always prevent collision, e.g. RACH is triggered by uplink transmissions. It is simpler to leave the decision to the UE. |
| LGE | Option 1 | Considering no RAR after sending preamble for early TA acquisition, collision may not be much and option 1 is enough. |
| Xiaomi | Option 1 | A smart UE implementation can choose the RACH procedure which is more important for the UE. |
| NTT DOCOMO | Option 1, or Option 2 with comment | We think UE implementation is sufficient. On the other hand, if RAN2 specify that the early RACH procedure has a lowest priority, should specify detail procedure about collision handling. |
| Charter Communications | Option 1 | Early RACH collision should be treated as legacy RACH collision, no need to introduce priority tagging to the RACH procedures. RAN2 can leave it up to UE implementation. |
| DENSO | Option 1 | Prefer UE implementation. we think it is under the discussion in RAN1. |
| Fujitsu | Option 1 | Option 1 is preferred since the UE has full information of initiated RACH and thus can decides whether to start a new RACH procedure. |
| Nokia, Nokia Shanghai Bell | 2 is preferred. FFS on how the prioritization is enabled | In case of two parallel RACH processes the early TA acquisition RACH process should be prioritized in order to avoid possible failures (e.g., Beam Failure) in the serving PCell. Anyway, maintaining the connection to the serving PCell is more important than obtaining TA from an LTM candidate cell. |
| Sharp | Option 2, Option 2-2 | For the collision between early RACH for candidate cell and legacy RACH for serving cell, early RACH procedure towards candidate cell has lower priority than any legacy RACH procedure towards serving cell. For the collision between early RACH for different candidate cells, this can be avoided by NW implementation because early TA RACH is PDCCH order. |
| CMCC | Option 1 | Up to UE implementation. |

According to the discussion, there are 24 companies participated in the discussion, and there are 19 companies think smart UE implementation can handle the collision where the early RACH is getting involved. And there are 5 companies think that is good to have a certain UE behavior if the collision is occurred since the legacy RACH procedure is much more critical than early RACH. To respect the majorities, rapporteur decides to propose to go for UE implementation. Considering the there have been a NOTE captured in the specification:

NOTE 1:   If a new Random Access procedure is triggered while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure (e.g. for SI request)

Rapporteur think the NOTE exhibited as above is enough for early RACH case, so it is proposed :

(19/24) It is up to UE implementation to handle the RACH collisions where the early RACH is getting involved. No specification change can be foreseen.

Power Ramping Counter for Early TA Acquisition

Apart from the early RACH modelling issue, another key issue is how to implement the power ramping for early RACH procedure. For this issue, RAN1 have concluded the agreement as below in the parallel WG meeting.

Agreement

For the power control of PDCCH-ordered CFRA in LTM, the UE can maintain only one power ramping counter

Likewise, there is one counter for calculating the power ramping step in the legacy RACH procedure which is PREAMBLE_POWER_RAMPING_COUNTER. In this sense, it shall be clarified whether the counter mentioned in RAN1 agreement is an existing counter (i.e. PREAMBLE_POWER_RAMPING_COUNTER) or a new counter dedicatedly for early RACH.

In rapporteur’s understanding:

If PREAMBLE_POWER_RAMPING_COUNTER is commonly applied for both early RACH and legacy RACH, the power ramping counter for early RACH toward one candidate cell might be interfered by the legacy RACH procedure in some cases. for example, the PREAMBLE_POWER_RAMPING_COUNTER is incremented by three times for one candidate cell, then a legacy RACH is initiated by UE itself (e.g. BFR for SpCell) and successful completion with only one attempt, the PREAMBLE_POWER_RAMPING_COUNTER comes back to the initial value (i.e. 1) according to the legacy RACH procedure, and the power ramping for the subsequent early RACH toward to the same candidate cell only can be recounted from 1 since there is only one bit in PDCCH order to indicate the preamble transmission type (i.e initial transmission or retransmission)

If introducing a new counter dedicatedly for early RACH, there is no any interference from legacy RACH procedure, but the price is that UE need to maintain extra power ramping counter for early RACH.

According to above analysis, companies are invited to answer the following question:

Q4: The COUNTER to calculate the power ramping step for RACH of early TA acquisition is to:

- Option 1: reuse the PREAMBLE_POWER_RAMPING_COUNTER

- Option 2: Introduce a new COUNTER dedicatedly for early RACH

| Company | Options | Comments |
| --- | --- | --- |
| ZTE | Option 2 | It can avoid the interference from the legacy RACH procedure. |
| Samsung | Option 1 | Multiple counters leads to increase UE complexity |
| Huawei | Option 1 | Let’s try to make it simple. |
| OPPO | Option 2 | In case UE triggers another RACH between initial transmission and retransmission of the preamble for early sync , separate counter is preferred. |
| Transsion | Option 1 | From question Q3, we had consensus on there is only RACH procedure at a time, we can reuse PREAMBLE_POWER_RAMPING_COUNTER for early TA acquisition. |
| Futurewei | Option 1 | No need to have a separate counter as long as the priority rule is setting on source cell RACH vs candidate cell RACH. |
| Panasonic | Option 1 | If only one RACH procedure is allowed at a time, no interference would happen. |
| NEC | Option 1 | Maintaining multiple power ramping counters may cause problem for UE implementation |
| Lenovo | Option 2 | It seems option 2 is simple because it can avoid interference. |
| ITRI | Option 1 | A preference is to keep it simple. |
| MediaTek | Option 1 | We do not expect RACH towards serving cell and different candidate cells to overlap with each other. Therefore, one counter is enough and it can be reset when performing RACH towards a different cell. |
| vivo | Option 1 | Option 1 is more straightforward, and we don’t think the issue raised by rapporteur in our understanding, simultaneous ongoing RACH procedures in one UE MAC should be avoided and it could be up to UE implementation to determine whether UE perform the early TA acquisition based RACH procedure or perform the legacy RACH (e.g. BFR for SpCell or MO intiated data arrival) when they collide. And even UE performs legacy RACH, the only impact is that the Network may trigger early TA acquisition for this candidate cell more times, we think the impact is not big and could be accepted. |
| Ericsson | Option 1 |  |
| Qualcomm | Option 1 | Assuming we will only have one RACH procedure at any given time, there won’t be any interference issues |
| LGE | Option 1 | Simple approach is preferred. |
| Xiaomi | Option 1 |  |
| NTT DOCOMO | Option 2 | We have same view as other companies supporting Option 2. |
| Charter Communications | Option 1 | Reuse PREAMBLE_POWER_RAMPING_COUNTER. |
| DENSO | Option 1 | Multiple counters may cause problem and complexity. |
| Fujitsu | Option 2 |  |
| Nokia, Nokia Shanghai Bell | Discussion needed 2 | With option 1 the same counter would be used for LTM RACH (no RAR) and RACH for other reasons. Since there is an agreement to support power ramping for LTM RACH, the MAC would need to maintain the state/value of the counter upon initiating another RA procedure (e.g., UE initiated or PDCCH ordered RACH). After completion of another RACH procedure, the power ramping should not be restarted unless e.g., initial transmission is indicated. With option 2, there is a separate counter at MAC layer that would not be affected by other RACH procedure (e.g., UE initiated) which may provide cleaner solution. In both options, the question is still on the RACH modelling i.e., whether the RACH procedure is completed from the MAC perspective upon providing PRACH TX indication to lower layer. |
| Sharp | Option 2 | Agree with Rapporteur’s understanding that Option 1 might cause some interferences between legacy RACH and early RACH. |
| CMCC | Option 1 |  |

According to the comments in discussion, there are 23 companies joined in the discussion, among companies, there are 6 companies support to have a separate COUNTER dedicatedly for early RACH while there are 16 companies support to share the same COUNTER (i.e. PREAMBLE_POWER_RAMPING_COUNTER) with the legacy RACH procedure. One company think this issue is quite related to how to model early RACH procedure. Just note that, in parallel RAN1 discussion, they have concluded as below:

Agreement For the power control of PDCCH-ordered CFRA in LTM, - When a UE receives a PDCCH order indicating a re-transmission of PRACH with the same associated SSB and same candidate cell as the previous PRACH, the counter is increased by 1. - In addition to case 1, power-ramping counter is reset in the following cases: - Case 2: The candidate cell indicated in the PDCCH order, indicating retransmission, is different from that indicated in the last PDCCH order. Note: the initial counter is 0 before receiving any PDCCH order.

As we all know, the PREAMBLE_POWER_RAMPING_COUNTER only can be reset to 1 not 0, it means for now, we have two ways to go, if we following RAN1 agreement, it means we need a separate counter dedicatedly for early RACH. Another way is to follow majorities, that is , resue PREAMBLE_POWER_RAMPING_COUNTER. In this sense, we need to inform RAN1 of our decision.

For counting the power ramping step for early RACH, to select one of the following:

  - (17/23)Reuse PREAMBLE_POWER_RAMPING_COUNTER，and sent the information to RAN 1 via LS to revert their agreement (i.e. the initial value of the counter for calculating power ramping step is 1)

  - (06/23)Introduce a new COUNTER, that the initial value of the counter is 0.

In the parallel RAN1 meeting, a following agreement regarding the power ramping counter reset is achieved:

Agreement For the power control of PDCCH-ordered CFRA in LTM, power-ramping counter is reset at least when UE receives a PDCCH order indicating the initial transmission of PRACH

According to the agreements, at least one condition for resetting the counter is determined, that is, the early RACH is initiated by a PDCCH order with an initial transmission indication. Also in R2-2307209, it is mentioned that the power ramping counter is reset by the reception of PDCCH order with an initial transmission indication.

In R2-2308840, one issue is identified: considering there is only one COUNTER for early RACH and the DCI loss is inevitable, the preamble counter value would not be reset if the PDCCH order with an initial transmission indication is lost, as a result, the power ramping counter should be incremented on the basis of the value left from the previous RACH procedure if the PDCCH order with a retransmission indication is received then. For resolving this issue, one direct way is to reset counter as long as the candidate cell for early RACH is changed compare to the previous RACH procedure regardless of the preamble transmission type (i.e. initial transmission/retransmission) indicated in the PDCCH order.

In R2-2307168, for resolving the same issue, another solution is to revert the previous RAN1 agreement:

Agreement On the determination of the PRACH transmission power when reception of RAR is not configured, a 1-bit field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH, FFS - UE will increase the power with the value of power ramping configuration if it is indicated as re-transmission, unless the max allowed power is achieved - whether/how to reset the counter

R2-2307168 suggest to indicate actual values of power ramping counter via PDCCH order rather than the transmission type indication.

Companies are invited to answer the following question:

Q4-1: In case of early RACH , the COUNTER for power ramping step is reset:

- Option 1: if the candidate cell indicated in PDCCH order is not as same as the previous RACH or the initial transmission of the preamble is indicated

- Option 2: if the initial transmission is indicated [RAN1 agreements]

- Option 3: The power ramping counter value is informed to UE by PDCCH order.

| Company | Options | Comments |
| --- | --- | --- |
| ZTE | Option 1 | It can avoid the TX power of preamble transmission go too higher due to the DCI loss where the initial transmission of preamble is indicated. |
| CATT | Option 2 | Option 2 is aligned with RAN1 agreements. And Option 2 is simple from UE perspective. If we go Option 1, UE need to check whether it is the new RACH or retransmission by comparing the new PDCCH triggered for the new RACH and the PDCCH order triggered for the last RACH procedure. If we go Option 3, this is conflicted with RAN1 agreement, and also option 3 causes more overhead for DCI, but as we know, DCI size is quite limited. |
| Samsung | Option 3/Option 1 | 1st preference is Option 3. Indication of counter in PDCCH order can take care of various cases (missing PDCCH order, failed transmission due to LBT failure, UE initiated RA between multiple PDCCH order for early TA for same candidate cell) of mismatch of counter between UE and network. 1 bit field for initial/Retx can be changed to 3/4 bits in DCI. It option 3 is not acceptable, prefer to agree on option 1. |
| Huawei | Option 2, but | Option 2 is implemented in the endorsed MAC running CR. We prefer to wait for RAN1 progress otherwise. |
| OPPO | Option 2 |  |
| Transsion | Option 1 |  |
| Futurewei | Option 2 | It seems Option 1 is not needed if the early RACH process is performed once a time. The topic is also under discussion in RAN1. Can wait for RAN1 conclusion. |
| Panasonic | Option 1 or Leave it to RAN1 | RAN1 is discussing the same topic. |
| NEC | See comments | Following RAN1 agreement may be enough. No need to discuss. |
| Lenovo | Option 1 | It can avoid a new field to be added in PDCCH order. |
| ITRI | Option 2 | Option 2 is aligned with RAN1 agreements. |
| MediaTek | Option 1 | (see above) |
| vivo | Option 1 | Compared with option2, option 1 could resolve the issue that PDCCH order with an initial transmission indication is lost. Hence, we prefer option1. Besides, for the case that UE miss the PDCCH order with an initial transmission indication and there is no previous early TA acquisition procedure, we think the COUNTER should also be reset when a PDCCH order with the retransmission indication is received. Hence we suggest to modify the option1 as following：. Option 1: if the candidate cell indicated in PDCCH order is not as same as the previous RACH or if the PDCCH order for early TA acquisition is the first one received after UE access the serving SpCell or the initial transmission of the preamble is indicated RAN1 is also discussing this issue, we could update the agreement later based on RAN1 conclusion. |
| Ericsson | Wait for RAN1 | Since this is currently discussed in RAN1 we should not have overlapping discussion. |
| Qualcomm | Option 2 | We can go with the RAN1 agreement; don’t see a good argument to revert it in RAN2. |
| LGE | Option 2 | We can follow RAN1 agreement unless there is a critical problem. We think that DCI for initial transmission may not be missed frequently. |
| Xiaomi | Option 2 | For Option 1, it seems that when the gNB indicates the change of the cell for early-TA RACH, the gNB shall always indicate the initial transmission of preamble. |
| NTT DOCOMO | Option 1 | In the RAN1 session yesterday, Option 1 was agreed additionally. We want to support the latest RAN 1 agreement. |
| Charter Communications | Option 1 |  |
| DENSO | See comments | Following RAN1 agreement. |
| Fujitsu | Option 1 | Same view as DOCOMO. |
| Nokia, Nokia Shanghai Bell | 1 | RAN1 has already made further agreements on this, so maybe no need to discuss this further in R2: Agreement For the power control of PDCCH-ordered CFRA in LTM, the UE can maintain only one power ramping counter Agreement For the power control of PDCCH-ordered CFRA in LTM, power-ramping counter is reset at least when UE receives a PDCCH order indicating the initial transmission of PRACH Agreement For the power control of PDCCH-ordered CFRA in LTM, - When a UE receives a PDCCH order indicating a re-transmission of PRACH with the same associated SSB and same candidate cell as the previous PRACH, the counter is increased by 1. - In addition to case 1, power-ramping counter is reset in the following cases: - Case 2: The candidate cell indicated in the PDCCH order, indicating retransmission, is different from that indicated in the last PDCCH order. Note: the initial counter is 0 before receiving any PDCCH order. |
| Sharp | Option 1 | Same view as DOCOMO and Nokia. |
| CMCC | Option 1 | Following the RAN1 agreements. |

In the discussion for this issue, there are 24 companies participated in the discussion, among them: 13 companies explicitly support option 1, and 8 companies explicitly support option 2, and 3 companies think we shall follow the RAN1 agreement. And as pointed out by NOKIA, RAN1 have achieved final agreement about the conditions to reset of the counter:

Agreement For the power control of PDCCH-ordered CFRA in LTM, - When a UE receives a PDCCH order indicating a re-transmission of PRACH with the same associated SSB and same candidate cell as the previous PRACH, the counter is increased by 1. - In addition to case 1, power-ramping counter is reset in the following cases: - Case 2: The candidate cell indicated in the PDCCH order, indicating retransmission, is different from that indicated in the last PDCCH order. Note: the initial counter is 0 before receiving any PDCCH order.

However, RAN1 agreement is literally not completely aligned with option 1, in RAN1 agreements, the candidate cell is different from that indicated in the last PDCCH order (e.g. early RACH) while the candidate cell is different from the previous RACH (e.g. including both early RACH and legacy RACH) , and there are 5 companies explicitly support option 2 comments they also would like to follow the RAN1 agreements, in this sense, rapporteur to down select the option 2 and leave option 1 and RAN1 agreements on the table for discussion, rapporteur to propose:

Apart from the condition where the initial transmission of preamble is indicated in the PDCCH order for early RACH, RAN2 confirms the power ramping counter also should be reset by an extra condition:

  - the candidate cell indicated in PDCCH order is not as same as the previous RACH

  - the candidate cell indicated in PDCCH order is not as same as the previous early RACH.

With the same reason:

in R2-2307209, it is mentioned that the power ramping counter is incremented by 1 if the PDCCH order with a re-transmission indication is received for a candidate cell.

In R2-2308840, the increment of COUNTER for power ramping have the same options to implement/

R2-2307168 suggest to indicate actual values of power ramping counter via PDCCH order rather than the transmission type indication which violate the RAN1 agreement.

so companies are invited to answer the following question:

Q4-2: In case of early RACH , the COUNTER for power ramping step shall be incremented by 1:

- Option 1: if the re-transmission is indicated and the indicated candidate cell is as same as the previous RACH.

- Option 2: if the re-transmission is indicated

- Option 3: The power ramping counter value is informed to UE by PDCCH order.

| Company | Options | Comments |
| --- | --- | --- |
| ZTE | Option 1 | As in Q4-1 |
| CATT | Option 2 | Detailed reasons can be seen as comments provided in Q4-1. |
| Samsung | Option 3, Option 1 | 1st preference is Option 3. Indication of counter in PDCCH order can take care of various cases (missing PDCCH order, failed transmission due to LBT failure, UE initiated RA between multiple PDCCH order for early TA for same candidate cell) of mismatch of counter between UE and network. 1 bit field for initial/Retx can be changed to 3/4 bits in DCI. It option 3 is not acceptable, prefer to agree on option 1. |
| Huawei | Option 2 | Option 2 is implemented in the endorsed MAC running CR. We prefer to wait for RAN1 progress otherwise. |
| OPPO | Option 1 |  |
| Option 1 | Option 1 |  |
| Futurewei | Option 2 |  |
| Panasonic | Option 1 or leave it to RAN1 | RAN1 is discussing the same topic. |
| NEC | See comments | Following RAN1 agreement may be enough. No need to discuss. |
| Lenovo | Option 1 | If PDCCH order indicates the same candidate cell (explicit indication for re-transmission is not needed), the COUNTER for power ramping step shall be incremented by 1. |
| ITRI | Option 2 | For option 1, the UE may need to verify whether the indicated candidate cell is the same as the previous RACH. This could require additional effort for the MAC. |
| MediaTek | Option 1 | RACH towards different cells should not overlap |
| vivo | Option 1 with comments | We wonder whether a timer needs to be introduced? For example, a early TA acquisition for candidate cell1 is initiated at T1. After a while (e.g. 3s), the serving DU think the TA for candidate cell1 is invalid. Hence, the serving DU initiates a early TA acquisition for candidate cell1 a second time in T2. Unfortunately, the PDCCH with the initial transmission indication is missed this time. As a result, UE receives a PDCCH order with retransmission indication for candidate cell1 after T2. If this case is valid, UE will increase the counter by 1 if option1 or 2 is adopted. We propose RAN2 to discuss whether a timer should be introduced to handle the case. RAN1 is also discussing this issue, we could update the agreement later based on RAN1 conclusion. |
| Ericsson | Wait for RAN1 | Since this is currently discussed in RAN1 we should not have overlapping discussion. |
| Qualcomm | Option 2 |  |
| LGE | Option 2 | We can follow RAN1 agreement unless there is a critical problem. |
| Xiaomi | Option 2 | As explained above, when the gNB indicates the change of the cell for early-TA RACH, the gNB shall always indicate the initial transmission of preamble. |
| NTT DOCOMO | Option 1 | Same as Q4-1. |
| Charter Communications | Option 1 |  |
| DENSO | See comments | Following RAN1 agreement may be enough. |
| Fujitsu | Option 1 | Option 1 is aligned with new RAN1 agreement. |
| Nokia, Nokia Shanghai Bell | 1 with comments | To our understanding option 1 is preferred but it is also needed to be indicated whether the same SSB has been used before for RACH. Specifically we propose to modify Option 1 to: if the re-transmission is indicated and the indicated candidate cell and SSB are as same as the previous RACH. |
| Sharp | Option 1 |  |
| CMCC | Option 1 |  |

In the discussion, there are 23 companies participated, 12 companies explicitly support option 1, and 7 companies explicitly support option 2, and others think to follow RAN1 conclusion. In rapporteurs’ understanding, in legacy RACH procedure , if SSB is changed compare to the previous preamble transmission, the power ramping should be pended, hence it is to good to follow RAN1 following agreements:

Agreement For the power control of PDCCH-ordered CFRA in LTM, - When a UE receives a PDCCH order indicating a re-transmission of PRACH with the same associated SSB and same candidate cell as the previous PRACH, the counter is increased by 1. - In addition to case 1, power-ramping counter is reset in the following cases: - Case 2: The candidate cell indicated in the PDCCH order, indicating retransmission, is different from that indicated in the last PDCCH order. Note: the initial counter is 0 before receiving any PDCCH order.

RAN2 confirms the counter for power ramping step is incremented by 1:

  - If the retransmission is indicated and the indicated candidate cell and SSB are as same as the previous RACH

In addition, R2-2307168 mention that preamble transmission may encounter the LBT failure and think there may be two ways go for:

- Option 1: UE autonomously retransmit PRACH preamble without power ramping upon LBT failure as in legacy

- Option 2: UE retransmits without power ramping upon reception of PDCCH order with retransmission indication

Companies are invited to answer the following question:

Q5: If the preamble transmission for candidate cell encounter the LBT failure, which option is preferred:

- Option 1: UE autonomously retransmit PRACH preamble without power ramping upon LBT failure as in legacy

- Option 2: UE retransmits without power ramping upon reception of PDCCH order with retransmission indication

| Company | Options | Comments |
| --- | --- | --- |
| ZTE |  |  |
| Samsung | Option 2 | UE retransmits without power ramping upon reception of PDCCH order with retransmission indication for an LTM candidate cell, if last Random Access Preamble transmission towards that LTM candidate cell was not transmitted due to LBT failure. |
| OPPO | Option 2 |  |
| Transsion | Option 1 |  |
| Futurewei |  | Can be further discussed. Option 2 would be a good candidate. |
| Lenovo | Option 2 |  |
| ITRI | Option 2 |  |
| vivo | Option2 | Autonomously retransmit of PRACH preamble is not preferred since UE has no idea which RACH resource is allocated to it for retransmission. |
| Qualcomm | Option 1 | We should keep existing NR-U functionality. If LBT fails, NW does not know this and the UE will not get TA. |
| LGE | See comment | We are not sure that discussion for unlicensed band is essential for now and like to defer this discussion at least until the basic procedure is defined. |
| Xiaomi | Option 2 |  |
| Charter Communications | Option 1 | Keep legacy LBT procedure. |
| DENSO | Option 2 |  |
| Fujitsu | Option 2 |  |
| Nokia, Nokia Shanghai Bell | 1 | Ok to follow legacy |
| CMCC | Option 2 |  |

In this discussion, there are 16 companies joined in the discussion, and 10 companies explicitly support option 2, and 4 companies explicitly support option 1, and 2 companies think it can be further discussed and not urgent for now. Rapporteur would like to propose by simply following majorities’ view:

(10/16)UE retransmits the preamble without the power ramping upon reception of PDCCH order with retransmission indication if preamble transmission encounter the LBT failure.

Early RACH Completition

For completion/terminating the early RACH procedure, the following opinions are collected from the following contributions:

| Tdocs | Proposal |
| --- | --- |
| R2-2308840 ZTE | Proposal 4: The PDCCH triggered RACH for early TA acquisition is a single-shot-RACH procedure, that is, the RACH triggered by PDCCH order toward a LTM candidate cell is considered as complete once the preamble transmission is instructed to lower layer. |
| R2-2307290 vivo | Proposal 5: Specify the way to determine early TA acquisition procedure completion, an ongoing early TA acquisition procedure is considered completed if any of the following occurs: - UE receives a PDCCH order for early TA acquisition procedure which indicates it is the initial transmission. - UE receives a PDCCH order for early TA acquisition procedure which indicates a candidate cell different from the ongoing early TA acquisition procedure. - the newly introduced timer expires, the timer is introduced for early TA acquisition which starts when UE sends the preamble. |
| R2-2307888 ITRI | Proposal 1: The UE start the ra-ResponseWindow when the UE transmits the preamble for LTM early TA acquisition. Proposal 2: The UE should stop the ra-ResponseWindow timer after receiving the cell switch MAC CE. Proposal 3: For the early TA acquisition, the UE should declare the LTM early TA acquisition is failure when the ra-ResponseWindow timer expires. Proposal 4: For the early TA acquisition, when the ra-ResponseWindow timer expires, UE should report the failure information to the serving cell. |
| R2-2308213 Transsion Holdings | Proposal 7 RAN2 should introduces a new mechanism, i.e. new window for LTM reception, to monitor if PDCCH-order triggered RA is successed or not for scenario that the network eception UE not to receive RAR. |
| R2-2307961 HuaWei | Endorsed 38.321 running CR: The MAC entity shall, for each Random Access Preamble: - if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and - if the notification of suspending power ramping counter has not been received from lower layers; and - if LBT failure indication was not received from lower layers for the last Random Access Preamble transmission; and - if SSB or CSI-RS selected is not changed from the selection in the last Random Access Preamble transmission: 2> increment PREAMBLE_POWER_RAMPING_COUNTER by 1. - if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell as preamble re-transmission: 2> increment PREAMBLE_POWER_RAMPING_COUNTER by 1. - select the value of DELTA_PREAMBLE according to clause 7.3; - set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA; - except for contention-free Random Access Preamble for beam failure recovery request and Random Access Preamble for an LTM candidate cell, compute the RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted; - instruct the physical layer to transmit the Random Access Preamble using the selected PRACH occasion, corresponding RA-RNTI (if available), PREAMBLE_INDEX, and PREAMBLE_RECEIVED_TARGET_POWER. - if the Random Access Procedure is triggered by a PDCCH order for a LTM candidate cell: 2> consider this Random Access procedure completed. |

According to above information, rapporteur think that is natural to finish the RACH procedure when preamble transmission is instructed to the lower layer since there is no any RAR present for the early RACH procedure. And the solution provided from other companies are some kind of optimization/enhancement, For easing our discussion at this stage, rapporteur suggest to give a high level proposal regarding the termination/completion of early RACH procedure:

Q6: The RACH for TA acquisition is considered as complete/terminated:

- Option 1: The preamble transmission is instructed to lower layer.

- Option 2: Enhancement/Optimization for early RACH termination is needed, detail is FFS.

| Company | Options | Comments |
| --- | --- | --- |
| ZTE | 1 | Simpler solution is preferred since the gain of optimization is not clear. |
| CATT | 1 |  |
| Samsung | 1 |  |
| Huawei | Option 1 | It is implemented in the endorsed MAC running CR. We prefer to wait for RAN1 progress otherwise. |
| OPPO | Option 1 |  |
| Transsion | Option 2 | Based on current specification, the UE shall start ra-ResponseWindow after preamble had been transmitted, which can be a implementation baseline. For the early TA acquisition with RAR reception is configured, the UE can starts the legacy ra-ResponseWindow for the RAR monitoring from servcing cell. For the case that early TA acquisition without RAR reception is configured, the UE can not to starts the legacy ra-ResponseWindow but receive LTM cell swtich command to identify if the RA preamble had been eceptio by candidate cell or not. As the legacy RA window was for RAR reception and to satisfied eceptio early TA acquisition without eception RAR but LTM cell swtich command, we should enhance on this. At the same time, the legacy procedure to response to UE’s preamble was RAR and using the LTM command to identify if the early TA acquisition can be a baseline and lower the specification impacts. |
| Futurewei | Option 1 |  |
| Panasonic | Option 1 |  |
| NEC | Option 1 |  |
| Lenovo | Option1 |  |
| ITRI | Option 2 | As mentioned in our contribution, the handling of preamble transmission failure for early TA acquisition has not been addressed. We believe that addressing this issue is important for the network to obtain the related information promptly. This information can be used to indicate an alternative candidate target cell for LTM in the event of preamble transmission failure by network. |
| MediaTek | Option 1 | Since UE does not monitor RAR, early RACH is a one-shot procedure and can be considered complete once the preamble is transmitted |
| vivo | Option 2 | We think option 1 couldn’t represent that the early TA acquisition for a candidate cell is completed, since the network may indicate UE to retransmit the Preamble. In our view, we had better to keep the principle that the UE has same undetstanding with the network. With option1, UE may not identify some RACH collision cases, details can be found in our response to Q4-1. It may lead to long latency of early TA acquisition and delay the LTM. |
| Ericsson | Option 1 or wait for RAN1 |  |
| Qualcomm | Option 1 |  |
| LGE | Option 1 |  |
| Xiaomi | Option 1 |  |
| NTT DOCOMO | Option 1 | UE don’t have to know whether RACH for early TA acquisition is completed. |
| Charter Communications | Option 1 |  |
| DENSO | Option 1 |  |
| Fujitsu | Option 1 |  |
| Nokia, Nokia Shanghai Bell | See comments | For early TA acquisition the RACH completion should be considered the preamble transmission instruction to lower layer (Option 1). However, since we cannot ensure that the RA will be successful, the power ramping counter for LTM should not be flushed out. The value of the latest power ramping counter should be stored so as to be used if in a later phase a PDCCH order is received, e.g., with a retransmission flag. |
| Sharp | Option 1 |  |
| CMCC | Option 1 |  |

According to the discussion for this issue, there are 24 companies participated in the discussion: 20 companies support option 1, 1 company have a concern about the counter value would be reset, and 3 companies support option 2, so the proposal would be implemented by following majorities:

(20/24) The RACH procedure toward a candidate cell is considered as complete once the preamble transmission is instructed to the lower layer.

RACH-less LTM

We already have some discussion online regarding the RACH-Less LTM and some initial agreements are achieved, and there are still some hot topics those are not visited in this meeting. In rapporteur understanding, we can touch these issues in the offline to see whether we can make some progress on these issues.

| Tdoc | Proposals |
| --- | --- |
| R2-2307060 Xiaomi | Proposal 4: The expiry of cg-RetransmissionTimer triggers the retransmission of the first UL data transmission to the target cell, as the legacy SDT procedure. |
| R2- 2308039 OPPO | Proposal 5 Timer-based retransmission on CG for the first UL transmission can be supported before the reception of ack from NW. |
| R2-2308841 ZTE | Proposal 6: Introduce the automatic (re)transmission of the first UL transmission with configured grant for CG-LTM, the detail solution (e.g. CG-SDT like mechanism, or NRIIOT like mechanism) is FFS. |
| R2-2307398 Fujitsu | Proposal 8: CG retransmission for the first UL transmission in target cell is allowed |

For CG-LTM, there is some possibilities that the first UL data transmission with CG is lost due to the lower SINR and NW may be not aware of the lost, so the automatic (re)transmission with the CG occasion seems necessary. For now, there are two automatic (re)transmission mechanisms are on the table, the first is retransmission timer based automatic retransmission (i.e. CG-SDT and/or NRU), the second is CG timer based automatic transmission for one MAC PDU (i.e. NRIIOT), Companies are invited to answer the following questions:

Q7: Whether the automatic (re)transmission with CG is supported for the first UL data transmission with CG?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| CATT | No strong view | It is not essential, more like some optimization. If due to lowest SINR, it is quite possible that the LTM cell switch will not succeed. And since there is already supervision timer for LTM, anyway upon the expire of the supervision timer, if the LTM cell switch is not succeed, e.g., the UE does not receive the C-RNTI addressed PDCCH, UE will consider the procedure is failed. |
| Samsung | No strong view | We can also rely on DG for retransmission. |
| Huawei | See comment | We prefer to make it as FFS for further thinking. |
| OPPO | Yes |  |
| Transsion | Yes |  |
| Futurewei |  | Under discussion in RAN1. Can wait for RAN1 input. Any possible following up retransmissions can be candidate cell scheduled. Either way is doable. |
| Panasonic | Yes | Re-transmission can be attempted before declaring cell switch failure. |
| NEC | Yes |  |
| Lenovo | No strong view | It was agreed that once transmitting cell switching command, source DU will indicate target DU via CU. Therefore, we can rely on DG. |
| ITRI | Yes |  |
| MediaTek | Yes |  |
| vivo | No strong view | The target cell can identify the arrival of UE by monitoring CG resources allocated for LTM. When the cell failed to decode the first UL transmission on CG, the cell can rely on DG for retransmission. Hence, we think retransmission with CG is just an optimization. |
| Ericsson | Yes | We are of with this but we are also fine to have companies more time to check. |
| Qualcomm | Yes | For NR-U, this should of course be supported since it is part of NR-U feature. For licensed, Rel-17 SDT introduced a similar mechanism. Therefore, it is fine to have it for LTM as well. |
| LGE | No | Share comments from CATT and Samsung. |
| Xiaomi | Yes | It is critical to ensure the transmission reliability of the first UL transmission. |
| NTT docomo | No strong view |  |
| Charter Communications | Yes |  |
| DENSO | Yes | Automatic re-transmission can be allowed for CG-LTM. |
| Fujitsu | Yes |  |
| Nokia, Nokia Shanghai Bell | Yes | Automatic (re)transmission with CG should be supported for the first UL data transmission with SG. |
| Sharp | Yes |  |
| CMCC | Yes |  |

Q6a: If companies think the automatic (re)transmission is allowed, which alternative is preferred？

- Option 1: CG retransmission Timer based automatic retransmission (e.g. CG-SDT/NRU)

- Option 2: CG timer based automatic transmission (e.g. NRIIOT)

Just note that, the main difference between those two options：in option 1, the CG retransmission timer based automatic retransmission for the first UL data can be performed until the expiration of CG timer, in option 2, the automatic transmission of the first UL data can be performed until the supervisor timer is expired.

| Company | Options | Comments |
| --- | --- | --- |
| ZTE | Option 1 | It is agreed that CG is associated with the SSB for CG-LTM which is asimilar as CG-SDT. It is safe to reuse the automatic retransmission in CG-SDT. |
| OPPO | Option 1 | The mechanism in SDT can be reused. |
| Transsion | Option 1 |  |
| NEC | Option 1 |  |
| ITRI | Option 1 |  |
| Mediatek | Option 1 |  |
| Ericsson | Option 1 |  |
| Qualcomm | Option 1 |  |
| Xiaomi | Option 1 |  |
| Charter Communications | Option 1 |  |
| DENSO | Option 1 | The mechanism in SDT can be reused. |
| Fujitsu | Option 1 |  |
| Nokia, Nokia Shanghai Bell | Option 1 | As long as the CG timer is configured the UE should be able to use it to transmit and re-transmit. To our understanding Option 2 restricts to single transmission, which limits the effectiveness of the CG use. |
| Sharp | Option 1 |  |
| CMCC | Option 1 |  |

According to the comments of this issue, there are 24 companies participated in the discussion, 16 companies support to have automatic-retransmission, only 1 company not support to have automatic retransmission. And 6 companies have no strong view, 1 company think it shall be FFS. And all companies think the CG-RetransmissionTimer based solution is preferred. So rapporteur suggest to propose:

(16/24) CG-RetransmissionTimer based automatic retransmission with CG is supported for the first UL data transmission with CG.

Conclusion

According to the discussion above, the conclusions are achieved as below:

Early TA Acuqisition

Proposal 1:	(24/24)All the RRC configurations related to early RACH are specific per LTM candidate cell and signalled separately from the candidate cell configuration (i.e. LTM Candidate configuration).

Proposal 2:	(24/24) The early RACH procedure share a same MAC entity with the legacy RACH procedure. (e.g. no extra MAC entity is needed for early RACH)

Proposal 3:	It is up to UE implementation to handle the RACH collisions where the early RACH is getting involved. No specification change can be foreseen.

Proposal 4:	For counting the power ramping step for early RACH, to select one of the following:

-	(17/23)Reuse PREAMBLE_POWER_RAMPING_COUNTER，and sent the information to RAN 1 via LS to revert their agreement (i.e. the initial value of the counter for calculating power ramping step is 1)

-	(06/23)Introduce a new COUNTER, that the initial value of the counter is 0.

Proposal 5:	Apart from the condition where the initial transmission of preamble is indicated in the PDCCH order for early RACH, RAN2 confirms the power ramping counter also should be reset by an extra condition:

-	the candidate cell indicated in PDCCH order is not as same as the previous RACH

-	the candidate cell indicated in PDCCH order is not as same as the previous early RACH.

Proposal 6:	RAN2 confirms the counter for power ramping step is incremented by 1:

-	If the retransmission is indicated and the indicated candidate cell and SSB are as same as the previous RACH

Proposal 7:	(10/16)UE retransmits the preamble without the power ramping upon reception of PDCCH order with retransmission indication if preamble transmission encounter the LBT failure.

Proposal 8:	(20/24) The RACH procedure toward a candidate cell is considered as complete once the preamble transmission is instructed to the lower layer.

RACH-Less LTM

Proposal 9:	(16/24) CG-RetransmissionTimer based automatic retransmission with CG is supported for the first UL data transmission with CG.