3GPP TSG-RAN WG2 Meeting #123bis                                                                    R2-2309544

Xiamen, China, October 9th – 13th, 2023

Source:	CATT

Title:	RRC Open issue list for CHO with candidate SCGs

Agenda Item:	7.4.1

Document for:	Discussion and Decision

Introduction

This contribution is to provide a list of the open issues to be addressed in order to complete the RRC CR for CHO with candidate SCGs.

Open issues

Following are the open issues captured in EN in the RRC Running CR for CHO with candidate SCGs [1].

| Issue | Relevant section in TS 38.331 | rapporteur’s suggestions on how to address |
| --- | --- | --- |
| If the CPA or CPC was not configured, FFS whether UE should remove the configuration for CHO with candidate SCG(s) when PSCell changes. | 5.3.5.3 | Discuss based on companies contributions |
| FFS whether the execution of CHO with candidate SCG is piriotized, if both PCell for CHO only or CHO including target MCG and target SCG, and the PCell and the associated PSCell for CHO with candidate SCG(s) is triggered. | 5.3.5.13.5 | Discuss based on companies contributions |
| CHO recovery details to handle the additions brought by this feature is FFS. | 5.3.7.3 | Discuss based on companies contributions |
| For CHO with candidate SCG(s), maxNrofCondCells is the max number of conditional configurations that the UE can store (is assumed to be a memory limitation), value FFS. | 6.3.2 | Discuss based on companies contributions |
| It is assumed to be discussed in RAN3 on the granularity of the configuration for CHO with candidate SCG(s) from candidate MN to source MN, e.g., per target MN, or per candidate PCell (with multiple associated candidate PSCells) or per candidate PCell with one candidate PSCell. | 11.2.2 | Up to RAN3 |
| FFS which parameters of the execution conditions for candidate PSCells to send from candidate MN to source MN. | 11.2.2 | Discuss based on companies contributions |
| R2 assumes Source MN initiates the preparation of the R18 CHO with candidate SCG(s), e.g., S-MN tells the T-MN whether it is allowed to configure candidate SCG(s). FFS the signalling details | 11.2.2 | Up to RAN3 |

Reference

[1] R2-2309319 RRC Running CR for CHO with candidate SCGs