3GPP TSG-RAN WG2 Meeting #123bis                                                                              R2-2309545

Xiamen, China, October 9th – 13th, 2023

Source:	CATT

Title:	Discussion on improvement on FR2 SCell/SCG setup delay

Agenda Item:	7.4.5

Document for:	Discussion and Decision

- 1. Introduction

Last meeting, RAN2 receive LS[1] from RAN4 about the improvement on FR2 SCell/SCG setup delay. RAN4 asks RAN2 to provide signaling input based on RAN4 agreement.

This contribution will further discuss from the signaling point view to support R18 enhancement take the R16 EMR signaling frame work as baseline.

- 2. Discussions

2.1 Scenarios

Firstly we should focus on the scenarios supported in R18. In R16, all the NRDC/NEDC/(NG)ENDC scenarios are supported, and both the 36.331 and 38.331 are impacted. Based on RAN4 LS we can find for R18 enhancement, FR2 is supported, but FR1 is also not excluded.

AN4 #106: R4-2303176 <Agreement> - RAN4 shall focus on inter-band target cell in FR2. If final solution to be agreed can cover intra-band and FR1 without extra standardization effort, it is unnecessary to exclude these two scenarios. - UE is allowed to reuse existing measurement, including legacy measurement for cell re-selection and EMR. UE is allowed to perform addition measurement starting from RRC connection setup/resume procedure

Considering if (NG)ENDC (including EUTRA CA) or NEDC is supported, the 36.331 should be impacted, if only NRDC is supported, only the 38.331 is impact. So we should firstly ask RAN4 to confirm the supporting scenarios.

Proposal 1: Send LS to ask RAN4 that which of the following scenarios are supported for R18:

- NRDC (including NR SCG/NR CA)

- NEDC (including NR CA/EUTRA SCG)

- (NG)ENDC (including EUTRA CA/NR SCG)

For simply , the following discussion will focus on NRDC scenario. If other senarios are confirmed to be supported, the proposals will extend to cover other scenarios.

2.2 idle/inactive measurement configuration

Currently, for R16 EMR it supports the idle/inactive measurement configuration is configured via RRCRelease message and via SIB11. Whether the UE should perform idle/inactive measurement when UE camps on the cell is indicated via indication broadcasted in SIB1 as following shows.

SIB1-v1610-IEs ::=               SEQUENCE {

idleModeMeasurementsEUTRA-r16    ENUMERATED{true}                                                   OPTIONAL,  -- Need R

idleModeMeasurementsNR-r16       ENUMERATED{true}                                                   OPTIONAL,  -- Need R

posSI-SchedulingInfo-r16         PosSI-SchedulingInfo-r16                                           OPTIONAL,  -- Need R

nonCriticalExtension             SIB1-v1630-IEs                                                     OPTIONAL

}

For supporting R18 enhancement, considering it has different UE behavior from R16 EMR, as following:

Starting point of the enhanced measurement <Agreement> - For MT originating call, UE starts to perform additional measurement after paging reception. - And for MO call, UE starts to perform additional measurement after first RACH preamble transmission, i.e. Msg1. - Note: the respective agreement can be used to derive requirements once feasibility of enhanced solution is confirmed Ending point of the enhanced measurement <Agreement> - The ending point of enhanced measurement at the reception of the RRC CONNECTED mode measurement configuration (the 1st RRC_reconfiguration message) - Note: the respective agreement can be used to derive requirements once feasibility of enhanced solution is confirmed

so new indication is needed to indicate UE to perform idle/inactive measurement based on R18 enhancement feature when UE camps on this cell. So we propose

Proposal 2: New indication in SIB1 is needed to indicate whether a UE that is configured for improvement on SCell/SCG setup delay shall perform the measurements and report availability of these measurements when establishing or resuming a connection in this cell.

As for the idle/inactive measurement configuration, we think the R16 mechnism can also reused for R18 i.e. idle/inactive measurement configuration can be provided via dedicated message  i.e. RRCRelease message and SIB.

Proposal 3: Measurement configuration for improvement on SCell/SCG setup delay  can be provided via system information and RRCRelease message, similar as R16 EMR.

As for the detail signaling e.g. the content of the measurement configurtion or whehter the R16 signalling could be used for R18 directly should wait for RAN4 disucssion, as the following[1] shows,the relationship between the legacy measurement and the R18 measurement is under RAN4 discussion, and the carriers needed to be measured is also under RAN4 discussion.

RAN4 #106: R4-2303176 <Agreement> - RAN4 shall focus on inter-band target cell in FR2. If final solution to be agreed can cover intra-band and FR1 without extra standardization effort, it is unnecessary to exclude these two scenarios. - UE is allowed to reuse existing measurement, including legacy measurement for cell re-selection and EMR. - UE is allowed to perform addition measurement starting from RRC connection setup/resume procedure. RAN4 #104-e R4-2214348 <Agreement>: - Reduce the number of EMR carriers to be measured for improved measurement. - FFS: how to select the frequency layers for improved measurement, how many frequency layers to measure and which scenario that is applicable.

Proposal 4: The details for measurement configuration(e.g. content, relationship between R16 EMR and R18 enhancement) for improvement on SCell/SCG setup delay should wait for more progress on RAN4.

2.3 Measurement report

Currently, for R16 EMR, idle UE report that it has available idle measurement results  via RRCSetupComplete message. And UE report the available idle measurement results via UE UEInformationRequest/UEInformationResponse procedure. For inactive UE, it could report it measurement results as idle UE via UEInformationRequest/UEInformationResponse procedure. Or via RRCResumeComplete message if NW requests via RRCResume.

Based on RAN4 agreement, UE report the idle/inactive measurement result during or after RRC/setup procedure which is similar with R16 EMR procedure.

<Agreement> - For UE which is not capable of EMR, when the measurement results during idle/inactive mode are available and valid, UE can report these results to the network during or after RRC setup or resume procedures. The reporting procedure is up to RAN2.

So the idle/inactive measurement results report procedure for R18 can reuse the similar R16 EMR signalling procedure. As for the details (e.g. the content of the measurement results, whether the same signaling used for R16 could be used for R18 enhancement) depend on the releationship between the two features which is under RAN4 discussion.

Proposal 5: R16 EMR idle/inactive measurment report signaling framework is reused for R18 improvement on FR2 SCell/SCG setup delay. i.e.

- UE in idle/inactive reports idle/inactive measurment result via UEInformationRequest/UEInformationResponse procedure

- UE in idle/inactive indicates the availability of idle/inactive measurements via RRCSetupComplete/RRCResumeComplete message

- UE in inactive reports measurment result via RRCResumeComplete if NW requests it via RRCResume message

Different from R16 EMR, the R18 enhancement doesn’t allow UE to report invalid measurement results while in R16 it allow UE to report measurement result after T331 expiry which may be invalid measurment results.

RAN4 #107: R4-2310064 o Agreements: • In new reporting in R18, UE does not report the invalid results to NW. RAN4 #106-bis-e: R4-2306396 <Agreement>: FFS : whether to introduce indication of availability and/or validity status of measurement results from UE to NW into the [existing] EMR reports.

Considering the coexist of R16 EMR and R18 enhancement, if separate signalling is defined for R18 and R16. The NW could identify which measurement result is reported due to R18 enhancment feature, and the NW could know the results reported due to R18 feature is not including invalid results. But if the same signalling is used for R16 and R18 feature. If no extra indication is included in the report message, the NW couldn’t distinguish the measurement result is reported due to R16 EMR feature or R18 enhancement feature, hence it couldn’t verify the measurement is valid. From this point, for R18 enhancement, one indication indicating the reported measurment results are valid is introduced. Anyway NW is needed to distinguish whether or which measurement results is reported for R18 feature, the details signalling should wait for RAN4 progress on the relationship between the 2 feaures.

Proposal 6: The signalling design for measurement report should ensure that NW knows whether the measurement results are for improvement on SCell/SCG setup delay. The signaling details are pending on RAN4 progress.

Besides, RAN2 should discuss another issue on the configuration for report measurement results based on the following agreement from RAN4 LS:

<Agreement> It is up to UE implementation how to select a band to report unless NW specify a band to report

Based on RAN4 agreement we can find, it allows NW to configure for which band UE should report. Though the measurement configuration could be provided via dedicated signalling and system information. But the report of the availability of the measurement result is based on system information. Furthermore the measurement results are useful for the current serving cell, so it is seems not suitable to configure the reported band via RRCRelease message via the last serving cell.  From this pointview, it is suitable to indicate the band to report via system information.

Proposal 7: The band to report in the measurement results for improvement on SCell/SCG setup delay is indicated in system information.

- 3. Conclusion

This paper further discusses the signaling on supporting improvement on FR2 SCell/SCG setup delay. The proposals of this paper are summarized as follows,

Scenarios

Proposal 1: Send LS to ask RAN4 that which of the following scenarios are supported for R18:

- NRDC (including NR SCG/NR CA)

- NEDC (including NR CA/EUTRA SCG)

- (NG)ENDC (including EUTRA CA/NR SCG)

idle/inactive measurement configuration

Proposal 2: New indication in SIB1 is needed to indicate whether a UE that is configured for improvement on SCell/SCG setup delay shall perform the measurements and report availability of these measurements when establishing or resuming a connection in this cell.

Proposal 3: Measurement configuration for improvement on SCell/SCG setup delay  can be provided via system information and RRCRelease message, similar as R16 EMR.

Proposal 4: The details for measurement configuration(e.g. content, relationship between R16 EMR and R18 enhancement) for improvement on SCell/SCG setup delay should wait for more progress on RAN4.

Measurement report

Proposal 5: R16 EMR idle/inactive measurment report signaling framework is reused for R18 improvement on FR2 SCell/SCG setup delay. i.e.

- UE in idle/inactive reports idle/inactive measurment result via UEInformationRequest/UEInformationResponse procedure

- UE in idle/inactive indicates the availability of idle/inactive measurements via RRCSetupComplete/RRCResumeComplete message

- UE in inactive reports measurment result via RRCResumeComplete if NW requests it via RRCResume message

Proposal 6: The signalling design for measurement report should ensure that NW knows whether the measurement results are for improvement on SCell/SCG setup delay. The signaling details are pending on RAN4 progress.

Proposal 7: The band to report in the measurement results for improvement on SCell/SCG setup delay is indicated in system information.

- 4. Reference

[1] R2-2309462, LS on improvement on FR2 SCell/SCG setup delay, RAN4