3GPP TSG-RAN WG2 Meeting #123bis	R2-2309594

Xiamen, China, October 9th – 13th, 2023

Agenda item:		7.5.4.1

Source:		Futurewei

Title:		Detailed Buffer Size table design for XR

Document for:		Discussion and Decision

# Introduction

In RAN2 meeting #122, RAN2 agreed on the following with respect to the new Buffer Size (BS) table for XR [1]:

Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases).

We do not support additional piecewise linear BSR table in Rel-18. Can consider piecewise linearity when discussing how the BSR table values are defined.

No progress was made on the BS table design during RAN2 meeting #123 [2].

In this contribution, we provide the detailed design of the new BS table for XR. We also compare it with some alternatives being proposed.

# Discussions

## Our proposed new BS table design

In our Tdoc [3] submitted to RAN2 meeting #122, we conclude that the new BS table should cover a buffer size up to the size of a single I frame for the optional video data rate scenario (i.e., 20Mbps), as described in [4], or roughly 200 Kbytes. This design is based on the assumptions that a Buffer Size field in the DSR MAC CE corresponds to the size of a single video frame and that any non-urgent data, of which the remaining time has not dropped below the pre-specified time threshold, can be reported using one of the legacy BSR MAC CEs and the legacy BS table because the quantization granularity is not critical for the non-urgent data, because they will likely be transmitted in an opportunistic manner, as long as they remain non-urgent. Once the non-urgent data become urgent (i.e., the remaining time drops below the DSR reporting threshold), a more accurate BS report will be provided via another DSR MAC CE.

If the new BS table starts from 0 Kbytes and has a linear quantization step size of 1024 bytes, it can cover up to a size of 260,095 bytes, which is sufficient. Table 1 below illustrates the values in such table using a single linear quantization step size of 1024 bytes.

Table 1. New Buffer Size Table with a single linear region

| Index | BS value (bytes) | Index | BS value (bytes) | Index | BS value (bytes) | Index | BS value (bytes) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 64 | ≤ 65535 | 128 | ≤ 131071 | 192 | ≤ 196607 |
| 1 | ≤ 1023 | 65 | ≤ 66559 | 129 | ≤ 132095 | 193 | ≤ 197631 |
| 2 | ≤ 2047 | 66 | ≤ 67583 | 130 | ≤ 133119 | 194 | ≤ 198655 |
| 3 | ≤ 3071 | 67 | ≤ 68607 | 131 | ≤ 134143 | 195 | ≤ 199679 |
| 4 | ≤ 4095 | 68 | ≤ 69631 | 132 | ≤ 135167 | 196 | ≤ 200703 |
| 5 | ≤ 5119 | 69 | ≤ 70655 | 133 | ≤ 136191 | 197 | ≤ 201727 |
| 6 | ≤ 6143 | 70 | ≤ 71679 | 134 | ≤ 137215 | 198 | ≤ 202751 |
| 7 | ≤ 7167 | 71 | ≤ 72703 | 135 | ≤ 138239 | 199 | ≤ 203775 |
| 8 | ≤ 8191 | 72 | ≤ 73727 | 136 | ≤ 139263 | 200 | ≤ 204799 |
| 9 | ≤ 9215 | 73 | ≤ 74751 | 137 | ≤ 140287 | 201 | ≤ 205823 |
| 10 | ≤ 10239 | 74 | ≤ 75775 | 138 | ≤ 141311 | 202 | ≤ 206847 |
| 11 | ≤ 11263 | 75 | ≤ 76799 | 139 | ≤ 142335 | 203 | ≤ 207871 |
| 12 | ≤ 12287 | 76 | ≤ 77823 | 140 | ≤ 143359 | 204 | ≤ 208895 |
| 13 | ≤ 13311 | 77 | ≤ 78847 | 141 | ≤ 144383 | 205 | ≤ 209919 |
| 14 | ≤ 14335 | 78 | ≤ 79871 | 142 | ≤ 145407 | 206 | ≤ 210943 |
| 15 | ≤ 15359 | 79 | ≤ 80895 | 143 | ≤ 146431 | 207 | ≤ 211967 |
| 16 | ≤ 16383 | 80 | ≤ 81919 | 144 | ≤ 147455 | 208 | ≤ 212991 |
| 17 | ≤ 17407 | 81 | ≤ 82943 | 145 | ≤ 148479 | 209 | ≤ 214015 |
| 18 | ≤ 18431 | 82 | ≤ 83967 | 146 | ≤ 149503 | 210 | ≤ 215039 |
| 19 | ≤ 19455 | 83 | ≤ 84991 | 147 | ≤ 150527 | 211 | ≤ 216063 |
| 20 | ≤ 20479 | 84 | ≤ 86015 | 148 | ≤ 151551 | 212 | ≤ 217087 |
| 21 | ≤ 21503 | 85 | ≤ 87039 | 149 | ≤ 152575 | 213 | ≤ 218111 |
| 22 | ≤ 22527 | 86 | ≤ 88063 | 150 | ≤ 153599 | 214 | ≤ 219135 |
| 23 | ≤ 23551 | 87 | ≤ 89087 | 151 | ≤ 154623 | 215 | ≤ 220159 |
| 24 | ≤ 24575 | 88 | ≤ 90111 | 152 | ≤ 155647 | 216 | ≤ 221183 |
| 25 | ≤ 25599 | 89 | ≤ 91135 | 153 | ≤ 156671 | 217 | ≤ 222207 |
| 26 | ≤ 26623 | 90 | ≤ 92159 | 154 | ≤ 157695 | 218 | ≤ 223231 |
| 27 | ≤ 27647 | 91 | ≤ 93183 | 155 | ≤ 158719 | 219 | ≤ 224255 |
| 28 | ≤ 28671 | 92 | ≤ 94207 | 156 | ≤ 159743 | 220 | ≤ 225279 |
| 29 | ≤ 29695 | 93 | ≤ 95231 | 157 | ≤ 160767 | 221 | ≤ 226303 |
| 30 | ≤ 30719 | 94 | ≤ 96255 | 158 | ≤ 161791 | 222 | ≤ 227327 |
| 31 | ≤ 31743 | 95 | ≤ 97279 | 159 | ≤ 162815 | 223 | ≤ 228351 |
| 32 | ≤ 32767 | 96 | ≤ 98303 | 160 | ≤ 163839 | 224 | ≤ 229375 |
| 33 | ≤ 33791 | 97 | ≤ 99327 | 161 | ≤ 164863 | 225 | ≤ 230399 |
| 34 | ≤ 34815 | 98 | ≤ 100351 | 162 | ≤ 165887 | 226 | ≤ 231423 |
| 35 | ≤ 35839 | 99 | ≤ 101375 | 163 | ≤ 166911 | 227 | ≤ 232447 |
| 36 | ≤ 36863 | 100 | ≤ 102399 | 164 | ≤ 167935 | 228 | ≤ 233471 |
| 37 | ≤ 37887 | 101 | ≤ 103423 | 165 | ≤ 168959 | 229 | ≤ 234495 |
| 38 | ≤ 38911 | 102 | ≤ 104447 | 166 | ≤ 169983 | 230 | ≤ 235519 |
| 39 | ≤ 39935 | 103 | ≤ 105471 | 167 | ≤ 171007 | 231 | ≤ 236543 |
| 40 | ≤ 40959 | 104 | ≤ 106495 | 168 | ≤ 172031 | 232 | ≤ 237567 |
| 41 | ≤ 41983 | 105 | ≤ 107519 | 169 | ≤ 173055 | 233 | ≤ 238591 |
| 42 | ≤ 43007 | 106 | ≤ 108543 | 170 | ≤ 174079 | 234 | ≤ 239615 |
| 43 | ≤ 44031 | 107 | ≤ 109567 | 171 | ≤ 175103 | 235 | ≤ 240639 |
| 44 | ≤ 45055 | 108 | ≤ 110591 | 172 | ≤ 176127 | 236 | ≤ 241663 |
| 45 | ≤ 46079 | 109 | ≤ 111615 | 173 | ≤ 177151 | 237 | ≤ 242687 |
| 46 | ≤ 47103 | 110 | ≤ 112639 | 174 | ≤ 178175 | 238 | ≤ 243711 |
| 47 | ≤ 48127 | 111 | ≤ 113663 | 175 | ≤ 179199 | 239 | ≤ 244735 |
| 48 | ≤ 49151 | 112 | ≤ 114687 | 176 | ≤ 180223 | 240 | ≤ 245759 |
| 49 | ≤ 50175 | 113 | ≤ 115711 | 177 | ≤ 181247 | 241 | ≤ 246783 |
| 50 | ≤ 51199 | 114 | ≤ 116735 | 178 | ≤ 182271 | 242 | ≤ 247807 |
| 51 | ≤ 52223 | 115 | ≤ 117759 | 179 | ≤ 183295 | 243 | ≤ 248831 |
| 52 | ≤ 53247 | 116 | ≤ 118783 | 180 | ≤ 184319 | 244 | ≤ 249855 |
| 53 | ≤ 54271 | 117 | ≤ 119807 | 181 | ≤ 185343 | 245 | ≤ 250879 |
| 54 | ≤ 55295 | 118 | ≤ 120831 | 182 | ≤ 186367 | 246 | ≤ 251903 |
| 55 | ≤ 56319 | 119 | ≤ 121855 | 183 | ≤ 187391 | 247 | ≤ 252927 |
| 56 | ≤ 57343 | 120 | ≤ 122879 | 184 | ≤ 188415 | 248 | ≤ 253951 |
| 57 | ≤ 58367 | 121 | ≤ 123903 | 185 | ≤ 189439 | 249 | ≤ 254975 |
| 58 | ≤ 59391 | 122 | ≤ 124927 | 186 | ≤ 190463 | 250 | ≤ 255999 |
| 59 | ≤ 60415 | 123 | ≤ 125951 | 187 | ≤ 191487 | 251 | ≤ 257023 |
| 60 | ≤ 61439 | 124 | ≤ 126975 | 188 | ≤ 192511 | 252 | ≤ 258047 |
| 61 | ≤ 62463 | 125 | ≤ 127999 | 189 | ≤ 193535 | 253 | ≤ 259071 |
| 62 | ≤ 63487 | 126 | ≤ 129023 | 190 | ≤ 194559 | 254 | ≤ 260095 |
| 63 | ≤ 64511 | 127 | ≤ 130047 | 191 | ≤ 195583 | 255 | > 260095 |

One advantage of choosing a quantization step size of a power of 2 is that the transmitter may determine the BS index for a given non-zero BS value by truncating certain number of LSBs of the BS value and adding 1 (due to index 0 being used for indicating zero BS level) to the remaining MSBs, which is much simpler for UE implementation than using a searching algorithm to search within a loop-up table or performing a logarithmic calculation, as in the case if the legacy or a new exponential BS table is used. We consider a quantization step size of 1024 bytes as acceptable, noting that in the legacy 8-bit Buffer Size table, the quantization step size around a buffer size of 200 Kbytes is about 12 Kbytes.

It has been proposed and suggested that a single 8-bit BS table with piecewise linearity may be beneficial to further reduce quantization error at the low end of the BS range, e.g., by using two different quantization step sizes to cover two linear regions. Table 2 below illustrates the values in such table where the quantization step size of 32 bytes is used, primarily for the pose traffic, to cover up to a BS level of 1023 bytes, which is sufficient for up to 10 pose packets being buffered at a time (100 bytes pose packet size, 10 msec PDB, and up to 1kHz packet rate). After 1023 bytes, the quantization step size of 1024 bytes is used, primarily for the video traffic. With two linear regions, the UE still can use truncating bits to derive the BS index from a given non-zero BS value, by truncating different numbers of LSBs and then adding different offset values to the remaining MSBs for different linear regions.

Table 2. New Buffer Size Table with two linear regions

| Index | BS value (bytes) | Index | BS value (bytes) | Index | BS value (bytes) | Index | BS value (bytes) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 64 | ≤ 33791 | 128 | ≤ 99327 | 192 | ≤ 164863 |
| 1 | ≤ 31 | 65 | ≤ 34815 | 129 | ≤ 100351 | 193 | ≤ 165887 |
| 2 | ≤ 63 | 66 | ≤ 35839 | 130 | ≤ 101375 | 194 | ≤ 166911 |
| 3 | ≤ 95 | 67 | ≤ 36863 | 131 | ≤ 102399 | 195 | ≤ 167935 |
| 4 | ≤ 127 | 68 | ≤ 37887 | 132 | ≤ 103423 | 196 | ≤ 168959 |
| 5 | ≤ 159 | 69 | ≤ 38911 | 133 | ≤ 104447 | 197 | ≤ 169983 |
| 6 | ≤ 191 | 70 | ≤ 39935 | 134 | ≤ 105471 | 198 | ≤ 171007 |
| 7 | ≤ 223 | 71 | ≤ 40959 | 135 | ≤ 106495 | 199 | ≤ 172031 |
| 8 | ≤ 255 | 72 | ≤ 41983 | 136 | ≤ 107519 | 200 | ≤ 173055 |
| 9 | ≤ 287 | 73 | ≤ 43007 | 137 | ≤ 108543 | 201 | ≤ 174079 |
| 10 | ≤ 319 | 74 | ≤ 44031 | 138 | ≤ 109567 | 202 | ≤ 175103 |
| 11 | ≤ 351 | 75 | ≤ 45055 | 139 | ≤ 110591 | 203 | ≤ 176127 |
| 12 | ≤ 383 | 76 | ≤ 46079 | 140 | ≤ 111615 | 204 | ≤ 177151 |
| 13 | ≤ 415 | 77 | ≤ 47103 | 141 | ≤ 112639 | 205 | ≤ 178175 |
| 14 | ≤ 447 | 78 | ≤ 48127 | 142 | ≤ 113663 | 206 | ≤ 179199 |
| 15 | ≤ 479 | 79 | ≤ 49151 | 143 | ≤ 114687 | 207 | ≤ 180223 |
| 16 | ≤ 511 | 80 | ≤ 50175 | 144 | ≤ 115711 | 208 | ≤ 181247 |
| 17 | ≤ 543 | 81 | ≤ 51199 | 145 | ≤ 116735 | 209 | ≤ 182271 |
| 18 | ≤ 575 | 82 | ≤ 52223 | 146 | ≤ 117759 | 210 | ≤ 183295 |
| 19 | ≤ 607 | 83 | ≤ 53247 | 147 | ≤ 118783 | 211 | ≤ 184319 |
| 20 | ≤ 639 | 84 | ≤ 54271 | 148 | ≤ 119807 | 212 | ≤ 185343 |
| 21 | ≤ 671 | 85 | ≤ 55295 | 149 | ≤ 120831 | 213 | ≤ 186367 |
| 22 | ≤ 703 | 86 | ≤ 56319 | 150 | ≤ 121855 | 214 | ≤ 187391 |
| 23 | ≤ 735 | 87 | ≤ 57343 | 151 | ≤ 122879 | 215 | ≤ 188415 |
| 24 | ≤ 767 | 88 | ≤ 58367 | 152 | ≤ 123903 | 216 | ≤ 189439 |
| 25 | ≤ 799 | 89 | ≤ 59391 | 153 | ≤ 124927 | 217 | ≤ 190463 |
| 26 | ≤ 831 | 90 | ≤ 60415 | 154 | ≤ 125951 | 218 | ≤ 191487 |
| 27 | ≤ 863 | 91 | ≤ 61439 | 155 | ≤ 126975 | 219 | ≤ 192511 |
| 28 | ≤ 895 | 92 | ≤ 62463 | 156 | ≤ 127999 | 220 | ≤ 193535 |
| 29 | ≤ 927 | 93 | ≤ 63487 | 157 | ≤ 129023 | 221 | ≤ 194559 |
| 30 | ≤ 959 | 94 | ≤ 64511 | 158 | ≤ 130047 | 222 | ≤ 195583 |
| 31 | ≤ 991 | 95 | ≤ 65535 | 159 | ≤ 131071 | 223 | ≤ 196607 |
| 32 | ≤ 1023 | 96 | ≤ 66559 | 160 | ≤ 132095 | 224 | ≤ 197631 |
| 33 | ≤ 2047 | 97 | ≤ 67583 | 161 | ≤ 133119 | 225 | ≤ 198655 |
| 34 | ≤ 3071 | 98 | ≤ 68607 | 162 | ≤ 134143 | 226 | ≤ 199679 |
| 35 | ≤ 4095 | 99 | ≤ 69631 | 163 | ≤ 135167 | 227 | ≤ 200703 |
| 36 | ≤ 5119 | 100 | ≤ 70655 | 164 | ≤ 136191 | 228 | ≤ 201727 |
| 37 | ≤ 6143 | 101 | ≤ 71679 | 165 | ≤ 137215 | 229 | ≤ 202751 |
| 38 | ≤ 7167 | 102 | ≤ 72703 | 166 | ≤ 138239 | 230 | ≤ 203775 |
| 39 | ≤ 8191 | 103 | ≤ 73727 | 167 | ≤ 139263 | 231 | ≤ 204799 |
| 40 | ≤ 9215 | 104 | ≤ 74751 | 168 | ≤ 140287 | 232 | ≤ 205823 |
| 41 | ≤ 10239 | 105 | ≤ 75775 | 169 | ≤ 141311 | 233 | ≤ 206847 |
| 42 | ≤ 11263 | 106 | ≤ 76799 | 170 | ≤ 142335 | 234 | ≤ 207871 |
| 43 | ≤ 12287 | 107 | ≤ 77823 | 171 | ≤ 143359 | 235 | ≤ 208895 |
| 44 | ≤ 13311 | 108 | ≤ 78847 | 172 | ≤ 144383 | 236 | ≤ 209919 |
| 45 | ≤ 14335 | 109 | ≤ 79871 | 173 | ≤ 145407 | 237 | ≤ 210943 |
| 46 | ≤ 15359 | 110 | ≤ 80895 | 174 | ≤ 146431 | 238 | ≤ 211967 |
| 47 | ≤ 16383 | 111 | ≤ 81919 | 175 | ≤ 147455 | 239 | ≤ 212991 |
| 48 | ≤ 17407 | 112 | ≤ 82943 | 176 | ≤ 148479 | 240 | ≤ 214015 |
| 49 | ≤ 18431 | 113 | ≤ 83967 | 177 | ≤ 149503 | 241 | ≤ 215039 |
| 50 | ≤ 19455 | 114 | ≤ 84991 | 178 | ≤ 150527 | 242 | ≤ 216063 |
| 51 | ≤ 20479 | 115 | ≤ 86015 | 179 | ≤ 151551 | 243 | ≤ 217087 |
| 52 | ≤ 21503 | 116 | ≤ 87039 | 180 | ≤ 152575 | 244 | ≤ 218111 |
| 53 | ≤ 22527 | 117 | ≤ 88063 | 181 | ≤ 153599 | 245 | ≤ 219135 |
| 54 | ≤ 23551 | 118 | ≤ 89087 | 182 | ≤ 154623 | 246 | ≤ 220159 |
| 55 | ≤ 24575 | 119 | ≤ 90111 | 183 | ≤ 155647 | 247 | ≤ 221183 |
| 56 | ≤ 25599 | 120 | ≤ 91135 | 184 | ≤ 156671 | 248 | ≤ 222207 |
| 57 | ≤ 26623 | 121 | ≤ 92159 | 185 | ≤ 157695 | 249 | ≤ 223231 |
| 58 | ≤ 27647 | 122 | ≤ 93183 | 186 | ≤ 158719 | 250 | ≤ 224255 |
| 59 | ≤ 28671 | 123 | ≤ 94207 | 187 | ≤ 159743 | 251 | ≤ 225279 |
| 60 | ≤ 29695 | 124 | ≤ 95231 | 188 | ≤ 160767 | 252 | ≤ 226303 |
| 61 | ≤ 30719 | 125 | ≤ 96255 | 189 | ≤ 161791 | 253 | ≤ 227327 |
| 62 | ≤ 31743 | 126 | ≤ 97279 | 190 | ≤ 162815 | 254 | ≤ 228351 |
| 63 | ≤ 32767 | 127 | ≤ 98303 | 191 | ≤ 163839 | 255 | > 228351 |

In our view, the UE and the gNB should always use the new BS table for the urgent data volume, with which the remaining time associated has dropped below a threshold, because the finer granularity is more critical when the remaining time is limited. Because the urgent data volume can only be reported using the new DSR MAC CE, the UE and the gNB should always use the new BS table for the BS field(s) in the DSR MAC CE. Meanwhile, the non-urgent data volume buffered for the same XR LCG can be reported using one of the legacy BSR MAC CEs, of which the BS field(s) will continue to use the legacy BS table, even if a BS field in it is for the XR LCG, because the coarse quantization is not so critical for the non-urgent data of the XR LCG as long as their remaining time has dropped below the threshold. Therefore, the UE determines which BS table to use based on the urgency of the data being reported. And there is no need for the UE to identify the BS table to the gNB, other than the LCID identifying the MAC CE used for the report, i.e., whether a legacy LCID defined for a legacy BSR MAC CE or a new LCID to be defined for the new DSR MAC CE.

## Other proposed BS table ranges

In [5], the upper bound of the BS table, Bmax, is derived as 150Mbps/8/24 = 781,250 B. We noticed that 150Mbps is the video data rate for 8k video on the DL, where retina resolution is desirable. However, BSR or DSR are for reporting UL data. Most commonly used UL video stream in AR devices today use 720p or 1080p resolution, because UL video in AR is mostly used for object recognition, where retina resolution is not required. This has been consistent with the 10Mbps (baseline) and 20Mbps (optional) data rates assumed for XR UL video stream in Rel-17 SI TR [4]. Therefore, we think the Bmax in [5] has been over-estimated for UL.

In [6], it was suggested that the maximum Buffer Size value be 16,777,216 bytes, because the SA4 LS [7] indicates that the maximum PDU Set size is encoded using a 24-bit value. First, the encoding of PDU Set Size specified by SA4 is for DL XR video, which tends to have a video frame size of one magnitude larger than that of the UL (AR) video. Secondly, in transport network protocols, fields tend to be defined to be octet-aligned. It is obvious that 16 bits or 2 octets (indicating up to 65535 bytes) are insufficient for DL video frame size, hence it is understandable that SA4 has chosen 24 bits or 3 octets to be the length of the PDU Set Size field, but it doesn’t necessarily mean that all possible values that can be expressed by 24 bits will be encountered in practice for DL video frame sizes, let alone for UL video frame sizes. Therefore, we think that the upper bound of the BS value in [6] was determined for an improper reason.

## Proposed way forward

There are proposals suggesting that RAN2 reverse an existing agreement on defining the new BS table as a linear table, and instead, define it as an exponential table. However, the benefit of using exponential BS table may have been artificially amplified when an unreasonably large upper bound is chosen in the evaluation. The question remains whether the gain brought by using exponential BS table is significant when the upper bound of the BS values is not as high as being assumed and whether that justifies the use of a more complex scheme. In our view, a linear table as described in Table 1 or Table 2 above is simple and good enough.

Proposal 1. RAN2 discuss and consider adopting one of Table 1 and Table 2 above for XR.

Proposal 2. The new BS table is always used for the BS field(s) in the new DSR MAC CE. The BS field(s) in the legacy BSR MAC CEs continue to use the legacy BS table, no matter the legacy BSR MAC CE includes a BS field (for reporting the non-urgent data) for an XR LCG or not.

Proposal 3. The UE determines which BS table to use based on the urgency (urgent vs. non-urgent) of the data being reported.

Proposal 4. There is no need for the UE to identify the BS table to the gNB, other than the LCID identifying the MAC CE used for the report, i.e., whether a legacy LCID defined for a legacy BSR MAC CE or a new LCID to be defined for the new DSR MAC CE.

# Conclusions

The followings are proposed:

Proposal 1. RAN2 discuss and consider adopting one of Table 1 and Table 2 above for XR.

Proposal 2. The new BS table is always used for the BS field(s) in the new DSR MAC CE. The BS field(s) in the legacy BSR MAC CEs continue to use the legacy BS table, no matter the legacy BSR MAC CE includes a BS field (for reporting the non-urgent data) for an XR LCG or not.

Proposal 3. The UE determines which BS table to use based on the urgency (urgent vs. non-urgent) of the data being reported.

Proposal 4. There is no need for the UE to identify the BS table to the gNB, other than the LCID identifying the MAC CE used for the report, i.e., whether a legacy LCID defined for a legacy BSR MAC CE or a new LCID to be defined for the new DSR MAC CE.

# References

R2-2306542, RAN2-122 Report on LTE MUSIM QoE XR, Vice Chairman (Nokia).

R2-2308962, RAN2-123 LTE MUSIM QoE XR, Vice Chairman (Nokia).

R2-2307762 Discussions on new Buffer Status table design for XR, Futurewei.

TR 38.838 V17.0.0.

R2-2307078, BSR and delay status reporting for XR, Qualcomm.

R2-2307532, BSR enhancements for XR, ZTE and Sanechips.

R2-2304659_S4-230739, LS out on the N6 PDU Set Identification, SA4.