3GPP TSG-RAN WG2 Meeting #123bis	R2-2309604

# Xiamen, China, October 9th – 13th, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | draft | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LPP Running CR for Redcap positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2 |  |  |  |  |  | Date: |  |  | 2023-09-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of positioning for Redcap |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enable Positioning for Redcap based on the agreement in RAN1 Agreement The previous agreement is updated as follows: Agreement For DL Rx hopping or UL Tx hopping, support the UE or gNB to report the following: - A single measurement based on receiving multiple hops of the DL PRS or UL SRS for positioning - One measurement where a measurement is associated with one received hop - FFS: indication of how many received hops / which received hops where used in the measurement report. - Note: no new measurement definition is introduced in RAN1 - FFS: conditions when the above measurements are reported, and whether the above measurements can be reported together. |
| Consequences if not approved: |  | Positioning for Redcap is not supported in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.10.4, 6.5.11.4, 6.5.12.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

6.5.10.4	NR DL-TDOA Location Information Elements

–	NR-DL-TDOA-SignalMeasurementInformation

The IE NR-DL-TDOA-SignalMeasurementInformation is used by the target device to provide NR DL-TDOA measurements to the location server.

NOTE 1:	The dl-PRS-ReferenceInfo defines the "RSTD reference" TRP. The nr-RSTD's and nr-RSTD-ResultDiff's in nr-DL-TDOA-MeasList are provided relative to the "RSTD reference" TRP.

NOTE 2:	The "RSTD reference" TRP may or may not be the same as the "assistance data reference" TRP provided by nr-DL-PRS-ReferenceInfo in IE NR-DL-PRS-AssistanceData.

NOTE 3:	The target device includes a value of zero for the nr-RSTD and nr-RSTD-ResultDiff of the "RSTD reference" TRP in nr-DL-TDOA-MeasList.

-- ASN1START

NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE {

dl-PRS-ReferenceInfo-r16		DL-PRS-ID-Info-r16,

nr-DL-TDOA-MeasList-r16			NR-DL-TDOA-MeasList-r16,

...,

[[

nr-UE-RxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL	-- Cond UERxTEG

]]

}

NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16								OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15										OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15								OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16						OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-r16						CHOICE {

k0-r16						INTEGER (0..1970049),

k1-r16						INTEGER (0..985025),

k2-r16						INTEGER (0..492513),

k3-r16						INTEGER (0..246257),

k4-r16						INTEGER (0..123129),

k5-r16						INTEGER (0..61565),

...

},

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126)								OPTIONAL,

nr-DL-TDOA-AdditionalMeasurements-r16

NR-DL-TDOA-AdditionalMeasurements-r16			OPTIONAL,

...,

[[

nr-UE-Rx-TEG-ID-r17					INTEGER (0..maxNumOfRxTEGs-1-r17)			OPTIONAL,

nr-DL-PRS-FirstPathRSRP-Result-r17	INTEGER (0..126)							OPTIONAL,

nr-los-nlos-Indicator-r17			CHOICE {

perTRP-r17						LOS-NLOS-Indicator-r17,

perResource-r17					LOS-NLOS-Indicator-r17

}																				OPTIONAL,

nr-AdditionalPathListExt-r17		NR-AdditionalPathListExt-r17				OPTIONAL,

nr-DL-TDOA-AdditionalMeasurementsExt-r17

NR-DL-TDOA-AdditionalMeasurementsExt-r17	OPTIONAL

]],

[[

nr-FrequencyHopping-Indicator-r18 	FFS {}

]]

}

Editor Notes:

Need further agreement from RAN1. FFS: indication of how many received hops / which received hops where used in the measurement report.

NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF

NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-DL-TDOA-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasTDOA-r17)) OF

NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-DL-TDOA-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16						OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-ResultDiff-r16			CHOICE {

k0-r16						INTEGER (0..8191),

k1-r16						INTEGER (0..4095),

k2-r16						INTEGER (0..2047),

k3-r16						INTEGER (0..1023),

k4-r16						INTEGER (0..511),

k5-r16						INTEGER (0..255),

...

},

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-ResultDiff-r16	INTEGER (0..61)									OPTIONAL,

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

...,

[[

nr-UE-Rx-TEG-ID-r17				INTEGER (0..maxNumOfRxTEGs-1-r17)				OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17

INTEGER (0..61)									OPTIONAL,

nr-los-nlos-IndicatorPerResource-r17

LOS-NLOS-Indicator-r17							OPTIONAL,

nr-AdditionalPathListExt-r17	NR-AdditionalPathListExt-r17					OPTIONAL

]]

}

-- ASN1STOP

NEXT CHANGE

6.5.11.4	NR DL-AoD Location Information Elements

–	NR-DL-AoD-SignalMeasurementInformation

The IE NR-DL-AoD-SignalMeasurementInformation is used by the target device to provide NR DL-AoD measurements to the location server.

-- ASN1START

NR-DL-AoD-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-DL-AoD-MeasList-r16			NR-DL-AoD-MeasList-r16,

...

}

NR-DL-AoD-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-AoD-MeasElement-r16

NR-DL-AoD-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16						OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15								OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15						OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16				OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16				OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126),

nr-DL-PRS-RxBeamIndex-r16		INTEGER (1..8)							OPTIONAL,

nr-DL-AoD-AdditionalMeasurements-r16

NR-DL-AoD-AdditionalMeasurements-r16	OPTIONAL,

...,

[[

nr-DL-PRS-FirstPathRSRP-Result-r17

INTEGER (0..126)						OPTIONAL,

nr-los-nlos-Indicator-r17		CHOICE {

perTRP-r17					LOS-NLOS-Indicator-r17,

perResource-r17				LOS-NLOS-Indicator-r17

}																		OPTIONAL,

nr-DL-AoD-AdditionalMeasurementsExt-r17

NR-DL-AoD-AdditionalMeasurementsExt-r17	OPTIONAL

]],

[[

nr-FrequencyHopping-Indicator-r18 	FFS {}

]]

Editor Notes:

Need further agreement from RAN1. FFS: indication of how many received hops / which received hops where used in the measurement report.

}

NR-DL-AoD-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..7)) OF

NR-DL-AoD-AdditionalMeasurementElement-r16

NR-DL-AoD-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasAoD-r17)) OF

NR-DL-AoD-AdditionalMeasurementElement-r17

NR-DL-AoD-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16				OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16				OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-DL-PRS-RSRP-ResultDiff-r16	INTEGER (0..30),

nr-DL-PRS-RxBeamIndex-r16		INTEGER (1..8)							OPTIONAL,

...

}

NR-DL-AoD-AdditionalMeasurementElement-r17 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r17				NR-DL-PRS-ResourceID-r16		OPTIONAL,

nr-DL-PRS-ResourceSetID-r17				NR-DL-PRS-ResourceSetID-r16		OPTIONAL,

nr-TimeStamp-r17						NR-TimeStamp-r16,

nr-DL-PRS-RSRP-ResultDiff-r17			INTEGER (0..30)					OPTIONAL, -- Cond rsrp

nr-DL-PRS-RxBeamIndex-r17				INTEGER (1..8)					OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17	INTEGER (0..61)					OPTIONAL, -- Cond rsrpp

nr-los-nlos-IndicatorPerResource-r17	LOS-NLOS-Indicator-r17			OPTIONAL,

...

}

-- ASN1STOP

NEXT CHANGE

6.5.12.4	NR Multi-RTT Location Information Elements

–	NR-Multi-RTT-SignalMeasurementInformation

The IE NR-Multi-RTT-SignalMeasurementInformation is used by the target device to provide NR Multi-RTT measurements to the location server.

-- ASN1START

NR-Multi-RTT-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-Multi-RTT-MeasList-r16		NR-Multi-RTT-MeasList-r16,

nr-NTA-Offset-r16				ENUMERATED { nTA1, nTA2, nTA3, nTA4, ... }		OPTIONAL,

...,

[[

nr-SRS-TxTEG-Set-r17			SEQUENCE (SIZE(1..maxTxTEG-Sets-r17)) OF

NR-SRS-TxTEG-Element-r17					OPTIONAL

-- Cond Case2-3

]],

[[

nr-UE-RxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL,-- Cond TEGCase3

nr-UE-TxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL,-- Cond TEGCase2-3

nr-UE-RxTxTEG-TimingErrorMargin-r17	RxTxTEG-TimingErrorMargin-r17	OPTIONAL -- Cond TEGCase1-2

]]

}

NR-Multi-RTT-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-Multi-RTT-MeasElement-r16

NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16								OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15										OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15								OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16						OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16						OPTIONAL,

nr-UE-RxTxTimeDiff-r16			CHOICE {

k0-r16						INTEGER (0..1970049),

k1-r16						INTEGER (0..985025),

k2-r16						INTEGER (0..492513),

k3-r16						INTEGER (0..246257),

k4-r16						INTEGER (0..123129),

k5-r16						INTEGER (0..61565),

...

},

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126)								OPTIONAL,

nr-Multi-RTT-AdditionalMeasurements-r16

NR-Multi-RTT-AdditionalMeasurements-r16			OPTIONAL,

...,

[[

nr-UE-RxTx-TEG-Info-r17				NR-UE-RxTx-TEG-Info-r17						OPTIONAL,

nr-DL-PRS-FirstPathRSRP-Result-r17	INTEGER (0..126)							OPTIONAL,

nr-los-nlos-Indicator-r17			CHOICE {

perTRP-r17					LOS-NLOS-Indicator-r17,

perResource-r17				LOS-NLOS-Indicator-r17

}																				OPTIONAL,

nr-AdditionalPathListExt-r17		NR-AdditionalPathListExt-r17				OPTIONAL,

nr-Multi-RTT-AdditionalMeasurementsExt-r17

NR-Multi-RTT-AdditionalMeasurementsExt-r17	OPTIONAL

]],

[[

nr-FrequencyHopping-Indicator-r18 	FFS {}

]]

Editor Notes:

Need further agreement from RAN1. FFS: indication of how many received hops / which received hops where used in the measurement report.

}

NR-Multi-RTT-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF

NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasRTT-r17)) OF

NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16			NR-DL-PRS-ResourceID-r16					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16			NR-DL-PRS-ResourceSetID-r16				OPTIONAL,

nr-DL-PRS-RSRP-ResultDiff-r16		INTEGER (0..61)								OPTIONAL,

nr-UE-RxTxTimeDiffAdditional-r16	CHOICE {

k0-r16							INTEGER (0..8191),

k1-r16							INTEGER (0..4095),

k2-r16							INTEGER (0..2047),

k3-r16							INTEGER (0..1023),

k4-r16							INTEGER (0..511),

k5-r16							INTEGER (0..255),

...

},

nr-TimingQuality-r16				NR-TimingQuality-r16,

nr-AdditionalPathList-r16			NR-AdditionalPathList-r16					OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

...,

[[

nr-UE-RxTx-TEG-Info-r17					NR-UE-RxTx-TEG-Info-r17			OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17	INTEGER (0..61)					OPTIONAL,

nr-los-nlos-IndicatorPerResource-r17	LOS-NLOS-Indicator-r17			OPTIONAL,

nr-AdditionalPathListExt-r17			NR-AdditionalPathListExt-r17	OPTIONAL

]]

}

NR-SRS-TxTEG-Element-r17 ::= SEQUENCE {

nr-TimeStamp-r17			NR-TimeStamp-r16							OPTIONAL,	-- Need OP

nr-UE-Tx-TEG-ID-r17			INTEGER (0..maxNumOfTxTEGs-1-r17),

carrierFreq-r17				SEQUENCE {

absoluteFrequencyPointA-r17		ARFCN-ValueNR-r15,

offsetToPointA-r17				INTEGER (0..2199)

}											OPTIONAL,

srs-PosResourceList-r17		SEQUENCE (SIZE (1..maxNumOfSRS-PosResources-r17)) OF

INTEGER (0..maxNumOfSRS-PosResources-1-r17),

...

}

NR-UE-RxTx-TEG-Info-r17 ::= CHOICE {

case1-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17)

},

case2-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

case3-r17				SEQUENCE {

nr-UE-Rx-TEG-ID-r17		INTEGER (0..maxNumOfRxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

...

}

-- ASN1STOP

END OF CHANGE