3GPP TSG-RAN WG2 Meeting #123bis	                     		  R2-2309607

Xiamen, China, Oct 9 - 13, 2023

Source:	CATT

Title:	Discussion on bandwidth aggregation for positioning

Agenda Item:	7.2.5

Document for:	Discussion and Decision

1 Introduction

In RAN1 #112, #112bis-e, #113, and #114 meeting [1][2][3][4], these RAN1 led objective which is bandwidth aggregation for positioning is discussed in RAN1 and LS was sent to RAN2 at #114 meeting [5]. This contribution will show the observed issue in the LS from RAN1 and the impact to RAN2 according to RAN1 agreement.

2 Discussion

2.1 Discussion on the LS on TRP ID from RAN1

RAN1 discussed the understanding of dl-PRS-ID, and sent their interpretations to RAN2 in LS [5]. However from RAN2’s perspective, the below interpretations are not correct.

1. Overall Description:

With regards to higher layer parameter dl-PRS-ID, RAN1 understands that the current RAN2 specification support two interpretations:

- Interpretation 1: PRS resource sets in different PFLs of a TRP are configured with the same dl-PRS-ID

- Interpretation 2: PRS resource sets in different PFLs of a TRP can be configured with different dl-PRS-ID

- What’s the meaning of dl-PRS-ID in LPP?

dl-PRS-ID is the id of a TRP associated with a single TRP, not the id of a DL-PRS Resource Set in a TRP.

So the PRS resource sets in different PFLs of a TRP (within this TRP) are definitely configured with the same dl-PRS-ID which is associated with this TRP.

Below please find the definition of dl-PRS-ID in LPP as proof.

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resource ID to uniquely identify a DL-PRS Resource, and is associated with a single TRP.

NR-DL-PRS-AssistanceDataPerTRP-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16			OPTIONAL,	-- Need ON

nr-CellGlobalID-r16				NCGI-r15					OPTIONAL,	-- Need ON

nr-ARFCN-r16					ARFCN-ValueNR-r15			OPTIONAL,	-- Need ON

nr-DL-PRS-SFN0-Offset-r16		NR-DL-PRS-SFN0-Offset-r16,

nr-DL-PRS-ExpectedRSTD-r16		INTEGER (-3841..3841),

nr-DL-PRS-ExpectedRSTD-Uncertainty-r16

INTEGER (0..246),

nr-DL-PRS-Info-r16				NR-DL-PRS-Info-r16,

Observation 1: With regards to higher layer parameter dl-PRS-ID, the current RAN2 specification only support: Interpretation 1: PRS resource sets in different PFLs of a TRP are configured with the same dl-PRS-ID. Because dl-PRS-ID is the id of a TRP associated with a single TRP, not the id of a DL-PRS Resource Set in a TRP.

- What’s the meaning of NR-DL-PRS-ResourceSetID in LPP?

The PRS resource set is identified by NR-DL-PRS-ResourceSetID in LPP which is a global id in a TRP. The PRS resource sets in different PFLs of a TRP are identified with different NR-DL-PRS-ResourceSetID.

Below please find the proof of NR-DL-PRS-ResourceSetID as global id: different frequency layers have different Resource Set IDs and the range is from 0 to 7, instead of from 0 to 1.

- NR-DL-PRS-Info field descriptions

nr-DL-PRS-ResourceSetID This field specifies the DL-PRS Resource Set ID, which is used to identify the DL-PRS Resource Set of the TRP across all the frequency layers.

–	NR-DL-PRS-ResourceSetID

The IE NR-DL-PRS-ResourceSetID defines the identity of a DL-PRS Resource Set of a TRP.

-- ASN1START

NR-DL-PRS-ResourceSetID-r16 ::= INTEGER (0..nrMaxNumDL-PRS-ResourceSetsPerTRP-1-r16)

-- ASN1STOP

nrMaxNumDL-PRS-ResourceSetsPerTRP-1-r16	INTEGER ::= 7

Observation 2: With regards to higher layer parameter NR-DL-PRS-ResourceSetID, the current RAN2 specification only support: PRS resource sets in different PFLs of a TRP are identified with different number. PRS resource sets of a TRP, which may be configured within the same PFL or across different PFLs, are identified with distinct PRS resource set IDs.

According to the understanding of dl-PRS-ID and NR-DL-PRS-ResourceSetID in LPP, we propose to reply to RAN1 to correct their interpretations of dl-PRS-ID and NR-DL-PRS-ResourceSetID.

Proposal 1: Send a reply LS to RAN1 to clarify the higher layer parameters, indicating that:

•	With regards to higher layer parameter dl-PRS-ID, the current RAN2 specification only support: Interpretation 1: PRS resource sets in different PFLs of a TRP are configured with the same dl-PRS-ID. Because dl-PRS-ID is the id of a TRP associated with a single TRP, not the id of a DL-PRS Resource Set in a TRP.

•	With regards to higher layer parameter NR-DL-PRS-ResourceSetID, the current RAN2 specification only support: PRS resource sets in the same PFL or different PFLs of a TRP are identified with different numbers. PRS resource sets of a TRP, which may be configured within the same PFL or across different PFLs, are identified with distinct PRS resource set IDs.

The reply LS in R2-2309599 Reply LS on TRP ID for positioning with bandwidth aggregation can be taken as baseline.

2.2 PRS aggregation linkage

According to the parameters from RAN1, the indication which two or three PFLs are used will be provided in DL-TDOA Location Information Request/ in Multi-RTT Location Information Request.

| Description | Value range | Per (UE, cell, TRP, …) | Specification |
| --- | --- | --- | --- |
| Request from the LMF to the UE indicating which two or three PFLs to be used for performing joint measurement | TBD | Per UE In DL-TDOA Location Information Request In Multi-RTT Location Information Request | 37.355 |
| This field indicates whether to perform joint measurement across aggregated PFLs for DL-TDOA. | Enabled | In NR-DL-TDOA-RequestLocationInformation | 37.355 |
| This field indicates whether to perform joint measurement across aggregated PFLs for Multi-RTT. | Enabled | In NR-Multi-RTT-RequestLocationInformation | 37.355 |
| PRS resource set IDs for the aggregated measurement which are used for RSRP/RSRPP and/or timing measurement results | PRS Resource Set IDs | In NR-DL-TDOA-MeasElement; [In NR-Multi-RTT-MeasElement] | 37.355 |

For each TRP, RAN1 support new signaling to indicate which PRS resource sets across PFLs are linked. And the information will be indicated in TS38.455.

Agreement

For PRS bandwidth aggregation across PFLs, support

•	Option 2: Per TRP basis and per PRS resource set basis.

o	For each TRP, support new signaling to indicate which PRS resource sets across PFLs are linked.

o	It is assumed that the PRS resources across the linked PRS resource sets are linked if the conditions are satisfied. For the non-linked PRS resource sets, no aggregation is assumed even if the conditions are satisfied.

Agreement

For PRS bandwidth aggregation, with regards to the signaling in the location information request message, introduce the following:

•	A request to indicate UE which two or three PFLs to be used for performing joint measurement

| Indication of DL PRS resource sets in the two or three DL PFLs that are linked for DL PRS BW aggregation from the NG-RAN node to the LMF | Per TRP Example: in PRS Configuration (as in 9.2.44) in PRS CONFIGURATION RESPONSE message | 38.455 |
| --- | --- | --- |

Usually the NR-DL-PRS-AssistanceData can be broadcast to UE in posSIB. So the aggregation configuration per TRP may follow the legacy mechanism.

| NR DL-TDOA/DL-AoD Assistance Data (clauses 6.4.3, 7.4.2) | posSibType6-1 | NR-DL-PRS-AssistanceData |
| --- | --- | --- |
|  | posSibType6-2 | NR-UEB-TRP-LocationData |
|  | posSibType6-3 | NR-UEB-TRP-RTD-Info |
|  | posSibType6-4 | NR-TRP-BeamAntennaInfo |
|  | posSibType6-5 | NR-DL-PRS-TRP-TEG-Info |

The BW aggregation configuration per TRP can be indicated to UE via posSIB. So in LPP protocol, the assistance data can be enhanced to indicate UE which PRS resource sets in a TRP are linked via posSIB.

Proposal 2: The configuration of linked DL-PRS resource sets across linked PFLs per cell can be broadcast in posSIB.

RAN1 request RAN2 to capture the condition of the same TRP in RAN2 specifications for PRS bandwidth aggregation [5]. On the other hand, resource SetID is a global id in a TRP, so when the linked PRS resource sets from two or three PFLs should be from the same TRP, the linked resource sets can be designed per TRP directly as below:

-- ASN1START

NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-ReferenceInfo-r16			DL-PRS-ID-Info-r16,

nr-DL-PRS-AssistanceDataList-r16	SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-DL-PRS-AssistanceDataPerFreq-r16,

nr-SSB-Config-r16					SEQUENCE (SIZE (1..nrMaxTRPs-r16)) OF

NR-SSB-Config-r16	OPTIONAL,	-- Need ON

...

}

NR-DL-PRS-AssistanceDataPerFreq-r16 ::= SEQUENCE {

nr-DL-PRS-PositioningFrequencyLayer-r16

NR-DL-PRS-PositioningFrequencyLayer-r16,

nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

NR-DL-PRS-AssistanceDataPerTRP-r16,

...

}

NR-DL-PRS-AssistanceDataPerTRP-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16			OPTIONAL,	-- Need ON

nr-CellGlobalID-r16				NCGI-r15					OPTIONAL,	-- Need ON

nr-ARFCN-r16					ARFCN-ValueNR-r15			OPTIONAL,	-- Need ON

nr-DL-PRS-SFN0-Offset-r16		NR-DL-PRS-SFN0-Offset-r16,

nr-DL-PRS-ExpectedRSTD-r16		INTEGER (-3841..3841),

nr-DL-PRS-ExpectedRSTD-Uncertainty-r16

INTEGER (0..246),

nr-DL-PRS-Info-r16				NR-DL-PRS-Info-r16,

...,

[[

prs-OnlyTP-r16				ENUMERATED { true }		OPTIONAL	-- Need ON

]],

[[

nr-DL-PRS-ExpectedAoD-or-AoA-r17

NR-DL-PRS-ExpectedAoD-or-AoA-r17	OPTIONAL	-- Need ON

]],

[[

nr-DL-PRS-BWAggregationInfo-r18		NR-BW-DL-PRS-IndexList-r18 	OPTIONAL	-- Need ON

]]

}

***skip the unchange part***

NR-BW-DL-PRS-IndexList-r18 ::=	SEQUENCE {

NR-TwoBW-DL-PRS-IndexList-r18	SEQUENCE (SIZE(1..nrMaxNum-TwoFreqs-PerTRP-r18)) OF

NR-BW-DL-PRS-TwoFreqs-r18 OPTIONAL,		--Need OP

NR-ThreeBW-DL-PRS-IndexList-r18 SEQUENCE (SIZE(1..nrMaxNum-ThreeFreqs-PerTRP-r18)) OF

NR-BW-DL-PRS-ThreeFreqs-r18 OPTIONAL,	--Need OP

...

}

NR-BW-DL-PRS-TwoFreqs-r18 ::=	SEQUENCE {

nr-SelectedDL-PRS-FrequencyLayerIndex-r16	INTEGER (0..nrMaxFreqLayers-1-r16),

nr-SelectedDL-PRS-FrequencyLayerIndex-r16	INTEGER (0..nrMaxFreqLayers-1-r16),

nr-SelectedDL-PRS-IndexListPerFreq-r16		SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

NR-SelectedDL-PRS-IndexPerTRP-r16

OPTIONAL	--Need OP

}

NR-BW-DL-PRS-ThreeFreqs-r18::=	SEQUENCE {

nr-SelectedDL-PRS-FrequencyLayerIndex-r16	INTEGER (0..nrMaxFreqLayers-1-r16),

nr-SelectedDL-PRS-FrequencyLayerIndex-r16	INTEGER (0..nrMaxFreqLayers-1-r16),

nr-SelectedDL-PRS-FrequencyLayerIndex-r16	INTEGER (0..nrMaxFreqLayers-1-r16),

nr-SelectedDL-PRS-IndexListPerFreq-r16		SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

NR-SelectedDL-PRS-IndexPerTRP-r16

OPTIONAL	--Need OP

}

nrMaxNum-TwoFreqs-PerTRP-r18					INTEGER ::= 6	-- Max combination num two freq layers

nrMaxNum-ThreeFreqs-PerTRP-r18					INTEGER ::= 4-- Max combination num three freq layers

Proposal 3: The TP of broadcast aggregated DL-PRS info can be merged into LPP running CR.

2.3 On-demand PRS bandwidth aggregation

The following agreements were made by RAN1 on supporting the on-demand PRS bandwidth aggregation. It can be seen that RAN1 left all on-demand PRS bandwidth aggregation related signaling to RAN2.

RAN1#112:

Agreement - Support LMF-initiated and UE-initiated on-demand PRS request for PRS bandwidth aggregation - FFS details - Support preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations - FFS details

RAN1#112bis:

Conclusion The details for on-demand PRS on PRS bandwidth aggregation are up to RAN2 and RAN3.

2.3.1 Pre-configured available on-demand PRS bandwidth aggregation configurations

As for R17 on-demand PRS, NW can pre-configure a list of available PRS configurations to UE. For each PRS configuration, the information that can be changed, e.g., the bandwidth, the combine size, the periodicity, the start offset, the repetition, the number of symbols and the QCL info can be requested. From our perspective, similar design can also apply to the on-demand PRS bandwidth aggregation. That is, NW also pre-configure a list of available PRS bandwidth aggregation configurations, within each PRS bandwidth aggregation configuration, there can include a full set of PRS configurations for bandwidth aggregation, i.e., the two or three PFLs that can be aggregated, the resources sets per TRP across the two or the three PFLs can be aggregated. Besides, for the resources can be aggregated, there are some conditions to be satisfied as we listed in the following table, but since it is NW to provide the available PRS configurations, we think NW can ensure such conditions are satisfied for each PRS bandwidth aggregation configuration within the list of the available PRS bandwidth aggregation.

Agreement For PRS bandwidth aggregation between PRS in two or three different PFLs, the following are needed for the aggregated PRS resources for a TRP: - The same periodicity and slot offset - The same muting pattern - The same NR-DL-PRS-SFN0-Offset value - UE expects to be configured with PRS resources that maintain a per-symbol uniformly spaced PRS pattern across aggregated bandwidths in frequency domain (Note: It does not preclude dropping some REs in the guardband between two PFLs). - FFS same antenna port from RAN1 perspective

Proposal 4: NW provides a list of available on-demand PRS bandwidth aggregation configurations, and for each on-demand PRS bandwidth aggregation within the list, there is a full PRS bandwidth aggregation configuration (PFLs, resource sets across the PFLs).

Proposal 5: For each PRS bandwidth aggregation configuration within the list, NW to ensure the conditions specified by RAN1 to ensure the bandwidth aggregation should be satisfied.

2.3.2 UE-initiated on-demand PRS request

As for UE-initiated on-demand PRS request in R17, two types of request are supported, i.e., the explicitly request of the detailed PRS configuration parameters, or the request of the configuration index based on the pre-configured available on-demand PRS configurations.

Observation 3: For R17 UE initiated on-demand PRS, both the explicit request of the detailed PRS configuration parameters and the request of the available PRS configuration index are supported.

In general, we think similar solutions can also be reused for UE-initiated on-demand PRS bandwidth aggregation. That is, UE can request the explicit PRS bandwidth aggregation configurations, or UE can also indicate the index of the PRS bandwidth aggregation configurations from the list of the available PRS bandwidth aggregation configurations provided by NW,

Proposal 6: For UE-initiated on-demand PRS bandwidth aggregation request, both the request of explicit PRS bandwidth aggregation configuration parameters and the request of the PRS bandwidth aggregation configurations from the list of the available PRS bandwidth aggregation configurations can be supported.

Besides, for explicit PRS bandwidth aggregation configuration parameters request type, since there are conditions agreed by RAN1 on which PRS configurations can be aggregated, so it is proposed that UE need to ensure that explicit PRS bandwidth aggregation parameters requested by UE should satisfy the requirement.

Proposal 7: For the UE initiated on-demand PRS bandwidth aggregation via explicit PRS configuration parameters, UE need to ensure that the explicit requested PRS bandwidth aggregation configuration parameters should satisfy the conditions as specified by RAN1 for PRS bandwidth aggregation.

3 Conclusions

In this contribution, we have discusses the LPP impact and potential solutions based on the RAN1 progress on band aggregation of DL-PRS.

Observation 1: With regards to higher layer parameter dl-PRS-ID, the current RAN2 specification only support: Interpretation 1: PRS resource sets in different PFLs of a TRP are configured with the same dl-PRS-ID. Because dl-PRS-ID is the id of a TRP associated with a single TRP, not the id of a DL-PRS Resource Set in a TRP.

Observation 2: With regards to higher layer parameter NR-DL-PRS-ResourceSetID, the current RAN2 specification only support: PRS resource sets in different PFLs of a TRP are identified with different number. PRS resource sets of a TRP, which may be configured within the same PFL or across different PFLs, are identified with distinct PRS resource set IDs.

Observation 3: For R17 UE initiated on-demand PRS, both the explicit request of the detailed PRS configuration parameters and the request of the available PRS configuration index are supported.

Proposal 1: Send a reply LS to RAN1 to clarify the higher layer parameters, indicating that:

•	With regards to higher layer parameter dl-PRS-ID, the current RAN2 specification only support: Interpretation 1: PRS resource sets in different PFLs of a TRP are configured with the same dl-PRS-ID. Because dl-PRS-ID is the id of a TRP associated with a single TRP, not the id of a DL-PRS Resource Set in a TRP.

•	With regards to higher layer parameter NR-DL-PRS-ResourceSetID, the current RAN2 specification only support: PRS resource sets in the same PFL or different PFLs of a TRP are identified with different numbers. PRS resource sets of a TRP, which may be configured within the same PFL or across different PFLs, are identified with distinct PRS resource set IDs.

The reply LS in R2-2309599 Reply LS on TRP ID for positioning with bandwidth aggregation can be taken as baseline.

Proposal 2: The configuration of linked DL-PRS resource sets across linked PFLs per cell can be broadcast in posSIB.

Proposal 3: The TP of broadcast aggregated DL-PRS info can be merged into LPP running CR.

Proposal 4: NW provides a list of available on-demand PRS bandwidth aggregation configurations, and for each on-demand PRS bandwidth aggregation within the list, there is a full PRS bandwidth aggregation configuration (PFLs, resource sets across the PFLs).

Proposal 5: For each PRS bandwidth aggregation configuration within the list, NW to ensure the conditions specified by RAN1 to ensure the bandwidth aggregation should be satisfied.

Proposal 6: For UE-initiated on-demand PRS bandwidth aggregation request, both the request of explicit PRS bandwidth aggregation configuration parameters and the request of the PRS bandwidth aggregation configurations from the list of the available PRS bandwidth aggregation configurations can be supported.

Proposal 7: For the UE initiated on-demand PRS bandwidth aggregation via explicit PRS configuration parameters, UE need to ensure that the explicit requested PRS bandwidth aggregation configuration parameters should satisfy the conditions as specified by RAN1 for PRS bandwidth aggregation.

4 Reference

- R1-2302065 Session notes for 9.5 (Study on expanded and improved NR positioning), Ad-Hoc Chair (Huawei)

- 3GPP TS 38.305 Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN V17.4.0