3GPP TSG-RAN WG2 Meeting#123bis	R2-2309652

# Xiamen, China, 9th-13th October 2023                                 resubmission of R2-2307113

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0931 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the Capability of TA Reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-core |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the description of the capability uplink-TA-Reporting-r17 in TS 38.306 as below, this indication is to indicate whether UE supports TA reporting in all scenarios since TS 38.321 is referenced and all scenarios have been captured clearly in TS 38.321 clause 5.4.8. Specifically, this indication can be used to indicate whether UE supports TA reporting during initial access. However, the capability of TA reporting during initial access is defined in section 6 in TS 38.306 as below, which leads to redundant definitions of TA reporting during initial access. To this end, some clarification of the capability of TA reporting is needed. uplink-TA-Reporting-r17 Indicates whether the UE supports UE reporting of information related to TA pre-compensation as specified in TS 38.321 [8]. UE indicating support of this feature shall also indicate support of uplinkPreCompensation-r17 for this band. This field is only applicable for bands in Table 5.2.2-1 in TS 38.101-5 [34] and HAPS operation bands in clause 5.2 of TS 38.104 [35].; Band; No; N/A; N/A. TA reporting during initial access; It is mandatory to support TA reporting during initial access for UEs supporting uplink-TA-Reporting-r17 as specified in TS 38.321 [8].. |
| Summary of change: |  | Clarify the indication uplink-TA-Reporting-r17 does not include the capability of TA reporting during initial access. Impact analysis Impacted 5G architecture options: NR standalone Impacted functionality: TA reporting Inter-operability: - If the network is implemented according to the CR and the UE is not, the UE and the network may have different understandings about the capability uplink-TA-Reporting. - If the UE is implemented according to the CR and the network is not, the UE and the network may have different understandings about the capability uplink-TA-Reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of TA reporting is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

#### 4.2.7.2	BandNR parameters

< Irrelevant part is omitted >

| uplink-TA-Reporting-r17 Indicates whether the UE supports UE reporting of information related to TA pre-compensation as specified in TS 38.321 [8] except TA reporting during initial access. UE indicating support of this feature shall also indicate support of uplinkPreCompensation-r17 for this band. This field is only applicable for bands in Table 5.2.2-1 in TS 38.101-5 [34] and HAPS operation bands in clause 5.2 of TS 38.104 [35]. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

End of change