3GPP TSG-RAN WG2 #123bis	R2-2309665

Xiamen China Oct 9~13 2023

Agenda Item:	7.20.2

Source:	OPPO

Title:	Discussion on multiple TAG

Document for:	Discussion, Decision

Introduction

At RAN2#122 meeting, following agreement was made:

Configure one TAT per TAG to support two TAs for a serving cell, i.e., in this case 2 TAGs are configured for the serving cell.

At RAN2#123 meeting, following agreement was made:

Each joint/UL TCI state is associated with either TAG1 or TAG2 by RRC configuration.

RAN2 do not assume any restriction on grouping serving cells/TRPs to TAGs unless RAN1 indication comes.

RAN2 assumes the current 4 TAGs per cell group is sufficient to support Rel-18 mDCI mTRP with 2 TAs.

At least when both TATs for a SpCell are expired, 1-8 are applied to all TRPs of all serving cells.

At least when both TATs for a SCell are expired (assuming PTAG(s) of the cell group still running), 1-7 are applied to all TRPs associated to the TAG with the expired TAT (including both TRPs of the concerned SCell).

- Working assumption:

We will use the 2-PTAG model, i.e., both TAGs of SpCell are PTAGs;

      - When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired,

      - when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired.

For inter-cell PDCCH order CFRA to the additionalPCI,

      - PDCCH order indicates which additionalPCI’s PRACH configuration to be used (according to RAN1 agreement),

The following is taken as baseline (for intra-cell case): for CBRA, we reuse the mechanism agreed for CFRA case, i.e. use the RA RAR to indicate the TAG.

In RAN1#114 there are also some agreements related to two TA feature as following:

Agreement:

For inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, support indication of additionalPCI in the PDCCH order

- as baseline capability: support PRACH triggering towards servingCell PCI or active additionalPCI.

Agreement

- For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement and PDCCH order CFRA, indicate a representation of the TAG ID with 1 bit (either the first TAG ID or the second TAG ID for the serving cell) as part of TA command in RAR

- Note: For intra-cell multi-DCI based Multi-TRP operation, only a single NTA,offset is configured.

Based on those progress, in this paper we would like to consolidate the PTAG issue and CFRA/CBRA issue.

Discussion

RACH procedure of inter-cell case:

For CFRA gNB will signal indication of TAG ID via PDCCH order indirectly. Our understanding is that when the indication parameter is within range of 1~7, it indicates one active additional PCI. When it is 0(zero), it refers to servingCellPCI. Next question is how can UE deduce TAG ID based on received PCI index? There could be two options:

Option 1: Implicitly reflected from the association between TCI state and TAG

Option 2: Explicit RRC configuration

The association between servingPCI (and/or additionalPCI) and TAG ID could be reflected in the association between TCI state and TAG. But the reference signal within TCI state could be SSB or CSI-RS or SRS. If the reference signal is not SSB, technically the association between TCI state and TAG ID can’t be used for the association between PCI index and TAG ID indirectly unless CSI-RS or SRS are QCLed with SSB. In order to avoid such problem and keep some flexibility for the association between TRP and TAG ID, the association between PCI index and TAG ID could be configurable i.e. option 2. It should be noted that this configured parameter should be aligned with the association between TCI state and TAG ID. We slightly prefer option 2.

Since the association between servingPCI and TAG ID shall be the same for both legacy UE and new UE supporting this new feature, the servingPCI shall be always associated with legacy parameter tag-Id instead of tag-Id2. In addition if the reference signal within TCI state is SSB, then SSB from same PCI should be associated with same TAG. In this way once PDCCH order is received, UE knows the PRACH configuration and TAG ID hence no additional work to be done in MAC layer.

The additional PRACH resource is added for CFRA purpose only in RAN1. Here is the relevant parameter in [1]:

| NR_MIMO_evo_DL_UL-Core | mTRP-2TA | Up to RAN2 | additionalCFRA-ToAddModList-r18 (or up to RAN2) | New | List of additional CFRA configurations where each of the CFRA configurations in the list correspond to one of the Additional PCIs. |  | The contents of the paramaters inside each of the CFRA configurations in the list is up to RAN2. This is for PDCCH order CFRA only. This is just an example design. The exact design is up to RAN2. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Even technically additional PRACH resources could be also used for new UE to trigger CBRA. For UE in connected state CBRA is triggered mainly due to uplink data arrival or failure of SR procedure or BFR procedure etc. Those triggers don’t occur so frequently and legacy PRACH resource associated with servingPCI can be reused and sufficient. Another point is that those additional PRACH resources are configured via dedicated signalling which is of high flexibility and resource efficiency compared to PRACH resource broadcast for all new UE in e.g. SIB. So it would desirable that UE trigger CBRA only based on legacy PRACH resource, which is associated with servingPCI. For legacy UE in this case TAG ID is zero, which should be the same for new UE also.

When UE chooses legacy PRACH resource to transmit preamble for CBRA purpose, UE also chooses TAG 0 just like legacy UE. Hence no additional work to be done in MAC layer.

Proposal 2: The association between PCI index including servingPCI and additionalPCI with TAG ID is configurable

Proposal 2a: ServingPCI is always associated with existing parameter tag-Id but not tag-Id2.

Proposal 2b: in SPCell servingPCI is always associated with TAG 0 to avoid backwards compatibility issue

Proposal 2c: the association between PCI index and TAG ID should be aligned with association between TCI state and TAG ID in same serving cell/BWP.

Proposal 3: When UE trigger CBRA for inter-cell case, UE always chooses PRACH associated with servingPCI and hence TAG 0.

Proposal 3a: PRACH resources associated with additionalPCI is only used for CFRA purpose

RACH procedure of intra-cell case:

For intra-cell case, CFRA is triggered by PDCCH order command without any new parameters. UE will transmit preamble of legacy PRACH configuration. Because there is no difference w.r.t. NTA,offset between two TAGs of the same serving cell, it doesn’t matter whether UE knows the concerned TAG when it transmits preamble. When corresponding RAR is received, TAG ID is indicated by TA Command within RAR.

At last RAN2 meeting some company believe the same solution could be used also for CBRA. But actually there is backwards compatibility issue.

The following is taken as baseline (for intra-cell case): for CBRA, we reuse the mechanism agreed for CFRA case, i.e. use the RA RAR to indicate the TAG.

When CBRA is triggered, UE could use either 2-step RACH or 4-step RACH. For 2-step RACH,RAN1 already agreed that:

RAN1#110bis-e [8]

Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support enhancements related to indicating TAG ID via absolute TA command: - FFS: whether the indication is implicit or explicit - Detailed indication schemes are FFS - This does not preclude indication of two TAG IDs (if supported) - Note: This applies at least to MSGB in case of C-RNTI

RAN1#112bis-e [9]

Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, how to indicate the TAG ID via absolute TA command MAC CE is left up to RAN2: - One of two TAG IDs configured in the SpCell can be indicated.

For 2-step RACH network will receive msgA along with preamble. And the content of the msgA could tell UE’s RRC state (e.g. RRCSetupRequest for IDLE etc.) and UE’s identity(resumeIdentity or ue-Identity or C_RNTI). So network can set Absolute TA Command properly for UE in RRC_CONNECTED state.

RAN1 solution can be also reused for UE in RRC_INACTIVE state. This is because once msgA is received, UE context can be also identified and network can differentiate legacy UE from new UE. If legacy UE or new UE chooses SSB from additional TRP, network can use legacy RAR or updated RAR with TAG1 respectively.

RAN1 solution however can’t be reused directly for UE in RRC_IDLE state. The problematic case is when legacy UE choose SSB from additional TRP to transmit preamble. Network can know UE’s RRC state by assessing RRC message within msgA but it can’t differentiate legacy UE from new UE without further UE context. So if network use updated RAR with TAG1 blindly there is mismatch between network and legacy UE in terms of TAG ID because legacy UE will always take TAG ID as 0 (zero). Detail analysis can be found in Annex 2.

Observation1: In case of CBRA based on 2-step RACH, for UE in RRC_CONNECTED state gNB can indicate TAG ID within absolute TA command without compatibility issue

Observation2: In case of CBRA based on 2-step RACH, for UE in RRC_INACTIVE state gNB can indicate TAG ID within TA command in RAR without compatibility issue

Observation3: In case of CBRA based on 2-step RACH, for UE in RRC_IDLE state there is backwards compatibility issue by indicating TAG ID within TA command in RAR when SSB from additionalTRP is chosen for preamble transmission by legacy UE

Similar ambiguous issue occurs for 4-step RACH. When UE chooses a SSB from serving TRP and transmit preamble, the TAG ID is always zero from both network and UE point of view. So it doesn’t matter whether gNB can differentiate legacy UE from new UE regardless of RRC state.

But when UE chooses a SSB from additional TRP there is mismatch between legacy UE and gNB w.r.t TAG ID. The problem for 4-step RACH is different from 2-step RACH in the sense network can’t differentiate legacy UE from new UE and RRC state purely based on received preamble. Network can only behave as if all the UE is new UE. In this case network will set TAG ID=1 in the RAR. However legacy UE can’t recognize this updated RAR and still consider the TAG ID as zero in all RRC states. Detail analysis can be found in ANNEX 1.

First of all similar to inter-cell case, the association between TCI state and TAG ID could also imply that the association between TRP and TAG ID is actually semi-static:

Figure 2.2-1 association between TCI state, TAG and TRP

In Figure 2.2-1, since TCI state 1 with SSB1 as reference signal is associated with TAG 0, other TCI states with SSBs from serving TRP will also associated with TAG 0. And the same story happen to another bunch of SSBs and TAG 1.

Observation4: the association between TAG and TRP (hence set of SSBs) is actually semi static.

For the problematic case i.e. SSB from additional TRP is chosen, network can take two approaches:

Approach 1: Network set TAG ID as TAG 1 in the TA command within RAR

Approach 2: Network set TAG ID as TAG 0 in the TA command within RAR

With approach 1, there is mismatch between network and legacy UE as explained before. If legacy UE is in RRC_CONNECTED or RRC_INACTIVE state, network however can correct its TAG from 1 to 0 once it identify UE as legacy UE after receiving msg3 assuming legacy UE will not look into the TAG ID within the RAR. But this correction can’t be done for UE in RRC_IDLE state because network can’t differentiate legacy UE from new UE without further UE context. Since network can know UE’s RRC state after receiving msg3, one adjustment for approach 1 is to set TAG 0 only for UE in RRC_IDLE state. This adjustment should be applied for both legacy UE and new UE. In this regard new UE’s behaviour is different between RRC_IDLE and RRC_CONNECTED/RRC_INACTIVE state.

With approach 2, there is no problem for new UE in RRC_IDLE. It basically mean SSBs from additional PCI (hence additional TRP) is associated with TAG 0, which is totally different from the association between TCI state and TAG as illustrated in Figure 2.2-1. The adjustment for approach2 could be to reconfigure the association between TCI state and TAG in e.g. msg4. In addition it is possible that one or two TAT timers are still running when CBRA is triggered. So it is also necessary to consider all TAT timer(s) expires in order to build new association in clean way.

Another solution direction is that TAG0 is used for 4-step CBRA regardless of chosen SSB i.e. without reusing solution for CFRA. The only problematic case is when new UE in RRC_CONNECTED or RRC_INACTIVE state. If UE choose SSB from additional TRP to transmit preamble, it will associate additional TRP (and hence SSB) with TAG 0, which may contradict with running TAT and association between TCI state and TAG. To resolve the problem the same solution as adjustment on above approach2 is needed.

Observation5: In case of CBRA based on 4-step RACH, network can’t differentiate legacy UE from new UE and RRC state purely based on received preamble

Observation6: In case of CBRA based on 4-step RACH, there is backward compatibility issue by the baseline solution if legacy UE chooses SSB from additionalTRP for preamble transmission in RRC_CONNECTED and RRC_IDLE state. But it could be corrected by network after receiving msg3.

Observation7: In case of CBRA based on 4-step RACH, there is backward compatibility issue by the baseline solution if legacy UE chooses SSB from additionalTRP for preamble transmission in RRC_IDLE state and it can’t corrected by network implementation.

Based on above observations the solution based on adjustment on approach 1 could be way forward. For UE in RRC_IDLE state regardless whether it is legacy UE and new UE, the TAG of the PCell is TAG 0 would be very natural result. And this could be also common solution between 2-step and 4-step RACH. It means the baseline solution can be reused for UE only in RRC_CONNECTED and RRC_INACTIVE state. For UE in RRC_IDLE state TAG 0 is used regardless chosen SSB. For 2-step RACH, network will know UE’s RRC state after receiving msgA, so it will be easy for gNB to set RAR within msgB. For 4-step RACH, gNB will know RRC state only after receiving msg3. So gNB will set TAG 1 within RAR in message 2 for all cases. gNB then correct its TAG as zero when it realizes UE is in RRC_IDLE state or the UE is a legacy UE in RRC_CONNECTED or RRC_INACTIVE state.

Proposal 4: agree following updated baseline:

for CBRA, we reuse the mechanism agreed for CFRA case for UE only in connected or inactive mode, i.e. use the RA RAR to indicate the TAG for both 2-step RACH and 4-step RACH.

Proposal 5: For CBRA based on 2-step RACH, TAG ID is signalled in MAC CE Absolute TA Command for UE in RRC_CONNECTED state.

Proposal 6: for CBRA based on both 2-step/4-step RACH, TAG 0 is used regardless of chosen SSB from UE perspective

Proposal 6a: for CBRA based on 4-step RACH, it is up to network’s implementation to correct TAG 1 to TAG 0 after it receive msg3 for both legacy/new UE in RRC_IDLE state and legacy UE in RRC_CONNECTED or RRC_INACTIVE state

PTAG issue

We understand some company take 2 PTAG on SPCell as working assumption is because they are sceptical about the impact on RACH procedure by such modelling. Based on above proposals, it is clear that there is no essential difference between two TAG on SPCell w.r.t CFRA and CBRA procedure.

Proposal to confirm following working assumption as agreement:

We will use the 2-PTAG model, i.e., both TAGs of SpCell are PTAGs;

      - When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired,

      - when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired.

The list of UE behaviour are as following:

List the actions to be applied at TAT expiry as follows.

1.	not perform any uplink transmission except the Random Access Preamble and MSGA transmission;

2.	flush all HARQ buffers;

3.	notify RRC to release PUCCH, if configured;

4.	notify RRC to release SRS, if configured;

5.	clear any configured downlink assignments and configured uplink grants;

6.	clear any PUSCH resource for semi-persistent CSI reporting;

7.	maintain NTA (defined in TS 38.211 [8]) of this TAG;

When TAT of one TAG expires but another is still running, the HARQ processes associated with TAG (hence TRP) can still work. So HARQ buffer of those HARQ processes should not be flushed. For another TRP, whose TAT timer expires, there is no uplink transmission is allowed except for preamble or msgA including HARQ ACK/NACK. However those HARQ ACK/NACK maybe able to go through another TRP if the backhaul is ideal.  The problem is that UE has no idea whether backhaul is ideal or not unless some configuration e.g. coherent joint transmission with NTRP>1 is configured, which is only feasible based on ideal backhaul. It means whether flush partial HARQ buffer could be conditional behaviour.

Proposal 7: When TAT of only one TAG expires (when there are two) UE should not flush HARQ buffer for those HARQ process associated with TRP(TAG), whose TAT timer is running.

Proposal 8: When TAT of only one TAG expires (when there are two) RAN2 need discuss whether conditional flush HARQ buffer is to be defined and how for those HARQ process associated with TRP(TAG), whose TAT timer expires.

The configured downlink assignment is configured per BWP but not per TRP. Same resource can be scheduled on any TRP by indicating coresetPoolIndex in PDCCH properly. That’s why UE can still keep the configured downlink assignments

Proposal 9: When TAT of only one TAG expires (when there are two), it is not necessary to flush configured downlink assignment

PUCCH resource will follow TCI state indicated via DCI[2]. In case M-DCI, which coresetPoolIndex to follow is configured via new RRC parameter applyIndicatedTCIState-r18 [1]. For the broken TRP, it is obviously that network will not indicate corresponding coresetPoolIndex anymore via DCI if TCI states of one coresetPoolIndex are only associated with one TAG and relevant PUCCH resource is not valid anymore unless it is reconfigured via RRC message. There are two options to treat those PUCCH resources:

Option1: UE release PUCCH resource relevant to broken TRP. When the TRP is recovered via CFRA, network reconfigure those PUCCH resource

Option2: UE doesn’t release PUCCH resource relevant to broken TRP. When the TRP is recovered via CFRA, UE continue to use those PUCCH resource i.e. UE suspend PUCCH resource relevant to broken TRP temporary

Which way to go actually depends on how long will last when the TAT timer of one TRP expires. If it can last relatively long time option 1 makes sense, or option 2 is better. UE can transmit on two TRPs only when UE is getting close to gNB. When it is bit far away from gNB, transmission power is limited due to high pathloss and transmission on 2 TRPs is not necessary. To keep two TAG while one TAG is out of synchronization sounds a temporary situation and hence it is not necessary for UE to release PUCCH resource since it can be recovered quickly via CFRA. The logic is the same for other resource including type1 CG and SRS resource.

Proposal 10: When TAT of only one TAG expires (when there are two), it is not necessary to release PUCCH, type1 CG PUSCH and SRS resource i.e. UE behaviour 3~6 are not conducted

Conclusion

Observation1: In case of CBRA based on 2-step RACH, for UE in RRC_CONNECTED state gNB can indicate TAG ID within absolute TA command without compatibility issue

Observation2: In case of CBRA based on 2-step RACH, for UE in RRC_INACTIVE state gNB can indicate TAG ID within TA command in RAR without compatibility issue

Observation3: In case of CBRA based on 2-step RACH, for UE in RRC_IDLE state there is backwards compatibility issue by indicating TAG ID within TA command in RAR when SSB from additionalTRP is chosen for preamble transmission by legacy UE

Observation4: the association between TAG and TRP (hence set of SSBs) is actually semi static.

Observation5: In case of CBRA based on 4-step RACH, network can’t differentiate legacy UE from new UE and RRC state purely based on received preamble

Observation6: In case of CBRA based on 4-step RACH, there is backward compatibility issue by the baseline solution if legacy UE chooses SSB from additionalTRP for preamble transmission in RRC_CONNECTED and RRC_IDLE state. But it could be corrected by network after receiving msg3.

Observation7: In case of CBRA based on 4-step RACH, there is backward compatibility issue by the baseline solution if legacy UE chooses SSB from additionalTRP for preamble transmission in RRC_IDLE state and it can’t corrected by network implementation.

Proposal 2: The association between PCI index including servingPCI and additionalPCI with TAG ID is configurable

Proposal 2a: ServingPCI is always associated with existing parameter tag-Id but not tag-Id2.

Proposal 2b: in SPCell servingPCI is always associated with TAG 0 to avoid backwards compatibility issue

Proposal 2c: the association between PCI index and TAG ID should be aligned with association between TCI state and TAG ID in same serving cell/BWP.

Proposal 3: When UE trigger CBRA for inter-cell case, UE always chooses PRACH associated with servingPCI and hence TAG 0.

Proposal 3a: PRACH resources associated with additionalPCI is only used for CFRA purpose

Proposal 4: agree following updated baseline:

for CBRA, we reuse the mechanism agreed for CFRA case for UE only in connected or inactive mode, i.e. use the RA RAR to indicate the TAG for both 2-step RACH and 4-step RACH.

Proposal 5: For CBRA based on 2-step RACH, TAG ID is signalled in MAC CE Absolute TA Command for UE in RRC_CONNECTED state.

Proposal 6: for CBRA based on both 2-step/4-step RACH, TAG 0 is used regardless of chosen SSB from UE perspective

Proposal 6a: for CBRA based on 4-step RACH, it is up to network’s implementation to correct TAG 1 to TAG 0 after it receive msg3 for both legacy/new UE in RRC_IDLE state and legacy UE in RRC_CONNECTED or RRC_INACTIVE state

Proposal 7: When TAT of only one TAG expires (when there are two) UE should not flush HARQ buffer for those HARQ process associated with TRP(TAG), whose TAT timer is running.

Proposal 8: When TAT of only one TAG expires (when there are two) RAN2 need discuss whether conditional flush HARQ buffer is to be defined and how for those HARQ process associated with TRP(TAG), whose TAT timer expires.

Proposal 9: When TAT of only one TAG expires (when there are two), it is not necessary to flush configured downlink assignment

Proposal 10: When TAT of only one TAG expires (when there are two), it is not necessary to release PUCCH, type1 CG PUSCH and SRS resource i.e. UE behaviour 3~6 are not conducted

Reference

R1-2308672 Consolidated_higher_layer_parameters_list_for_Rel18.xlsx

R1-2308630 Moderator summary on extension of unified TCI framework (Final)

Annex_1 4-step RACH

| case | SSB_s vs SSB_a | legacy vs new UE | RRC state | take UE as new UE in connected state | alternative B: always TAG0 |
| --- | --- | --- | --- | --- | --- |
| 1 | SSB_s | new UE | connected/inactive | network signal TAG0 in RAR(updated MAC CE) blindly UE start TAT of TAG0 | UE choose TAG0 network choose TAG0 |
| 2 | SSB_s | legacy UE | connected/inactive | network signal TAG0 in RAR(updated MAC CE) blindly. UE doesn't recognize this is updated MAC CE, but it still start TAT of TAG0 | UE choose TAG0 network choose TAG0 |
| 3 | SSB_s | new UE | idle | network signal TAG0 in RAR(updated MAC CE) blindly UE start TAT of TAG0 | UE choose TAG0 network choose TAG0 |
| 4 | SSB_s | legacy UE | idle | network signal TAG0 in RAR(updated MAC CE) blindly. UE doesn't recognize this is updated MAC CE,and UE start TAT of TAG0 | UE choose TAG0 network choose TAG0 |
| 5 | SSB_a | new UE | connected/inactive | network signal TAG1 in RAR(updated MAC CE) blindly UE start TAT of TAG1 | UE choose TAG0 network choose TAG0 UE consider all TAT expires and start TAT of TAG0 network may reconfigure TCI state and TAG association if it is not aligned with current association between TRP(SSB) and TAG |
| 6 | SSB_a | legacy UE | connected/inactive | network signal TAG1 in RAR(updated MAC CE) blindly UE doesn't recognize this is updated MAC CE, UE start TAT of TAG0 there is mismatch between UE and network, but network can fix it by changing to TAG0 after receiving msg3 | UE choose TAG0 network choose TAG0 |
| 7 | SSB_a | new UE | idle | network signal TAG1 in RAR(updated MAC CE) blindly UE start TAT of TAG0 since it is in IDLE state network correct TAG ID to be zero after receiving msg3 containing RRCConnectionRequest(1) message | UE choose TAG0 network choose TAG0 |
| 8 | SSB_a | legacy UE | idle | network signal TAG1 in RAR(updated MAC CE) blindly UE doesn't recognize this is updated MAC CE, UE think this is TAG0 there is mismatch between UE and network network correct TAG ID to be zero after receiving msg3 containing RRCConnectionRequest(1) message | UE choose TAG0 network choose TAG0 |

Annex_2 2-step RACH

| case | SSB_s vs SSB_a | legacy vs new UE | RRC state | Analysis |
| --- | --- | --- | --- | --- |
| 1 | SSB_s | new UE | connected | network signal TAG0 in absolute TA command (updated MAC CE) based on C_RNTI UE start TAT of TAG0 |
| 2 | SSB_a | new UE | connected | network signal TAG1 in absolute TA command (updated MAC CE) based on C_RNTI UE start TAT of TAG1 |
| 3 | SSB_s | legacy UE | connected | network signal TAG0 in absolute TA command (legacy MAC CE) based on C_RNTI UE start TAT of TAG0 |
| 4 | SSB_a | legacy UE | connected | network signal TAG0 in absolute TA command ((legacy MAC CE) UE this is TAG0 |
| 5 | SSB_s | new UE | inactive | network response RAR with TAG0 in msgB UE start TAT of TAG0 |
| 6 | SSB_a | new UE | inactive | network response RAR with TAG1 in msgB UE start TAT of TAG1 |
| 7 | SSB_s | legacy UE | inactive | network response RAR with TAG0 in msgB UE start TAT of TAG0 |
| 8 | SSB_a | legacy UE | inactive | network response RAR with TAG0 in msgB UE start TAT of TAG0 |
| 9 | SSB_s | new UE | idle | network response RAR with TAG0 in msgB UE start TAT of TAG0 |
| 10 | SSB_a | new UE | idle | network response RAR with TAG1 in msgB UE start TAT of TAG1 |
| 11 | SSB_s | legacy UE | idle | network response RAR with TAG0 in msgB UE start TAT of TAG0 |
| 12 | SSB_a | legacy UE | idle | network response RAR with TAG1 in msgB UE start TAT of TAG0 |