3GPP TSG-RAN WG2 Meeting #123bis		R2-2309721

Xiamen, China, 9th Oct. – 13th Oct. 2023

Source:	vivo

Title:	Contents of LTM MAC CE and other MAC related issue for LTM

Agenda Item:	7.4.2.2

Document for:	Discussion and Decision

- Introduction

In RAN2#123 meeting, we discuss the content of LTM cell switch command and LTM impacts on MAC specification including the early RACH collisions issues. In this contribution, we will further discuss some remaining issues on dynamic cell switch, mainly including the content of LTM cell switch command MAC CE and other MAC related issues for LTM.

- Discussion

  - Content of LTM MAC CE

In RAN2#123 meeting, the following agreements on content of LTM cell switch MAC CE were reached [1].

BWP ID is not in the LTM cell switch MAC CE, but only based on the RRC configuration. Scell activation state is not in the LTM cell switch MAC CE, but only based on the RRC configuration Will have CFRA resource related information field in LTM cell switch MAC CE (unless serious issues are found). Not introduce UL grant related information field in LTM cell switch MAC CE. Not introduce C-RNTI information field in LTM cell switch MAC CE. Not introduce LTM supervisor timer value field in LTM cell switch MAC CE.

Besides, there is no doubt that the TA value will be optionally introduced in LTM cell switch command. And based on our agreements in RAN2#119bis-e meeting [2] and agreements in RAN1 as follows, the candidate configuration index and beam indication should also be included in LTM cell switch command.

RAN2 agreement: RAN2 assumes the MAC CE for L1/2 mobility trigger contains at least a candidate configuration index. RAN1 agreement: The agreement on scenario 2 (Beam indication together with cell switch command) at RAN1#111 is further clarified as the following: - Beam indication for the target cell(s) is conveyed in the MAC CE used for LTM triggering for scenario 2

Observation 1:  The LTM triggering MAC CE contains at least the following:

- Candidate configuration index

- TA of the target cell (including exact TA value or TAG index of Source CG), optionally

- TCI state or beam indication, details depend on RAN1

- CFRA resource related information

2.1.1 CFRA Resource for cell switch related information in LTM cell switch command

In this section, we will further discuss the detailed CFRA resource related information included in LTM cell switch MAC CE. In our view, the motivation of including the CFRA resource related information in LTM cell switch MAC CE is similar to the early RACH resource, i.e., to save the RACH resource in candidate cells as much as possible. To achieve this purpose, a similar solution for early RACH as follows could be re-used for CFRA resources for LTM cell switch.

From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order).

In detail, for CFRA based LTM, shared preamble resources among multiple UEs is supported and the allocated CFRA resource for LTM cell switch is indicated by LTM cell switch MAC CE. In this solution, candidate cell won’t allocate the dedicated CFRA resource for a UE, instead, it will allocate a shared RACH resource pool to a source DU, and the source DU will select a resource from the pool and inform UE about the selected resource in the LTM cell switch MAC CE when triggering an LTM. The same CFRA related information as PDCCH order, i.e. the Random Access Preamble index, UL/SUL indicator, SS/PBCH index and PRACH Mask index should be included in LTM cell switch command.

Proposal 1: For CFRA based LTM, shared RACH resource among multiple UEs is supported and the CFRA resource related information in LTM cell switch MAC CE includes the following:

- Random access preamble index

- UL/SUL indicator

- SS/PBCH index

- PRACH Mask index.

Since the RACH resource(s) for cell switch is shared by UEs within the same source DU, the candidate cell couldn’t determine the UE identity based on the shared preambles received from UEs within the CFRA based LTM procedure. To resolve this issue, one solution is that UE sends the UE_ID using the UL grant allocated in RAR, e.g. the C-RNTI MAC CE to the target cell, in this way the target cell could identify the coming UE which used the shared RACH resource.

Observation 2: Candidate DU couldn’t determine which UE is handover to it in case the UEs use the shared preambles resource to perform CFRA based LTM cell switch.

Proposal 2: For CFRA based LTM, if shared RACH resource is used, UE sends C-RNTI MAC CE after performing the RACH procedure to indicate its identity to target cell.

2.1.2 Scenarios that TA of target cell is same as source cell

In RAN2#123 meeting, we agreed that UE performs RACH-less LTM when target cell with TA=0 or with the same TA value as the source. And for the scenario that TA =0, TA value is provided in the LTM cell switch MAC CE to indicate UE to perform RACH-less LTM.

The UE will do RACH-less when: - TA value is provided in the cell switch MAC CE (already agreed, TA=0 is assumed to be covered by this) - When the UE shall apply the same TA value as the source (already agreed) FFS how the UE knows this.

For the scenario that target cell has the same TA value as source cell, we should further study how to indicate the UE to perform RACH-less LTM. The same RACH-less scenario that target cell has same TA value as source is supported in LTE and it uses the current TAG to indicate the TA value of the target cell, detail is as follows[3]:

And for NR LTM, the following options could be considered:

- Option 1: The exact TA value is included in LTM cell switch MAC CE

- Option 2: Similar to LTE, source TAG indication (i.e. MCG-pTAG/SCG-pTAG/MCG-sTAG/SCG-sTAG) is included in LTM cell switch MAC CE

In our view, option 1 couldn’t work, since the exact TA value is maintained by UE.  UE may adjust the TA value to adapt the changed DL synchronization by itself without notifying the source gNB, hence the source gNB couldn’t know the UE’s TA value accurately. Hence, option 2 should be adopted.

Since LTM only involves one cell group, it is not likely for UE to apply the TA of source SCG-pTAG or SCG-sTAG to target PCell. Similarly, it is not like for UE to apply the TA of source MCG-pTAG or MCG-sTAG to target PSCell. Hence, option 2 can be improved as follows: Indication of pTAG or sTAG may be included in LTM cell switch MAC CE to indicate UE to apply the TA value of source TAG to target SpCell. Hence we propose:

Proposal 3: TAG information should be included in LTM cell switch MAC CE at least for RACH-less scenario that target cell has the same TA value as source cell.

- The TAG ID is used to indicate UE to apply the TA of indicated source TAG for the target SpCell.

- The source TAGs are related to the MAC entity which receives the LTM cell switch MAC CE.

2.1.3 Beam indication for target SCell

Since LTM supports CA scenario, and according to our agreement in RAN2#123 the target SCell could be activated based on the RRC configuration. One issue that whether the beam indication for target SCell should be included in LTM cell switch command should be discussed. For the target SCell in intra-band CA case, if it is configured for simultaneous TCI update with the target SpCell, the indicated TCI state in the cell switch command should be applied to the target SCell. In other cases, even the beam information could not be used for Scell, it is not necessary to indicate separate beam indication for the target SCell considering the larger measurement overhead for beam measurement in target SCell.

Proposal 4: Only the beam indication for target SpCell is included in the cell switch command, detail is up to RAN1.

  - Early TA acquisition

2.2.1 RACH collision for early RACH

In RAN2#123 meeting, RAN2 agreed that reuse the current PREAMBLE_POWER_RAMPING_COUNTER[1], and in RAN1 last meeting, RAN1 agreed that the counter should be reset when the candidate cell in the PDCCH order is different from the candidate cell indicated in the last PDCCH order as follows[4].

RAN2 agreement - R2 assumes For counting the power ramping step for early RACH, Reuse PREAMBLE_POWER_RAMPING_COUNTER. RAN1 Agreement For the power control of PDCCH-ordered CFRA in LTM, - When a UE receives a PDCCH order indicating a re-transmission of PRACH with the same associated SSB and same candidate cell as the previous PRACH, the counter is increased by 1. - In addition to case 1, power-ramping counter is reset in the following cases: - Case 2: The candidate cell indicated in the PDCCH order, indicating retransmission, is different from that indicated in the last PDCCH order. Note: the initial counter is 0 before receiving any PDCCH order.

Besides, we have discussed the MAC model for early RACH procedure and we have agreed that it is up to UE implementation to handle the RACH collisions when early RACH is involved.

- The early RACH procedure share a same MAC entity with the legacy RACH procedure. (e.g. no extra MAC entity is needed for early RACH) - It is up to UE implementation to handle the RACH initiation collisions where the early RACH is getting involved. No specification change can be foreseen.

Observation 3: It is up to UE implementation to decide to perform which RACH procedure when early RACH collides with the normal RACH in serving cell.

However, it is not specified how to handle the power-ramping counter in the following cases shown as figure 1: one UE performs a UE triggered CBRA procedure towards the serving cell in between two early TA acquisition procedures for initial transmission and re-transmission respectively towards the same candidate cell with the same associated SSB.

Figure 1: Case that early RACH collides with RACH to serving cells.

According to RAN1’s agreement, in the above case, the power-ramping counter should be increased based on the  power-ramping counter used by the legacy RACH(i.e. RACH towards the serving cell) when UE starts the 2nd early RACH procedure since the candidate cell indicated in this associated PDCCH order is same as the last PDCCH order.  However, it is not sensible considering the power-ramping counter has been disturbed by the legacy RACH procedure, hence the power-ramping counter should be reset. Therefore, we propose:

Proposal 5: Upon the initiation of an early RACH triggered by PDCCH order (no matter initial transmission or retransmission is indicated), power-ramping counter should be reset if the last performed RACH is toward current serving cell.

2.2.2 Impact of DRX on early TA acquisition

In RAN2#123 meeting, we agreed that the early RACH procedure is considered as complete when the preamble transmission is performed[1].

- Confirm that the RACH procedure toward a candidate cell is considered as complete once the preamble transmission is instructed to the lower layer.

However, the network will likely send the PDCCH order for preamble re-transmission upon it detects the early RACH for TA acquisition is not successful. Since UE only monitors PDCCH in DRX active time then the PDCCH order with re-transmission indication may be delayed for a long time if UE leaves the DRX active time when UE completes the last early RACH procedure. To solve the issue, the UE should keep monitoring PDCCH for a time duration even UE completes early RACH procedure for potential PDCCH order triggering re-transmission of early TA acquisition procedure.

Proposal 6: After the preamble transmission for early TA acquisition, UE should keep monitoring PDCCH for potential scheduling of preamble retransmission in a time window, i.e. UE should avoid entering DRX off during the time window.

2.2.3 LBT

In RAN2#123 meeting, we have the following FFS on LBT failure for early RACH procedure.

- FFS if UE transmits the preamble without the power ramping upon reception of PDCCH order with retransmission indication if preamble transmission encounter the LBT failure.

In the current MAC specification, the power ramping counter is not increased for a RACH attempt if the last preamble transmission is not performed due to LBT failure, since the failure of the last preamble transmission is not caused by low uplink power. We think the legacy mechanism can be reused to early TA acquisition.

Proposal 7: If the last early RACH fails due to the LBT failure, UE transmits the preamble without the power ramping upon reception of PDCCH order with retransmission indication.

  - Security concerns for LTM cell switch command

Currently, security is not applied to MAC CE. To address the security concern for LTM cell switch command, it was agreed that target configuration ID instead of target cell id should be included in the LTM cell switch MAC CE. Since the mapping relationship between target configuration ID and target cell id is configured to UE via RRC which is encrypted, the attacker can neither track a handover UE nor indicate a UE to handover to a certain target cell(e.g. fake cell).

However, another type of attack to LTM may occur. According to the current MAC running CR, upon the reception of LTM cell switch command, UE MAC will perform MAC reset. Based on this mechanism, an attacker may generate and send the LTM cell switch MAC CE to UEs randomly. Since the attacker couldn’t acquire the pre-configured RRC configurations of candidate cells, the target configuration ID in the fake LTM cell switch command MAC CE may be a unconfigured configuration ID. But the UE MAC entity doesn’t know which configuration IDs are configured and which are not configured. Hence, the MAC layer will always perform MAC reset even the cell switch command indicates a unconfigured configuration ID.

Observation 4: According to the current MAC running CR, upon the reception of LTM cell switch command, UE MAC will perform MAC reset, even the target configuration ID in the cell switch command is a unconfigured configuration ID.

In the MAC specification, handling of unknown, unforeseen and erroneous protocol data are specified. For example, if a received MAC PDU containing an LCID or eLCID value which is not configured, the MAC entity shall at least discard the received subPDU. In our understanding, similar handling can be adopted for the case unknown target configuration ID is included in the cell switch command MAC CE, i.e. discard/ignore the LTM cell switch command MAC CE.

Proposal 8: UE should ignore a received LTM cell switch command if it includes unknown information, e.g. unconfigured target configuration ID.

However, the UE MAC entity may not know whether the target configuration ID is a configured one if the UE RRC has not indicated MAC the all the configuration IDs that have been configured. Hence, it is straightforward to let RRC to confirm whether a received LTM cell switch command MAC CE is valid or not. Therefore, we think the MAC layer should inform RRC layer about the content of the received LTM cell switch command MAC CE. And MAC entity only perform MAC related action according to the indication from RRC layer after RRC layer has confirmed the LTM cell switch command MAC CE is valid.

With the above analysis, regarding the following ffs in the MAC running CR:

FFS on whether MAC or RRC triggers the “MAC reset”.

Editor’s note: FFS on the need of RRC layer indication on the time when the target configuration has been applied, so that MAC layer can start to process the later fields e.g. TA/TCI state.

We propose:

Proposal 9: UE MAC only performs LTM cell switch (including reset MAC, apply TCI state and TA value received, etc) upon the reception of indication from RRC, after the content in the received LTM cell switch command MAC CE has been confirmed as valid by RRC.

Regarding the following ffs in the MAC running CR:

FFS on the need of indication to RRC layer that those fields has been applied in MAC, so that RRC can submit the RRCReconfigurationComplete message for transmission.

We think there is no need for RRC to wait for MAC layer indication for RRCReconfigurationComplete message transmission, RRC layer could transmit the RRCReconfigurationComplete message to RLC layer after it indicates L2 to perform LTM cell switch related action（e.g. MAC reset, RLC re-establishment and PDCP data recovery if necessary）. And only when MAC layer applies the target configuration, it will indicate the new transmission opportunity for RRCReconfigurationComplete message according to current specification. There is no specification impact.

Proposal 10: RRC can submit the RRCReconfigurationComplete message for transmission to lower layer,  after it indicates L2 to perform LTM cell switch related action（e.g. MAC reset, RLC re-establishment and PDCP data recovery if necessary）, i.e. without the indication from MAC that those fields indicated in LTM cell switch command MAC CE have been applied in MAC.

  - MAC indication to RRC to trigger/skip RACH procedure

In the 38321 mobility CR [5], it is agreed that the MAC will indicate the RRC layer that LTM cell switch command MAC CE is received and the target configuration ID when it receives the LTM cell switch command MAC CE. While there is still some Editor’s note. In this section, we will discuss the remaining FFS in [5].

Regarding the following issues:

Editor’s note: FFS on the need of MAC indicating to RRC to trigger/skip the Random Access procedure for this LTM cell switch (since RA will not be triggered if the TA is valid when sending the RRCReconfigurationComplete message).

In our view, the RRCReconfigurationComplete message is generated by RRC layer, and RRC layer sends the data to PDCP layer for integrity protected and ciphered, then the PDCP layer will send the PDCP PDU to RLC layer, RLC generates RLC PDU for this message and it will send it to MAC layer when MAC layer informs that there is a transmission chance and the UL resource size which is acquired by RAR for RACH-based LTM or acquired by dynamic schedule or configured grant for RACH-less LTM.  However, no matter RACH-based LTM or RACH-less LTM, it has nothing with the RRC layer to generate the RRCReconfigurationComplete message according to the above RRCReconfigurationComplete message transmission procedure.

Proposal 11: It is transparent to RRC whether MAC triggers/skips the Random Access procedure for this LTM cell switch.

- Conclusion

In this contribution, we discuss some issues on dynamic cell switch including the content of LTM MAC CE and MAC issues. Based on the discussion, we have the following observations and proposals:

Observation 1:  The LTM triggering MAC CE contains at least the following:

- Candidate configuration index

- TA of the target cell (including exact TA value or TAG index of Source CG), optionally

- TCI state or beam indication, details depend on RAN1

- CFRA resource related information

Observation 2: Candidate DU couldn’t determine which UE is handover to it in case the UEs use the shared preambles resource to perform CFRA based LTM cell switch.

Observation 3: It is up to UE implementation to decide to perform which RACH procedure when early RACH collides with the normal RACH in serving cell.

Observation 4: According to the current MAC running CR, upon the reception of LTM cell switch command, UE MAC will perform MAC reset, even the target configuration ID in the cell switch command is a unconfigured configuration ID.

Content of LTM MAC CE

Proposal 1: For CFRA based LTM, shared RACH resource among multiple UEs is supported and the CFRA resource related information in LTM cell switch MAC CE includes the following:

- Random access preamble index

- UL/SUL indicator

- SS/PBCH index

- PRACH Mask index.

Proposal 2: For CFRA based LTM, if shared RACH resource is used, UE sends C-RNTI MAC CE after performing the RACH procedure to indicate its identity to target cell.

Proposal 3: TAG information should be included in LTM cell switch MAC CE at least for RACH-less scenario that target cell has the same TA value as source cell.

- The TAG ID is used to indicate UE to apply the TA of indicated source TAG for the target SpCell.

- The source TAGs are related to the MAC entity which receives the LTM cell switch MAC CE.

Proposal 4: Only the beam indication for target SpCell is included in the cell switch command, detail is up to RAN1.

Early TA acquisition

Proposal 5: Upon the initiation of an early RACH triggered by PDCCH order (no matter initial transmission or retransmission is indicated), power-ramping counter should be reset if the last performed RACH is toward current serving cell.

Proposal 6: After the preamble transmission for early TA acquisition, UE should keep monitoring PDCCH for potential scheduling of preamble retransmission in a time window, i.e. UE should avoid entering DRX off during the time window.

Proposal 7: If the last early RACH fails due to the LBT failure, UE transmits the preamble without the power ramping upon reception of PDCCH order with retransmission indication.

Security concerns for LTM cell switch command

Proposal 8: UE should ignore a received LTM cell switch command if it includes unknown information, e.g. unconfigured target configuration ID.

Proposal 9: UE MAC only performs LTM cell switch (including reset MAC, apply TCI state and TA value received, etc) upon the reception of indication from RRC, after the content in the received LTM cell switch command MAC CE has been confirmed as valid by RRC.

Proposal 10: RRC can submit the RRCReconfigurationComplete message for transmission to lower layer,  after it indicates L2 to perform LTM cell switch related action（e.g. MAC reset, RLC re-establishment and PDCP data recovery if necessary）, i.e. without the indication from MAC that those fields indicated in LTM cell switch command MAC CE have been applied in MAC.

MAC indication to RRC to trigger/skip RACH procedure

Proposal 11: It is transparent to RRC whether MAC triggers/skips the Random Access procedure for this LTM cell switch.

- References

- R2#123 Chair Notes (MediaTek).

- R2#119bis Chair Notes (MediaTek).

- 3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol Specification"

- Chairman’s Note, RAN1#114

- R2-2309281, 38.321 running CR for introduction of NR further mobility enhancements, Huawei, HiSilicon