3GPP TSG-RAN WG2 Meeting #123bis	R2- 2309940

Xiamen, China, 9th – 13th October, 2023

Agenda Item:	7.12.2.2

Source: 	Lenovo

Title: 	Mobility enhancement for UE in idle or inactive mode

Document for:	Discussion and decision

- Introduction

A new WID for mobile IAB-node [1] has been approved to be discussed in Rel-18, and following detail objectives are identified mainly focus on RAN3 and RAN2.

- Define Procedures for migration/topology adaptation to enable IAB-node mobility, including inter-donor migration of the entire mobile IAB-node (full migration) [RAN3, RAN2] - The mobile IAB-node can connect to a stationary (intermediate) IAB-node. Optimizations specific to the scenarios, where the mobile IAB-node connects to a stationary (intermediate) IAB-node, or where it directly connects to an IAB-donor-DU are de-prioritized. - The mobility of dual-connected IAB-nodes is down-prioritized. - Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2] Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility. - Mitigation of interference due to IAB-node mobility, including the avoidance of potential reference and control signal collisions (e.g. PCI, RACH). [RAN3, RAN2]

In this contribution, enhancements for mobile IAB-node migration are discussed, including aspects for mobility enhancement for UE in idle or inactive mode.

- Discussion

RAN2 123 [2]: Confirm the WA for inter-frequency cell reselection (scenarios: For a UE that is “on-board”, irrespective whether it is camped on the mobile IAB cell or a stationary cell, it can prioritize another frequency for which a mobile IAB cell is the best cell). No enhancement is needed for intra-frequency and equal-priority cell reselection. The procedure that UE searches and measure for mIAB cells on different frequencies is unspecified. RAN2 assumes that As assistance information, the NW can optionally provide inter-frequency mIAB list in SIB4, details FFS. It is left to UE implementation to determine whether the UE is physically on a moving vehicle and when it applies mobile IAB cell reselection prioritization for agreed scenarios. Chair Comment: The last agreement is different to and supersedes earlier agreements that states that RAN2 shall specify on-board criterion.

Based on the agreement from last meeting, no enhancements are introduced for intra-frequency and equal-priority cell reselection, while for the inter-frequency cell reselection, and assistance information in SIB4 is introduced for the UE to locate the mobile IAB cells on different frequencies. However, as for the detail information provided by the SIB4, there is still no consensus and may have two candidate options.

- Option 1: SIB4 only contains a list of frequencies on which mobile IAB cells are deployed.

- Option 2: SIB4 contains a list of frequencies and the PCI list of mobile IAB cells on these frequencies.

For Option 1, the UE thinks the frequencies provided by SIB4 as highest priority during cell reselection, and then the UE needs to perform cell reselection evaluation for the selected frequency and select the suitable cell to camp. However, the selected cell may be not a mobile IAB cell after reading the SIB1 of the selected cell, and then the UE needs to search for another suitable mobile IAB cell in the selected frequency.

While for Option 2, the PCI list of mobile IAB cells is also provided by the SIB4 accompanies with the frequency information. In this case, the UE only needs to perform cell reselection evaluation on these PCI lists and search for the suitable cell to camp, which is easier and timesaving than Option 1.

Proposal 1: SIB4 contains a list of frequencies and the PCI list of mobile IAB cells on these frequencies, to enable inter-frequency cell reselection of the served UEs.

Proposal 2: UE identifies the cell as a mobile IAB-cell based on frequency and PCI information in the SIB4 and selects the best cell in the PCI list.

And potential following corrections can be adopted for RRC Running CR.

MobileIAB-InterFreqCarrierFreqList-r18 ::= SEQUENCE (SIZE (1..maxFreq)) OF MobileIAB-InterFreqCarrierFreqInfo-r18 MobileIAB-InterFreqCarrierFreqInfo-r18 ::= SEQUENCE { targetFrequency-r18 ARFCN-ValueNR mobile-IABcellList-r18 PCI-List OPTIONAL -- Need R }

# Conclusion

This contribution aims to discuss mobility enhancements for UE in idle or inactive mode. And following proposals are concluded.

Proposal 1: SIB4 contains a list of frequencies and the PCI list of mobile IAB cells on these frequencies, to enable inter-frequency cell reselection of the served UEs.

Proposal 2: UE identifies the cell as a mobile IAB-cell based on frequency and PCI information in the SIB4 and selects the best cell in the PCI list.

# Reference

- RP-222671. Revised WID for NR_mobile_IAB.

- Chairman notes of RAN2 #123 meeting.