3GPP TSG-RAN WG2 Meeting #123bis	R2-2309959

Xiamen, China, October 9th – 13th, 2023                     Revision of R2-2308009

Agenda item:		7.6.4

Source:		Lenovo

Title:		Views on some remaining issues for discontinuous coverage in IoT NTN

Document for:		Discussion

- Introduction

The latest RAN2#123 meeting made the following agreements regarding discontinuous coverage in IoT NTN:

Agreements: RAN2 understands that UE may directly go to RRC_IDLE after RLF is triggered, if there is not enough time for the UE to finish the procedure of RRC re-establishment due to the discontinuous coverage (FFS whether this needs to be captured in the specs, e.g. a NOTE)

In this contribution we further discuss some remaining issues in discontinuous coverage based on previous discussions and agreements.

- Discussion

## State transition or RRC release due to discontinuous coverage

According to the agreement made in the latest RAN2#123 meeting, UE is at least allowed to enter IDLE when recovering from RLF is impossible before coverage interruption occurs. In our understanding this is a UE behavior that needs to be specified, e.g., by adding a note to indicate the possible handling at UE. Meanwhile, how to define or determine the condition of “there is not enough time for the UE to finish the procedure of RRC re-establishment” could be based on UE implementation e.g., when the remaining time before coverage interruption is less than n*UE-eNB/gNB-RTT.

Proposal 1: Add a note as "If RLF is triggered and RRC re-establishment cannot be finished in time before coverage interruption, UE may go to RRC_IDLE. The metric of determining whether RRC re-establishment cannot be finished in time is up to UE implementation".

Another case corresponding to discontinuous coverage is UE autonomous RRC Release, for which UE can release RRC connection autonomously before or upon coverage interruption even if no RLF is triggered. In our understanding such behavior is reasonable and beneficial in UE power saving, and it is better to have an aligned understanding on the autonomous RRC release (and its timing or reason) between network and UE. Therefore, UE reporting of some assistance information like its estimated coverage interruption duration is necessary.

Proposal 2: Al least to ensure aligned understanding on RRC release, UE reports its estimation for the duration of coverage interruption to network.

## CONNECTED measurement suspending

In Rel-17 to avoid unnecessary measurements and to reduce UE power consumption in IDLE due to coverage interruption in discontinuous coverage, the UE is not required to perform AS functions when out of coverage. However, unnecessary procedures including measurements may also occur before due to coverage interruption.

The criteria of triggering neighbour cell measurement in RRC_CONNECTED for NB-IoT is a combination of serving cell RSRP and its variance. That is, in CONNECTED mode if the measured serving cell RSRP is lower than a threshold and its variance compared to a reference RSRP is higher than a threshold, the UE performs neighbour cell measurements. This solution is designed for continuous coverage deployment, wherein the RSRP and its variation can well reflect whether an NB-IoT UE is possibly approaching RLF (i.e., due to leaving the serving cell), and the scenarios of discontinuous coverage have not been considered.

The discontinuous coverage may happen in space and time domain due to sparse constellation of satellites and satellite movement. It may lead to additional and unnecessary power consumption which is at least essential to NB-IoT devices. Unlike the temporary coverage interruptions in TN, in NTN the coverage interruptions are expected to last for a longer time duration and are predictable with the help of satellite assistance information. In such scenario there could be no neighbour cell available to be measured, but unnecessary neighbour cell measurement could be still triggered by NB-IoT UE.

Therefore, CONNECTED neighbour cell measurement triggering for NB-IoT shall be suspended or disabled according to UE estimated coverage interruption in discontinuous coverage.

Proposal 3: CONNECTED neighbour cell measurement triggering for NB-IoT can be suspended when no neighbour cell coverage is expected in discontinuous coverage.

Similarly, for eMTC, unnecessary measurement reporting for CONNECTED mobility could be still triggered when there is no available neighbour cell.

Proposal 4: Measurement reporting for eMTC can be suspended when no neighbour cell coverage is expected in discontinuous coverage.

- Conclusion

In this contribution we discuss some remaining issues in discontinuous coverage based on previous discussions and agreements. It is proposed that:

Proposal 1: Add a note as "If RLF is triggered and RRC re-establishment cannot be finished in time before coverage interruption, UE may go to RRC_IDLE. The metric of determining whether RRC re-establishment cannot be finished in time is up to UE implementation".

Proposal 2: Al least to ensure aligned understanding on RRC release, UE reports its estimation for the duration of coverage interruption to network.

Proposal 3: CONNECTED neighbour cell measurement triggering for NB-IoT can be suspended when no neighbour cell coverage is expected in discontinuous coverage.

Proposal 4: Measurement reporting for eMTC can be suspended when no neighbour cell coverage is expected in discontinuous coverage.