3GPP TSG-RAN WG2 Meeting #123bis	R2-2309965

Xiamen, China, October 9th-13th, 2023

Agenda item:		7.5.4.1

Source:		Lenovo

Title:		Discussion on BSR enhancements for XR

Document for:		Discussion and Decision

- Introduction

- RAN#98-e has approved the XR WID in RP-223502 including BSR and delay report enhancement for XR capacity:

- Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2); - Delay reporting of buffered data in uplink (RAN2);

In this contribution we further discuss UE feedback enhancements e.g., BSR format and trigger.

- Discussion

In RAN2#121bis-e and RAN2#122 meeting following agreements for BSR enhancements have been reached:

- Support of new BSR table(s) is based on NW configuration and UE capability. FFS whether the UE capability can apply to non-XR UEs. - As a working assumption, at most one BS index or BS value is reported by an LCG. This assumption can be revisited if new BSR table design cannot achieve a target level of quantization error. FFS what this target level should be. - Design/configuration for new BSR table(s) should include support for narrower ranges (i.e. finer granularity) than the legacy. Details can be discussed after an agreement on how UE obtains new BSR table(s) (e.g. pre-definition vs RRC configuration) is made. - At least linear distribution is used for generating code points in new BSR table(s). FFS whether exponential distribution can be considered too. FFS if piecewise linear distribution is supported. - New BSR table(s) can be used by any UEs that support such a capability. However, design of the new BSR table(s) should be based on XR-specific use cases and requirements. - Network can configure which BSR table(s) an LCG is eligible to use. UE determines which BSR table (i.e. legacy or something else) the LCG should use. FFS details of this determination (e.g. based on buffer size) and how network knows which BSR table each LCG uses. - As working assumption (depending on how we create the new BSR table(s) and the MAC CE format), If more than one new BSR table are introduced, all of them have the same size BS field. FFS on the exact size. - Deprioritize Option 2c (static + dynamic BSR tables) and Option 2d (reference table + scaling factor). - Have more discussions on Option 2a (static BSR tables) vs Option 2b (RRC configured BSR tables). In next meeting, companies should explain how BSR table(s) are created and how many tables would be needed, and how the MAC CE structure will look like. Should also explain what is the expected quantization error. - UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this. - When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this. - Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases). - We do not support additional piecewise linear BSR table in Rel-18. Can consider piecewise linearity when discussing how the BSR table values are defined.

BSR triggering and BSR MAC CE format enhancements

As mentioned above, IP packets belonging to the same PDU set are correlated to each other, and the end-user performance may depend on if all these IP packets belonging to a single PDU set are successfully delivered for some codecs. Providing the amount of data of the PDU set is beneficial for gNB to schedule the suitable UL grant size to schedule them as a whole. However, the legacy BSR procedure does not well support the PDU set level buffer size reporting.

Current BSR format indicates all the available data volume with LCHs in an LCG. If there’s more than one PDU set available for transmission for an LCH, NW cannot identify the buffer size for each PDU set in the BSR. RAN2 can discuss how to enhance the PDU set buffer size report format. For example, if an LCG only has one PDU set, the BSR includes the PDU set size for the PDU set available for transmission in Figure 1. In Figure 2, multiple PDU set buffer size can be included for one LCG. LCG index indicates buffer size of LCG is for which LCG, and PDU set bit indicates which PDU set has buffer status of an LCG. For each LCG, at most 8 buffer size fields.

Figure 1. An example of one PDU set in one LCG

Figure 2. An example of multiple PDU sets in one LCG

The enhanced BSR MAC CE format is only suitable for certain LCHs which are e.g. LCH carrying XR traffic. Therefore, NW should configure per LCH or LCG whether to use the enhanced BSR MAC CE format.

Proposal 1: RAN2 to discuss Long/short (truncated) BSR format enhancements to report multiple PDU set buffer sizes in one BSR MAC CE.

Proposal 2: NW configures UE to use a new enhanced BSR MAC CE format for specific logical channels group.

There are 4 types of BSR trigger events currently specified:

1) UL data with an LCH in an LCG with higher priority becomes available or UL data with an LCH in an LCG becomes available again after no ULs data in an LCG is available;

2) number of padding bits is larger enough to include an BSR;

3) retxBSR-Timer expires and data is available in an LCG ;

4) periodicBSR-Timer expires.

The first 3 trigger conditions are not designed for periodic data. Due to the UL arrival jitter caused by variable coding delay depending on the variable packet size, it is difficult to set a suitable value for the periodicBSR-Timer to adapt to the periodical arrival considering also the jitter. When SDUs of a PDU set belonging to a LCH arrive and there are still PDUs of a previous PDU set pending for transmission in the buffer, no legacy BSR for the newly arrived PDU set can be triggered. Therefore, the BSR trigger for the PDU set may be delayed.

Proposal 3: RAN2 to discuss whether there is a need to enhance BSR trigger events to avoid BSR trigger delay.

BSR report based on New Buffer Size Table

In the RAN2#122 meeting, RAN2 has agreed to introduce one new static BSR table with 8 bits BS field for Rel-18 XR to reduce the quantisation errors.

As a working assumption, UE always sends only one BSR. UE may use either the legacy BSR table or a new BSR table with smaller quantization error. UE chooses which BSR table to use based on its buffer size, e.g.

- Option 1. use a new BSR table if its buffer size is within the range of the new BSR table: otherwise use the legacy BSR table , or

- Option 2. use a new BSR table if the quantisation error is lower when the new BSR table is used; otherwise use the legacy BSR table

We consider that some buffer sizes in the new BSR table and in the old BSR table most probably overlap, resulting in a similar quantisation error when using the old BSR table for some data volume to report. If the old table can meet the similar quantisation error as the new table for a data volume to report, UE can use the old BSR table. Therefore, it seems option 2 has more flexible implementation for UE and achieves the same intention as option 1. And this principle can be specified in specification for the BS table determination procedure.

If UE selects a different BS table to report the BS index for different LCGs, the table information needs to be reported along with BS index.

There are two alternatives BSR format to indicate the new table:

- Alt 1. As legacy, using an assigned LCID in the MAC sub-header to indicate the new table.

old BSR format is reused but associated with new LCID, however two BSRs may be reported in a MAC PDU. For example, UE triggers BSRs for two LCGs, new table is selected for one LCG, and old table is selected for another LCG. In this case, UE will generate two separate BSR MAC CEs which corresponding to the two LCGs. UE will send two BSR MAC CEs to the network.

- Alt 2. Using additional bitmap field to indicate the BS table per LCG.

If the value of a field Ti corresponding to the LCG i is set to a specific value, the buffer size i corresponding to LCG i is present, and the value is in the new BS table. Based on alt 2, if UE triggers BSRs for two LCGs as above stated in alt1. UE can use one BSR MAC CE to include the BSRs for two LCGs.

Figure 3. An example of BSR format to indicate BS table information

Proposal 4: RAN2 to define a principle that UE use a new BSR table if the quantisation error is lower if the new BSR table is used or use the legacy BSR table instead.

Proposal 5: RAN2 to use a table information field corresponding to a LCG field to indicate the buffer size corresponding to the LCG is in the new BS table.

Proposal 6: UE reports the BSRs using the new BSR MAC CE with table information field if more than one LCG has data available for transmission and new BS table is selected for at least one LCG.

RAN2 can further evaluate whether the quantization errors based on the new BS table for Rel-18. UE selects a base buffer size and in addition indicates another relative buffer size if the remaining quantization error is larger than a threshold. The actual Buffer status of a PDU set is comprised of two parts, as shown in Figure 3. UE reports the two BSR components in the new BSR MAC CE so that the NW can derive the actual PDU set buffer size based on the two BS components for each LCG, i.e., the actual BS of PDU set = 1st BS + 2nd BS.

Based on the example of the legacy Table 6.1.3.1-2: Buffer size levels (in bytes) for 8-bit Buffer Size field, the quantization error can be reduced to about one hundred bytes. We observe this can offer sufficiently good performance based on legacy table.

| Original Quantization error | First Buffer Size | (BS Index) Second buffer size | Reduced error to (bytes) | Reduced error rate to (%) |
| --- | --- | --- | --- | --- |
| 2415 | 48676 | (88) ≤ 2533 | 118 | 4% |
| 4700 | 97221 | (98) ≤ 4751 | 51 | 1% |
| 530 | 58784 | (64) ≤ 560 | 30 | 5% |
| 899 | 117407 | (72) ≤ 926 | 27 | 3% |

Of course, if RAN2 intends to design a new fixed table to adapt the linear step to indicate this quantization error value can also work. One example, the range value is (10, 10000), if using BS index size is 8 bits, the linear step size = 40 bytes, if using BS index size is 6 bits, the linear step size = 156 bytes.

Proposal 7: RAN2 can evaluate the quantization errors based on the new BS table, if the remaining quantization error is larger than a threshold after UE selects a base buffer status, UE selects a relative BS index to indicate the quantization error.

Priority for BSR and DSR MAC CE

In case of both BSR and DSR is triggered, if the UL grant is not sufficient to accommodate both regular BSR MAC CE and DSR MAC CE, how to prioritize which MAC CE needs to be discussed. For example, whether to prioritize the DSR MAC CE over regular BSR MAC CE regardless of the logical channel priority with the DSR and BSR triggered. For example, whether to prioritize one MAC CE based on the priority of the LCG. In addition, as discussed above for the two alternatives BSR formats to indicate the new BS table, if Alt1 is adopted, there is also an issue to how to prioritize which BSR MAC CE if two BSR MAC CEs are triggered. Therefore, RAN2 needs to discuss the priority for the legacy BSR MAC CE and DSR MAC CE, BSR MAC CE to indicate the new BS table.

Proposal 8: RAN2 can discuss the priority for the DSR MAC CE and BSR MAC CE, e.g., prioritize the DSR MAC CE over regular BSR MAC CE.

- Conclusion

In this contribution, we discuss BSR enhancements for XR capacity. We have the following proposals:

Proposal 1: RAN2 to discuss Long/short (truncated) BSR format enhancements to report multiple PDU set buffer sizes in one BSR MAC CE.

Proposal 2: NW configures UE to use a new enhanced BSR MAC CE format for specific logical channels group.

Proposal 3: RAN2 to discuss whether there is a need to enhance BSR trigger events to avoid BSR trigger delay.

Proposal 4: RAN2 to define a principle that UE use a new BSR table if the quantisation error is lower if the new BSR table is used or use the legacy BSR table instead.

Proposal 5: RAN2 to use a table information field corresponding to a LCG field to indicate the buffer size corresponding to the LCG is in the new BS table.

Proposal 6: UE reports the BSRs using the new BSR MAC CE with table information field if more than one LCG has data available for transmission and new BS table is selected for at least one LCG.

Proposal 7: RAN2 can evaluate the quantization errors based on the new BS table, if the remaining quantization error is larger than a threshold after UE selects a base buffer status, UE selects a relative BS index to indicate the quantization error.

Proposal 8: RAN2 can discuss the priority for the DSR MAC CE and BSR MAC CE, e.g., prioritize the DSR MAC CE over regular BSR MAC CE.

- References

RAN2#121bis chairman note.

R2-2303203, Discussion on UE Feedback enhancements, Lenovo