3GPP TSG RAN WG2 #123-bis										     	 	        R2-2310037

Xiamen, China, October 9 –13, 2023

Agenda Item:	7.7.3

Source:	Thales

Title:	Discussion on network verified UE location in NR NTN

Document for:	Discussion and decision

# Introduction

As part of Release 18, a new work item is proposed to define enhancements for NG-RAN based Non-Terrestrial Networks [1]. Among other objective, this work item aims to:

Based on RAN1 conclusions of the study phase, RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view [RAN1, 2, 3, 4]. Note 1: Enhancements assume reuse of the RAT dependent positioning framework Note 2: The target accuracy for position verification purposes is as documented in clause « recommendations » of the 3GPP TR 38.882 (i.e. 10 km granularity) Note 3 : Multiple satellite in view by the UE may be considered if time allows Note 4 : The enhancements may be subject to relevant SA WGs (e.g. SA3/SA3-LI) feedbacks on the reliability of UE reports involved Note 5 : The enhancements should take into account the mirror-image ambiguity Note 6 : Network verified UE location is an optional UE feature

RAN1 part of the network verified UE location topic was agreed in Toulouse meeting [5].

In this contribution we discuss assistance data needed to multi-RTT positioning with single satellite method to support the network verified UE location in NTN.

# Assistance data for Multi-RTT positioning with single Satellite

## Information to be transferred from the UE to LMF

As specified in clause 8.10.2.2 of TS 38.305 [3], the information transferred from the UE to LMF are listed in Table 1.

Information

PCI, GCI, and PRS ID, ARFCN, PRS resource ID, PRS resource set ID for each measurement

DL-PRS-RSRP measurement

UE Rx-Tx time difference measurement

Time stamp of the measurement

Quality for each measurement

TA offset used by UE

UE Rx TEG IDs, UE Tx TEG IDs, and UE RxTx TEG IDs associated with UE Rx-Tx time difference measurements

LOS/NLOS information for UE measurements

DL-PRS-RSRPP measurement

The association of UE Tx TEG ID and SRS

Table 1: Information to be transferred from the UE to LMF [3]

Figure 1: UE Rx-Tx time difference

The legacy R17 definition of UE Rx-Tx time difference is adopted for NTN with an offset that is determined based on the following: - UE reports the actual index difference between subframe j and subframe i - The uplink subframe j is closest in time to the DL subframe #i received from the TP - The DL timing drift due to Doppler over the service link associated with the UE RX-TX time difference measurement period is reported

The absolute UE Rx-Tx time difference measurement corresponds in essence to the UE timing advance with regards to the uplink synchronization reference point.

RAN1 discussed the UE Rx-Tx time difference to use in NTN in RAN1#114 meeting. The above agreement has been achieved where the UE report the actual index difference between the closest UL subframe j and DL subframe i as illustrated on the Figure 1 (UE Rx-Tx Time Difference offset. To fully report the UE Rx-Tx time difference as existing in legacy, the UE also report the legacy UE Rx-Tx time difference. Finally, the UE need to report the DL timing drift due to service link Doppler for the measurement period.

Proposal 1:

For multi-RTT based positioning in NTN, the following measurements should be added to the existing measurement results transferred from UE to the LMF:

- The actual index difference between the closest UL subframe j and DL subframe i UE Rx-Tx time difference

  - NOTE: The uplink subframe j is closest in time to the DL subframe #i received from the TP

- DL timing drift over the service link associated with the UE Rx-Tx time difference measurement period

- NOTE: RAN1 to define value Range, granularity and bit allocation of these measurements

## Information to be transferred from the gNB to LMF

As specified in clause 8.10.2.3 of TS 38.305 [3] the assistance data that may be transferred from gNB to the LMF is listed in the following Table 2.

Information

PCI, GCI, ARFCN and TRP IDs of the TRPs served by the gNB

Timing information of TRPs served by the gNB

DL-PRS configuration of the TRPs served by the gNB

SSB information of the TRPs (the time/frequency occupancy of SSBs)

Spatial direction information of the DL-PRS Resources of the TRPs served by the gNB

Geographical coordinates information of the DL-PRS Resources of the TRPs served by the gNB

TRP type

On-demand DL-PRS information

TRP Tx TEG association information

Table 2: Information to be transferred from the gNB to LMF [3]

Figure 1: Time Alignment Reference Point

Because the reference point for TgNB-RX  and TgNB-TX used for the gNB Rx – Tx time difference measurement is located at the satellite payload which may be different from the reference point for uplink synchronization, it is necessary for the gNB to indicate to the LMF the value of ta-Common at the time of gNB Rx – Tx time difference measurement is performed.

Proposal 2:

For multi-RTT based positioning in NTN, the following assistance data may be transferred from gNB to the LMF:

- The value of ta-Common when the gNB Rx – Tx time difference measurement is performed

Upon the receipt of NRPPa Measurement Request from the LMF, the gNB performs UL SRS measurements at different (consecutive) vTRP. Although the LMF may has already assisted the gNB by indicating the vTRP positions or the time intervals at which the SRS should be measured based on vTRP determination using the GDOP, it is reasonable that the gNB includes within the measurement report the position of the satellite at UL-SRS measurements for Multi-RTT. The following RAN1 agreement confirms the accurate satellite position and velocity should be available at LMF for an accurate UE location determination.

Ephemeris information for UE location verification, including accurate satellite position and velocity at the time of measurement, should be available at LMF.

The gNB may need to provide the LMF with the ephemeris data in PVT state vector format or Keplerian format and associated epoch time.

Proposal 3:

For multi-RTT based positioning in NTN, the gNB includes the position and velocity of the satellite when UL-SRS measurements are performed to the assistance data that should be transferred from gNB to the LMF.

For RTT-based positioning in NTN with single satellite in view, based on GDOP calculation, the LMF determines the appropriate geometry and vTRP positions with the suitable time to perform Multi-RTT measurements. To this aim, in the response to NRPPa TRP INFORMATION REQUEST message the gNB indicate as part of TRP configuration information the serving satellite ID. Satellite ID is not used by the NG-RAN nodes but it may give satellite ephemeris data access (e.g. via OAM) to the LMF. It may trigger NRPPa Ephemeris INFORMATION REQUEST towards the gNB to retrieve ephemeris data in PVT state vector format or Keplerian format along with the associated epoch time.

Proposal 4:

For multi-RTT based positioning in NTN, the gNB may provide the LMF with assistance data including:

- Satellite ID

- The ephemeris data in PVT state vector format or Keplerian format along with the associated epoch time.

# Conclusion

In this contribution. we made the following observations and proposals:

Proposal 1:

For multi-RTT based positioning in NTN, the following measurements should be added to the existing measurement results transferred from UE to the LMF:

- The actual index difference between the closest UL subframe j and DL subframe i UE Rx-Tx time difference

  - NOTE: The uplink subframe j is closest in time to the DL subframe #i received from the TP

- DL timing drift over the service link associated with the UE Rx-Tx time difference measurement period

- NOTE: RAN1 to define value Range, granularity and bit allocation of these measurements

Proposal 2:

For multi-RTT based positioning in NTN, the following assistance data may be transferred from gNB to the LMF:

- The value of ta-Common when the gNB Rx – Tx time difference measurement is performed

Proposal 3:

For multi-RTT based positioning in NTN, the gNB includes the position and velocity of the satellite when UL-SRS measurements are performed to the assistance data that should be transferred from gNB to the LMF.

Proposal 4:

For multi-RTT based positioning in NTN, the gNB may provide the LMF with assistance data including:

- Satellite ID

- The ephemeris data in PVT state vector format or Keplerian format along with the associated epoch time.

# Appendix

## Main WGs’ study outcomes

During the study carried out in the RAN WGs on the potential solutions for the network verified UE location, the following was agreed (see details in clause 7.2):

| WG | Main outcomes |
| --- | --- |
| SA2 | AMF may trigger the verification in parallel with session set-up. LMF to provide a UE location with the verification status. Consistency between UE reported location (GNSS coordinates) and the network based assessment to within targeted accuracy (e.g. similar to terrestrial network macro cell size) => LMF implementation (may combine one or several 3GPP defined RAT dependent positioning methods) « If the AMF, based on the ULI, is not able to determine the UE's location with sufficient accuracy to make a final decision or if the received ULI is not sufficiently reliable, the AMF proceeds with the Mobility Management or Session Management procedure and may initiate UE location procedure after the Mobility Management or Session Management procedure is complete, as specified in clause 6.10.1 of TS 23.273 [87]” If verification unsuccessful, AMF may initiate to deregister the UE and provide a cause to NG-RAN S2-2211199: LS Response on Latency impact for NTN verified UE location: « SA2 requests that location verification be capable of being completed within a period of approximately 1 minute maximum and 30 seconds preferably” |
| SA1 | [S1-223539] Reply LS on Latency impact for NTN verified UE location: SA1 would like to provide the following answers to the questions: •Q1: Is there any constraint on the latency (from trigger to result) of the verification procedure? Answer from SA1: There are no related 3GPP SA1 requirements. •Q2: Can the verification procedure be run independently from the targeted services (e.g. in parallel to prevent any set-up delay)? If not, what is the estimate of set-up delay? Answer from SA1: Yes |
| RAN1 | The feasibility of Multi-RTT and DL-TDOA methods with single satellite have been assessed: For network verification of UE location in NR NTN with single satellite in view with multi-RTT positioning: - From RAN1 perspective, if the UE’s Rx-Tx time difference measurements report can be assumed to be trusted, multi-RTT positioning method using Rx-Tx time difference measurements can meet the accuracy requirement of less than 10km with 90% confidence, in case of: - At least LEO600 based deployment - Earth fixed cells - Earth moving cell at least if UE dwell time within the cell is enough to perform at least two RTT measurements - Note: the required over-the-air latency reported in evaluations ranged from less than 10s up to 180s For network verification of UE location in NR NTN with single satellite in view with DL-TDOA positioning: if the UE’s RSTD measurements report can be assumed to be trusted, DL-TDOA positioning method can meet the accuracy requirement of less than 10km with 90% confidence, in case of: - At least LEO600 based deployment - Earth fixed cells - Earth moving cell at least if UE dwell time within the cell is enough to perform at least two RSTD measurements Note 1: the above is based on evaluation results that didn’t account for UE Clock drift Note 2: the required over-the-air latency reported in evaluations ranged from less than 20s up to 180s Note 3: The requirements of Network verification of UE location may not be met if realistic assumption on UE clock drift is considered. |
| RAN2 | The network verification of the UE reported location may provide measurements/information associated to one or several 3GPP defined RAT dependent positioning methods (e.g. Multi RTT, DL/UL-TDOA, DL-AoA, NR E-CID, etc.). the verification of the consistency (within 5-10 km) between the actual reported UE location with the UE location(s) computed by the network is up to the 5GC re-use of the LCS framework of the LMF for the network verification of UE reported location information in NTN |
| RAN3 | It was agreed that: • The verification is performed in the CN. • If the reported UE location is not correct, the CN will take necessary action and Rel-17 behavior can be kept as baseline. The RAN3 agreements are fully consistent with the current LCS architecture. Once the UE is connected, the AMF triggers the location services request toward the LMF, which processes it and returns the result to the AMF. The AMF can then take the necessary action. RAN3 not impacted by UE reporting (in connected mode) |

## Main WGs’ study outcomes

The RAN1 agreements on network verified UE location achieved so far:

RAN2#121-bis-e, April 17 – 26th 2023, Online: Agreements: For RTT determination in NTN, discuss further the accuracy, and reporting details of combinations of the following UE and gNB receive-transmit time difference measurements: - Alt-1: UE Rx-Tx time difference based on Option 3 and gNB Rx-Tx time difference as defined in TS 38.215. - Note 1: The signaling method of UE Rx-Tx time difference definition option 1 is not precluded if Alt1 is adopted - Alt-2: UE Rx-Tx time difference based on Option 2 and gNB Rx-Tx time difference as defined in TS 38.215. - Note 2: The LMF will use the time stamp of the PRS and the time stamp of SRS to calculate the time difference between the transmission of PRS and the reception of SRS - Alt-3: UE Rx-Tx time difference based on Option 2 and gNB Rx-Tx time difference based on Option 4 - FFS: One or multiple SRS can be used in determining the arrival time - FFS: Additional enhancement including additional information to be reported, if justified Note 3: The impact of UE autonomous adjustment of TA (when applied) should be taken into account Note 4: The gNB Rx-Tx time difference option in the above alternatives may need updates accordingly based on the outcome of discussion on reference point for the gNB Rx – Tx time difference

RAN2#113, May 22 – 26th 2023, Incheon: Agreements: For network verified UE location in NTN, satellite ephemeris information should be available at the LMF. For network verified UE location in NTN common TA information should be reported at least from gNB to LMF. WA: In NTN, gNB receive-transmit time difference calculated at uplink time synchronization reference point is reported to the LMF.

RAN2#114, August 21 – 25th 2023, Toulouse: Agreements: The legacy R17 definition of UE Rx-Tx time difference is adopted for NTN with an offset that is determined based on the following: - UE reports the actual index difference between subframe j and subframe i - The uplink subframe j is closest in time to the DL subframe #i received from the TP - The DL timing drift due to Doppler over the service link associated with the UE RX-TX time difference measurement period is reported WA: In NTN, gNB receive-transmit time difference calculated at uplink time synchronization reference point is reported to the LMF. Note: This does not imply that the actual gNB receive-transmit time difference measurement is necessarily made at the uplink time synchronization reference point No need to support common TA information report from UE to LMF with consideration that common TA information report from gNB has been agreed supported. To resolve the mirror positions ambiguity for multi-RTT positioning, the following methods can be used without RAN1 specification impact from RAN1 perspective: - by gNB or LMF implementation - existing ECID method - UL-AoA Ephemeris information for UE location verification, including accurate satellite position and velocity at the time of measurement, should be available at LMF.

The RAN2 agreements on network verified UE location achieved so far:

RAN2#119-e agreements – August meeting: Agreements: The UE location information is considered verified if the reported GNSS position is consistent with the network based assessment to within 5-10 km (similar to terrestrial network macro cell size) (it is assumed that there is no RAN2 spec impact due to this) RAN2 should consider, as starting point, the re-use of the LCS framework of the LMF network for the network verification procedure. Send an LS to SA2 indicating RAN2 assumption on this The network verification of the UE reported location may combine one or several 3GPP defined RAT dependent positioning methods (e.g. Multi RTT, DL/UL-TDOA, DL-AoA, NR E-CID, etc.). LS sent to SA2 (R2-2208779): “RAN2 is considering the re-use of the LCS framework of the LMF for the network verification of UE reported location information in NTN. RAN2 would like to inform SA2 about this agreement and ask for any related feedback.”

RAN2#119-bis-e, October 10 – 19th 2022, e-meeting: Agreements: - RAN2 assumes that the network is able to compute possible UE locations independently from the GNSS location reported by UE - RAN2 assumes that the UE location verification procedure can be triggered by the CN and it is up to the CN to decide when to trigger the procedure - RAN2 should consider in priority the NGSO case with earth moving and earth fixed beams for the definition of the UE location verification procedure - Multi-connectivity involving multiple NTN NG-RAN nodes or NTN NG-RAN node and TN NG-RAN node is not part of the Rel-18 study on UE location verification - RAN2 assumes that the verification of the consistency (within 5-10 km) between the actual reported UE location with the UE location(s) computed by the network is up to the 5GC. (this doesn’t mean that RAN2 has nothing to do for this WI objective)

RAN2#120, November 14 – 18th 2022, Toulouse: Agreements: From RAN2 point of view, assuming the NW may allow the UEs access to services before verifying the UE reported location, the latency of the NW verification can be handled by the NW. Agreements: RAN2 agrees the re-use of the LCS framework of the LMF for the network verification of UE reported location information in NTN. RAN2 will work on the details of radio protocol aspects of the verification procedure based on the solution investigated by RAN1

RAN2#121, February 20 – 25th 2023, Athens: Agreements: For network verified UE location, the verification procedure can only be triggered by the CN, as already decided also by SA2. Network initiated verification procedure can be triggered by the NW when the UE is in RRC Connected. Agreements: RAN2 assumes that, as a baseline, legacy signalling procedure of location service can be reused for the purpose of network verified UE location in NTN. Agreements: RAN2 assumes that in general the mirror point issue can be resolved by properly configuring neighbor cell measurement to UE, for example, measurement of two neighbor cells in the opposite side of a satellite beam.

RAN2#121-bis-e, April 17 – 26th 2023, Online: Agreements: NTN UE doesn’t support positioning measurement and report in RRC INACTIVE

RAN2#122, May 22 – 26th 2023, Incheon: Agreements: In order to resolve the mirror point ambiguity issue, the network relies on the legacy signaling and procedure to configure NTN UE to measure and report neighbor cells or reference signals/beams. No spec changes to radio interface are needed from RAN2 perspective. Unclear if changes are needed to other interfaces, NRPPa protocol (RAN2 will no longer discuss this) Working Assumption: A Rel-18 UE capability is needed for indicating whether UE supports the feature of network verified UE location in NR NTN network

RAN2#123, August 21 – 25th 2023, Toulouse: Agreements: A Rel-18 UE capability is needed for indicating whether UE supports the feature of network verified UE location in NR NTN network (FFS whether this is an additional capability on top of FG 44-3) RAN2 assumption is that how the network handles the access to NR NTN cells for R18 UEs that do not support the new Rel-18 NR NTN “network verified UE location” capability is up to NW implementation, with no need for specs impact (RAN2 can still introduce needed changes to RAN2 specs for this, if requested by other groups)

RAN3 agreements on network verified UE location achieved so far:

RAN3#117-e agreements- August meeting: Agreements: The verification is performed in the CN. If the reported UE location is not correct, the CN will take necessary action and Release-17 behavior can be kept as baseline. FFS on new cause value. RAN3 wait for RAN1/2 progress on the specific position method to be used for verification.

RAN3#117-bis-e, October 10 – 18th 2022, e-meeting: Agreements: RAN3 is not affected by UE location reporting No additional RAN3 impact if UE location is not correct

RAN3#118, November 14 – 18th 2022, Toulouse: Documents endorsed R3-226102 UE Location Verification by the Network (Ericsson, CATT, Thales, Huawei, Samsung, ZTE, Nokia, Nokia Shanghai Bell) draftCR

# References

- Revised WID: NR NTN (Non-Terrestrial Networks) enhancements. RP-232669. Thales

- Status report for WI NR NTN (Non-Terrestrial Networks) enhancements. RP-230119, Thales

- 3GPP TS 38.305. « NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Release 15)  »

- R1-2210948. Discussion on network verified UE location in NR NTN. Thales. November 2022

- RAN1#114 Chair’s Notes, August 2023.