3GPP TSG-RAN WG2 Meeting #123bis	R2-2310050

Xiamen, China, October 9th – 13th, 2023

Agenda Item:    7.13.7

Source:         Xiaomi

Title:         	Discussion on the SON/MDT enhancement for NPN

Document for:	Discussion and Decision

Introduction

The agreements achieved in the RAN2#123 meeting on the SON/MDT enhancement for NPN scenario are as follows: [1]

Agreements:

- Include SNPN ID (list) in the logged MDT area configuration following RAN3 agreement to align with the future NPN evolution.

- No new UE variables will be introduced for PNI-NPNs.

- UE performs SNPN ID checking before transmitting the information for corresponding SON and MDT reports, upon the network requests for it.

- Assuming ESNPN is supported, include a list of SNPN IDs in the logged MDT report.

And there are still some open issues to be resolved as follows:

FFS1: Include UE CAG subscription information in the RLF/HOF report:

- CAG subscription statues indication;

- CAG-only indication.

FFS2: RAN2 to discuss whether and how to address the loss issue of logged MDT report when UE switches between SNPN and PN and then send RAN2 decision to RAN3.

- Option 1: Introducing new variables for SNPNs;

- Option 2: Storing only the collected MDT measurements report (UE deletes the MDT configuration as legacy);

- Option 3：No enhancement is needed;

FFS3:RAN2 to discuss:

- Whether and how to introduce information reporting for OOC analysis involving NPN network;

- Whether and which to introduce other SON/MDT enhancements for NPN in this Release.

In this contribution, we provide our considerations on the remaining issues for the SON/MDT enhancement in NPN scenario.

Discussion

FFS: Include UE CAG subscription information in the RLF/HOF report:

-	CAG subscription statues indication;

-	CAG-only indication.

For the UE operating in the CAG only access mode, the cell outside allowed CAG is unavailable for the UE. Reporting the CAG-only indication in the RLF/HOF is beneficial for the source cell to optimize HO-related parameters, assists operators in analysing the PNI-NPN performance, and helps identify potential mismatches between NPN network deployment and user requirements. With this indication, network operators can ensure seamless connectivity and better align their network infrastructure with the needs of their users.

During the discussion, some companies indicated that the CAG-only indication has already been known by the network through the NG interface. However, since the UE may enter idle mode upon RLF/HOF and the UE context may be discarded by NG-RAN, the network might not retain such information when the RLF/HOR report is retrieved. As such, compared to relying on network storage, a more straightforward and effective way is for the UE to log and report this information to the network, which can then be used for MRO.

Proposal 1: UE includes the CAG-only indication into the RLF/HOF report.

FFS: RAN2 to discuss whether and how to address the loss issue of logged MDT report when UE switches between SNPN and PN.

As we discussed before, this issue also exists for the existing logged MDT report in the case of mobility between PLMNs. For such issue in the legacy PLMN scenarios, there is no enhancement and it is up to the network implementation to retrieve the logged MDT report. As specified in TS 38.413 quoted below, when the AMF requests the NG-RAN to release the UE context in the deregistration procedure, the NAS cause can be set to "Deregister" so that the NG-RAN is aware that the UE will perform the deregistration procedure. At that time, if the network considers the report of the UE's currently registered network to be very important, it can retrieve the report before releasing the UE. Otherwise, there is no need for further enhancement to retain an unimportant report.

- 9.3.1.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the NGAP protocol.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| >>NAS Cause | M |  | ENUMERATED (Normal release, Authentication failure, Deregister, Unspecified, …, UE not in PLMN serving area) |  |
| … | … | … | … | … |

Observation 1: NG-RAN is aware of the deregistration procedure based on the UE context release cause "Deregister" and can decide whether to retrieve the existing network report to avoid the data loss at that time.

Based on the above analysis, we think RAN2 can confirm that there is no need to have further enhancements to address the loss issue of logged MDT when UE switches between SNPN and PN, as the data loss in such scenario can be handled and avoided through network implementation.

Proposal 2: RAN2 confirms to have no enhancements for the loss issue of logged MDT when UE switches between SNPN and PN.

FFS: RAN2 to discuss whether and how to introduce information reporting for OOC analysis involving NPN network.

Currently, if there is no suitable cell to camp on, UE will enter to any cell selection state, and provide the OOC indication to network (e.g. anyCellSelectionDetected, noSuitableCellFound) to identify the coverage hole. In NPN, a UE operating in SNPN access mode or with CAG-only indication can only camp on a cell that belongs to SNPN or a UE-allowed CAG ID during the mobility in IDLE/INACTIVE/CONNECTED mode. Otherwise, the UE will enter to the any cell selection state. In other words, a UE may experience an out-of-coverage situation due to the access mode. As such, in order to identify the root cause of the UE entering any cell selection state, the UE access mode can be reported along with the existing OOC indication, which is beneficial for network analysis of the coverage hole.

Observation 2: UE may experience the OOC situation due to the limited access mode.

Proposal 3: The UE access mode can be reported along with the existing OOC indication for the network analysis of the coverage hole.

Conclusions

During the discussion above, we have the following observations and proposals:

Proposal 1: UE includes the CAG-only indication into the RLF/HOF report.

Proposal 2: RAN2 confirms to have no enhancements for the loss issue of logged MDT when UE switches between SNPN and PN.

Proposal 3: The UE access mode can be reported along with the existing OOC indication for the network analysis of the coverage hole.

References

R2-23xxxx Report of 3GPP TSG RAN WG2 meeting #123, Toulouse, France.