3GPP TSG-RAN WG2 Meeting #123bis	R2-2310068

Xiamen, China, October 9-13, 2023

Agenda Item:	7.5.4.1

Source:	Honor

Title: 	Discussion on BSR and DSR enhancements for XR

Document for:	Discussion and decision

1	Introduction

In previous RAN2 meetings, the following agreements for BSR enhancements were achieved:

RAN2 #121bis[1]:

- Network can configure which BSR table(s) an LCG is eligible to use. UE determines which BSR table (i.e. legacy or something else) the LCG should use. FFS details of this determination (e.g. based on buffer size) and how network knows which BSR table each LCG uses.

- At least linear distribution is used for generating code points in new BSR table(s).  FFS whether exponential distribution can be considered too.  FFS if piecewise linear distribution is supported

RAN2 #122[2]:

- Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases)

For DSR, the following agreements were achieved in RAN2 #123[3]:

- When UE triggers reporting delay information for a LCG, and UE also reports the buffer status associated with the remaining time.

- Working assumption: Define a new separate MAC CE for DSR (remaining delay and associated data volume) reporting, e.g. DSR reporting is not coupled with BSR reporting. Detailed Definition of associated data volume is FFS.

- Support threshold based DSR reporting, e.g. DSR reporting is triggered when remaining delay of a PDU/PDU set is below a NW configured threshold. The threshold is configured per LCG. FFS whether configuring multiple thresholds for a LCG is supported. Definition of remaining time is FFS.

For BSR, the remaining issues will be discussed in this document:

- How the new BSR table looks like?

- How to report the new BSR?

For DSR, the left issues will be discussed:

- How the DSR looks like?

- How to report the DSR

- Discussion

2.1 BSR

2.1.1 How the new BSR table looks like?

The maximum and minimum values of the new BSR table

In TR 38.838-h00, the packet size of a video frame for single-stream is formulised as following:

| Parameter | unit | Baseline values for evaluation | Optional values for evaluation for single eye buffer |
| --- | --- | --- | --- |
| Mean: M | byte | R×1e6 / F / 8 | R×1e6 / F / 8 |
| STD | byte | 10.5% of M | 3 % of M |
| Max | byte | 150% of M | 109% of M |
| Min | byte | 50% of M | 91% of M |
| R: data rate of the flow in Mbps. F: frame generation rate of the flow in fps. Note that the mean and STD apply before truncation applies. Note that the value of R, F depend on application. |  |  |  |

According to above table, packet size of a video frame is determined by frame rate (F) and data rate (R). For frame rate, typical values of 15, 24, 25, 30, 50, 60, 90, and 120 fps can be considered as baseline.

Observation 1: Packet size of a video frame is determined by frame rate and data rate. Values 15, 24, 25, 30, 50, 60, 90, and 120 fps can be considered as baseline for frame rate.

For data rate, some reference values in bit rate have been collected in TR 26.928 [3], some values of different cases and scenarios are still FFS (e.g. for 3D mesh, the detail information of UL bitrate for XR in section 6.3).

The bitrates of 2D and 3D in TR 26.928 can be used for calculating UL packet size of a video frame.

- For 2D Compression Technologies

The following table is provided in TR 26.928:

| # Codec | # Coding performance # (Random-Access) |  | # Targeted bitrate # (Random Access) |
| --- | --- | --- | --- |
|  | Objective | Subjective |  |
| AVC |  |  | 4k: - Statmux: 20-25 Mbps - CBR: 35 - 50 Mbps 8k: - CBR: 80 - 100 Mbps - High quality: 100 - 150 Mbps [33][34][35] |
| HEVC | -40% vs AVC [33][34][35] | -60% vs AVC [33][34][35] | 4k: - Statmux: 10-13 Mbps - CBR: 18-25 Mbps 8k: - CBR: 40-56 Mbps - High quality: 80-90 Mbps [33][34][35] |

According to the table, the value range of bit rate is [10, 150] Mbps. Thus, according to formula 150% of (R×1e6 / F / 8), the maximum packet size can be calculated as 1875000 bytes by using maximum data rate R=150Mbps and the minimum frame rate F=15fps. The minimum packet size is 5208 bytes.

- For 3D and XR Visual Formats

In TR 26.928:

“According to clause 4.2.1, 1k by 1k per eye is a minimum for the signal in the viewport, and for stereoscopic rendering this results in basically a signal for 2k by 1k, typically at a frame rate of 50 or 60fps.With current codecs according to clause 4.5.1, the pure viewport data can be represented with around 4-10 Mbit/s. However, a viewport typically only covers around 100 degree horizontal and 60 degree vertical. Hence, to present a full omnidirectional presentation, about 20 times more data may be necessary, leading to 80 – 200 Mbit/s. Viewport-dependent coding and delivery, in particular tiling, can support to reduce the required bitrates.”

The bitrate can be up to 200 Mbit/s, the maximum packet size can be obtained as 2500000 bytes by using the above parameters.

The information obtained by the above analyses for all cases (e.g. 2D, 3D and XR visual) can be summarised as following:

Table 1

| Range of bit rate (Mbps) | Frame rate (fps) | Packet size (bytes) |
| --- | --- | --- |
| [10, 200] | 15, 24, 25, 30, 50, 60, 90, and 120 | [5208(10Mbps,120 fps, 50% of M), 2500000(200Mbps,15fps, 150% of M)] |

Due to there is no more detailed information for UL bit rate in TR 26.928, the maximum value of UL bit rate for XR is assumed as 200Mbps. Using the frame rate of 15 fps, the maximum value of packet size can be 2500000 bytes, this value can be used as the maximum value of new BSR tables.

Proposal 1: According to TR 26.928 and TR 26.926, 2500000 bytes (bitrate = 200Mbps, frame rate = 15 fps) can be used as baseline of the maximum value for the new BSR table.

2.2.2 How to report the new BSR?

The condition to use new table

Quantization error of new BSR table should be less than legacy one, thus, if buffer size of the LCG is within the range of the new table, the UE should select the new BSR table to report the buffer size of LCG which is configured to use new BSR table. Otherwise, the UE selects the legacy one.

Proposal 2: The new BSR table should be used for the LCG with buffer size within the range of the new table. Otherwise, the legacy BSR table is used.

How to differ the new index and legacy index of BSR tables

When reporting BSR, UE may report the index of new BSR table or the index of legacy BSR table. However, based on the current BSR format, it is unclear to the network whether the index is from new BSR table or legacy BSR table. It is simple that the different LCIDs are used to identify the MAC CE with new BSR table and the MAC CE with legacy BSR table without introducing new field(s) in BSR.

Proposal 3: Using new LCID to identify the BSR MAC CE with new BSR table.

The BSR format for the new BSR table

At the last meeting, RAN2 supported one static BSR table with 8 bits BS field. If only one LCG with available UL data can use the new BSR table, the short BSR format which contains 8 bits BS field should be reported. In current specification, the extended short BSR format for IAB contains 8 bits BS field, however the LCG field is occupied 8-bits to indicate 256 number of LCGs. Since 8 number of LCGs are supported in XR, 3-bits LCG field is enough. If more than one LCG with available UL data can use the new BSR table, the current long BSR format can be reused.

Proposal 4: A new short BSR format with 8-bits BS field and 3-bits LCG field is introduced.

## 2.2 DSR

2.2.1 How the DSR looks like?

In the online discussion, a new separate MAC CE is cleaner and simpler than extended BSR containing DSR. And triggered conditions for DSR and BSR can be decoupled by using a new MAC CE. Therefore, the working assumption “Define a new separate MAC CE for DSR (remaining delay and associated data volume) reporting, e.g. DSR reporting is not coupled with BSR reporting” can be confirmed.

Proposal 5: Confirm that “Define a new separate MAC CE for DSR (remaining delay and associated data volume) reporting, e.g. DSR reporting is not coupled with BSR reporting.”

There were still some remaining issues  left to this meeting:

- Number of DSR info(s) per LCG and one or multiple threshold(s) per LCG

- Remaining delay info and associated data volume format

Number of DSR info(s) per LCG and one or multiple threshold(s) per LCG

There are two options on the number of DSR info(s) per LCG:

•Option 1: Single (lowest remaining delay/most urgent) DSR info is reported per LCG

•Option 2: multiple DSR (remaining delay + BS) info is reported per LCG

o Details FFS (how many DSR info, which are reported)

Some companies raised that there may be different PDB requirements for LCH(s) belonging to a LCG, therefore, multiple DSR info is needed per LCG. In current specification, NW configures the LCH(s) with similar QoS/priority into one LCG, thus reporting buffer status per LCG can satisfy the QoS requirements of services. With the same assumption, the NW can configure LCH(s) with similar PDB requirements into one LCG that configured to report DSR, thus, the MAC entity can report one delay status (i.e remaining time) per LCG. As a LCG may contain one or more LCH(s) and each LCH has associated remaining time, the MAC entity can obtain the minimum remaining time from LCH(s) for the LCG. With the same assumption, one threshold per LCG is enough.

Proposal 6: Single delay status information with lowest remaining delay is reported per LCG.

Proposal 7: One threshold is supported for a LCG for triggering DSR.

Remaining delay info and associated data volume format

For the remaining delay information format, implicit and explicit method are provided.

For the implicit method, DSR contains buffer size and associated LCG index. The LCG index reported in DSR can inform the network that its remaining delay is below the triggering threshold.

For the explicit method, DSR contains remaining delay information, buffer size and associated LCG index whose remaining delay is less than DSR triggering threshold.

When the remaining delay of LCG is below its configured triggering threshold, the DSR is triggered for that LCG, however, UL resources may not be available for DSR reporting at this time. Therefore, there may be a delay between available UL resources determining and DSR triggering. If just report LCG index instead of remaining delay information, there may be a delay gap for the network to use this DSR to optimize scheduling.

Proposal 8: Remaining delay is reported as an index to indicate a range of remaining delay, a new DSR table containing DS index and corresponding remaining delay is introduced, similar to BS table.

Proposal 9: DSR contains the buffer size of UL data with remaining delay below the DSR triggering threshold.

2.2.2 How to report the DSR?

In current specification, when BSR is triggered, if UL-SCH resources are available for a BSR, the BSR MAC CE can be generated. If there is no available UL-SCH resources for BSR, the SR should be triggered for BSR when SR delay timer is not running (i.e. logicalChannelSR-DelayTimer is not configured or logicalChannelSR-DelayTimer expires). The SR delay timer is used to avoid too frequent SRs for LCH(s). Since the DSR may be triggered more frequently than BSR, the similar procedure can be reused for DSR.

Since the BSR is used to report regular data volume, the value of logicalChannelSR-DelayTimer is more than 20ms. The delay requirments of XR services are generally within the range of [10ms, 30ms], the current value of logicalChannelSR-DelayTimer is so large for DSR. Therefore, a SR delay timer with smaller value can be introduced to cover DSR requirements. Since the UL data may arrive concentratedly in a data burst, to avoid UL data in data burst arrived at different time triggers SR frequently, the typical duration of data burst can be considered.

Proposal 10: If no UL-SCH resources are available for the DSR, SR can be triggered when the SR delay timer (e.g. extended logicalChannelSR-DelayTimer with smaller values) is not running , similar to BSR.

In BSR procedure, the triggered BSR can be cancelled in the following conditions:

- UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC CE plus its subheader;

- when a MAC PDU is transmitted and this PDU includes a Long, Extended Long, Short, or Extended Short BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR prior to the MAC PDU assembly.

The same mechanism can be used for DSR. For simplest, the first condition can be reused instead of considering that urgent pending data available for transmission.

Proposal 11: All triggered DSR can be cancelled when:

- UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the DSR MAC CE plus its subheader;

- a MAC PDU is transmitted and this PDU includes latest triggered DSR MAC CE.

Different from BSR, the DSR contains remaining delay information, this information may have a valid period after DSR is triggered. If there is no available UL-SCH resources for a long time, the information in DSR may not be available before or after UL-SCH resources are determined. This can be illustrated in the following figures:

Figure 1 Associated UL data of LCG(s) that trigger DSR is discarded

For this case, triggered DSR is pended for a long time and discard-timer expires before UL resources are determined, the associated UL data of LCG(s) that trigger DSR is discarded, therefore, the triggered DSR is invalid as the data volume is empty.

Figure 2 The remaining time of LCG(s) is not useful for triggered DSR

For this case, since there is transmission delay for DSR and UL data, the remaining delay information may not be useful when it is below a threshold though the associated UL data is not dropped.

Based on above analysis, the triggered DSR can be invalid in the following scenarios:

- the associated UL data is discarded;

- the remaining delay information is no longer useful.

Proposal 12: 	Consider the triggered DSR to be invalid in the following cases:

- the associated UL data is discarded before UL-SCH resources are determined;

- the remaining delay information is no longer useful.

3	Conclusions

In this contribution, we discussed the issues related to BSR for XR-specific capacity improvement. Based on the  discussion, the following observation and proposals are concluded:

Observation 1: Packet size of a video frame is determined by frame rate and data rate. Values 15, 24, 25, 30, 50, 60, 90, and 120 fps can be considered as baseline for frame rate.

Proposal 1: According to TR 26.928 and TR 26.926, 2500000 bytes (bitrate = 200Mbps, frame rate = 15 fps) can be used as baseline of the maximum value for the new BSR table.

Proposal 2: The new BSR table should be used for the LCG with buffer size within the range of the new table. Otherwise, the legacy BSR table is used.

Proposal 3: Using new LCID to identify the BSR MAC CE with new BSR table.

Proposal 4: A new short BSR format with 8-bits BS field and 3-bits LCG field is introduced.

Proposal 5: Confirm that “Define a new separate MAC CE for DSR (remaining delay and associated data volume) reporting, e.g. DSR reporting is not coupled with BSR reporting.”

Proposal 6: Single delay status information with lowest remaining delay is reported per LCG.

Proposal 7: One threshold is supported for a LCG for triggering DSR.

Proposal 8: Remaining delay is reported as an index to indicate a range of remaining delay, a new DSR table containing DS index and corresponding remaining delay is introduced, similar to BS table.

Proposal 9: DSR contains the buffer size of UL data with remaining delay below the DSR triggering threshold.

Proposal 10: If no UL-SCH resources are available for the DSR, SR can be triggered when the SR delay timer (e.g. extended logicalChannelSR-DelayTimer with smaller values) is not running , similar to BSR.

Proposal 11: All triggered DSR can be cancelled when:

- UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the DSR MAC CE plus its subheader;

- a MAC PDU is transmitted and this PDU includes latest triggered DSR MAC CE.

Proposal 12: 	Consider the triggered DSR to be invalid in the following cases:

- the associated UL data is discarded before UL-SCH resources are determined;

- the remaining delay information is no longer useful.

4	References

- RAN2 #121bis-e meeting report.

- RAN2 #122 meeting report.

- RAN2 #123 meeting report.

- 3GPP TR 26.928 V18.0.0 (2023-03) Extended Reality (XR) in 5G