3GPP TSG-RAN WG2 #123bis	R2-2310128

# Xiamen, China, 9 – 13 Oct 2023

Title:	[Draft] LS reply to RAN4 LS R4-2314351

Response to:	R4-2314351

Release:	Rel-18

Work Item:	NR_SL_enh2

Source:	RAN2

To:	RAN4

Cc:	RAN1

Contact person:	Min Wang

min.w.wang@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN4 has discussed RRM requirements for following sidelink procedure:

- Procedure for initiation/cease of SLSS transmissions

Based on the discussion, RAN4 respectfully asks RAN2 provide feedback on the expected UE actions upon exceeding the maximum allowed CCA failures during the evaluation period .

RAN2 understands that RAN4 discussions concerns a scenario where a UE is synchronized to a selected SyncRef UE and the UE can itself become a new SyncRef UE by checking the RSRP of its selected SyncRef UE is below a preconfigured threshold. In this scenario, the UE measures PSBCH-RSRP of the SyncRef UE and compares with the preconfigured threshold syncTxThreshOoC in SidelinkPreconfigNR. Due to the selected SyncRef has been experiencing consistent LBT failures, the UE cannot obtain measurement results of a sufficient number of SLSS transmission occasions. The UE can therefore not able to derive an accuarate PSBCH-RSRP measurement result, which cannot be used to compared with syncTxThreshOoC.

For the three options listed in the RAN4 LS

  - Option 1: cease all SLSS transmissions,

  - Option 2: initiate SLSS transmissions

  - Option 3: UE keeps current SLSS transmission status.

In this scenario, RAN2 recommends that the UE initiates SLSS transmissions upon exceeding the maximum allowed CCA failures during the evaluation period, i.e., option 2.

Therefore, RAN2 would like to inform RAN4 of the below RAN2 agreement:

Adopt Option 2, i.e., the UE with a selected SyncRef UE shall initiate SLSS transmissions after reaching maximum unavailable S-SSB periods.

# 2	Actions

To RAN4:

ACTION: 	RAN2 respectfully asks RAN4 to consider the above RAN2 agreement in the future work.

# 3	Dates of next TSG-RAN WG2 meetings

TSG RAN WG2 Meeting #124	November 13 – November 17, 2023	Chicago, US

TSG RAN WG2 Meeting #125	February 26 – March 1, 2024	             Athens, GR