3GPP TSG-RAN WG2 Meeting #123-bis                                                             R2-2310189

Xiamen, China, October 9 – 13, 2023

Agenda item: 		7.12.1

Source: 		Qualcomm Incorporated

Title: 	Mobile IAB-nodes connecting at Rel-16/17 cells

Document for:	Discussion

# 1	Introduction

RAN2 achieved the following agreements related to the access of the mobile IAB-node to the RAN [1]:

R2#120 A mobile IAB node may camp on and connect to legacy Rel-16/Rel-17 IAB capable cell. R2 assumes "supporting mobile-IAB" indication is provided by Rel-18 Mobile IAB capable parent cell. R2#121bis-e The mobile IAB-MT to include a mobile-IAB indication in Msg. 5.

RAN3 achieved the following agreements related to the access of the mobile IAB-node to the RAN and CN [2]:

R3#119 NGAP Initial Context Setup Request, UE Context Modification Request and HO Request to include an IE with code points “mobile-IAB authorized”, “mobile-IAB not-authorized”. R3#119bis-e The IAB-donor-CU selects an AMF that supports mobile IAB-node based on the mobile IAB-node indication received via Msg5. R3#121 It is agreed that a Mobile-IAB indication is included in the NGAP Initial UE message. Agree to introduce a Mobile-IAB-authorization status indication in the Path Switch Request Ack for the mIAB-MT. Agree that the mIAB-DU’s CU to be informed about the mIAB-authorized status by mIAB-MT´s CU when IAB MT and IAB DU connect to different IAB donor CUs. In case the mIAB-DU’s CU obtains “non-authorized” indication for the mIAB-node, it performs an orderly F1 release.

TS 23.501 (v18.3) Section 5.35A includes the following related for Rel-16/17 IAB-node authorization:

### 5.35.2 5G System enhancements to support IAB … The following aspects are enhanced to support the IAB operation: - the Registration procedure as defined in clause 4.2.2.2 of TS 23.502 [3] is enhanced to indicate IAB-node's capability to the AMF; - The IAB-node provides an IAB-indication to the IAB-donor-CU when the RRC connection is established as defined in TS 38.331 [28]. When the IAB-indication is received, the IAB-donor-CU selects an AMF that supports IAB and includes the IAB-indication in the N2 INITIAL UE MESSAGE as defined in TS 38.413 [34] so that the AMF can perform IAB authorization. - the UE Subscription data as defined in clause 5.2.3 of TS 23.502 [3] is enhanced to include the authorization information for the IAB operation; - Authorization procedure during the UE Registration procedure is enhanced to perform verification of IAB subscription information; - If the IAB operation is not authorized, the AMF may reject the IAB-UE's registration or de-register the IAB-UE. Or the AMF may initiate UE Context setup/modification procedure by providing IAB authorized indication with the value set to "not authorized" to the NG-RAN, but the IAB-UE is still registered. - If the IAB operation is authorized, UE Context setup/modification procedure is enhanced to provide IAB authorized indication with the value set to "authorized" to NG-RAN. …

TS 23.501 (v18.3) Section 5.35A includes the following related to Rel-18 MBSR authorization:

### 5.35A.1 General … For a MBSR node to operate as a MBSR, it provides a mobile IAB-indication to the IAB-donor-CU when the RRC connection is established as defined in TS 38.331 [28]. When the mobile IAB-indication is received, the IAB-donor-CU selects an AMF that supports IAB-node with mobility and includes the mobile IAB-indication in the N2 INITIAL UE MESSAGE as defined in TS 38.413 [34] so that the AMF can perform mobile IAB authorization as described in clause 5.35A.4. If the MBSR node does not operate as a MBSR, e.g. due to the MBSR authorization indication from AMF, it does not provide the indication when establishing new RRC connection. … ### 5.35A.4 MBSR authorization For a MBSR, the subscription information stored in the HPLMN indicates whether it is authorized to operate as MBSR, and the corresponding location and time periods. … When the MBSR (IAB-UE) performs initial registration with the serving PLMN, it indicates the request to operate as a MBSR as described in clause 5.35A.1. The AMF authorizes the MBSR based on the subscription information, and provides MBSR authorized indication to the MBSR node over NAS and NG-RAN over NGAP as described in the registration procedure in TS 23.502 [3]. …

This paper discusses an inconsistency between RAN2’s initial agreement to allow a mIAB-node to connect to a Rel-16/17 legacy IAB-cell and the various agreements by RAN2, RAN3 and SA2 that followed later.

# 2	Discussion

RAN2 agreed that a mobile IAB node may camp on and connect to a legacy Rel-16/Rel-17 IAB capable cell. The intention of this agreement was to enable backhaul connectivity for a mobile IAB-node in case it moves to a Rel-16/17 RAN that does not support mobile IAB. This agreement is based on the assumption that mobile IAB represents an enhancement to Rel-16/17 IAB, so that a mobile IAB-node could still obtain support for Rel-16/17 at the Rel-16/17 cell.

This assumption no longer holds. The recent work on VMR by SA2 as well as efforts on mobile IAB by RAN2 and RAN3 have made the mobile IAB-node a different type of node than the Rel-16/17 IAB-node.

- According to TS 23.501, the mobile IAB-node follow a different authorization procedure than the Rel-16/17 IAB-node. This implies that a separate subscription is necessary for a mobile IAB-node vs. a Rel-16/17 IAB-node.

- According to RAN2/3 agreements, the mobile IAB-nodes uses a different indication in Msg.5 used for AMF selection and authorization than the Rel-16/17 IAB-node. This allows for selection of a different AMF mobile IAB-nodes vs. Rel-16/17 IAB-nodes.

- According to RAN3 agreements, the donor-CU sends a different indicator to the AMF for a mobile IAB-node than for a Rel-16/17 IAB-node, and it also receives a different type of authorization indication for a mobile IAB-node than a Rel-16/17 IAB-node.

- Finally, according to RAN2 agreements, parent-node selection for mobile IAB-nodes is based on a different cell broadcast indicator than used for Rel-16/17-IAB nodes.

Observation 1: Based on SA2, RAN3 and RAN2 agreements, the mobile IAB-node does not represent an enhanced Rel-16/17 IAB-node but a different type of node.

Consequently, the mobile IAB-node cannot connect to a Rel-16/17 cell since the corresponding Rel-16/17 donor does not understand the mobile IAB-node indication and authorization procedure.

Observation 2: A mobile IAB-node cannot connect to a Rel-16/17 cell since the corresponding Rel-16/17 donor does not understand the mobile-IAB-specific indications and the mobile-IAB-specific authorization procedure.

To retain some of the benefits envisioned by RAN2’s original agreement, one could allow the node to concurrently operate as both, a mobile IAB-node and a Rel-16/17 IAB-node. When the node integrates at a Rel-18 donor, it can be authorized for mobile IAB as well as for Rel-16/17 IAB. In case the node needs to be migrated to another donor that only supports Rel-16/17 IAB, mobile IAB operation needs to be suspended, but the node could still provide Rel-16/17 IAB service to UEs at this Rel-16/17 donor.

Obviously, the node cannot be migrated any further since this is not supported by Rel-16/17 IAB. The node can only be migrated back to the Rel-18 donor, where it came from. When it returns to this Rel-18 donor, it could resume mobile-IAB support.

Observation 3: Some limited benefits could be achieved by allowing a node to operate concurrently as a mobile IAB-node and a Rel-16/17 IAB-node.

Such dual-mode operation requires further scrutiny. Further, the mobile IAB WID does not allow mobile IAB-nodes to have child nodes or to operate in DC. In turn, the Rel-16/17 IAB-node does not support with node mobility. If a node concurrently operated as a mobile IAB-node and a Rel-16/17 IAB-node, it could combine the features supported for both types of nodes, i.e., enjoy node-mobility support while having child-nodes and being dual-connected. This is not in the spirit of the Rel-18 WID and should therefore be precluded.

Observation 4: A node that concurrently operates as mobile IAB-node and as Rel-16/17 IAB-node could enjoy node-mobility support while having child nodes and being dual-connected. This is not in the spirit of the Rel-18 mobile IAB WID.

Therefore, the mobile IAB-node should not be allowed to concurrently operate as a Rel-16/17 IAB-node. This also implies that the mobile IAB-node cannot connect to a Rel-16/17 cell.

Proposal 1: The mobile IAB-node should not concurrently operate as a Rel-16/17 IAB-node.

Proposal 2: The mobile IAB-node cannot connect to a Rel-16/17 cell (in reversal of prior RAN2 agreement).

Proposal 3: Agree to TP to Running CR to TS 38.300 in the Annex.

# Conclusion

This paper discussed an inconsistency between RAN2’s initial agreement to allow a mIAB-node to connect to a Rel-16/17 legacy IAB-cell and the various agreements by RAN2, RAN3 and SA2 that followed later. The following observations and proposals have been made:

Observation 1: Based on SA2, RAN3 and RAN2 agreements, the mobile IAB-node does not represent an enhanced Rel-16/17 IAB-node but a different type of node.

Observation 2: A mobile IAB-node cannot connect to a Rel-16/17 cell since the corresponding Rel-16/17 donor does not understand the mobile-IAB-specific indications and the mobile-IAB-specific authorization procedure.

Observation 3: Some limited benefits could be achieved by allowing a node to operate concurrently as a mobile IAB-node and a Rel-16/17 IAB-node.

Observation 4: A node that concurrently operates as mobile IAB-node and as Rel-16/17 IAB-node could enjoy node-mobility support while having child nodes and being dual-connected. This is not in the spirit of the Rel-18 mobile IAB WID.

Proposal 1: The mobile IAB-node should not concurrently operate as a Rel-16/17 IAB-node.

Proposal 2: The mobile IAB-node cannot connect to a Rel-16/17 cell (in reversal of prior RAN2 agreement).

Proposal 3: Agree to TP to Running CR to TS 38.300 in the Annex.

# References

- Chair Notes from TSG RAN WG2 Meetings during Rel-18

- Chair Notes from TSG RAN WG3 Meetings during Rel-18

# Annex

--- Start Changes ---

## 4.7	Integrated Access and Backhaul

>>>> Skip

### 4.7.X	Mobile IAB

Mobile IAB provides enhancements to the IAB framework. t A mobile IAB-node can conduct physical mobility across the RAN while providing backhaul connectivity to UEs. An IAB-node supporting mobile-IAB enhancements is referred to as a mobile IAB-node, which includes a mobile IAB-MT and a mobile IAB-DU. Mobile IAB supports the same functionality as IAB unless explicitly specified. The following enhancements/restrictions only apply to mobile IAB:

- The mobile IAB-node cannot have descendent nodes.

- The mobile IAB-node uses an enhanced network integration procedure as defined in TS 38.401 [4].

Editor’s NOTE: The exact terminology to be aligned with TS 38.401.

Editor’s NOTE: From RAN2 perspective, the network integration procedure for mobile IAB-node has to include the mobile-IAB-indication in Msg. 5. It may include other mobile-IAB-specific enhancements.

- The mobile IAB-MT can perform the mobile IAB-MT migration procedure as defined in TS 38.401 [4].

- The mobile IAB-MT can perform both, Xn and NG handover.

Editor’s NOTE: FFS if mobile IAB-MT migration can use NG handover.

- The mobile IAB-node can perform a DU migration procedure, where a new logical mobile IAB-DU is established on the mobile IAB-node and the old logical mobile IAB-DU is released. During this procedure, the UEs connected via the mobile IAB-node are handed over from the old logical mobile IAB-DU to the new logical mobile IAB-DU. The details of this procedure are defined in TS 38.401 [4].

Editor’s NOTE: FFS enhancements on RACH-less handover

Editor’s NOTE: FFS on enhancements related to BAP that need to be captured on ST2.

- Dualconnectivity is not supported for the mobile IAB-MT.

The mobile IAB-node cannot concurrently operate as an IAB-node. The mobile IAB-node can only connect to a RAN that supports mobile IAB.

--- End Changes ---