3GPP TSG-RAN WG2 Meeting #123bis                                                 R2-2310250

Xiamen, China, 9-13 October, 2023

Agenda Item:	7.8.3

Source: 	CMCC

Title: 	Discussion on flight path reporting

Document for:	Discussion, Decision

Introduction

In previous meetings, the following agreements on flight path reporting have been made:

RAN2 #119bis [1]:

Agreements:

- The time information reported as part of flight path plan is optional. UE includes time info, if configured by the network and available at the UE.  FFS on flight path details (waypoints and what is time information).

- Allow the flight path to be updated.  FFS on the details.

- FFS on reporting format and initial flight path reporting (i.e. what information to report and how) – next meeting

RAN2 #120 [2]:

Agreements:

- A waypoint is a planned location for the UE along the flight path and is described via the existing parameter type LocationCoordinates defined in TS 37.355.

- A timestamp provides the UTC time associated with estimated time of arrival to a waypoint as baseline.  FFS on granularity

- No requirements are placed on spatial distribution of waypoints

- A UE indicates whether flight plan information is available within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message.   Flight path reporting uses at the UE Information request/response procedure as baseline.

- UE indicates to the network a new flight path is available in the UE (whether it is initial or update). Then, reuse the normal request/response procedure of flight path report.

- UAI message can also be used to indicate the UE has flight path availability.

- FFS whether and what triggering conditions are specified for flight update.  FFS The maximum number of waypoints within flight path plan is left FFS.

RAN2 #121 [3]:

Agreements:

- The granularity of flightpath timestamp is 1s.

- Timestamp in flightpath is encoded using AbsoluteTimeInfo-r16 IE

RAN2 #121bis [4]:

Agreements:

- Flightpath update indication in UAI is configurable by the network

- Maximum number of waypoints is set to 20 same as in LTE and number of waypoints is configurable by network as in LTE

- Flightpath information should be forwarded from source gNB to target gNB during handover. Send LS to RAN3 to check for feasibility [LS to RAN3 over email 307]

- As a baseline, we can consider a simple network control mechanisms (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. FFS if new threshold or the kind of threshold(s)

- As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flight path indication.  FFS if further differentiation is needed if we decide to have delta signaling

RAN2 #122 [5]:

Agreements

- The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration.

RAN2 #123 [6]:

Agreements:

Flight path update/reporting

- No delta signaling for updated flight path will be supported

- Confirm that same indication is used for both initial and updated flightpath plan

- If any waypoint/timestamp has changed more than the configured threshold the UE triggers flightpath update indication.

- When both distance and time threshold are configured, the flightpath update can be triggered if either of those conditions are met.   No prohibit timer will be introduced.

In this contribution, we would like to provide the some considerations about the left issues and possible mobility enhancements for flight path reporting enhancements for NR UAV.

Discussion

    - 2.1 Content of flight path

Regarding the specific content of flight path, we have agreed to include waypoints list and timestamp encoded using AbsoluteTimeInfo-r16 IE. Further, the waypoint is agreed to be described via the existing parameter type LocationCoordinates defined in TS 37.355 [9]. And in LTE, the FlightPathInfoReport IE [7] is as following:

FlightPathInfoReport-r15 ::=		SEQUENCE {

flightPath-r15	SEQUENCE (SIZE (1..maxWayPoint-r15)) OF WayPointLocation-r15    OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

Then the WayPointLocation IE [7]is as follows:

WayPointLocation-r15 ::=			SEQUENCE {

wayPointLocation-r15					LocationInfo-r10,

timeStamp-r15							AbsoluteTimeInfo-r10 		OPTIONAL

}

However, there are some misalignments between LocationInfo-r10 [7] and LocationCoordinates defined in TS 37.355 [9].

LocationInfo information element

-- ASN1START

LocationInfo-r10 ::=	SEQUENCE {

locationCoordinates-r10					CHOICE {

ellipsoid-Point-r10						OCTET STRING,

ellipsoidPointWithAltitude-r10			OCTET STRING,

...,

ellipsoidPointWithUncertaintyCircle-r11					OCTET STRING,

ellipsoidPointWithUncertaintyEllipse-r11				OCTET STRING,

ellipsoidPointWithAltitudeAndUncertaintyEllipsoid-r11	OCTET STRING,

ellipsoidArc-r11										OCTET STRING,

polygon-r11												OCTET STRING

},

horizontalVelocity-r10					OCTET STRING				OPTIONAL,

gnss-TOD-msec-r10						OCTET STRING				OPTIONAL,

...,

[[	heightCoordinates-r15		CHOICE  {

verticalVelocity-r15				OCTET STRING,

verticalVelocityAndUncertainty-r15	OCTET STRING

}			OPTIONAL

]]

}

-- ASN1STOP

LocationCoordinates ::= CHOICE {

ellipsoidPoint								Ellipsoid-Point,

ellipsoidPointWithUncertaintyCircle			Ellipsoid-PointWithUncertaintyCircle,

ellipsoidPointWithUncertaintyEllipse		EllipsoidPointWithUncertaintyEllipse,

polygon										Polygon,

ellipsoidPointWithAltitude					EllipsoidPointWithAltitude,

ellipsoidPointWithAltitudeAndUncertaintyEllipsoid

EllipsoidPointWithAltitudeAndUncertaintyEllipsoid,

ellipsoidArc								EllipsoidArc,

...,

highAccuracyEllipsoidPointWithUncertaintyEllipse-v1510

HighAccuracyEllipsoidPointWithUncertaintyEllipse-r15,

highAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-v1510

HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15,

ha-EllipsoidPointWithScalableUncertaintyEllipse-v1680																				HA-EllipsoidPointWithScalableUncertaintyEllipse-r16,

ha-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-v1680

HA-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-r16

}

Similar to LTE, we also have LocationInfo IE for NR, that is LocationInfo-r16 [8] which directly reuses the locationCoordinate in the TS 37.355, therefore, for the location information in the flight path, we could consider to utilize the LocationInfo-r16.

Proposal 1: It is proposed that for the location information in the flight path, we could consider to utilize the LocationInfo-r16.

Then with respect to the open issue that which messages should be used to (re)configure the distance/time threshold for Flight path update. In LTE, the FlightPathInfoReportConfig IE is used to specify the flight path information report configuration, for NR, there would be a similar IE according to the discussion up to now. Therefore, the distance/time threshold for Flight path update could be included in the same IE (e.g. FlightPathInfoReportConfig).

Proposal 2: The distance/time threshold for Flight path update could be included in the IE used to specify the flight path information report configuration as in LTE (e.g. FlightPathInfoReportConfig).

    - 2.2 Flight path for mobility

For terrestrial UEs, the strongest cell is in general from the closest eNB, even though the shape is not very smooth due to the shadow fading. However, for a drone UE, it is possible to be served by a sidelobe of some neighbour eNB far away from the drone UE instead of the main lobe of the closest eNB. The coverage of a given cell is fragmented into several small parts but not a continuous coverage. Especially, when the height of the drones reaches 300m, the shape of the serving cell is like the ripple spreading far away. Therefore, the situation of radio condition is possibly degraded, results in more handover number and more HOF rates, although the UE will almost always have a LOS connection to the base station due to lack of obstructions at flying altitude.

Thence, observations from field trials on RSRP/RSRQ and other measurements

- The radio environment including the RSRP, RSRQ and RSSI characteristics of aerial UE in the air are different from terrestrial UEs at ground level.

- RSRQ in general decreases for airborne UEs with increase in altitude compared to terrestrial UEs.

- RSSI is in general higher for airborne UEs compared to terrestrial UEs and the average RSSI increase

However, in LTE, only flight reporting is approved under the agenda of “Mobility Enhancement”, no further enhancement had been agreed.

Furthermore, nowadays, UAVs can be used in a wide range of applications, with innovators constantly identifying new, beneficial applications for UAS – goods delivery, infrastructure inspection, search and rescue, agricultural monitoring. Correspondingly, the requirements of UAV communication are different in diverse applications, including coverage, data rate and latency. In order to avoid the negative impact on terrestrial UE, current LTE UAV mechanism is basically conservative with restricted QoS performance. Hence, further improvement in NR is needed to meet the requirements.

A straightforward way to improve the aerial coverage performance is to introduce additional antenna which directions are to the air. However, these demands upgrade or replace the existing hardware, such as deploying dedicated RAN for drones or Massive MIMO antenna to provide special beams pointing to aerial objects, which has to cost large expense for equipment and constructions.

Therefore, we could consider mobility enhancements based on for example flight path information or UAV height. And we have agreed to forward the flight path information from S-gNB to T-gNB for handover enhancements. Maybe we could consider further enhancements, such as that network could provide several more accurate CHO configurations corresponding to different waypoint or UAV height value, which is beneficial for resource overhead reduction and UE power consumption reduction.

Proposal 3: Suggest RAN2 introduce additional mobility enhancement(e.g. CHO) utilizing the flight path information or UAV height.

- 3 Conclusions

Based on the discussions mentioned above, in this contribution we provide some discussions on flight path reporting enhancements for NR UAV and have the following proposals:

Proposal 1: It is proposed that for the location information in the flight path, we could consider to utilize the LocationInfo-r16.

Proposal 2: The distance/time threshold for Flight path update could be included in the IE used to specify the flight path information report configuration as in LTE (e.g. FlightPathInfoReportConfig).

Proposal 3: Suggest RAN2 introduce additional mobility enhancement(e.g. CHO) utilizing the flight path information or UAV height.

- Reference

- R2_119bis-e_Notes_R18 UAV NES (Diana)_18-10-22_14-40

- R2-2213003_R2_120_Notes_R17IIoT_SDT_RAPartition_R18 UAV NES SDT_R15-16 UP (Diana)_11-18-22_final

- R2-2301903_R2_121_Notes_R18 UAV NES SDT_R15-17 UP_R17 SData (Diana)_03-03_final

- R2-2304203_R2_121bis-e_Notes_R18 UAV NES R15-17 UP_SDT (Diana)_04-26

- R2-2306543_R2_122_Notes_R18 UAV NES MT-SDT R17 UP_SDT (Diana)_05-26

- R2-2308963_R2_123_Notes_R18 UAV NES MT-SDT R17 UP_SDT (Diana)_08-25_final

- TS 36.331 V17.5.0. Evolved Universal Terrestrial Radio Access (E-UTRA);Radio Resource Control (RRC);Protocol specification (Release 17)

- TS 38.331 V17.5.0. NR; Radio Resource Control (RRC) protocol specification (Release 17)

- TS 37.355 LTE Positioning Protocol (LPP) (Release 17)

- R2-1713897 Simulation Results and Analysis of Mobility Issues for Drones