3GPP TSG-RAN WG2 Meeting #123bis	R2-2310333

Xiamen, China, 9 – 13 October 2023

Agenda item:	7.5.4.2

Source:	Apple, Huawei, HiSilicon, ITRI, Lenovo, NTT Docomo, Qualcomm

Title:	Views on Threshold Signaling for PSI-based Discarding Mechanism

WID/SID:	NR_XR_enh-Core

Document for:	Discussion / Decision

# Introduction

For Rel-18 XR, each PDU Set may be associated with a PDU Set Importance (PSI) level, which could be used for discarding when network is congested. In general, PSI indicates the relative importance of a PDU Set compared to other PDU Sets within a QoS flow, and the less important PDU Sets may be discarded (or at least more likely/easily to be discarded) when congestion is detected in RAN. SA4 has defined 16 levels for PSI.

During RAN2 #121, we have agreed UL PDU Sets importance should be introduced, and have aimed to introduce a PSI-based discarding mechanism to cope with congestion:

RAN2 #121 Agreements: 5. Introduce UL PDU Set Importance. How UE derives this will be handled in UE implementation. RAN2 thinks PSI can be useful for PDU set-based discard. RAN2 aims to introduce a mechanism to allow UE to handle discarding of packets with different PSI in case of congestion. FFS for other cases.

Furthermore, in RAN2 #122 we have confirmed that the network can detect the uplink congestion, and send a dedicated signaling to trigger the PSI-based discarding mechanism:

RAN2 #122 Agreements: Network indicates UE to apply PSI-based XR discard mechanism via dedicated signalling. FFS how/whether to minimize additional UL signalling after this indication. FFS if the NW indication is a one-shot or also subsequent packets

Some extensive discussions (both online and offline) on the PSI-based discarding mechanism have been carried out in RAN2#123, but no decision can be made between two main solutions on the table:

- Timer-based Approach: When PSI-based discarding mechanism is activated, the UE applies different discard timer values to important PDU Sets and non-important PDU Sets.

- Threshold-based Approach: Assuming the UE knows a PSI threshold (e.g. a PSI threshold indicated by the network), the UE directly discard the PDU Sets less important than the threshold.

In our understanding, in order to have differentiated handling for PDU Sets that are considered as “important” and “less important”, most companies agree the PSI levels should be classified into two groups, regardless of which of the two solutions above is adopted. Instead of discussing which solution should be agreed by RAN2, this paper aims to discuss the need for the network to indicate the PSI threshold to distinguish important and non-important PDU Sets.

# Discussion

## Meanings of 16 PSI Levels

The 16 PSI levels are defined by SA4, and therefore we should refer to SA4 agreements/specifications in order to understand the meaning of the 16 PSI levels. According to the latest version of TS 26.522 (V0.1.1) agreed by SA4 in Aug/2023 [1], some rules about how PSI should be determined for a PDU Set are included. The PSI selection may vary across different codecs such as H.264 or HEVC, but a common rule is that, the PSI value should be based on if this PDU Set is needed for the processing of other PDU Sets (i.e. inter-dependency) – this is especially crucial for XR:

- 4.4.2.6.2.5 PSI mapping based on PDU set dependencies Senders should consider that multiplexed RTP streams are treated as a single QoS flow and set the PSI field accordingly, i.e., the PSI field for one bitstream will affect the PDU sets in other multiplexed streams as well. In some cases, dependencies may exist across bitstreams even when they are not multiplexed, particularly for XR services. In case of such dependencies, it may not be enough to have PSI values based on codecs and media types alone. PSI values shall be set in this case based on the following, which are listed in an increasing order of importance, i.e., decreasing order of PSI values. - The PDU set is not necessary for the processing of any other PDU set. Such PDU sets can be discarded, if needed and should be assigned the highest PSI values 14-15. When multiplexing, discardable packets of all streams should be given value 15 to prevent unfair treatment. If interdependency is known, e.g., in stereo streams (left eye is more important than right eye), then the more important stream can be given the PSI value 14. - The PDU set is necessary for the processing of some PDU sets of the stream to which it belongs. Such PDU sets should be assigned a PSI value in the range 9-13 (inclusive). The lower end of the range should be used for IDR/IRAP pictures since they are more important for the bitstream. - The PDU set is necessary for the processing of all the other PDU sets of the stream to which it belongs. Such PDU sets should be assigned a PSI value in the range 6-8 (inclusive). - The PDU set is necessary for the processing of some PDU sets of the stream to which it belongs and also necessary for the processing of some PDU sets of some other streams to which it does not belong. Such PDU sets should be assigned a PSI value in the range 4-5 (inclusive). - The PDU set is necessary for the processing of all PDU sets of the stream to which it belongs and also of some other streams to which it does not belong. Such PDU sets should be assigned a PSI value in the range 2-3 (inclusive). - The PDU set is necessary for the processing of all PDU sets of all streams. Such PDU sets should be assigned the lowest PSI value 1.

Based on TS 26.522, it is very clear that only PDU Sets with PSI = 14 or 15 are mentioned to be “discardable”, because these PDU Sets do not have any dependency to other PDU Sets. Also, by default the discardable PDU Sets always have PSI = 15, the PSI level 14 is only used in some special cases where certain prioritization between streams is known. All other PSI levels (i.e. PSI = 0~13) have some dependency to other PDU Sets and the lower the PSI the higher impact of the discarded PDU set. That is, discarding a PDU Set with PSI level 0~13 is equivalent to making a group of multiple PDU Sets useless for the Applications – the PDU Sets in this group may include the PDU Sets that are already successfully delivered, or the PDU Sets that will arrive subsequently (even after the congestion is over). Hence, at RAN level, it is very difficult to assess the severity of such impact, and how long such impact will last.

Observation 1: According to TS 26.522, intentionally discard a PDU Set with dependency to other PDU Sets (i.e. PSI=0~13) can cause error propagation in Application that result in impacts to user experience. It is very difficult (if not impossible) for the RAN to assess the damage to user experience.

Since the user experience is a critical factor for the end users to consider using cellular technologies (instead of using other connectivity technologies such as WiFi) for XR services, we must make sure that the damage to the user experience is minimized when introducing the capacity improvement schemes. One could argue that, the application developers and/or XR content providers may take such RAN feature into account to minimize the impacts from the application end. However, since nowadays a very large portion of smart phone traffics are delivered over WiFi rather than cellular, it is quite unlikely that application developers would be willing to sacrifice their own implementation flexibility in order to make it more compatible to an optional 5G feature that may be used rarely (assuming congestion is a rare scenario). For the time being at least, there is no obvious incentives for the Application developers to do so.

Observation 2: To make XR successful in 5G, the end user experience is a critical factor. Otherwise the users can simply choose other connectivity technologies such as WiFi.

## RAN Knowledge about Uplink PSI

On the other hand, if we want the RAN to signal some PSI-threshold for discarding to cope with uplink congestion, a prerequisite is RAN’s knowledge about how PSIs are used by the UE Application. Without sufficient knowledge of PSI, the gNB is not able to properly set a sensible PSI threshold that can strike a balance between alleviation of congestion and the resultant user experience.

As mentioned in TS 26.522, PSI selection can by influenced by many different things such as type of application, details of codec, and inter-dependency among PDU Sets. In addition, different applications (e.g. video, audio, text/metadata, image) can have different ways of PDU Set marking (Although the PSI rule based on PDU Set inter-dependency described in 4.4.2.6.2.5 of TS 26.522 is common for all applications). Therefore, various types of implementation choices of applications that are executed by the UE can result in different PSI distributions, as well as different levels of impacts to application performance caused by packet discarding. We do not have any mechanism in the XR-awareness framework that can allow the gNB to get such detailed knowledge about applications for neither DL nor UL. Therefore, the gNB is unable to set the PSI threshold properly due to lack of awareness in terms of impacts to application layer performance. We must also note that, not all XR applications will be implemented based on PSI concept defined by SA4 (e.g. some Applications may not employ RTP header extensions to convey PSI information), and the gNB may not know whether PSI of PDU Sets from the application can be identified or not. If the gNB signals a PSI threshold for a traffic flow where no PSI can be identified, this feature is totally useless.

Observation 3: The PSI distribution and impacts of dropping PDU Sets with particular PSI can be very different across various types/implementations of applications, but the gNB does not have such detailed knowledge about applications to set the PSI threshold properly. The gNB may not know whether the PSI of PDU Sets on a traffic flow are identifiable.

A common argument for the network to indicate the PSI threshold is that, the gNB is able to cope with different levels of congestions. For example, when the congestion gets worse, the network can lower the threshold, and the UE would therefore discard more packets. However, as we have observed above, the network does not have sufficient knowledge about the impacts of packet discarding to application performance (and hence user experience). Hence, in the end the gNB can only select the threshold based on congestion severity, which may not make any sense if the threshold is not even within the range of available PSIs of an application. For example, if the gNB sets a threshold of PSI = 7 while the application only has PSI = 0~3, no packet will be discarded, and it is unhelpful for congestion at all. In another example, if the application does not use RTP header extensions, the UE cannot identify the PSI and hence the PSI threshold signaling may not trigger any action at the UE side to reduce congestion.

Besides, it is worth highlighting that, congestion is not a new problem at all. There already exist many other solutions in either specification or based on implementation for the RAN to deal with congestion, such as activating more serving cells, or increasing the MCS levels of PUSCH scheduling. Thus, the PSI-based discarding should be deemed as one of the supplementary tools to alleviate congestions just like many other existing mechanisms, instead of a feature that aims to mitigate congestion entirely. If RAN is able to deal with congestion in the previous releases, we do not see why the RAN cannot deal with congestion for Rel-18 XR even if there is no PSI-based mechanism. Therefore, this is not convincing why we need to introduce a feature that can put user experience at risk when there are already many other ways to cope with congestions. To handle different levels of congestions, the gNB may simply control the number of UL DRBs where the PSI-based discarding should be activated. If the congestion is really that bad, the gNB can even handover the UE to another cell.

Furthermore, many modern applications have their own mechanisms to deal with congestion (e.g. the applications may reduce the frame rate autonomously), so there is no need to solely rely on RAN operations to deal with congestion.

Observation 4: Congestion is not a new problem, and there already exist many other ways for the RAN to deal with congestion. It is not necessary to put XR user experience at risk when introducing the PSI-based discarding mechanism.

## On the Need of PSI-Threshold Signalling

Based on what we have observed above, we do not see any need for the gNB to signal the PSI threshold, because:

- Only PDU Sets with PSI = 14 or 15 are discardable (by default all discardable PDU Sets have PSI=15) without risks of propagation errors in Application, so it is undesirable for RAN to ask the UE to intentionally discard the PDU Sets with other PSI values.

- This is very difficult (if not impossible) for the RAN to assess the impact to user experience if PDU Sets with inter-dependency (i.e. PSI = 0~13) are discarded, which may result in unpredictable degradation of user experience.

- The gNB does not have sufficient knowledge about the UE applications (in terms of PSI distribution and/or whether PSI information is available) to set the PSI threshold in a more sensible way.

- It is not worthwhile to intentionally damage XR user experience, when there already exist many other ways for the RAN to cope with congestion.

This is applicable regardless of what PSI-based discarding mechanism we will agree in the end (i.e. whether the UE applies different discard timer values to non-important PDU Sets, or the UE directly discard the non-important PDU Sets).

Proposal 1: Regardless of which PSI-based discarding mechanism (applying a different discard timer, or directly discard) is to be adopted, there is no need for the network to signal any PSI-threshold.

One may ask, if the network does not signal any PSI threshold, how does a UE determine which PDU Sets should be handled differently when congestion is detected by the network? We think it is much more feasible to rely on interaction between UE AS and UE Application since they are hosted on the same device. With a suitable implementation, the UE is able to identify which PDU Sets that are more/less important for an application, based on its knowledge of the application that is being executed, the codec that is being used, and the information in the RTP header etc. Moreover, it is aligned with the previous agreement made in RAN2, wherein the identification of PDU Set importance in uplink is up to UE implementation.

Proposal 2: It is up to UE implementation to identify the UL PDU Sets that should be handled differently (e.g. to direct discard or to apply a different discard timer value) when the congestion-triggered discarding mechanism is activated by the gNB.

# Conclusions

In this contribution, we have discussed our views about signaling of PSI-threshold for PSI-based discarding mechanism. We have observed the following:

Observation 1: According to TS 26.522, intentionally discard a PDU Set with dependency to other PDU Sets (i.e. PSI=0~13) can cause error propagation in Application that result in impacts to user experience. It is very difficult (if not impossible) for the RAN to assess the damage to user experience.

Observation 2: To make XR successful in 5G, the end user experience is a critical factor. Otherwise the users can simply choose other connectivity technologies such as WiFi.

Observation 3: The PSI distribution and impacts of dropping PDU Sets with particular PSI can be very different across various types/implementations of applications, but the gNB does not have such detailed knowledge about applications to set the PSI threshold properly. The gNB may not know whether the PSI of PDU Sets on a traffic flow are identifiable.

Observation 4: Congestion is not a new problem, and there already exist many other ways for the RAN to deal with congestion. It is not necessary to put XR user experience at risk when introducing the PSI-based discarding mechanism.

With these observations, we propose:

Proposal 1: Regardless of which PSI-based discarding mechanism (applying a different discard timer, or directly discard) is to be adopted, there is no need for the network to signal any PSI-threshold.

Proposal 2: It is up to UE implementation to identify the UL PDU Sets that should be handled differently (e.g. to direct discard or to apply a different discard timer value) when the congestion-triggered discarding mechanism is activated by the gNB.

# References

- S4-231544, 3GPP TS 26.522, V0.1.1 (2023-08)