3GPP TSG-RAN WG2 Meeting #123bis	R2-2310337

Xiamen, China, 9 – 13 October 2023

Agenda item:	7.4.3

Source:	Apple

Title:	UE reporting of sk-counter for S-CPAC

WID/SID:	NR_Mob_enh2-Core

Document for:	Discussion / Decision

# Introduction

In RAN2-123 and the following post email discussion [1], RAN2 had a discussion on handling of the security key with respect to sk-counter at the UE and at the MN. While there was good progress, the issue on whether the UE should inform the MN about the selected sk-counter did not reach a conclusion.

In this short paper, we provide reasons for allowing such indication to convince RAN2 to go this path.

# Discussion

## Background

RAN2 has discussed the LS from SA3 on the aspect of UE being provided multiple sk-counter values (and the aspect of having these provided in groups in some signaling form) such that the UE would select the sk-counter in situations where the key change is warranted. The below table captures the summary of this from the discussion[1] (to be discussed in RAN2-123bis.

Proposal 1: Rel-18 Conditional-Reconfiguration Information element include - List of Group-ID and associated SK-counter values outside the candidate conditional configurations. - The Group-ID parameter is included within each candidate conditional configuration(CondConfigAddMod) marked for subsequent CPAC. Proposal 2: Common Understanding on SK-Counter maintenance: The List of Group-ID and SK-Counter-values configured by network is maintained at UE after S-CPAC executions and the SK-Counter-Values are also updated after CPAC execution depending on the usage of SK-counter. RAN2 to discuss further how the UE maintains the SK-counter list across cell changes.

Also, the same discussion [1] proposes that the UE does not need to report the selected sk-counter, and also proposes that RAN2 discuss how to handle the case where the UE selected sk-counter is a mis-match

Proposal 3 : - UE need not include the selected SK-counter value in the RRC Reconfiguration message as the selection of SK-counter for Inter-SN cell change follows the defined pattern according to SA3 solution. RAN2 to discuss the scenarios where the RRC-Reconfiguration message needs to include selected SK-counter value (i.e. mismatch of SK counter at UE and NW). In this case, RAN2 also discusses the NW behavior if the reported SK-counter is a mismatch.

## Views

We list below some scenarios/aspects where it would be useful to have the UE signal the selected sk-counter:

### SCG Failure handling

One of the open items as part of the S-CPAC feature to be addressed in RAN2-123bis is the below, from the feature rapporteur summary[2].

In LTM, it is already agreed that if there is a failure in the current LTM cell (or during the LTM cell switch), the UE can use one of the candidate LTM cells to continue the connection. Similar logic for S-CPC is to be discussed in RAN2-123bis where the UE might select a candidate SN in case the current SN has a SCG failure. In these cases it helps with the UE provides the selected sk-counter to allow the UE and MN to be in sync.

Observation 1: UE providing the selected sk-counter to MN helps with the use case where SCG failure at current SN allows the UE to select one of the candidate SNs (like UE selection of candidate LTM cells at link failure).

### Reconciliation at MN

The offline discussion summary[1] proposes that RAN2 discuss the handling in case the NW detects the mis-match.

Proposal 3 : - UE need not include the selected SK-counter value in the RRC Reconfiguration message as the selection of SK-counter for Inter-SN cell change follows the defined pattern according to SA3 solution. RAN2 to discuss the scenarios where the RRC-Reconfiguration message needs to include selected SK-counter value (i.e. mismatch of SK counter at UE and NW). In this case, RAN2 also discusses the NW behavior if the reported SK-counter is a mismatch.

In our view, the detection itself would be based on a SCG failure if the UE does not inform the MN about the sk-counter, which is very inefficient, esp when the UE is expected to perform multiple S-CPC without an explicit RRC reconfiguration message.

Also, if RAN2 needs to discuss on how to handle the mis-match, it would be best to just use the UE selected sk-counter and have MN inform SN about this: in effect, this result in a normal S-CPC without considering this as a failure.

In other words, MN can monitor the sk-counter provided by the UE at S-CPC and validate/cross-check this before MN informs the SN (and this can be done in a spec independent way as well).

Observation 2: Without UE informing the selected sk-counter, the detection of the mismatch at MN is with SCG failure which is inefficient.

Observation 3: MN can use the information from UE (of selected sk-counter) as a validation/sync of the sk-counter maintenance (and this can be done as NW implementation) and can also indicate the correct sk-counter to SN, without ever having to discuss the case of mis-match as specification procedures.

### Future enhancements and inter-operability

RAN will very likely has R19 mobility enhancements, and also independent of this, the NW deployment can include multiple nodes where some of the nodes have R18 (and possibly R19) implementations while other nodes might be using R17 CPC implementations.

This can result in UE (through mobility) being configured with R18 S-CPC and R17 CPC (and possibly R19 enhanced version of CPC). This can lead to a scenario where the UE selected SN node might be part of more than one configuration, in which case, it is always useful for the UE to explicitly inform the MN about whether sk-counter (and which value) is selected.

Observation 4: Having the UE provide the selected sk-counter (if it did apply) is always useful in mixed deployments (R17/R18) and is also future proof – provides platform for backward compatibility for future extensions.

Based on all of the above, we think it is better to have RAN2 agree to UE reporting the sk-counter at every S-CPC.

Proposal 1: At every S-CPC, the UE informs the MN in RRCReconfigurationComplete message the selected sk-counter.

# Conclusions

Observation 1: UE providing the selected sk-counter to MN helps with the use case where SCG failure at current SN allows the UE to select one of the candidate SNs (like UE selection of candidate LTM cells at link failure).

Observation 2: Without UE informing the selected sk-counter, the detection of the mismatch at MN is with SCG failure which is inefficient.

Observation 3: MN can use the information from UE (of selected sk-counter) as a validation/sync of the sk-counter maintenance (and this can be done as NW implementation) and can also indicate the correct sk-counter to SN, without ever having to discuss the case of mis-match as specification procedures.

Observation 4: Having the UE provide the selected sk-counter (if it did apply) is always useful in mixed deployments (R17/R18) and is also future proof – provides platform for backward compatibility for future extensions.

Proposal 1: At every S-CPC, the UE informs the MN in RRCReconfigurationComplete message the selected sk-counter.

# References

- R2-23xxxxx  Summary of [Post123][046][feMob] subsequent CPAC security (Nokia)

- R2-23xxxxx  Summary of Rel-18 Mobility WI: Pre-RAN2#123bis Small Group Discussion Major Open Issues 09.25.2023