3GPP TSG-RAN WG2 Meeting #R2-123-bis		R2-2310537

Xiamen, China, Oct 09-13, 2023

Agenda Item:	7.4.2.1

Source: 	Rakuten Symphony Inc.

Title:  	TA acquisition related open issues

Document for:	Discussion and agreement

# Introduction

In RAN2#121-bis-e and RAN2#119-bis-e meeting, the following agreements were made:

- #121-bis-e - From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order). - RRC RACH configuration for early TA acquisition (e.g., including whether RAR needs to be received) is specific per target cell and is signalled separately (separate IEs) from the candidate cell configuration (the part that need to be applied at cell switch). #119-bis-e - RAN2 assumes that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1 L2 mobility switch may be supported. RACH-less if the UE doesn’t need to acquire TA during the cell switch. RAN2 understands that the feasibility of RACH-less may depend on RAN1, and expect that RAN1 is working on this. - RAN2 assumes RACH resource for CFRA for L1 L2 dynamic switch may be provided in RRC configuration (or potentially by MAC CE FFS). - FFS if the MAC CE can indicate TCI state(s) (or other beam info) to activate for the target Cell(s), dep on RAN1 progress.

In this contribution, we analyse a couple of key factors that influence the mobility latency, discuss how they can be addressed.

# Discussion

Based on the above highlighted agreements/working assumptions, we consider the possibility of the UE performing TA acquisition of the candidate/target cell before execution of L1/L2 triggered mobility (LTM) to realize RACH-less HO and its implications on the network and UE.

## UL synchronization before LTM execution

Figure 1: A UE performing RACH (UL sync) procedure with a candidate/target gNB-DU to acquire TA, while connected to serving gNB-DU

NOTE: The message names are exemplary and signify functionality only.

A UE is expected to perform RACH to an indicated target cell whenever the timing advance (TA) of the UE in target cell is different from that of the source cell. The UE acquiring TA of the target cell any time before the execution of LTM has latency gains during LTM execution. Hence it was agreed to investigate solutions to perform UL sync after the cell switch decision has been made. A UE can have multiple LTM target cells at any point of time and all of them may not be eligible for cell switch simultaneously. So, it is sensible for the network to indicate to the UE, the candidate target cell(s) to which UL sync needs to be performed, regardless of the cell switch decision.

The agreements highlighted above indicate that the RACH configuration for TA acquisition before LTM HO is provided over RRC. But it is also clear that the TA acquisition is performed by the UE based on a PDCCH order by the gNB-DU. Since TA acquisition may have to be performed by the UE more than once, for eg: if the TA timer expires, two aspects need clarification.

- Whether the same RACH configuration provided to the UE while at the time of performing TA acquisition first time, can be used on subsequent occasions and also for a potential RACH-based LTM cell switch.

- How to distinguish between normal RACH i.e RACH performed for a cell switch and RACH performed for TA acquisition. This is essential as the candidate gNB-DU is expected behave differently in either cases.

Observation 1: There is a need to distinguish the RACH performed to acquire TA from the RACH performed for a cell switch.

Proposal 1: RAN2 discusses and clarifies whether the RACH configuration provided during first TA acquisition can be re-used for subsequent attempt(s) and distinguish the RACH performed to acquire TA from the RACH performed for a cell switch.

## Re-use of Timing Advances post LTM cell switch

A serving gNB-DU may ask a UE to acquire timing advances of more than one LTM candidate cell before performing LTM cell switch. In any case, the cell switch can happen to only one target cell. If the TA thus acquired at the old serving cell has to be re-acquired at the new serving cell, despite the TAT still active, it causes unnecessary overhead on both UE and the network. Hence, there has to be a way to carry forward and continue the use of the acquired timing advances of different candidate cells at the new serving gNB-DU, unless the TAT has expired. For example, the acquired timing advances and their corresponding candidate cell Ids can be either sent by the UE to the new serving gNB-DU post LTM cell switch or from old serving gNB-DU to the new serving gNB-DU via the gNB-CU.

Proposal 2: RAN2 discusses and agrees a mechanism to continue use of acquired timing advances of different candidate cells at the new serving gNB-DU, post LTM cell switch.

## Timing Advance acquisition from a candidate/target cell

The serving gNB-DU sends a PDCCH order to acquire UE’s TA at the candidate cell. The UE performs a RACH based on this and the candidate gNB-DU determines the UE’s best DL beam based on the beam used by the UE in ULto perform RACH. The candidate gNB-DU further provides the UE’s TA to the serving gNB-DU via the gNB-CU.

During this operation, even a slight miscalculation in determination of the best DL beam at the candidate gNB-DU or minor UE movement could render the TA invalid soon. This also entails that the TA acquisition will have to be repeated again, which is expensive both for the network and the UE.

Hence, we think it is better to provide UE’s TAs of best neighboring beams/beam-groups to the serving gNB-DU so that the serving gNB-DU could choose the appropriate beam for the UE to use while/after performing LTM cell switch.

This ensures that slight UE movements are taken care of and a TA acquisition doesn’t have to be repeated.

Proposal 3: RAN2 discusses and agrees a mechanism to provide TAs of best neighboring beams/beam-groups of the UE to the serving gNB-DU along with the TA of the best beam.

# Conclusion

We have the following observations:

Observation 1: For LTM, the handover latency can be reduced, if network (gNB-DU) decides that the UE may perform RACH (UL sync) to a target cell in advance of the LTM SCC and indicates using a PDCCH order, the target cell(s) to which the RACH is to be performed.

Observation 2: DL data transmission at the serving cell is impacted if the UE performs UL sync with a target cell, while still being connected to serving cell.

Observation 3: Indexing the candidate LTM target cells for each individual UE and using the target cell Index rather than the cell identity during LTM execution addresses security concerns.

We have the following proposals:

Proposal 1: RAN2 discusses and clarifies whether the RACH configuration provided during first TA acquisition can be re-used for subsequent attempt(s) and distinguish the RACH performed to acquire TA from the RACH performed for a cell switch.

Proposal 2: RAN2 discusses and agrees a mechanism to continue use of acquired timing advances of different candidate cells at the new serving gNB-DU, post LTM cell switch.

Proposal 3: RAN2 discusses and agrees a mechanism to provide TAs of best neighboring beams/beam-groups of the UE to the serving gNB-DU along with the TA of the best beam.

# Reference

- RP-222332, “Revised WID on Further NR mobility enhancements,” MediaTek