3GPP TSG-RAN WG2 Meeting #123bis	R2-2310549

xiamen, China, October 9-13, 2023

Agenda item:	7.11.2.1

Source:	Sharp

Title:	Coexistence of SDT and Multicast reception in RRC_INACTIVE

WID/SID:	NR_MBS_enh-Core

Document for:	Discussion and Decision

1	Introduction

In current running CR[1], whether SDT and MBS multicast reception in RRC_INACTIVE can be configured together is still FFS:

Editor’s Note: FFS whether SDT and MBS multicast reception in RRC_INACTIVE can be configured together. And if yes, whether UE configured for MBS multicast reception in RRC_INACTIVE monitors group paging during SDT.

In this contribution, we discussed the FFS and proposed that SDT and MBS multicast reception in RRC_INACTIVE can be configured together.

2 Discussion

Firstly, on one hand, the use cases identified that could benefit from broadcast/multicast include public safety and mission critical, V2X applications, IPTV, live video, software delivery over wireless and IoT applications, etc[2]:

To enable resource-efficient delivery of multicast/broadcast services, 3GPP has developed NR broadcast/multicast in Rel-17 according to the WID in RP-201038, aiming to enable general MBS services over 5GS. The use cases identified that could benefit from this feature include public safety and mission critical, V2X applications, IPTV, live video, software delivery over wireless and IoT applications, etc. Two delivery modes have been agreed for Rel-17 MBS with delivery mode 1 (only for multicast) capable of addressing higher QoS services and delivery mode 2 (only for broadcast) focusing on lower QoS services. Given that Rel-17 MBS already provide the basic function to support MBS services, the general main goal for Rel-18 should be to enable better deployment of MBS, such as improvement of resource efficiency and capacity based on Rel-17 MBS.

On the other hand, the use cases identified that could benefit from SDT include [3]:

Specific examples of small and infrequent data traffic include the following use cases:

- Smartphone applications:

  - Traffic from Instant Messaging services (whatsapp, QQ, wechat  etc)

  - Heart-beat/keep-alive traffic from IM/email clients and other apps

  - Push notifications from various applications

- Non-smartphone applications:

  - Traffic from wearables (periodic positioning information etc)

  - sensors (Industrial Wireless Sensor Networks transmitting temperature, pressure readings periodically or in an event triggered manner etc)

  - smart meters and smart meter networks sending periodic meter readings

According to the WIDs, multicast and SDT can be applied to the same applications, for example, IoT applications.

In addition, in Rel-17, the UE enabled of multicast reception in RRC_CONNECTED is allowed to be configured with SDT. Support of multicast in RRC_INACTIVE is Rel-18 aims to resolve the system congestion and to achieve power efficiency of the UE which cannot be a justification of not supporting configure SDT and Multicast in RRC_INACTIVE together. So, it should be allowed to configure SDT and Multicast in RRC_INACTIVE together.

Proposal 1 SDT and Multicast in RRC_INACTIVE can be configured together.

Secondly, UE should monitor group paging when receiving multicast in RRC_INACTIVE for unicast paging or notification of activation of jointed multicast sessions. In Rel-17, considering that multicast can only be received in RRC_CONNECTED, if there is any multicast session is activated during SDT, UE will be switched to RRC_CONNECTED by the network. So, during SDT, Rel-17 UE can avoid monitor group paging for notification of multicast session activation. But in Rel-18, without monitoring the group paging, it should be ensured by network to switch UE to RRC_CONNECTED when any multicast session is activated even if the multicast session can be received by UE in RRC_INACTIVE. So, to avoid network congestion UE should monitor group paging during SDT to avoid missing notification of multicast session activation and avoid UE entering RRC_CONNECTED for multicast reception.

Proposal 2: UE configured with MBS multicast reception in RRC_INACTIVE should monitor group paging during SDT.

3 Summary

In this section, we summarized our proposals and observation as showed below:

Proposal 1 SDT and Multicast in RRC_INACTIVE can be configured together.

Proposal 2: UE configured with MBS multicast reception in RRC_INACTIVE monitors group paging during SDT.

4 References

[1] [Post123][603][eMBS] RRC running CR update (Huawei)

[2] RP-213568 New WID: Enhancements of NR Multicast and Broadcast Services

[3] RP-201305 Work Item on NR small data transmissions in INACTIVE state