3GPP TSG-RAN WG2 Meeting #123bis										   R2-2310580

Xiamen, China, October 9th – 13th, 2023

Source: 	Xiaomi

Title:  	RACH-less LTM and early TA

Agenda Item:	7.4.2.3

Document for:	Discussion and Decision

# Introduction

According to the recent RAN1/2/3 agreements as given in the Annex, the RACH-less solution is targeting at the early TA acquisition for the candidate cell before the cell switch. In this contribution, we provide our understandings on the remaining RAN2 issues regarding the RAN2 specification impacts by consolidating the RACH-less procedures in RAN1/2/3.

# Discussion

## MAC modelling for early-TA RACH procedure

38.321： When the UE is configured with SCG, two MAC entities are configured to the UE: one for the MCG and one for the SCG. … If the UE is configured with LTM candidate cell, there may be zero or one RACH for each LTM candidate cell, which shares the same MAC entiy as Serving Cell. … Figure 4.2.2-2: MAC structure overview with two MAC entities

As quoted above from the running MAC CR [1], “RACH for each LTM candidate cell” shares the same MAC entiy as Serving Cell. As given in Figure 4.2.2-2 from the 3GPP TS 38.321, when the UE is configured with DC, the UE could have two MAC entities. Then it is unclear whether “RACH for LTM candidate cell” is associated with MCG MAC or SCG MAC.

Observation 1: According to the current MAC running CR, it is unclear whether the RACH for LTM candidate cell is associated with MCG MAC or SCG MAC.

Since the early-TA RACH procedure is triggered by the PDCCH-order, we think that the UE can use the MAC entity sending the PDCCH-order for the early-TA RACH procedure. The potential specification change would be as follows:

Text proposal: If the UE is configured with LTM candidate cell, there may be zero or one RACH for each LTM candidate cell, which shares the same MAC entiy as Serving Cell triggering the RACH.

Proposal 1: The early-TA RACH procedure is associated with the MAC entity providing the PDCCH-order triggering the early-TA RACH procedure.

## CG-based RACH-less LTM

RAN2#123 meeting agreement: - RAN2 assumes for RACH-less LTM, the UE determines successful reception of its first UL data based on receiving a PDCCH addressing the UE’s C-RNTI in the target cell scheduling a new transmission after the first UL data, (FFS if specified contents should be transmitted with this transmission, e.g. as LTE MAC CE).

Regarding the termination of the RACH-less LTM procedure, we could have the following two solutions:

- Option 1: C-RNTI PDCCH

- Option 2: Contention Resolution MAC CE

38.321: - UE Contention Resolution Identity: This field contains the UL CCCH SDU. If the UL CCCH SDU is longer than 48 bits, this field contains the first 48 bits of the UL CCCH SDU.

38.321: ### 5.3.1 DL Assignment reception … When the MAC entity has a C-RNTI, Temporary C-RNTI, CS-RNTI, G-RNTI or G-CS-RNTI, the MAC entity shall for each PDCCH occasion during which it monitors PDCCH and for each Serving Cell: 1> if a downlink assignment for this PDCCH occasion and this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI, or Temporary C-RNTI, or G-RNTI configured for multicast MTCH: … 2> stop the cg-SDT-RetransmissionTimer, if it is running, for the corresponding HARQ process for initial transmission with CCCH message; ### 5.4.1 UL Grant reception … If the MAC entity has a C-RNTI, a Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion and for each Serving Cell belonging to a TAG that has a running timeAlignmentTimer or a running cg-SDT-TimeAlignmentTimer and for each grant received for this PDCCH occasion: 1> if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or … 2> stop the cg-SDT-RetransmissionTimer for the corresponding HARQ process, if running.

For the CG-based RACH-less LTM, we think that it is simpler to use Option 1, as Option 2 would require more standard efforts to define the content of the Contention Resolution MAC CE (or a new MAC CE), since “UE Contention Resolution Identity” is created via the UL CCCH SDU, which is not used for the handover procedure for RRC_CONNECTED UE. Similar to the SDT procedure quoted above, the C-RNTI PDCCH for Option 1 can be either UL grant or DL assignment, and either new transmission or retransmission.

Proposal 2: For CG-based RACH-less LTM, the reception of the C-RNTI PDCCH terminates the CG-based RACH-less LTM, same as CG-SDT.

## DG-based RACH-less LTM

38.321: 2> else if the Serving Cell is SCell, and a PDCCH addressed to C-RNTI indicating uplink grant for a new transmission is received for the HARQ process used for the transmission of the MAC CE for BFR which contains beam failure recovery information of this Serving Cell; or 2> if the SCell is deactivated as specified in clause 5.9: 3> set BFI_COUNTER to 0; 3> consider the Beam Failure Recovery procedure successfully completed and cancel all the triggered BFRs for this Serving Cell.

38.331 running CR: 1> if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and the RRCReconfiguration message is applied due to an LTM cell switch execution without performing a Random Access procedure and upon an indication from lower layer that the LTM cell switch execution has been successfully completed: 2> stop timer T304 for that cell group if running;

For the DG-based RACH-less LTM, we think that we can have an aligned solution (i.e. C-RNTI PDCCH) with the CG-based RACH-less LTM for terminating the LTM procedure to stop T304 (as quoted above from the running RRC CR), as the expiry of T304 would trigger the link recovery procedure. However, the gNB may send several C-RNTI PDCCH(s) indicating UL grants for the UE’s uplink transmission, so as to reduce the data transmission interruption during handover. Then, it is possible that the first UL data transmission may not be received by the gNB when the UE receives a second C-RNTI PDCCH. As the RRCReconfigurationComplete message is included in the first UL data transmission for handover, if the UE only uses the C-RNTI PDCCH to terminate the DG-based RACH-less LTM, the LTM procedure could be terminated before the successful transmission of the RRCReconfigurationComplete message. The loss of the RRCReconfigurationComplete message will cause handover failure. To avoid the loss of the first UL data transmission including RRCReconfigurationComplete message, we think that the solution (i.e. “a PDCCH addressed to C-RNTI indicating uplink grant for a new transmission is received for the HARQ process”) for terminating the BFR procedure can be reused for terminating the DG-based RACH-less LTM.

Observation 2: The DG-based RACH-less LTM should be terminated after the reception of the network confirmation, to stop the running T304.

Proposal 3: For DG-based RACH-less LTM, as BFR, if a C-RNTI PDCCH indicating uplink grant for a new transmission is received for the HARQ process used for the transmission of the first UL data transmission to the target cell, the LTM procedure is terminated.

## Reusing TA of the source cell

RAN2#123 meeting agreement: - The UE will do RACH-less when: - TA value is provided in the cell switch MAC CE (already agreed, TA=0 is assumed to be covered by this) - When the UE shall apply the same TA value as the source (already agreed) FFS how the UE knows this.

When the UE switches from the source cell to the target cell during the LTM, the target cell could be the source SpCell or SCell. The source TA reused by the target cell could be the PTAG or any STAG. As such the UE should be indicated which TA value of TAG is reused.

Proposal 4: LTM Cell Switch Command MAC CE provides the TAG ID for which the UE reuses the TA for LTM.

# Conclusion

According to the above analysis, we have the following observations and proposals.

Observation 1: According to the current MAC running CR, it is unclear whether the RACH for LTM candidate cell is associated with MCG MAC or SCG MAC.

Observation 2: The DG-based RACH-less LTM should be terminated after the reception of the network confirmation, to stop the running T304.

Proposal 1: The early-TA RACH procedure is associated with the MAC entity providing the PDCCH-order triggering the early-TA RACH procedure.

Proposal 2: For CG-based RACH-less LTM, the reception of the C-RNTI PDCCH terminates the CG-based RACH-less LTM, same as CG-SDT.

Proposal 3: For DG-based RACH-less LTM, as BFR, if a C-RNTI PDCCH indicating uplink grant for a new transmission is received for the HARQ process used for the transmission of the first UL data transmission to the target cell, the LTM procedure is terminated.

Proposal 4: LTM Cell Switch Command MAC CE provides the TAG ID for which the UE reuses the TA for LTM.

# Reference

[1] R2-2309281, Huawei, HiSilicon, “38.321 running CR for introduction of NR further mobility enhancements”

# Annex

## RAN1 agreements for RACH-less LTM

The RAN1#111 meeting agreements regarding “Timing advance management to reduce latency” are quoted as follows:

Agreement On mechanism to acquire TA of the candidate cell(s) in Rel-18 LTM, at least support PDCCH ordered RACH.  The PDCCH order is only triggered by source cell  FFS: the details including content of DCI, RACH resource configuration, RAR transmission mechanism, etc.  Note: any other RACH-based solutions are for discussion separately Agreement (Made in RAN1#110b-e) Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility. - FFS: whether this can be applied to candidate cell when it is deactivated SCell (if defined in RAN2) Agreement For PDCCH ordered RACH in LTM, at least the following enhancements are supported · Introduce indication of candidate cell and/or RO of candidate cell in DCI · configuration of RACH resource for candidate cell(s) is provided prior to the PDCCH order · FFS: whether/how to transmit RAR Agreement On whether RAR is needed for PDCCH ordered RACH for a candidate cell in LTM, the following alternatives are considered for further study • Alt 1: RAR is needed • Alt 2: RAR is not needed - Note: If Alt 2 is supported, TA value of candidate cell is indicated in cell switch command • Alt 3: whether RAR is needed can be configured Agreement · TA updating (i.e. re-acquisition of TA) for candidate cell can be triggered by NW. · same triggering mechanism reuse the initial TA acquisition, i.e., PDCCH order triggered RACH in a candidate cell

The RAN1#112 meeting agreements regarding “Timing advance management to reduce latency” are quoted as follows:

Agreement For Rel-18 LTM, Random Access Preamble indices and indication of RACH occasions with the associated SSB indices are configured for each candidate cell. Note: the detailed signalling is left to RAN2 Agreement The PDCCH order from the source cell contains the indication of candidate cell. § The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity Agreement For PDCCH ordered-RACH for candidate cell(s), RAR reception can be configured/indicated § If reception of RAR is not configured/indicated (without RAR) - TA value of candidate cell is indicated in cell switch command - FFS: whether UE should re-transmit PRACH when reception of RAR is not configured/indicated - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order § If reception of RAR is configured/indicated (with RAR), FFS - whether RAR is received from serving cell or candidate cell u if RAR is received from candidate cell, whether Type1-PDCCH CSS of the candidate cell is configured to the UE - content of RAR § FFS: signaling for configuration/indication of whether RAR needs to be received § UE can report the support combination of with RAR only and without RAR only, where support of one default scheme is the baseline UE approach for LTM § Send LS to RAN2 and RAN3 to check the feasibility about this agreement § Note: Definition of candidate cells is up to RAN2 Agreement • For PDCCH-order based RACH for TA measurement for candidate cells, legacy CBRA is not supported Agreement on whether UE should initiate re-transmit PRACH when reception of RAR is not configured/indicated, down select one from the following alternatives. • Alt 1: UE autonomous re-transmission of PRACH is not allowed (e.g., by setting the number of allowed PRACH transmission to the minimum value of PreambleTransMax=1) • Alt 2: UE autonomous Re-transmission of PRACH is allowed, · The number of PRACH transmission will be defined e.g. set the times of RACH transmission to the minimum value of PreambleTransMax Agreement If reception of RAR is configured/indicated, RAR contains at least TA of candidate cell. • The maximum number of TA values memorized by UE is a UE capability • FFS: whether other parameters such as UE ID, candidate cell ID etc. is contained in RAR Agreement Whether RAR needs to be received is configured by RRC. Agreement study at least the following issues on PDCCH-order based PRACH for candidate cell that is not UL serving cell, i.e. without PUCCH/PUSCH configured • Whether gap between the DCI and PRACH longer than timeline defined in spec is needed • Any impact/interruption on UL Tx of serving CCs due to the PRACH Tx Working Assumption UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. • Corresponding UE capability is to be introduced to support UE-based TA measurement • For a UE reports support of this capability, configuration of UE-based TA measurement is supported • FFS: other impacts on RAN1 spec Agreement For Rel-18 LTM, Random Access Preamble indices and indication of RACH occasions with the associated SSB indices are configured for each candidate cell. Note: the detailed signalling is left to RAN2 Agreement The PDCCH order from the source cell contains the indication of candidate cell. § The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity

The RAN1#112bis meeting agreements regarding “Timing advance management to reduce latency” are quoted as follows:

- Time gap between a PDCCH order and the corresponding PRACH transmission RAN1 discussed the time gap between a PDCCH order and the corresponding PRACH transmission for LTM. RAN1 believes that this will require that the time gap is increased at least for the following scenario - For PDCCH-order based PRACH on a candidate cell that is not a current serving cell with PUCCH/PUSCH or inter-frequency with the current serving cell RAN1 relies on RAN4: - to verify the need for the above additional latency and, if so, the corresponding value is needed. - to investigate any impact/interruption on UL Tx of serving cell due to the PRACH Tx on a candidate cell that is not a current serving cell with PUCCH/PUSCH - to verify the need for any update is required to ΔBWPSwitching, ΔDelay if so, the corresponding values and whether UE capability is needed Potential RAN1 spec update will be based on RAN4’s feedback. - RAN1 #112bis-e agreements Agreement From RAN1 point of view, at least the following information can be included in the cell switch command, which is conveyed by MAC CE - Information to identify the target cell(s) - The details including bit number are designed by RAN2 - TA related information (details up to the discussion in A.I. 9.10.2) - 1 joint or 1 pair of UL and DL unified TCI State index for the target Cell - Note: discussion on target SpCell is not precluded - Active DL and UL BWPs for the target cell - FFS: Triggering of aperiodic TRS transmitted from the target cell - FFS: Triggering the CSI acquisition of the target cell and reporting to the target cell - FFS: Triggering of aperiodic SRS transmission to the target cell - FFS: C-RNTI - FFS: the presence of each field (i.e. always present or configurable) Agreement - RRC parameter ss-PBCH-BlockPower for candidate cells is included in the LTM configuration. - UE needs the parameter to (at least) perform RACH towards candidate cells - Note: how to capture this parameter and RRC structure are up to RAN2 Agreement For PDCCH ordered-RACH, if reception of RAR is not configured, UE autonomous re-transmission of PRACH is not allowed, regardless of the configuration of PreambleTransMax. Agreement When reception of RAR is configured, support RAR is received from serving cell at least in intra-DU case. Agreement When reception of RAR is configured, support RAR is received from serving cell in inter-DU case. - FFS: RA response window related issues Agreement For PDCCH ordered RACH mechanism in R18 LTM, when reception of RAR is configured, - the UE stores(remembers/maintains/handles) a TA for at least one candidate cell - storing(remembering/maintaining/handling) corresponding TAs for more than one candidate cell is up to UE capability - detailed number of candidate cell is up to UE capability Agreement For PDCCH-order based PRACH for candidate cell study the following issues: - whether/how prioritizations for transmission power reduction for a PRACH transmission to a LTM candidate cell is performed - whether/how prioritizations for prioritization of a PARCH transmission to a LTM candidate cell compared to an overlapped (in time and frequency) serving cell UL transmission Agreement For PDCCH ordered-RACH, if reception of RAR is not configured - Whether power ramping is performed or not is determined from PDCCH order - If power ramping is performed, - whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order (FFS exact indication mechanism) - power ramping-up value is configured - else, the power should be determined by open-loop power control

RAN1#113 meeting agreements:

Agreement On the determination of the PRACH transmission power when reception of RAR is not configured, a 1-bit field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH, FFS - UE will increase the power with the value of power ramping configuration if it is indicated as re-transmission, unless the max allowed power is achieved - whether/how to reset the counter Agreement · For PDCCH-order based PRACH for candidate cell, If UE capability does not support simultaneous/parallel transmissions, when the PRACH transmission to a candidate cell other than current serving cell (including any interruption due to processing time to build the PRACH transmission, carrier or/and BWP switching time if any, UL or DL RF retuning time if any, additional preparation time (if any) happen to overlap over one or more symbols or have a time gap below a certain threshold (e.g., N symbols, FFS: the value of N) with following UL transmission to one of the serving cells • PRACH transmission • PUCCH/PUSCH transmission carrying HARQ-ACK, SR, P/SP CSI, aperiodic CSI • SRS transmission • Any other PUCCH/PUSCH transmission · Down-select the UE behavior in this case • Alt 1: Dropping rule is needed • Alt 2: up to UE implementation

RAN1#114 meeting agreements:

Agreement For the power control of PDCCH-ordered CFRA in LTM, the UE can maintain only one power ramping counter Agreement For the power control of PDCCH-ordered CFRA in LTM, power-ramping counter is reset at least when UE receives a PDCCH order indicating the initial transmission of PRACH Agreement when a PDCCH order is sent for a candidate cell, • The bit size of N in DCI format 1_0 for cell indicator is determined by the number (e.g., C) of configured candidate cells with RACH configuration provided for early TA acquisition, downselect one from the following alternatives. • Alt 1: N= • Alt 2: N= (update the equation with (C+1)) · The number of cells used to calculate the bit width is the number of candidate cells with RACH configuration provided for early TA acquisition + 1 (serving cell) Agreement For the power control of PDCCH-ordered CFRA in LTM, • When a UE receives a PDCCH order indicating a re-transmission of PRACH with the same associated SSB and same candidate cell as the previous PRACH, the counter is increased by 1. • In addition to case 1, power-ramping counter is reset in the following cases: - Case 2: The candidate cell indicated in the PDCCH order, indicating retransmission, is different from that indicated in the last PDCCH order. Note: the initial counter is 0 before receiving any PDCCH order. Agreement when a PDCCH order is sent for a candidate cell, • The bit size of N in DCI format 1_0 for cell indicator is determined by the number (e.g., C) of configured candidate cells with RACH configuration provided for early TA acquisition, the following alternative is supported: • Alt 2: N= · The number of cells used to calculate the bit width is the number of candidate cells with RACH configuration provided for early TA acquisition + 1 (serving cell) Agreement When the UE does not support simultaneous/parallel transmissions of PRACH in candidate cell and UL channels and signals in serving cell, support - serving cell UL TX is dropped. Agreement If the UE supports simultaneous/parallel transmissions of PRACH in candidate cell and UL channels and signals in serving cell in the same frequency range, support: • A PRACH transmission to a LTM candidate cell has the highest priority for power allocation Note: up to UE whether performs power scale-down or drop of UL transmission with lower priority when UL transmission power is insufficient.

## RAN2 agreements for RACH-less LTM

- RAN2#121bis-e meeting agreements:

From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order). RRC RACH configuration for early TA acquisition (e.g., including whether RAR needs to be received) is specific per target cell and is signalled separately (separate IEs) from the candidate cell configuration (the part that need to be applied at cell switch). R2 assumes that Early TA RACH option 3 (with RAR from candidate cell) is not needed in Rel-18. With the assumption that the UE will skip RACH in the target cell if a TA value is given in the cell switch command: It is FFS if the following TA values can be given to the UE: - - Value 0, - - Value indicating that the UE shall apply the TA of one source cell.

RAN2#122 meeting agreements:

- Dynamic grant can be used for RACH-less LTM, for the first UL data transmission to the target cell: - the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch. - upon cell switch decision, R2 assumes that the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant. - Configured grant can be used for RACH-less LTM, for the first UL data transmission to the target cell, the UE selects the configured grant occasion, which is associated with the beam indicated in the LTM MAC CE (as set by source cell). FFS further optimization For early TA acquisition for candidate Cells: - For PDCCH ordered early TA acquisition without RAR, there is no need for UE to maintain the TA timer for candidate cell (i.e. it is NW implementation to determine the TA validity), TA is given in the cell switch MAC CE (when available in the network). - RAN2 doesn’t see a need for a solution with RAR in for Rel-18. - Observation: Without RAR (without UE maintaining TA), the UE will need to do RACH for link recovery and/or conditional (if supported), which is acceptable in Rel-18 - The UE determines to trigger RACH-less cell switch in MAC layer, if the LTM cell switch MAC CE provides the TA value (no RAR is assumed).

RAN2#123 meeting agreements:

- Define the association between CG occasion and beam in RRC and specify that the UE uses a CG occasion associated with the indicated beam in MAC - Observation: In cases for which it is desired that CG used for LTM is not used further once the UE has made the cell its new serving cell, it is assumed that the network could release Type1 CG resource on LTM completion (existing functionality). - Before RACH-less LTM procedure completion, the UE shall not trigger RACH (when the UE has no valid PUCCH resource for triggered SRs), as in LTE RACH-less. - RAN2 assumes for RACH-less LTM, the UE determines successful reception of its first UL data based on receiving a PDCCH addressing the UE’s C-RNTI in the target cell scheduling a new transmission after the first UL data, (FFS if specified contents should be transmitted with this transmission, e.g. as LTE MAC CE). - All the RRC configurations related to early RACH are specific per LTM candidate cell and signalled separately from the candidate cell configuration (i.e. LTM Candidate configuration). - The early RACH procedure shares a same MAC entity with the legacy RACH procedure. (e.g., no extra MAC entity is needed for early RACH). - It is up to UE implementation to handle the RACH initiation collisions where the early RACH is getting involved. No specification change can be foreseen. - R2 assumes for counting the power ramping step for early RACH, Reuse PREAMBLE_POWER_RAMPING_COUNTER. - FFS if UE transmits the preamble without the power ramping upon reception of PDCCH order with retransmission indication if preamble transmission encounter the LBT failure. - P8: Confirm that the RACH procedure toward a candidate cell is considered as complete once the preamble transmission is instructed to the lower layer. - automatic retransmission by timer with CG (similar to NR-U, SDT) is supported for the first UL data transmission with CG. - The UE will do RACH-less when: - TA value is provided in the cell switch MAC CE (already agreed, TA=0 is assumed to be covered by this) - When the UE shall apply the same TA value as the source (already agreed) FFS how the UE knows this.

## RAN3 agreements for RACH-less LTM

RAN3#119bis-e meeting agreements:

RAN3 would like to provide the following feedback for RAN1’s question: RAN1 respectfully asks RAN2 and RAN3 to check the feasibility and potential impact on specs of RAN2 and RAN3 of all options, i.e. with RAR (from serving or candidate cell) and without RAR, in the agreement described in section B. From RAN3 perspective, all options are feasible. The potential impacts of the following options on specifications are being taken into consideration by RAN3. - Case 1: with RAR and the RAR is received from the candidate cell - RAN3 will not pursue any standardization work in Rel-18 considering other groups output of this option. - Case 2: with RAR and the RAR is received from the serving cell - RAN3 will further discuss how to specify how the information required for the RAR transmission is signalled from the candidate DU to the source DU through the CU. - Case 3: without RAR - For the RACH-less solution, RAN3 will further discuss how to specify how the TA value(s) is (are) transmitted from the candidate DU to the source DU through the CU.

RAN3#120 meeting agreements:

- CU can request RACH resources for early TA acquisition together with the LTM candidate cell configuration to a Candidate DU in the UE Context Setup Request or UE Context Modification Request messages (for inter-DU LMT and intra-DU LTM respectively). - If the Candidate DU accepts the RACH resource request for early TA acquisition, the Candidate DU responds the CU with RACH configuration in the UE Context Setup Response or UE Context Modification Response messages.

RAN3#121 meeting agreements:

- RAN3 prioritizes specifying support for early TA acquisition “without RAR” and mark the method “with RAR” as open issue. - CU shall send the TA value to the source DU. - In inter-DU LTM, the gNB-CU provides the CFRA resource received from the candidate gNB-DU to the source gNB-DU via UE Context Modification procedure. - Working assumption: the CFRA resources could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different DU. need align with RAN2. - WA: Indicate the source gNB-DU ID when request the CFRA resources to the candidate DU. - WA: to introduce (2)new class 2 non-UE associated F1 signalings to deliver the TA information from the candidate gNB-DU to the source gNB-DU via gNB-CU. - How the source DU detect failure of TA info acquisition is up to implementation (eg., based on timer in source DU). - Source DU is in charge of checking the TA validity.