3GPP TSG-RAN WG2 Meeting #123bis										   R2-2310586

Xiamen, China, October 9th – 13th, 2023

Source: 	Xiaomi

Title:  	Discussion on the CFR location for shared MBS capability

Agenda Item:	7.11.3

Document for:	Discussion and Decision

# Introduction

According to the RAN2 discussion on the shared processing for MBS broadcast and unicast reception, RAN2 made the following agreements [1] in RAN2#123 meeting:

- As per the previous agreement, if the UE is able to get the additional information (i.e. its current configuration does not prevent it from doing so), the UE shall do this (if capable and configured by the network) - In case additional information (SCS, bandwidth) is not available at the time of sending the MII to the unicast serving cell (e.g. the UE is not able to read SIB1 from the non-serving cell), the UE reports whatever is available information at that time (i.e. at least the frequency, and optionally SCS and/or BW as available). - UE reports updated MII after acquiring additional information from the non-serving cell (if previously it reported only frequency) or if the information in the non-serving cell changes. - The SCS in the MII is set to the SCS of the CORESET#0 for the MBS broadcast cell. - Combination of FreqBandIndicatorNR and ARFCN-ValueNR is used to signal the frequency information in the MII for shared processing. - At least CFR bandwidth is reported by the UE in MII. FFS whether "location" needs to be also reported and how exactly this is captured in RRC (i.e. which IE is used).

In this contribution, we provide some clarifications on the frequency location information reported by the UE for the sharing processing capability.

# Discussion

## Frequency location information of CFR

According to the CFR configuration [2] specified in the Rel-17 broadcast MBS as quoted below, the frequency location of CFR is provided based on the initial BWP configuration, including the frequency location (i.e. the lowest subcarrier ARFCN of PRB#0) of the first PRB of initial BWP, staring PRB and number of PRBs (i.e. locationAndBandwidthBroadcast (or locationAndBandwidth)) of CFR.

Observation 1: If RAN2 includes the PRB location (i.e. locationAndBandwidth) of CFR for the shared capability reporting, the lowest subcarrier ARFCN of PRB#0 needs to be included.

According to the running RRC CR of Rel-18 MBS, the UE assistance information for shared capability includes the followings:

Figure 1: Frequency location for MBS reception

As illustrated in Figure 1, the above information (i.e. freqInfoMBS, bandwidthMBS and subcarrierSpacing) is sufficient to locate the frequency allocation/range used by the UE for the reception of MBS. The carrierFreqMBS and freqBandIndicatorMBS can represent the central frequency of the UE’s operating frequency for CFR while receiving MBS. The bandwidthMBS and subcarrierSpacing can present the frequency range for MBS reception.

Observation 2: freqInfoMBS, bandwidthMBS and subcarrierSpacing are sufficient for locating the operating frequency range for receiving MBS.

From our understanding, RAN2 only needs to clarify that when the bandwidthMBS and subcarrierSpacing are reported, the freqInfoMBS represents the central frequency of the UE’s operating frequency for MBS reception.

Proposal: When the bandwidthMBS (i.e. number of PRBs) and subcarrierSpacing are reported, the freqInfoMBS (i.e. ARFCN and band) represents the central frequency for MBS reception. No extra signalling for frequency location is needed.

# Conclusions

According to the analysis given above, we have the following observations and proposal:

Observation 1: If RAN2 includes the PRB location (i.e. locationAndBandwidth) of CFR for the shared capability reporting, the lowest subcarrier ARFCN of PRB#0 needs to be included.

Observation 2: freqInfoMBS, bandwidthMBS and subcarrierSpacing are sufficient for locating the operating frequency range for receiving MBS.

Proposal: When the bandwidthMBS (i.e. number of PRBs) and subcarrierSpacing are reported, the freqInfoMBS (i.e. ARFCN and band) represents the central frequency for MBS reception. No extra signalling for frequency location is needed.

# Reference

[1] RAN2#123 meeting minutes

[2] 3GPP TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”.