3GPP TSG-RAN WG2 Meeting #123bis									R2-2310607

Xiamen, China, October 9th – 13th, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	On height-based SSB-ToMeasure configuration

Agenda item:		7.8.2

Document for:		Discussion and Decision

- Introduction

In RAN2#121bis-e meeting [1], following agreements were reached:

Agreements

- Height-dependent more-than-one configurations is supported on parameter/field level (i.e. different fields/values within the same MO) where different values (or value ranges) of the parameter/field applies to different height or height range.

- For MO configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: SSB-ToMeasure. Details on how to specify is FFS.    FFS on UE behavior on L1 and L3 measurement.

- For MR configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: Event A4 threshold and numberoftriggeringcells.  Details on how to specify is FFS (i.e. maybe it can be achieved by combination of events).

- When height-dependent more-than-one configurations are provided, UE applies the new value once it moves to new height (or height range) similar to the case of RRC reconfiguration. Need Codes, field descriptions, etc. as in legacy specifications apply

- If a height-specific value is not explicitly configured for certain height, whether to keep using the value that was used or consider the parameter as released (i.e. parameter/value not applicable at this height) should be looked into case by case, and can be clarified by need code, field description, or procedural text as needed.   FFS details

In RAN2#122[2], following agreements were reached:

Agreements

- Add height-based list of SSB-ToMeasure with corresponding height ranges and hysteresis in MeasObjectNR.  FFS on the number of height ranges

- As a basic principle, if no height-specific SSB-ToMeasure is configured for a specific height region, the legacy behaviour applies.

- For UE behavior on L1 and L3 measurement, it is left to UE implementation whether to keep/discard the old samples while UE moves to a new height region with a different SSB-ToMeasure value

- New event types will be introduced on the combination of event Ax and event Hx, at least for event A4 + event H1/H2. FFS for other event Ax + event H1/H2. FFS on details, e.g. whether to include one height threshold (H1 or H2 threshold) or a height range (both H1 and H2 threshold) in the new event, how to configure height-dependent numberOfTriggeringCells, etc.    This will be applied to all height dependent MR parameters.

- Whether UE height is included when UAV specific MR is triggered is configurable by the network.

- We will use LTE UEheight.

In RAN2#123[3], following agreements were reached:

Agreements:

Measurement reporting

- New event type is introduced to implement both height-dependent MR configuration and combination of events

- The new event type includes only one of H1 or H2 thresholds.   A hysteresis for each threshold will be introduced.  A single time to trigger for the event will be introduced. Rapporteur will implement the solution and we can come back next meeting if there are technical issues.

- Event combination of H1/H2 with A3, A4, A5 will be support

- When the UE moves to a new height region with a different numberOfTriggeringCells value for one configured event, it is up to UE implementation whether the UE keeps the old measurement report or cellstriggeredlist

- At least 300m above ground level will be supported.  FFS if higher ranges can/need to be considered.

- Signalling should allow the network to simultaneously configure height-based event thresholds for the whole range of possible UE heights

Flight path update/reporting

- No delta signaling for updated flight path will be supported

- Confirm that same indication is used for both initial and updated flightpath plan

- If any waypoint/timestamp has changed more than the configured threshold the UE triggers flightpath update indication.

- When both distance and time threshold are configured, the flightpath update can be triggered if either of those conditions are met.   No prohibit timer will be introduced.

There are still some FFS issues related to height-dependent SSB-ToMeasure configuration. This contribution provides considerations and proposals on the FFS issues and remaining issues based on the agreements reached in previous meetings.

- Discussion

In RAN2#122 meeting[2], following agreement is reached about the height-dependent SSB-ToMeasure:

- Add height-based list of SSB-ToMeasure with corresponding height ranges and hysteresis in MeasObjectNR.  FFS on the number of height ranges

In our understanding, it is more flexible and future proof to allow more than one height range. Although in previous meetings, only above/below rooftop case is discussed, the real radio environment may be more complicated. Thus we propose RAN2 to agree that more than two height ranges can be configured. Considering the potential height range to apply this mechanism, we think maybe 8 is enough.

Proposal 1: The maximum number of height ranges can be 8.

In current running CR, the height-based SSB-ToMeasure is defined as following:

SSB-ToMeasureHeightBased-r18 ::=     SEQUENCE {

heightRange-r18     SEQUENCE {

heightMin-r18       INTEGER (0..maxHeight-r18)   OPTIONAL,

heightMax-r18       INTEGER (0..maxHeight-r18)   OPTIONAL,

heightHyst-r18      INTEGER (0..maxHyst-r18)     OPTIONAL

},

ssb-ToMeasure-r18   SSB-ToMeasure

}

MeasObjectNR field descriptions

ssb-ToMeasureHeightBasedList List of height-dependent ssb-ToMeasure. If this list is included, while the UE is within a height range indicated by heightRange, it applies corresponding ssb-ToMeasure-r18 for that height range and ignores ssb-ToMeasure (without suffix). When the UE is in a height range for which a corresponding ssb-ToMeasure-r18 is not configured, ssb-ToMeasure (without suffix) applies.

It is unclear how heightHyst-18 is applied. In LTE event H1 and H2, hysteresis is applied for both entering condition and leaving condition. For this heightHyst-r18 defined for height-based SSB-ToMeasure, we think it can be applied similarly. I.e. it is applied when UAV UE flying in and flying out of the height range.

For example, the entering condition can be defined as following (flying into the range)

current height – heightHyst-r18 > heightMin-r18, or,

current height + heightHyst-r18 < heightMax-r18

The leaving condition can be defined as following (flying out of the range):

current height + heightHyst-r18 < heightMin-r18, or

current height – heightHyst-r18 > heightMax-r18

Proposal 2: The heightHyst-r18 IE is applied for both entering and leaving the height range.

Proposal 3: To capture in the specification how to apply the heightHsyt-r18 in height-based SSB-ToMeasure configuration:

- The entering condition can be defined as following (flying into the range):

- current height – heightHyst-r18 > heightMin-r18, or,

- current height + heightHyst-r18 < heightMax-r18

- The leaving condition can be defined as following (flying out of the range):

- current height + heightHyst-r18 < heightMin-r18, or

- current height – heightHyst-r18 > heightMax-r18

If more than two height range can be configured, the adjacent height ranges may be overlapped. Following is an example of height range overlapping without taking account hysteresis. Note when the hysteresis is taking into account, the overlapping situation maybe a little more complicated.

Figure 1: illustration of overlapping in height range

First of all, RAN2 should decide whether such overlapping configuration in height range is allowed. In our understanding, such overlapping in height range is contradictory to the intention of height-based SSB-ToMeasure mechanism, i.e. to reduce unnecessary beam measurement. On the other hand, the coverage of beams may be overlapped in a height range. But the network can configure an separate configuration for this range if necessary. So we propose this kind of overlapping is not allowed for simplicity.

Proposal 4: RAN2 to agree the overlapping in height range is not allowed for height-based SSB-ToMeasure.

If RAN2 decides above proposal: two possible options can be considered:

Option 1: To leave to network configuration to ensure there is no overlapping configuration.

Option 2: To specify a standard mechanism to avoid overlapping configuration. For example, for each height range, only min height (heightMin) is configured. The min height of next height range is the max height of this height range. And all height ranges share same hysteresis value.

Figure 2: illustration of non-overlapping configuration

Proposal 4a: If RAN2 agrees overlapping in height range is not allowed, RAN2 to choose one of following options to avoid the overlapping:

- Option 1: Left to network configuration to ensure there is no overlapping in height ranges.

- Option 2: To specify a mechanism to ensure there is no overlapping in height range.

If RAN2 decides the overlapping configuration is allowed, we need to decide the UE behavior when it is in the overlapped height range. One straight forward solution is to combine the SSB-ToMeasure of overlapped height ranges. Since different height ranges share same SMTC, this solution is feasible.

As to the impact on specification, maybe nothing needs to be captured considering the field description of current running CR. i.e. when the UE is within more than one height ranges, it applies SSB-ToMeasure-r18 corresponding to these height ranges.

MeasObjectNR field descriptions

ssb-ToMeasureHeightBasedList List of height-dependent ssb-ToMeasure. If this list is included, while the UE is within a height range indicated by heightRange, it applies corresponding ssb-ToMeasure-r18 for that height range and ignores ssb-ToMeasure (without suffix). When the UE is in a height range for which a corresponding ssb-ToMeasure-r18 is not configured, ssb-ToMeasure (without suffix) applies.

Proposal 4b: If RAN2 decides overlapping in height ranges is allowed, the UE applies combination of SSB-ToMeasure of overlapped height ranges. FFS on the spec impact.

Besides RRC_CONNECTED mode, we believe it is also beneficial to extend height-based SSB-ToMeasure to RRC_IDLE/RRC_INACTIVE mode RRM measurement and RACH resource selection in Rel-18. Considering that the UAV UE is not always in RRC_CONNECTED state when in the air, the benefit of height-based SSB-ToMeasure for RRC_CONNECTED UE is also valid for RRC_IDLE UE and RRC_INACTIVE UE, i.e. avoid unnecessary SSB measurement and avoid improper cell reselection due to the impact of side lobes from faraway cells.

Someone argues this is out of Rel-18 scope. But considering height-based SSB-ToMeasure and measurement report configuration for RRC_CONNECTED are adopted for Rel-18, they can be naturally extended to RRC_IDLE/RRC_INACTIVE for Rel-18 too.

Proposal 5: RAN2 to extend height-dependent SSB-ToMeasure for RRM measurement to cell re-selection, Idle/Inactive measurements and RACH resource selection.

As to the FFS issue “Should the numberOfTriggeringCells mechanism be allowed to consider all SSBs despite the SSB to Measure configuration for the purpose of interference reporting (which limits the use to some specific SSBs only)?”, we share the similar view that different configuration for interference detection may be necessary.

The intention of height-based SSB-ToMeasure is to save UE power and avoid impact of side lobes from far away cells. However, for interference detection, it may be necessary to perform measurement on all SSBs.

In addition, the cell quality is derived based on the consolidation of one or multiple beam measurements. So the interference detection requirement in NR may differ from the LTE, e.g. one or two beams from the neighbor cells with strong RSRP are possible to cause interference to the serving cell.

So RAN2 can consider whether some enhancements on the interference detection in NR is needed, e.g. consider all SSBs for interference detection, and configure another criteria or parameters for cell quality derivation on interference detection, e.g. a less RS number to average for cell measurement derivation (e.g. nrofSS-BlocksToAverage) but a higher threshold for the consolidation of measurement results per RS from L1 filter (absThreshSS-BlocksConsolidation).

Proposal 6:	RAN2 to consider all SSBs for interference detection despite the height-based SSB-ToMeasure configuration, and to introduce separate nrofSS-BlocksToAverag and absThreshSS-BlocksConsolidation for interference detection.

- Conclusion and proposals

Proposal 1: The maximum number of height ranges can be 8.

Proposal 2: The heightHyst-r18 IE is applied for both entering and leaving the height range.

Proposal 3: To capture in the specification how to apply the heightHsyt-r18 in height-based SSB-ToMeasure configuration:

- The entering condition can be defined as following (flying into the range):

- current height – heightHyst-r18 > heightMin-r18, or,

- current height + heightHyst-r18 < heightMax-r18

- The leaving condition can be defined as following (flying out of the range):

- current height + heightHyst-r18 < heightMin-r18, or

- current height – heightHyst-r18 > heightMax-r18

Proposal 4: RAN2 to agree the overlapping in height range is not allowed for height-based SSB-ToMeasure.

Proposal 4a: If RAN2 agrees overlapping in height range is not allowed, RAN2 to choose one of following options to avoid the overlapping:

- Option 1: Left to network configuration to ensure there is no overlapping in height ranges.

- Option 2: To specify a mechanism to ensure there is no overlapping in height range.

Proposal 4b: If RAN2 decides overlapping in height ranges is allowed, the UE applies combination of SSB-ToMeasure of overlapped height ranges. FFS on the spec impact.

Proposal 5: RAN2 to extend height-dependent SSB-ToMeasure for RRM measurement to cell re-selection, Idle/Inactive measurements and RACH resource selection.

Proposal 6:	RAN2 to consider all SSBs for interference detection despite the height-based SSB-ToMeasure configuration, and to introduce separate nrofSS-BlocksToAverag and absThreshSS-BlocksConsolidation for interference detection.

- Reference

- RAN2#121-bis chairman notes

- RAN2#122 chairman notes

- RAN2#123 chairman notes

- TS 38.331