3GPP TSG-RAN WG2 #123bis 	R2-2310659

Xiamen, China, 9th –13th Oct 2023

Agenda Item:	7.5.4.1 BSR enhancements for XR

Source:	NEC

Title:	Details of the New BS table

Document for:	Discussion, Decision

# Introduction

RAN2 agreed to support only one static 8bits BSR table for XR service with big data size.

Detailed agreements regarding new BS table during RAN2#121bis-e and RAN2#122 meeting are listed here for reference:

At RAN2#121bis-e: - As a working assumption, at most one BS index or BS value is reported by an LCG. This assumption can be revisited if new BSR table design cannot achieve a target level of quantization error. FFS what this target level should be. - Design/configuration for new BSR table(s) should include support for narrower ranges (i.e. finer granularity) than the legacy. Details can be discussed after an agreement on how UE obtains new BSR table(s) (e.g. pre-definition vs RRC configuration) is made. - At least linear distribution is used for generating code points in new BSR table(s). FFS whether exponential distribution can be considered too. FFS if piecewise linear distribution is supported. - New BSR table(s) can be used by any UEs that support such a capability. However, design of the new BSR table(s) should be based on XR-specific use cases and requirements. - Network can configure which BSR table(s) an LCG is eligible to use. UE determines which BSR table (i.e. legacy or something else) the LCG should use. FFS details of this determination (e.g. based on buffer size) and how network knows which BSR table each LCG uses. At nRAN2#122: - Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases). - We do not support additional piecewise linear BSR table in Rel-18. Can consider piecewise linearity when discussing how the BSR table values are defined.

In this document, we discuss the code points of the new static BS table, and corresponding new BSR MAC CE format, and necessary configuration.

# Discussion

## New static BSR table

According to the agreement made in last meeting, RAN2 will design one static BSR table with 8bits BS field for Rel-18 XR.

Also it is majority opinion to support linear distribution to generate code points in the new BSR table. But based on previous meeting’s agreement it is still need to confirm whether exponential distribution and or piecewise linearity can be considered

- At least linear distribution is used for generating code points in new BSR table(s). FFS whether exponential distribution can be considered too. FFS if piecewise linear distribution is supported. - We do not support additional piecewise linear BSR table in Rel-18. Can consider piecewise linearity when discussing how the BSR table values are defined.

And according to contributions promoting piecewise, the intention seems to keep the same step size within the typical XR data burst size, and with bigger step size out of the typical XR data size. Since we agree to cover a narrower size range with the new BS table, and within this narrower range, there seems no point to have different step size.  For exponential distribution, it could only work together with linear distribution if it becomes configurable, but this is not the case anymore.

Moreover, based on the analysis in [1], a linear distribution BS table already can achieve an average quantization error of ~1%, to cover the maximum 150Mbps bitrate and minimum 24 frame rates, it is much better comparing with the average quantization error of ~6.5% with legacy BSR table. To keep the thing simple:

RAN2 confirm to support linear distribution only

Then next step is to determine the size range of the new BS table, following are the text from TR38.835, where it says

- the XR video frame rate could varies from15fps to 120 fps

- the XR video bit rate could be between 10 up to 200Mbps

### 4.5.2 Video The frame rate for XR video varies from 15 frames per second up to 90 or even 120 frames per second, with a typical minimum of 60 for VR (see TR 26.918 [5] and TR 26.926 [6]). According to TR 26.918 [5], the latency of action of the angular or rotational vestibulo-ocular reflex is known to be of the order of 10 ms or in a range from 7-15 milliseconds and it seems reasonable that this should represent a performance goal for XR systems. This results in a motion-to-photon latency of less than 20 milliseconds, with 10ms being given as a goal. Regarding the bit rates, between 10 and 200Mbps can be expected for XR depending on frame rate, resolution and codec efficiency (see TR 26.926 [6] and TR 26.928 [7]).

Use the same method proposed in [1], and align the documented maximum bit rate and minimum framerate in TR, we can calculate

- Bmax=maximum bit rate/8/minimum frame rate= 200Mbps/8/15fps= 1666,667Bytes

- Bmin >= Bmax /(1+256*6.5%) = 94,482bytes

- Step size is about 6141 Byte

In legacy 8bits BS table, this size range is covered by code points from 145 to 192 , where the step size is larger than 106,951. now the step size of new table is reduced to 6000. The step size is still quite big but much smaller than legacy. The quantization error of the new table is from ~0.4% up to ~6.4%.

kindly ask RAN2 to support the new BSR table in appendix with:

    - Bmin = 94,482 Bytes

    - Step size = 6141 Bytes

## BSR MAC CE format

It is likely that new BS table is used for some LCG(s)( i.e., the LCG include the LCHs carrying XR service) but legacy BS table is for other LCGs(e.g., the LCG for SRBs and DRB not carrying XR traffic). BS fields of some LCGs should be encode/decode always based on legacy BS table if it is not configured with new BS table.

- Observation 1: it should be possible not to apply new BS table for SRB(s) and DRB not for XR traffic

From a given LCG configured with the new BS table, it is still possible that the buffer size corresponding to this LCG at some point of time, is small, e.g., when only PDCP/RLC control PDU is in buffer or when most of buffered data has been scheduled away. When the buffered data is small, precise BS information is very important for gNB in order to schedule a right size of UL grant. Falling back to use legacy BS table would be preferable. Moreover, the new BSR table only covers a certain BS range, the buffer size may fall out of the new BS table coverage.

- Observation 2: UE should be able to fall back to legacy BS table(s) for a LCG which is configured/enabled with new BS table (e.g., in case the buffer size becomes small after being scheduled)

hence it is not enough for UE always use new or legacy BS table based on NW RRC configuration, it would be better:

- When the buffer size is in (min, max) of the new BS table, use new BS table which provides more precise buffer size

- When the buffer size is out of (min, max) of the new BS table, use legacy BS table(s)

To support this dynamic BS table switch, table indication should be introduced in BSR MAC CE. Moreover there might be some uncertain period between the reconfiguration message and the reflection of the received new configuration. If this uncertain period may exist for this new BSR mechanism, the BS table indication will be useful. Hence, It is proposed:

Introduce BS table indication in new BSR MAC CE

For long or long truncated BSR MAC CE, in principle, BS table indication would be only needed for the LCGs configured with additional new BSR table, but it is simple just to introduce table indication for every LCG.

For short or short truncated BSR MAC CE, currently, 5bit BS field/table is used. However, there was proposal to also use 8bits field for finer granularity. Apparently, new BS table is also for finer granularity, this should be discussed together. we could then have 4 options to increase the granularity:

- keep the 5bits BS field but use a new 5bits BS table for higher granularity

- introduce 8bits BS field and use only the legacy 8bit BS table

- introduce 8bits BS field and use only the new 8bit BS table to align with the BS table used for long or long truncated BSR MAC CE

- introduce 8bit BS filed, and UE choose either to use legacy 8bits BS table or new 8bits BS table depends on if the Buffer Size fall in (min, max)

Option 2 and option 3 would request 2bytes of MAC CE format to accommodate 8bits Buffer Size field and 3bits LCG ID field, still 5bits will not be used yet. Instead, with option 4, 1 of the unused 5bits can be used for BS table indication. For option1, it can keep 1byte MAC CE format, but a new (e)LCID would be still required to distinguish from legacy short/short truncated MAC CE. Our preference is to have 8bits field to having same granularity for both short/short truncated BSR MAC CE and long/long truncated BSR MAC CE, with the cost of 1byte more payload.

if new BS table is configured/enabled for a LCG, for a short or short truncated BSR MAC CE, BS field has fixed length of 8 bits too, either legacy or new 8bit BS table could be used.

Together with all above proposals, we further propose to introduce two new BSR MAC CEs:

introduce following two new BSR MAC CEs:

- New long or long truncated BSR MAC CE

- New short or short truncated BSR MAC CE

## Other aspect of new BS table

Since the new table is designed in a way that if the buffer size falls in the size range of the new BS table, the quantization error will be lower if using the new table.

- UE use new BS table if the buffer size falls in the size range of the new BS table when 8bits BS field is used

If it is agreeable to have 1bit table indicator per LCG in long or long truncated BSR MAC CE, there is no necessary to enable new static BS table per LCG.  per UE enable bit is enough. And once new static BS table is enabled, it means new BSR MAC CE with table indicator is also enabled:

- New BSR MAC CE with new BS table could be enabled/disabled per UE.

# Summary

In this contribution, we discussed several aspect around new BSR table and new BSR trigger:

New BS table:

- RAN2 confirm to support linear distribution only

kindly ask RAN2 to support the new BSR table in appendix with:

    - Bmin = 94,482 Bytes

    - Step size = 6141 Bytes

New BSR MAC CEs:

- Observation 1: it should be possible not to apply new BS table for SRB(s) and DRB not for XR traffic

- Observation 2: UE should be able to fall back to legacy BS table(s) for a LCG which is configured/enabled with new BS table (e.g., in case the buffer size becomes small after being scheduled)

Introduce BS table indication in new BSR MAC CE

if new BS table is configured/enabled for a LCG, for a short or short truncated BSR MAC CE, BS field has fixed length of 8 bits too, either legacy or new 8bit BS table could be used.

introduce following two new BSR MAC CEs:

- New long or long truncated BSR MAC CE

- New short or short truncated BSR MAC CE

UE use new BS table if the buffer size falls in the size range of the new BS table when 8bits BS field is used

New BSR MAC CE with new BS table could be enabled/disabled per UE.

# References

- R2-2304711 BSR and delay status reporting for XR Qualcomm Incorporated

# Appendix: new BS table

New Buffer size levels (in bytes) for 8-bit Buffer Size field

| index | BS value | index | BS value | index | BS value | index | BS value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 94482< <=100623 | 64 | <=493647 | 128 | <=886671 | 192 | <=1279695 |
| 1 | <=106764 | 65 | <=499788 | 129 | <=892812 | 193 | <=1285836 |
| 2 | <=112905 | 66 | <=505929 | 130 | <=898953 | 194 | <=1291977 |
| 3 | <=119046 | 67 | <=512070 | 131 | <=905094 | 195 | <=1298118 |
| 4 | <=125187 | 68 | <=518211 | 132 | <=911235 | 196 | <=1304259 |
| 5 | <=131328 | 69 | <=524352 | 133 | <=917376 | 197 | <=1310400 |
| 6 | <=137469 | 70 | <=530493 | 134 | <=923517 | 198 | <=1316541 |
| 7 | <=143610 | 71 | <=536634 | 135 | <=929658 | 199 | <=1322682 |
| 8 | <=149751 | 72 | <=542775 | 136 | <=935799 | 200 | <=1328823 |
| 9 | <=155892 | 73 | <=548916 | 137 | <=941940 | 201 | <=1334964 |
| 10 | <=162033 | 74 | <=555057 | 138 | <=948081 | 202 | <=1341105 |
| 11 | <=168174 | 75 | <=561198 | 139 | <=954222 | 203 | <=1347246 |
| 12 | <=174315 | 76 | <=567339 | 140 | <=960363 | 204 | <=1353387 |
| 13 | <=180456 | 77 | <=573480 | 141 | <=966504 | 205 | <=1359528 |
| 14 | <=186597 | 78 | <=579621 | 142 | <=972645 | 206 | <=1365669 |
| 15 | <=192738 | 79 | <=585762 | 143 | <=978786 | 207 | <=1371810 |
| 16 | <=198879 | 80 | <=591903 | 144 | <=984927 | 208 | <=1377951 |
| 17 | <=205020 | 81 | <=598044 | 145 | <=991068 | 209 | <=1384092 |
| 18 | <=211161 | 82 | <=604185 | 146 | <=997209 | 210 | <=1390233 |
| 19 | <=217302 | 83 | <=610326 | 147 | <=1003350 | 211 | <=1396374 |
| 20 | <=223443 | 84 | <=616467 | 148 | <=1009491 | 212 | <=1402515 |
| 21 | <=229584 | 85 | <=622608 | 149 | <=1015632 | 213 | <=1408656 |
| 22 | <=235725 | 86 | <=628749 | 150 | <=1021773 | 214 | <=1414797 |
| 23 | <=241866 | 87 | <=634890 | 151 | <=1027914 | 215 | <=1420938 |
| 24 | <=248007 | 88 | <=641031 | 152 | <=1034055 | 216 | <=1427079 |
| 25 | <=254148 | 89 | <=647172 | 153 | <=1040196 | 217 | <=1433220 |
| 26 | <=260289 | 90 | <=653313 | 154 | <=1046337 | 218 | <=1439361 |
| 27 | <=266430 | 91 | <=659454 | 155 | <=1052478 | 219 | <=1445502 |
| 28 | <=272571 | 92 | <=665595 | 156 | <=1058619 | 220 | <=1451643 |
| 29 | <=278712 | 93 | <=671736 | 157 | <=1064760 | 221 | <=1457784 |
| 30 | <=284853 | 94 | <=677877 | 158 | <=1070901 | 222 | <=1463925 |
| 31 | <=290994 | 95 | <=684018 | 159 | <=1077042 | 223 | <=1470066 |
| 32 | <=297135 | 96 | <=690159 | 160 | <=1083183 | 224 | <=1476207 |
| 33 | <=303276 | 97 | <=696300 | 161 | <=1089324 | 225 | <=1482348 |
| 34 | <=309417 | 98 | <=702441 | 162 | <=1095465 | 226 | <=1488489 |
| 35 | <=315558 | 99 | <=708582 | 163 | <=1101606 | 227 | <=1494630 |
| 36 | <=321699 | 100 | <=714723 | 164 | <=1107747 | 228 | <=1500771 |
| 37 | <=327840 | 101 | <=720864 | 165 | <=1113888 | 229 | <=1506912 |
| 38 | <=333981 | 102 | <=727005 | 166 | <=1120029 | 230 | <=1513053 |
| 39 | <=340122 | 103 | <=733146 | 167 | <=1126170 | 231 | <=1519194 |
| 40 | <=346263 | 104 | <=739287 | 168 | <=1132311 | 232 | <=1525335 |
| 41 | <=352404 | 105 | <=745428 | 169 | <=1138452 | 233 | <=1531476 |
| 42 | <=358545 | 106 | <=751569 | 170 | <=1144593 | 234 | <=1537617 |
| 43 | <=364686 | 107 | <=757710 | 171 | <=1150734 | 235 | <=1543758 |
| 44 | <=370827 | 108 | <=763851 | 172 | <=1156875 | 236 | <=1549899 |
| 45 | <=376968 | 109 | <=769992 | 173 | <=1163016 | 237 | <=1556040 |
| 46 | <=383109 | 110 | <=776133 | 174 | <=1169157 | 238 | <=1562181 |
| 47 | <=389250 | 111 | <=782274 | 175 | <=1175298 | 239 | <=1568322 |
| 48 | <=395391 | 112 | <=788415 | 176 | <=1181439 | 240 | <=1574463 |
| 49 | <=401532 | 113 | <=794556 | 177 | <=1187580 | 241 | <=1580604 |
| 50 | <=407673 | 114 | <=800697 | 178 | <=1193721 | 242 | <=1586745 |
| 51 | <=413814 | 115 | <=806838 | 179 | <=1199862 | 243 | <=1592886 |
| 52 | <=419955 | 116 | <=812979 | 180 | <=1206003 | 244 | <=1599027 |
| 53 | <=426096 | 117 | <=819120 | 181 | <=1212144 | 245 | <=1605168 |
| 54 | <=432237 | 118 | <=825261 | 182 | <=1218285 | 246 | <=1611309 |
| 55 | <=438378 | 119 | <=831402 | 183 | <=1224426 | 247 | <=1617450 |
| 56 | <=444519 | 120 | <=837543 | 184 | <=1230567 | 248 | <=1623591 |
| 57 | <=450660 | 121 | <=843684 | 185 | <=1236708 | 249 | <=1629732 |
| 58 | <=456801 | 122 | <=849825 | 186 | <=1242849 | 250 | <=1635873 |
| 59 | <=462942 | 123 | <=855966 | 187 | <=1248990 | 251 | <=1642014 |
| 60 | <=469083 | 124 | <=862107 | 188 | <=1255131 | 252 | <=1648155 |
| 61 | <=475224 | 125 | <=868248 | 189 | <=1261272 | 253 | <=1654296 |
| 62 | <=481365 | 126 | <=874389 | 190 | <=1267413 | 254 | <=1660437 |
| 63 | <=487506 | 127 | <=880530 | 191 | <=1273554 | 255 | <=1666578 |

# Appendix: new BSR MAC CEs

- New long or long truncated BSR MAC CE

- New short or short truncated BSR MAC CE