3GPP TSG-RAN WG2 Meeting #123	R2-2310687

Xiamen, China, 09 – 13 October 2023

Agenda item:	7.5.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	BSR enhancements for XR

WID/SID:	NR_XR_enh-core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

This contribution address the following open issues on the BSR and delay information report for XR.

- BSR - Entries in the new BSR table - How the MAC entity determines which BSR table a logical channel group is eligible to use - Whether the legacy BSR MAC CE or a new BSR MAC is used to support the use of new BSR table - DSR - Definition of remaining time - Whether one or more values of remaining time are reported for an LCG - Whether data with delay below the threshold is reported - The fields included in the DSR MAC CE and their respective definitions, and which type of LCID to use

# 2	Discussion

## 2.1	Code points for a new fixed 8-bit BS table

Based on the above agreements, RAN2 should define the values to fulfill the new 8-bit BS table (i.e., 28 = 256 code points). This table should provide a quantization error significantly lower than the provided by legacy 8-bit table (6.5% across all code points). The table’s range should cover all possible combinations of bit rate and frame rate for XR video traffic. For example, considering an XR application applies either AVC or HEVC codecs, with 4K or 8K resolution, the bit rate will be in the range of 10~150 Mbps as shown in Table 4.5-1 [1].

Table 4.5-1: Expected Video coding standards performance and bitrate target

| # Codec | # Coding performance # (Random-Access) |  | # Targeted bitrate # (Random Access) |
| --- | --- | --- | --- |
|  | Objective | Subjective |  |
| AVC |  |  | 4k: - Statmux: 20-25 Mbps - CBR: 35 - 50 Mbps 8k: - CBR: 80 - 100 Mbps - High quality: 100 - 150 Mbps [33][34][35] |
| HEVC | -40% vs AVC [33][34][35] | -60% vs AVC [33][34][35] | 4k: - Statmux: 10-13 Mbps - CBR: 18-25 Mbps 8k: - CBR: 40-56 Mbps - High quality: 80-90 Mbps [33][34][35] |

By looking at the frame rate values, the most common are 24, 25, 30, 50 ,60, 90, 120, 144 and 240.

In TS 38.838 [2], it is defined that the XR video traffic follows a truncated gaussian distribution. Figure 1 shows the truncated gaussian distribution of all possible combinations of bit rate and frame rate.

Figure 1: Video size truncated Gaussian distribution for each combination of bit rate and frame rate

Based on the previous analysis, the minimum value Bmin corresponds to minimum value of the truncated Gaussian distribution of the combination of bit rate and frame rate values of 10 Mbps and 240 fps, respectively. It is obtained as follows:

Bmin=0.5×10×106/8/240=2642B

On the contrary, the maximum value Bmax corresponds to maximum value of the truncated Gaussian distribution of the combination of bit rate and frame rate values of 150 Mbps and 24 fps, respectively. It is obtained as follows:

Bmax=1.5×150×106/8/24=1171875B

Within the options to determine the 256 code points, we analyzed the quantization error with linear and exponential distributions:

- Linear function

- Exponential function (same as legacy)

To evaluate how the different options perform, we determined the maximum quantization error (QEmax), as in [3], for each of the 255 code points, as follows:

QEmaxn%=BSn+1-BSnBSn×100%, n∈0,254.

Within the options to determine the 256 code points, we have the following:

- Linear function

It consists of having equidistant code points between Bmin and Bmax as follows:

step size=floorBmax-Bmin256=4567B

For this option, the QEmaxn% across all code points fluctuates between 0.39~173%. The high values are experienced at the first code points since the distance between Bmin and Bmax is considerably big, making the granularity at the small BS values insufficient.

Observation 1: the quantization error is very large for the lower end code points with linear distribution if with full range Bmin and Bmax.

- Exponential function (same as legacy)

It consists of following the exponential function used in current specification [4] as follows:

Bk=Bmin×1+pk, with p=Bmax/Bmin1/N-1-1

This function provides a reduction of the QEmaxn% until 2.42% across all code points.

Observation 2: QEmaxn ≈2.4 % across all code points is achieved when filling the range with the legacy exponential formula. The main reason for the better performance is, of course, the narrower range than legacy, but still not close to 1%.

Observation 3: with a single BS table covering the full range of XR traffic, it is impossible to achieve 1% quantization error as analyzed in [4] with either linear or exponential distribution.

Possible options to achieve low quantization error:

- Option 1: Limit the range of Bmin and Bmax to cover only certain data and frame rates.

- Option 2: introduce multiple tables covering different ranges.

  - Predefined tables or UE generated tables based on NW configurations.

With the current RAN2 decision for a single fixed 8-bit table, the opportunities to optimize and lower the quantization errors are limited. Moreover, there has not been an agreement on the target quantization error value. Initial proposals for 1% quantization error are not possible to achieve in a meaningful range covering the currently considered video data and frame rates. While the range of possibilities for data and frame rates is wide (even if it might not be future-proof), simulation criteria listed in [2] Table 5.5.2.1-1 are the most used in RAN2, and any new table definition should provide improvements within the traffic model defined for evaluation of capacity.

Any definition of the Bmin/Bmax range for the single 8-bit table should cover, at least partially, the range of the currently defined baseline capacity evaluation traffic model from [2]. The baseline traffic model is a 10 Mbps video stream @ 60 frames per second and PDB of 30 ms. The resulting minimum frame size is 10417 Bytes, and the maximum frame size is 31250 Bytes with a frame arrival periodicity of 16.67 ms. This means that within the 30 ms PDB, there can be at most two full frames in the buffer. Narrowing the range to this extreme level leads to a range of Bmin=10417 B and Bmax=62500 B, which, if covered by a table generated with the exponential distribution, leads to an excellent accuracy of 0.7% quantization error. Nevertheless, this is too narrow of a solution as it only improves the accuracy between indexes 110 and 139, approximately, of the legacy table.

Several companies proposed an easy, single threshold checked at the UE side, where only the Bmin value is re-defined, and the upper bound is set by the current table  Bmax=81 338 368 B. If we restrict the evaluation of the benefits with current traffic models of 10 Mbps with 60 frames per second, then Bmin=10417 B, and the achieved quantization error, using exponential distribution, is 3.45%. Accuracy is improved between indexes 110 and 255 of the legacy table.

An alternative is to use Bmin=2642 Bytes and Bmax=2 343 750 Bytes based on currently known video rates as described above. Then, the achieved error rate is 2.6%, and the range of re-defined indexes is from approximately 89 to 196. Note that the Bmax value is achieved for two frames from a 150 Mbps @24 FPS stream with the optional PDB=60 ms from [2]. Of the options discussed above, this one covers all currently considered video data and frame rates and considers the frame sizes and PDBs defined in [2].

Proposal 1: the following values are defined for the new 8-bit table: Bmin=2642 Bytes and Bmax=2 343 750 Bytes

Finally, it should be noted that with such wide ranges, the exponential distribution is most suitable for the table values definition.

Proposal 2: exponential distribution is used to define the new table values.

Based on the proposal 1 and 2, the resulting BSR table is shown in Table 1.

Table 1 - Proposed BSR table implementing proposals 1-2

| Index | BS value | Index | BS value | Index | BS value | Index | BS value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | ≤ 2642 | 64 | ≤ 14516 | 128 | ≤ 79745 | 192 | ≤ 438115 |
| 1 | ≤ 2714 | 65 | ≤ 14907 | 129 | ≤ 81897 | 193 | ≤ 449934 |
| 2 | ≤ 2787 | 66 | ≤ 15309 | 130 | ≤ 84106 | 194 | ≤ 462071 |
| 3 | ≤ 2862 | 67 | ≤ 15722 | 131 | ≤ 86375 | 195 | ≤ 474537 |
| 4 | ≤ 2939 | 68 | ≤ 16146 | 132 | ≤ 88705 | 196 | ≤ 487338 |
| 5 | ≤ 3019 | 69 | ≤ 16582 | 133 | ≤ 91098 | 197 | ≤ 500485 |
| 6 | ≤ 3100 | 70 | ≤ 17029 | 134 | ≤ 93555 | 198 | ≤ 513987 |
| 7 | ≤ 3184 | 71 | ≤ 17489 | 135 | ≤ 96079 | 199 | ≤ 527852 |
| 8 | ≤ 3270 | 72 | ≤ 17960 | 136 | ≤ 98671 | 200 | ≤ 542092 |
| 9 | ≤ 3358 | 73 | ≤ 18445 | 137 | ≤ 101333 | 201 | ≤ 556716 |
| 10 | ≤ 3448 | 74 | ≤ 18942 | 138 | ≤ 104067 | 202 | ≤ 571735 |
| 11 | ≤ 3541 | 75 | ≤ 19453 | 139 | ≤ 106874 | 203 | ≤ 587158 |
| 12 | ≤ 3637 | 76 | ≤ 19978 | 140 | ≤ 109757 | 204 | ≤ 602998 |
| 13 | ≤ 3735 | 77 | ≤ 20517 | 141 | ≤ 112718 | 205 | ≤ 619265 |
| 14 | ≤ 3836 | 78 | ≤ 21071 | 142 | ≤ 115759 | 206 | ≤ 635971 |
| 15 | ≤ 3939 | 79 | ≤ 21639 | 143 | ≤ 118882 | 207 | ≤ 653128 |
| 16 | ≤ 4045 | 80 | ≤ 22223 | 144 | ≤ 122089 | 208 | ≤ 670747 |
| 17 | ≤ 4154 | 81 | ≤ 22822 | 145 | ≤ 125382 | 209 | ≤ 688842 |
| 18 | ≤ 4267 | 82 | ≤ 23438 | 146 | ≤ 128765 | 210 | ≤ 707425 |
| 19 | ≤ 4382 | 83 | ≤ 24070 | 147 | ≤ 132238 | 211 | ≤ 726509 |
| 20 | ≤ 4500 | 84 | ≤ 24720 | 148 | ≤ 135806 | 212 | ≤ 746108 |
| 21 | ≤ 4621 | 85 | ≤ 25386 | 149 | ≤ 139469 | 213 | ≤ 766235 |
| 22 | ≤ 4746 | 86 | ≤ 26071 | 150 | ≤ 143232 | 214 | ≤ 786906 |
| 23 | ≤ 4874 | 87 | ≤ 26775 | 151 | ≤ 147096 | 215 | ≤ 808134 |
| 24 | ≤ 5005 | 88 | ≤ 27497 | 152 | ≤ 151064 | 216 | ≤ 829935 |
| 25 | ≤ 5140 | 89 | ≤ 28239 | 153 | ≤ 155139 | 217 | ≤ 852324 |
| 26 | ≤ 5279 | 90 | ≤ 29000 | 154 | ≤ 159324 | 218 | ≤ 875318 |
| 27 | ≤ 5421 | 91 | ≤ 29783 | 155 | ≤ 163622 | 219 | ≤ 898931 |
| 28 | ≤ 5568 | 92 | ≤ 30586 | 156 | ≤ 168036 | 220 | ≤ 923181 |
| 29 | ≤ 5718 | 93 | ≤ 31411 | 157 | ≤ 172569 | 221 | ≤ 948086 |
| 30 | ≤ 5872 | 94 | ≤ 32259 | 158 | ≤ 177225 | 222 | ≤ 973662 |
| 31 | ≤ 6030 | 95 | ≤ 33129 | 159 | ≤ 182006 | 223 | ≤ 999929 |
| 32 | ≤ 6193 | 96 | ≤ 34023 | 160 | ≤ 186916 | 224 | ≤ 1026904 |
| 33 | ≤ 6360 | 97 | ≤ 34940 | 161 | ≤ 191958 | 225 | ≤ 1054607 |
| 34 | ≤ 6532 | 98 | ≤ 35883 | 162 | ≤ 197137 | 226 | ≤ 1083057 |
| 35 | ≤ 6708 | 99 | ≤ 36851 | 163 | ≤ 202455 | 227 | ≤ 1112274 |
| 36 | ≤ 6889 | 100 | ≤ 37845 | 164 | ≤ 207916 | 228 | ≤ 1142280 |
| 37 | ≤ 7075 | 101 | ≤ 38866 | 165 | ≤ 213525 | 229 | ≤ 1173095 |
| 38 | ≤ 7266 | 102 | ≤ 39914 | 166 | ≤ 219286 | 230 | ≤ 1204742 |
| 39 | ≤ 7462 | 103 | ≤ 40991 | 167 | ≤ 225201 | 231 | ≤ 1237242 |
| 40 | ≤ 7663 | 104 | ≤ 42097 | 168 | ≤ 231276 | 232 | ≤ 1270619 |
| 41 | ≤ 7870 | 105 | ≤ 43233 | 169 | ≤ 237516 | 233 | ≤ 1304896 |
| 42 | ≤ 8082 | 106 | ≤ 44399 | 170 | ≤ 243923 | 234 | ≤ 1340099 |
| 43 | ≤ 8300 | 107 | ≤ 45597 | 171 | ≤ 250503 | 235 | ≤ 1376250 |
| 44 | ≤ 8524 | 108 | ≤ 46827 | 172 | ≤ 257261 | 236 | ≤ 1413377 |
| 45 | ≤ 8754 | 109 | ≤ 48090 | 173 | ≤ 264201 | 237 | ≤ 1451506 |
| 46 | ≤ 8990 | 110 | ≤ 49387 | 174 | ≤ 271328 | 238 | ≤ 1490663 |
| 47 | ≤ 9232 | 111 | ≤ 50719 | 175 | ≤ 278648 | 239 | ≤ 1530877 |
| 48 | ≤ 9481 | 112 | ≤ 52088 | 176 | ≤ 286165 | 240 | ≤ 1572175 |
| 49 | ≤ 9737 | 113 | ≤ 53493 | 177 | ≤ 293885 | 241 | ≤ 1614588 |
| 50 | ≤ 10000 | 114 | ≤ 54936 | 178 | ≤ 301813 | 242 | ≤ 1658144 |
| 51 | ≤ 10270 | 115 | ≤ 56418 | 179 | ≤ 309955 | 243 | ≤ 1702876 |
| 52 | ≤ 10547 | 116 | ≤ 57940 | 180 | ≤ 318317 | 244 | ≤ 1748814 |
| 53 | ≤ 10831 | 117 | ≤ 59503 | 181 | ≤ 326904 | 245 | ≤ 1795992 |
| 54 | ≤ 11123 | 118 | ≤ 61108 | 182 | ≤ 335723 | 246 | ≤ 1844442 |
| 55 | ≤ 11423 | 119 | ≤ 62757 | 183 | ≤ 344780 | 247 | ≤ 1894200 |
| 56 | ≤ 11731 | 120 | ≤ 64450 | 184 | ≤ 354081 | 248 | ≤ 1945300 |
| 57 | ≤ 12048 | 121 | ≤ 66188 | 185 | ≤ 363633 | 249 | ≤ 1997778 |
| 58 | ≤ 12373 | 122 | ≤ 67974 | 186 | ≤ 373442 | 250 | ≤ 2051672 |
| 59 | ≤ 12707 | 123 | ≤ 69807 | 187 | ≤ 383517 | 251 | ≤ 2107020 |
| 60 | ≤ 13049 | 124 | ≤ 71691 | 188 | ≤ 393863 | 252 | ≤ 2163861 |
| 61 | ≤ 13401 | 125 | ≤ 73625 | 189 | ≤ 404488 | 253 | ≤ 2222235 |
| 62 | ≤ 13763 | 126 | ≤ 75611 | 190 | ≤ 415400 | 254 | ≤ 2282184 |
| 63 | ≤ 14134 | 127 | ≤ 77651 | 191 | ≤ 426606 | 255 | ≤ 2343750 |

## 2.2	Co-existence of the legacy BSR tables and the new table

The new table provides finer granularity for the legacy table from index 89 to 196. In terms of table switch criteria this can be done as follows:

- For the low-end range check, if an LGC is enabled to use the new XR BSR table, it will:

  - Use legacy table index 88 with value of 2533 Bytes for low switching threshold.

  - A BS value below 2533 Bytes will trigger a fallback to legacy table.

  - A BS value between 2533 and 2642 Bytes would be reported with the new XR BSR table of index 0.

- For the high-end range check, if an LGC is enabled to use the new XR BSR table, it will:

  - Use new XR BSR table index 255 with value of 2343750 Bytes for high switching threshold.

  - A BS value above 2343750 will trigger a fallback to legacy table.

For the LCH/LCG configured to use new BSR table, only Long BSR is supported when new BS table is used for the LCG since only 8 bits new table is supported. Thus, even if there is only one LCG with data available for transmission, it should report Long BSR if the new table is used.

It is also to be noted that even though 8-bit BS also supported for short BSR for IAB, it makes more sense to use Long BSR since the UE anyway supports Long BSR but not necessarily 8-bit BS Short BSR.

Proposal 3: for regular BSR, UE uses 8-bit table if new BS table is configured for the LCG with Long BSR even if there is only one LCG with data available for transmission.

Since there could also be traffic other than the video traffic, e.g., voice, pose, RRC signaling etc., legacy tables are still to be used for those LCGs. Besides, when the remaining data of the video traffic becomes out of range of the new table, it makes sense to fallback to legacy table to provide enough information for lower range BS values.

Two possible ways to design the BSR to report LCGs with both new table and the legacy table:

- Option 1: Define new MAC CE which could be used to report LCGs using new table as well as the legacy table (for both fallback case and the LCGs not configured to use new table) and explicitly indicate in the MAC CE which table is used.

- Option 2: the new MAC CE is only used to report LCGs using the new table, while the legacy MAC CE is used to report the LCGs using the legacy table (for both fallback case and the LCGs not configured to use new table).

With option 1, there would always be the overhead of table indication in the MAC CE even if there is only one LCG or only LCGs using either new or legacy table. While with option 2, the LCID for new MAC CE or legacy MAC CE could indicate which table is used which is more efficient for only one LCGs to be reported but on the other hand less efficient when there are LCGs using both new and legacy table.

Proposal 4: when the remaining data for the LCG configured with new table falls out of the range of the new table, it falls back to use legacy 8-bits BSR table, i.e., a LCG configured to use new table can be reported in legacy MAC CE as well.

Proposal 5: the MAC CE using new BSR table and the MAC CE using legacy BSR table for different LCGs are identified with different LCIDs and they can be included in the same TB, the NW knows those reported LCGs used legacy table or the new table without other explicit indication.

When there are BSR MAC CEs with both legacy BS table and new BS table for different LCGs to be reported, the priority between the two MAC CEs should be defined in case there are not enough space to include both in the MAC PDU.

Proposal 6: priority of the BSR MAC CEs is according to the highest priority of the LCH with data available for transmission.

It could also be possible to have exception for padding BSR since it is best effort basis, e.g., to always use the legacy BS table for all the LCG to be reported if there is at least one LCG using the legacy table since the legacy table covers the full range. Or it could also be separate reported according to the priority of the BSR MAC CE. Also, in case there is only one byte left, it makes sense to use legacy 5 bits BS table for the LCG configured with new table.

Proposal 7: legacy 5-bit BS table can be used for the LCG configured with new table for padding BSR in case there is only one byte left for BS payload.

## 2.3	Discard impact on BSR

The impact of discarding at the PDCP on the lower layers is often overlooked as the assumption is that the discarded PDUs will not be visible to RLC/MAC. However, discarding packets in the transmitter makes previous BSR obsolete (for as long as a new BSR is not received) and could lead to scheduling more resources than needed, which in turn increases power consumption and decrease overall cell capacity. It is therefore proposed to trigger a BSR whenever discard occurs.

Proposal 8: packet discard of amount above a threshold triggers a BSR.

Besides, currently BS value is calculated based on current buffer status after the TB for the BSR is built. If discard happens more often for XR than in legacy, it is worth considering the potential discarding as well in case the data would be discarded before the next possible transmission possibility, e.g., due to TDD configuration.

Proposal 9: the UE takes into account the potential discarding and next possible UL transmission opportunity when determine the data available for transmission for BS value.

## 2.4	Delay Information

The following FFSes were identified from the previous discussions on delay stature report:

- Whether configuring multiple thresholds for a LCG is supported.

- Definition of remaining time

- Detailed definition of associated data volume

Since it was already agreed the threshold is configured per LCG, it could be enough to leave with only one threshold and the NW may configure different thresholds for different LCGs taking the delay budget of the LCHs within the LCGs into account.

Proposal 10: one threshold per LCG is enough for delay status report.

There has been some discussion about whether the remaining time value would need to be reported in the delay status report if there is only one threshold per LCG as all the data reported would be associated to the remaining time below the configured threshold.  It should be noted that there might be some delay between the delay status report is triggered and when it can be actually included in a TB. The NW typically configure some margin for the trigger threshold considering the potential delay between the trigger and report. It is beneficial to include the actual remaining time in the report to let the gNB how much remaining time is left.

For the remaining time report definition, we think it could be enough to report the shortest remaining time . The value can be reported by sending an integer or index pointing to a predefined table. The table includes different threshold ranges such that the UE selects the one fitting its remaining time.

Proposal 11: the shortest remaining time of buffered data is reported for the LCG.

Proposal 12: the remaining time can be reported as an integer or an index pointing out the threshold fitting its remaining time from a predefined table.

Since the PDUs could have different remaining time, simplest could be to report the data volume with remaining time below the configured threshold.

Proposal 13: data volume associated with remaining time reported in the delay status report is the data with remaining time below the configured threshold.

How would the UE handle the delay report multiplexed in a MAC PDU that is not being able to be transmitted in PHY due to Intra-UE prioritization was briefly discussed in the previous meeting. In our view, it could be left to UE implementation similar to the NOTE in MAC which captured the case for NR-U when the built MAC PDU cannot be sent.

NOTE 5: If a HARQ process is configured with cg-RetransmissionTimer and if the BSR is already included in a MAC PDU for transmission on configured grant by this HARQ process, but not yet transmitted by lower layers, it is up to UE implementation how to handle the BSR content.

Proposal 14: similar to BSR, up to UE implementation to update the delay information content in case the TB with delay information multiplexed is deprioritized due to intra-UE prioritization.

# 3	Conclusion

This contribution has discussed BSR enhancements and delay information report for XR with the following proposals proposed:

Proposal 1: the following values are defined for the new 8-bit table: Bmin=2642 Bytes and Bmax=2 343 750 Bytes

Proposal 2: exponential distribution is used to define the new table values.

Proposal 3: for regular BSR, UE uses 8-bit table if new BS table is configured for the LCG with Long BSR even if there is only one LCG with data available for transmission.

Proposal 4: when the remaining data for the LCG configured with new table falls out of the range of the new table, it falls back to use legacy 8-bits BSR table, i.e., a LCG configured to use new table can be reported in legacy MAC CE as well.

Proposal 5: the MAC CE using new BSR table and the MAC CE using legacy BSR table for different LCGs are identified with different LCIDs and they can be included in the same TB, the NW knows those reported LCGs used legacy table or the new table without other explicit indication.

Proposal 6: priority of the BSR MAC CEs is according to the highest priority of the LCH with data available for transmission.

Proposal 7: legacy 5-bit BS table can be used for the LCG configured with new table for padding BSR in case there is only one byte left for BS payload.

Proposal 8: packet discard of amount above a threshold triggers a BSR.

Proposal 9: the UE takes into account the potential discarding and next possible UL transmission opportunity when determine the data available for transmission for BS value.

Proposal 10: one threshold per LCG is enough for delay status report.

Proposal 11: the shortest remaining time of buffered data is reported for the LCG.

Proposal 12: the remaining time can be reported as an integer or an index pointing out the threshold fitting its remaining time from a predefined table.

Proposal 13: data volume associated with remaining time reported in the delay status report is the data with remaining time below the configured threshold.

Proposal 14: similar to BSR, up to UE implementation to update the delay information content in case the TB with delay information multiplexed is deprioritized due to intra-UE prioritization.

# Reference

- TR 26.998, Support of 5G glass-type Augmented Reality / Mixed Reality (AR/MR) devices, V18.0.0, Jan 2023.

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR, V17.0.0, Dec 2021.

- R2-2300256, Dynamic BSR formulation and reporting for XR, Dell, Feb 2023.

- R2-2304711, BSR and delay status reporting for XR, Qualcomm, May 2023.