3GPP TSG-RAN WG2 Meeting #123bis	R2-2310702

Xiamen, China, 09 – 13 October 2023

Agenda item:	7.9.3

Source:	Nokia, Nokia Shanghai Bell

Title:	SL Relay service continuity considerations

WID/SID:	NR_SL_relay_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The approved release 18 WI on enhanced NR SL Relay [RP-221262] includes the following objective on enhancement of service continuity:

- Specify mechanisms to enhance service continuity for single-hop Layer-2 UE-to-Network relay for the following scenarios [RAN2, RAN3]:

  - Inter-gNB indirect-to-direct path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> gNB Y”)

  - Inter-gNB direct-to-indirect path switching (i.e., “remote UE <-> gNB X” to “remote UE <-> relay UE A <-> gNB Y”)

  - Intra-gNB indirect-to-indirect path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB X”)

  - Inter-gNB indirect-to-indirect path switching (i.e., “remote UE<-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB Y”)

Note 2A: Scenario D is to be supported by reusing solutions for the other scenarios without specific optimizations.

This contribution discusses the issues that impact the service continuity of L2 U2N relay for inter-gNB path switching scenario (i.e., scenario A, B and D) and proposes the solutions to be discussed in RAN2.

Moreover, this paper also discusses the issue related to SL-RSRP and SD-RSRP reporting.

# 2	Discussion

## 2.1	Issues related to relay UE’s inter-gNB HO during connected remote UE’s path switching

When the remote UE has already established U2N relay connection via the relay UE with the serving gNB, the remote UE’s UP and CP traffic transmission with the serving gNB are forwarded via the relay UE. In rel-17, when the relay UE is configured by the serving gNB to handover (HO) to another gNB, the relay UE sends a NotificationMessageSidelink message to the remote UE to indicate relay UE’s HO, which triggers the remote UE to initiate RRC connection re-establishment procedure. The RRC connection re-establishment procedure will impact the remote UE’s service continuity. In Rel-17, the remote UE can only be triggered to initiate the RRC connection re-establishment procedure as no inter-gNB path switching is supported for the remote UE in rel-17. As inter-gNB I2I or I2D path switching is supported in rel-18, the remote UE can be configured by the serving gNB to make the path switching. The main benefit of this enhancement is that there will be no break in the service of the remote UE.

Observation 1.1: The remote UE would benefit from being able to perform path switching instead of initiating RRC connection re-establishment when the connected relay UE performs a HO.

Proposal 1.1: RAN2 to agree the remote UE can be configured to perform path switching when the connected relay UE performs a HO.

If Proposal 1 is agreed, the serving gNB may configure the remote UE to perform path switching when it configures the relay UE to perform inter-gNB HO. When the relay UE receives RRC reconfiguration message (i.e., HO command message in HO case) from its own serving gNB to configure the relay UE to perform HO, the relay UE will start execution of the configured HO by detaching from the serving cell and synchronize to the target cell. As the remote UE’s RRC reconfiguration message for path switching is transmitted from the serving gNB to the remote UE via the relay UE, it may happen that the relay UE may detach from the serving cell before the relay UE receives the RRC reconfiguration message of the remote UE over Uu interface from the serving gNB. In this case the remote UE will not receive the RRC reconfiguration message from the serving gNB, thus the remote UE’s RRC connection to the current serving gNB will be interrupted and the ongoing services of the remote UE will be impacted.

It may be considered to solve the problem by the gNB triggering the remote UE’s RRC reconfiguration procedure for path switching earlier than the relay UE’s RRC reconfiguration for HO. However, it is not practical or feasible due to the following reasons:

- It may be too late to initiate RRC reconfiguration procedure to the relay UE for HO if the gNB needs to wait for the completion of the remote UE’s RRC reconfiguration procedure. This may impact robustness of relay UE’s HO, especially, in high mobility scenario that is typical scenario for many V2X use cases.

- In normal HO procedure, the RRCReconfigurationComplete message from the UE is not sent to the serving gNB (source gNB), but to the target gNB. In this case with regards to path switching of the remote UE, the serving gNB is not able to ensure the RRC reconfiguration message has been delivered to the remote UE before it initiates the RRC reconfiguration procedure to the relay UE for HO as there is no end-to-end RLC or MAC protocol between the gNB and the remote UE to allow the gNB be aware of the delivery of RRC reconfiguration message by ARQ or HARQ feedback.

Observation 1.2: The path switch command for the remote UE and the HO command for the relay UE are independent messages, and it is not possible for the gNB to ensure the path switch configuration arrives before the HO command.

To avoid the relay UE detaching from the serving cell before the relay UE receives the RRC Reconfiguration message of the remote UE from the serving gNB over Uu interface, the relay UE should be explicitly informed that the remote UE’s RRC Reconfiguration message has been delivered to the remote UE. For instance, the remote UE may indicate to the relay UE the reception of the RRC Reconfiguration message from the gNB if the relay UE sent the NotificationMessageSidelink message to indicate relay UE’s HO. Upon the indication from the remote UE, the relay UE can detach from the serving cell to perform the configured HO.

Proposal 1.2: RAN2 to agree that the relay UE should be explicitly informed about the reception of the RRC reconfiguration message by the remote UE and then the relay UE can detach from the serving cell for the HO.

## 2.2	Procedure of inter-gNB path switching to indirect path

The stage 2 procedure on inter-gNB path swithing from direct path to indirect path or from indirect-to-indirect path has not been added in the endorsed stage 2 TS38.300 running CR yet. Assuming that the HO preparation procedure over Xn interface is reused for inter-gNB path switching, the proposed flow charts for inter-gNB D2I and I2I path switching are illustrate in Figure 1 and Figure 2 respectively for the case when the target relay UE is in RRC_Connected.

Figure 1: Proposed procedure for inter-gNB path switching from direct path to indirect path

Figure 2: Proposed procedure for inter-gNB path switching from indirect path to indirect path

Proposal 2.1: RAN2 to agree to use Figure 1 and 2 as the baseline for inter-gNB path switching to indirect path.

The proposed flow charts in Figure 1 and Figure 2 follow the procedure of intra-gNB indirect path switching, the target gNB should first configure the target relay UE for the remote UE using RRC Reconfiguration procedure before it can send Handover Req Ack to the source gNB of the remote UE. The additional RRC reconfiguration procedure between the target gNB and the selected target relay UE will prolong the path switching preparation time before the source gNB can send path switching command to the remote UE.

Observation 2.1: When inter-gNB path switch to indirect path is performed, the Relay UE configuration delays the target gNB to send the acknowledgement for path switching.

During the prolonged path switching preparation time, the probability of missing the path switching command transmitted from the source gNB for configuring path switching becomes higher than in normal inter-gNB HO scenario. For example, during the prolonged path switching preparation procedure over Xn, the remote UE may already move out of coverage of the current serving cell if the remote UE has direct path with the source gNB, or if the remote UE has indirect path with the source gNB, it may happen that the relay UE moves out of coverage of source cell or the SL connection between the remote UE and the currently connected relay UE fails and thus sending the RRCReconfiguration message to the remote UE may fail. Missing of the RRCReconfiguration message by the remote UE, as illustrated in Figure 3, will cause path switching failure, which will impact the remote UE’s service continuity.

Observation 2.2: Missing of the RRCReconfiguration message by the remote UE can easily become a frequent problem due to prolonged path switching preparation time caused by inter-gNB path switching.

Figure 3: Procedure of inter-gNB path switching to indirect path with failure of RRC reconfiguration

To enhance the remote UE’s service continuity for inter-gNB path switching, RAN2 should discuss the possible solution to deal with the issue. One of the solutions is to allow the source gNB to send an “early” RRCReconfiguration message to the remote UE for path switching before performing the RRC reconfiguration for the relay connection in the relay UE as depicted in Figure 4. In this early RRCReconfiguration message, the source gNB may provide the RRC reconfiguration parameters in the similar way as the target relay UE is in RRC_Idle/Inactive state according to the configuration parameter provided by the target gNB in step 4 of Figure 4. Upon the successful RRC Reconfiguration procedure between relay UE and the target gNB as illustrated in step 6 of Figure 4, the target relay UE can assist the remote UE’s inter-gNB path switching by forwarding the RRC Reconfiguration message to the remote UE.

Figure 4: Enhanced procedure of inter-gNB path switching to indirect path

Proposal 2.2: RAN2 to agree to allow early RRCReconfiguration message to the remote UE for path switching and to use the target relay UE to assist the remote UE’s inter-gNB path switching.

## 2.3	SL-RSRP and SD-RSRP

It has been confirmed that the direct comparison of measurements of SL-RSRP with measurements of SD-RSRP is not meaningful (see LS from RAN1 in R2-2304617 and LS from RAN4 in R2-2304637).

At RAN2#122 the following agreement were made related to SL-RSRP and SD-RSRP:

Agreements:

Any operation based on direct comparison between the SD-RSRP and SL-RSRP measured at the Remote UE side is not supported in Rel-18.

The previous agreement from RAN2#119bis-e is revised as follows: For i2i scenario, for serving U2N relay UEs, when SL-RSRP is unavailable, SD-RSRP is used as the measurement quantity. And for candidate U2N relay UEs, only SD-RSRP is used as the measurement quantity.”

At RAN2#123 the following agreement was made related to SL-RSRP and SD-RSRP:

Separate thresholds for SL-RSRP and SD-RSRP can be configured for the threshold1 in Z1.  This does not imply that the network is required to configure two different values.

The agreement made at RAN2#123 solves the issue for Z1 event trigger related to SL-RSRP and SD-RSRP comparison. However, it does not solve the issue that the gNB does not know whether SL-RSRP or SD-RSRP is reported as the measurement result when the UE reports the SL measurement on serving L2 U2N relay.

Observation 3.1: The ambiguity that the network does not know whether SL-RSRP or SD-RSRP of the serving L2 U2N relay is reported by the L2 U2N remote UE has not been solved.

This may cause a problem in the gNB to decide whether path switch should be performed or not; e.g., if the reported RSRP is lower than the SL-RSRP threshold, then the gNB cannot know if it is an SL-RSRP or SD-RSRP, and thus the gNB cannot know that SD-RSRPs reported about candidate relay UEs mean better or worse SL radio connections.

Observation 3.2: The ambiguity that the network does not know whether SL-RSRP or SD-RSRP of the serving L2 U2N relay is reported by the L2 U2N remote UE may make it difficult for the gNB to make a good decision whether to perform a path switch or not.

Proposal 3.1: RAN2 agree that a solution is needed to remove the ambiguity of the measured RSRP values of the serving L2 U2N relay in the reports provided by the L2 U2N remote UE.

A simple solution for differentiating SL-RSRP and SD-RSRP reports would be to add an indication to the report which measurement is reported. The drawback of this approach is that it requires ASN.1 changes.

Another approach that requires no ASN.1 change is to enhance the reported measurement result by compensating SL-RSRP measurement with the level of the applied transmit power control on SL transmission so that the reported SL-RSRP measurement result of the serving L2 U2N relay can be comparable with SD-RSRP measurement report of the candidate L2 U2N relay UEs. There are a couple of options how this compensation could happen.

Option A: During the Relay discovery a hysteresis is used on top of SL-RSRP threshold to determine the SD-RSRP thresholds for candidate U2N relay UEs (clause 5.8.15.3 of TS 38.331):

4>	consider a candidate NR sidelink U2N Relay UE for which SD-RSRP exceeds sl-RSRP-Thresh by sl-HystMin has met the AS criteria;

Therefore, it is reasonable to assume that compensating the SL-RSRP measurements with this HystMin makes it comparable to SD-RSRP measurements. Thus, the L2 U2N remote UE reports the SD-RSRP or SL-RSRP compensated with this hysteresis can help to eliminate the problem that the gNB does not know if it is an SD-RSRP or an SL-RSRP value.

Another approach could be to use SL-RSRP and SD-RSRP thresholds provided for Z1. It can be assumed that the gNB provided SL-RSRP and SD-RSRP thresholds for Z1 event correspond to similar level SL radio link quality, thus compensating the measured SL-RSRP value with difference between the SL-RSRP and SD-RSRP thresholds could make the reported RSRP value independent whether it is based on SL-RSRP or SD-RSRP measurement.

Option B: Since the L2 U2N Remote UE knows its own power control parameters for a given PC5 unicast link, the UE may compensate the measured SL-RSRP with the difference between its maximum and the actually used transmission power over the given unicast link. In this way the compensated SL-RSRP value will be similar to a measured RSRP value transmitted with the maximum power of the L2 U2N Remote UE. As the gNB may know the difference between the maximum transmission power of the UEs, the report of SL-RSRP value compensated with the difference between the maximum transmission power and the used transmission power can be used by the gNB to estimate the PC5 link radio quality and compare the reported RSRP values regardless whether it is SL-RSRP or SD-RSRP.

Option C: The L2 U2N Remote UE knows the pathloss measured over a PC5 unicast link. As it can be assumed that the pathloss is similar in both directions, the measured SL-RSRP compensated with the value corresponding to the pathloss would give a good estimation of the SL-RSRP measurement if the maximum transmission power used by the L2N Relay UE. When the pathloss based compensated SL-RSRP measurement is reported to the gNB, the gNB can compare the PC5 link quality of serving L2 U2N relay UE and candidate L2 U2N relay UEs as the gNB can know the maximum transmission value of the serving L2 U2N Relay UE and the SL discovery message transmission power of the candidate L2 U2N relay UEs.

Proposal 3.2: RAN2 to discuss how to enhance the value reported in sl-MeasResult to remove the ambiguity on the reported value:

1)	Adding a new flag that indicates whether the reported value is an SL-RSRP or an SD-RSRP

2)	Reporting a compensated value instead of the measured SL-RSRP:

2a)		The measured SL-RSRP is increased, by the reporting UE by the offset provided for discovery or by the difference of SL-RSRP and SD-RSRP thresholds provided for Z1.

2b)		The measured SL-RSRP is increased, by the reporting UE by the difference between its maximum and the actually used transmission power over the given PC5 unicast link.

2c)		The measured SL-RSRP is increased, by the reporting UE, by the value corresponding to the pathloss over the given PC5 unicast link.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1.1: The remote UE would benefit from being able to perform path switching instead of initiating RRC connection re-establishment when the connected relay UE performs a HO.

Proposal 1.1: RAN2 to agree the remote UE can be configured to perform path switching when the connected relay UE performs a HO.

Observation 1.2: The path switch command for the remote UE and the HO command for the relay UE are independent messages, and it is not possible for the gNB to ensure the path switch configuration arrives before the HO command.

Proposal 1.2: RAN2 to agree that the relay UE should be explicitly informed about the reception of the RRC reconfiguration message by the remote UE and then the relay UE can detach from the serving cell for the HO.

Proposal 2.1: RAN2 to agree to use Figure 1 and 2 as the baseline for inter-gNB path switching to indirect path.

Observation 2.1: When inter-gNB path switch to indirect path is performed, the Relay UE configuration delays the target gNB to send the acknowledgement for path switching.

Observation 2.2: Missing of the RRCReconfiguration message by the remote UE can easily become a frequent problem due to prolonged path switching preparation time caused by inter-gNB path switching.

Observation 3.1: The ambiguity that the network does not know whether SL-RSRP or SD-RSRP of the serving L2 U2N relay is reported by the L2 U2N remote UE has not been solved.

Observation 3.2: The ambiguity that the network does not know whether SL-RSRP or SD-RSRP of the serving L2 U2N relay is reported by the L2 U2N remote UE may make it difficult for the gNB to make a good decision whether to perform a path switch or not.

Proposal 3.1: RAN2 agree that a solution is needed to remove the ambiguity of the measured RSRP values of the serving L2 U2N relay in the reports provided by the L2 U2N remote UE.

Proposal 3.2: RAN2 to discuss how to enhance the value reported in sl-MeasResult to remove the ambiguity on the reported value:

1)	Adding a new flag that indicates whether the reported value is an SL-RSRP or an SD-RSRP

2)	Reporting a compensated value instead of the measured SL-RSRP:

2a)		The measured SL-RSRP is increased, by the reporting UE by the offset provided for discovery or by the difference of SL-RSRP and SD-RSRP thresholds provided for Z1.

2b)		The measured SL-RSRP is increased, by the reporting UE by the difference between its maximum and the actually used transmission power over the given PC5 unicast link.

2c)		The measured SL-RSRP is increased, by the reporting UE, by the value corresponding to the pathloss over the given PC5 unicast link.