3GPP TSG-RAN WG2 #123bis	R2-2310747

Xiamen, China, 2023-10-09 - 2023-10-13

Agenda Item:	7.13.7

Source:	Ericsson

Title:	SON Support for NPN

Document for:	Discussion, Decision

# 1	Introduction

During RAN2#123 meeting the following agreements were taken

Agreements:

1	Include SNPN ID (list) in the logged MDT area configuration following RAN3 agreement to align with the future NPN evolution.

2	No new UE variables will be introduced for PNI-NPNs.

3	UE performs SNPN ID checking before transmitting the information for corresponding SON and MDT reports, upon the network requests for it.

4	Assuming ESNPN is supported, include a list of SNPN IDs in the logged MDT report.

Furthermore, the following FFSs were identified and are under discussion.

FFS1: Include UE CAG subscription information in the RLF/HOF report:

-	CAG subscription statues indication;

-	CAG-only indication.

FFS2: RAN2 to discuss whether and how to address the loss issue of logged MDT report when UE switches between SNPN and PN and then send RAN2 decision to RAN3.

-	Option 1: Introducing new variables for SNPNs;

-	Option 2: Storing only the collected MDT measurements report (UE deletes the MDT configuration as legacy);

-	Option 3：No enhancement is needed;

FFS3:RAN2 to discuss:

-	Whether and how to introduce information reporting for OOC analysis involving NPN network;

-	Whether and which to introduce other SON/MDT enhancements for NPN in this Release.

This contribution aims to discuss the SON and MDT enhancements for private networks optimization including both SNPN and PNI-NPN scenarios considering the above agreements and the FFSs into account.

# 2	Discussion

## 2.1	Enhancements of SON Reports

### 2.1.1 Enhancements of RA report

In RAN2#120 meeting, the discussions have been around checking NID for RLF/HOF report. In our opinion, it should be extended to RA Report also.

As per current specifications, UE checks the PLMN information to determine if the stored variable should be cleared.

1>	if the RPLMN or the PLMN selected by upper layers (see TS24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1 is not included in plmn-IdentityList stored in a non-empty VarRA-Report:

2> clear the information included in VarRA-Report;

However, if the UE moves between SNPN-1 and SNPN-2, current specifications does not restrict the UE to log RA report for SNPN-1 into RA report for SNPN-2. This is against RACH principle and needs to be blocked at the UE.

Upon logging the RA report for a new SNPN, the UE deletes the existing RA reports belonging to the previous SNPNs/PLMN.

On the other hand, if two SNPNs are served by same cell and the cell has separate RACH resources allocated for each SNPN, it would be useful for the network to receive NID in the RA Report from the UE. The network can perform additional optimizations and load balancing between RACH resources based on the resource utilization reported by the UE.

UE logs NID in the RA report.

### 2.1.2 Enhancements of the RLF/HOF report

An FFS under discussion is to how to include CAG information in the RLF/HOF report. The following options are under discussion-

-	Option 1: CAG subscription status indication (if the UE has subscription with any of the CAG IDs broadcast by the cell, and whether the UE only allowed to access CAG cells);

-	Option 2: CAG-only indication

-	Option 3: Other information, if any.

In our observation, neither option 1 nor option 2 provides any clear benefits for MRO purposes. From the indication, network can find the UEs CAG subscription status and if the UE is a CAG-only UE or not. Although this adds observability to the network, it does not aid in MRO. In addition, as of now RAN is not aware about the UE subscription status and we are uncertain about potential concern from privacy point of view.

Inclusion of CAG information in RLF/HOF report may be used for observability purpose but has no clear benefit for MRO.

CAG subscription status is not known in RAN and may be subject to user privacy.

UE does not add CAG information in the RLF/HOF report.

## 2.2 Enhancements of MDT framework

### 2.2.1 Wastefulness of MDT mechanism in co-existence of SNPN and PN

RAN3 has sent an LS [R3-232118] to RAN2; the content of the LS is copied here.

RAN3 noticed that logged MDT reports collected in one type of network, e.g., an SNPN, are lost when UE moves to a network of another type, e.g., a Public Network, due to the UE deregistering from the SNPN network. RAN3 believes that this would lead to a loss of the stored logged MDT reports if the reports are not retrieved before moving to the network of another type.

RAN3 would like to check with RAN2 if there are any solutions to avoid the loss of stored logged MDT reports upon moving from a network of one type to another type, even upon deregistration.

Considering the scenario that SNPN is deployed/managed by the enterprises independently from the PN, the data collection as part of MDT features lend itself special considerations when users connected to the SNPN moves frequently between SNPN and PN. According to the RAN3 observation in such scenario collecting data as part of MDT reports when SNPN and PN co-exist, becomes significantly inefficient specially when the UE moves back and forth between SNPN and PN. Therefore, when a user (holding a UE) moves frequently between SNPN and PN, the SON reports collected in one network (PN) should not be ruined by moving to the other network and vice versa.

A SNPN user (holding an SNPN allowed UE) may move frequently between SNPN and PN which may ruin collected measurements for one network in the other networks.

The current logged MDT framework is wasteful when PN and SNPN co-exist.

In our opinion, a UE with subscription to both public and private networks i.e., SNPN, can log MDT reports associated to the SNPN in a new variable instead. Therefore, the MDT reports associated to PN is not overridden by the other network types i.e., SNPN. With this, the UE can report the associated logged MDT related information to a network wherein the event that triggered the report occurred. However, it should also be considered that resuming collection of logged MDT report once the UE is returned from SNPN to PN is an additional overhead on the UE side. One solution is that UE may delete the logged MDT configuration but store the collected report in a separate variable.

Upon moving to SNPN (and deregistration in PN) UE stores logged MDT report collected in PN in a separate variable and releases the MDT configuration received in PN.

RAN2 to agree on the draft LS reply in the Annex.

If the above proposals are agreeable RAN2 needs to discuss how to assure co-existence of the PN and SNPN MDT reports in particular whether the UE needs to consider additional memory or it is possible to share the existing memories between PN and SNPN MDT reports e.g., sharing 68KB of memory allocated to the logged MDT report or doubling the memory size i.e., 64 KB memory for PN MDT and 64 KB memory for NPN MDT.

RAN2 discuss whether to consider additional memory (e.g., additional 64KB) for storing the PLMN MDT measurements when going to SNPN network.

### 2.2.2 Enhancements of MDT configuration

RAN3 has sent an LS (R3-234744) informing RAN2 regarding MDT are scope configuration enhancements and requested RAN2 to evaluate and perform corresponding changes.

RAN3 would like to ask RAN2 and SA5 to take the above into account, to apply corresponding modification to their specifications, if needed, and to provide feedback on the progress achieved so far on MDT for NPN, if any

The changes mentioned in the LS are shown below.

#### 9.3.1.169	MDT Configuration-NR

This IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace, …) |  | - |  |
| CHOICE Area Scope of MDT | M |  |  |  | - |  |
| >Cell based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  |  |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >TA based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TA List for MDT |  | 1..<maxnoofTAforMDT> |  |  |  |  |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. | - |  |
| >PLMN wide |  |  | NULL | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >TAI based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  |  |  |
| >>>TAI | M |  |  |  | - |  |
| >PNI-NPN Based MDT |  |  |  |  | YES | Ignore |
| >>CAG List for MDT |  | 1..<maxnoofCAGforMDT> |  |  |  |  |
| >>>PLMN ID |  |  | 9.3.3.5 |  |  |  |
| >>>CAG ID | M |  | 9.3.3.43 |  |  |  |
| >SNPN Cell Based MDT |  |  |  |  | YES | ignore |
| >>SNPN Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  |  |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - | - |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity in the NR CGI IE. | - | - |
| >SNPN TAI Based MDT |  |  |  |  | YES | ignore |
| >>SNPN TAI List |  | 1..<maxnoofTAforMDT> |  |  | - | - |
| >>>TAI | M |  | 9.3.3.11 |  | - | - |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity in the TAI IE. | - | - |
| >SNPN Based MDT |  |  |  |  |  |  |
| >>MDT SNPN List |  | 1..<maxnoofMDTSNPNs> |  |  |  |  |
| >>>PLMN Identity | M |  | 9.3.3.5 |  |  |  |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity IE. |  |  |
| CHOICE MDT Mode | M |  |  |  | - |  |
| >Immediate MDT |  |  |  |  |  |  |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [41]. First Bit = M1, Second Bit= M2, Third Bit = M4, Fourth Bit = M5, Fifth Bit = M6, Sixth Bit = M7, Seventh Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, other bits reserved for future use. Value “1” indicates “activate” and value “0” indicates “do not activate”. | - |  |
| >>M1 Configuration | C-ifM1 |  | 9.3.1.171 |  | - |  |
| >>M4 Configuration | C-ifM4 |  | 9.3.1.172 |  | - |  |
| >>M5 Configuration | C-ifM5 |  | 9.3.1.173 |  | - |  |
| >>M6 Configuration | C-ifM6 |  | 9.3.1.174 |  | - |  |
| >>M7 Configuration | C-ifM7 |  | 9.3.1.175 |  | - |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  | - |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  | - |  |
| >>MDT Location Information | O |  | 9.3.1.176 |  | - |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  | - |  |
| >Logged MDT |  |  |  |  |  |  |
| >>Logging interval | M |  | ENUMERATED (320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms, 20480ms, 30720ms, 40960ms, 61440ms, infinity, …) | This IE is defined in TS 38.331 [18]. | - |  |
| >>Logging duration | M |  | ENUMERATED (10, 20, 40, 60, 90,120, …) | This IE is defined in TS 38.331 [18]. Unit: [minute]. | - |  |
| >>CHOICE Report Type | M |  |  |  | - |  |
| >>>Periodical |  |  | NULL |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>>Event Trigger Logged MDT Configuration | M |  | 9.3.1.180 |  | - |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  | - |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  | - |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  | - |  |
| >>Area Scope of Neighbour Cells | O |  | 9.3.1.182 |  | - |  |
| >>Early Measurement | O |  | ENUMERATED (true, ...) | This IE indicates whether the UE is allowed to log measurements on early measurement related frequencies in logged MDT as specified in TS 38.331 [18]. | YES | ignore |
| Signalling Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.168 |  | - |  |
| PNI-NPN Area Scope of MDT | O |  | 9.3.3.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |
| maxnoofCAGforMDT | Maximum no. of CAG IDs for MDT scope. Value is 256. |
| maxnoofMDTSNPNs | Maximum no. of SNPNs in the MDT SNPN list. Value is FFS. |

| Condition | Explanation |
| --- | --- |
| C-ifM1 | This IE shall be present if the Measurements to Activate IE has the first bit set to “1”. |
| C-ifM4 | This IE shall be present if the Measurements to Activate IE has the third bit set to “1”. |
| C-ifM5 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to “1”. |
| C-ifM6 | This IE shall be present if the Measurements to Activate IE has the fitth bit set to “1”. |
| C-ifM7 | This IE shall be present if the Measurements to Activate IE has the sixth bit set to “1”. |

In the running CR, changes corresponding to PNI-NPN Based MDT and SNPN Based MDT has been implemented, whereas changes corresponding to SNPN Cell Based MDT remains open.

RAN2 running CR has implemented changes mirroring RAN3 implementation of PNI-NPN Based MDT and SNPN Based MDT.

In our view, when the cell ID or TAC ID are provided NID checking is not needed. The reason is that the UE as part of cell selection/reselection to an SNPN cell already checks the NID and hence a UE accessing a cell in SNPN is already known to be allowed to perform MDT measurements. In other words, a UE without access right to the SNPN (without having the allowed NID list) will not be able to access the SNPN cell, hence a (double) checking of the NID as part of MDT measurements for a specific cell is redundant.

In the network provides SNPN cell ID or TAI as part of area configuration, i.e., in “SNPN Cell Based MDT” and “SNPN TAI Based MDT” checking the NID when logging MDT measurement is not needed.

Thus, any UE not belonging to a SNPN will not be able to select the cell and such granularity of data collection is not required. Hence, we propose RAN3 to revert their decision on configuration of SNPN Cell Based MDT and SNPN TAI based MDT as the legacy cell based and TAI based MDT configuration can be reused for the SNPN.

RAN2 requests RAN3 to revert their implementation on configuring SNPN Cell Based MDT and SNPN TAI Based MDT configuration.

RAN2 to agree on the draft LS reply in the Annex.

## 2.3	Enhancements for OoC events

In addition, some companies proposed to discuss out of coverage (OoC) in MDT report when UE is detecting coverage holes e.g., an area that no sufficient signal level is detected by UE for accessing specific services. In our opinion, when there is no suitable cell for a UE to camp on while the UE is requested to access a specific service (e.g., from upper layers), the UE logs an OoC indication in either RA report or CEF or new report. With this information, the network can identify the OoC for NPN. It is worthwhile to mention that particularly in scenarios that the upper layers (e.g., an SNPN service/application) request the UE to join an SNPN network and UE find no suitable cell for the requested SNPN, there might not be an appropriate logged MDT configuration from the same network at the UE to log the OoC event in the corresponding MDT report. Hence for such critical scenarios (in which the SNPN applications request the UE to register to an SNPN network) we cannot rely on the MDT reports. Hence, we propose:

RAN2 discuss to log an SNPN out-of-coverage (OoC) indication either in

  - an RA report, or

  - a CEF report, or

  - a new report.

## 2.4	Enhancements for MHI

In NR Mobility History information (MHI) report, if the UE is out of NR or LTE coverage, an indication is included to indicate that UE was out of NR or LTE coverage.

1> upon entering 'camped normally' state in NR (in RRC_IDLE or RRC_INACTIVE) or E-UTRA (in RRC_IDLE) while previously in 'any cell selection' state or 'camped on any cell' state in NR or LTE: 2> include an entry in variable VarMobilityHistoryReport possibly after removing the oldest entry, if necessary, according to following: 3> set the field timeSpent of the entry as the time spent in 'any cell selection' state and/or 'camped on any cell' state in NR or LTE.

However, if the UE registers to SNPN, it is not necessarily out of coverage of NR. Thus, upon returning to PN, it is uncertain what it should log in the MHI report for NR.

If a UE served by PN, registers to SNPN and later returns to PN, it is unclear what is logged and reported by the UE in the MHI.

In this scenario, it is important to identify in the MHI of the PN that the UE was served by an SNPN and was not out of coverage. Hence, we propose the UE to log a timeSpent entry in the PN MHI containing time spent in the SNPN indicating it was connected to an SNPN.

UE logs time spent in the SNPN network in an entry in the existing PN MHI report.

Furthermore, as per current specification, if the UE has a MHI report available it includes indication in the RRCRsetupComplete and RRCResumeComplete messages without performing PLMN check. Upon receiving request from the network, UE transmits the MHI to the network. Hence, there is a possibility that UE may report SNPN MHI to PN and vice versa. Such reporting could be avoided if UE performs PLMN check before transmitting MHI report to the network. Moreover, in RAN2#123 meeting, there was an agreement- “UE performs SNPN ID checking before transmitting the information for corresponding SON and MDT reports, upon the network requests for it.”

It is agreed that the UE performs SNPN ID checking before transmitting the information for corresponding SON and MDT reports, upon the network requests for it.

Following the same principle, we propose RAN2 confirms the following proposal.

RAN2 agree that MHI of PLMN should not be reported to the SNPN i.e., PLMN check is needed before MHI availability indication to the network.

RAN2 agree that MHI of the SNPN should not be reported to the PLMN i.e., SNPN checking is needed before MHI availability indication to the network.

## 2.5	Building coverage map for NPN and PN

One important use case of the logged MDT report is to build and analyze the coverage map for a given network. When both PN and NPN are deployed in an area and MDT reports are used to build coverage map of the area, it is important to the network knowing from the measurements in the MDT report which cells provide coverage to NPN and which cells provide coverage to PN. As of now the UE includes the measurements results of the neighboring cells associated with the PCI of the neighboring cells in the logged MDT report. Based on the current solution OAM is not capable to differentiate the network type based on the provided measurements (i.e., PCI and the RSRP measurements of the cells in an area). Hence it is not possible to build a coverage map for a specific network.

In the following, we discuss the required enhancements in the MDT report as well as MDT configuration, to enable the network to build and analyze the coverage map for a specific network e.g., PN or NPN.

    - Enhancement on the MDT report

In our understanding, since a UE only includes PCI and carrier frequency information for the neighbor cells in the logged MDT report (on frequency level), it is not possible for the network to identify whether the cell provides coverage to NPN or not (on cell level). This is shown in the following excerpt from the RRC specification.

MeasResultLoggingNR-r16 ::=          SEQUENCE {

physCellId-r16                       PhysCellId,

resultsSSB-Cell-r16                  MeasQuantityResults,

numberOfGoodSSB-r16                  INTEGER (1..maxNrofSSBs-r16) OPTIONAL

}

UE only logs the PCI and the radio measurements of the neighboring cells in the logged MDT report.

From the current logged MDT reports it is impossible for the network to deduce if the provided coverage is by a PN or an NPN cell.

By collecting logged measurements from the UE, where the network type is also included, the network can derive an accurate coverage map of each of the network types. This allows for optimization of network performance and better fulfilment of the service level agreements for each of the networks.

In addition, the following FFS is captured in RAN2#122 meeting notes.

=>	FFS: Include the SNPN ID/CAG ID(s) in the logged MDT report or cell type indication (e.g., NPN cell).

Thus, to address the above FFS, we propose the following.

RAN2 to enhance the logged MDT report with cell type indication (e.g., NPN cell) as part of the measurement results.

If a UE configured with LoggedMeasurementConfiguration goes to anyCellSelection state, it logs the following information upon performing a PLMN check-

3> if the UE is in any cell selection state (as specified in TS 38.304 [20]): 4> set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found; 4> if the reportType is set to eventTriggered in the VarLogMeasConfig; and 4> if the RPLMN at the time of entering the any cell selection state is included in plmn-IdentityList stored in VarLogMeasReport; and 4> if areaConfiguration is not included in VarLogMeasConfig or if the last suitable cell that the UE was camping on is part of the area indicated by areaConfig of areaConfiguration in VarLogMeasConfig: 5> set the servCellIdentity to indicate global cell identity of the last suitable cell that the UE was camping on; 5> set the measResultServingCell to include the quantities of the last suitable cell the UE was camping on;

However, as per Rel-18 running CR, the UE can be configured with a CAGConfigList that includes a list of CAG identities. If the UE is camped normally on a cell that does not belong to the plmn-IdentityList provided in the LoggedMeasurementConfiguration but broadcasts CAG identities that are part of the CAGConfigList and goes to anyCellSelection state, i.e., UE goes to out of coverage, it will not log any information in the logged MDT report since it does not satisfy the PLMN checking condition. This is however, an important information for network to perform coverage analysis of the NPN network. Hence, we propose that upon going to anyCellSelection state, UE logs information of the last suitable cell if CAG identities broadcasted by the last suitable cell was part of the CAGConfigList.

RAN2 enable MDT measurements in UEs in anyCellSelection state in NPN network.

UE in anyCellSelection state logs cell identity and measurements of the last suitable cell if the CAG identities broadcasted by the cell that is part of the CAGConfigList received in LoggedMeasurementConfiguration.

    - Enhancement on the MDT Configuration

Similarly, a PN UE may collect MDT measurements on configured target frequencies for neighbor cells belonging to NPN when the cells operate on the same target carrier frequency. In other words, the UE collects both PN and NPN related MDT measurements in logged MDT report wherein two neighbor NPN and PN cells are sharing the target carrier frequency (e.g., configured as part of InterFreqTargetInfo). Therefore, building a coverage map for network types e.g., PN, or NPN for a given frequency may not be enough accurate.

As of now, building coverage map for an area (including both PN and NPN coverage) can be misleading for coverage analysis purpose, as MDT reports does not reflect the cell type (e.g., PN or NPN type), and a coverage of NPN maybe counted as coverage of PN and vice versa. Note that neighbouring cells measurement in MDT reports includes only PCI info of the cell.

We request RAN2 to discuss UE behavior for collecting logged MDT measurements concerning the target carrier frequency:

RAN2 to enhance the MDT configuration (interFreqTargetInfo) to enable logging only NPN or PN cells per frequency.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Inclusion of CAG information in RLF/HOF report may be used for observability purpose but has no clear benefit for MRO.

Observation 2	CAG subscription status is not known in RAN and may be subject to user privacy.

Observation 3	A SNPN user (holding an SNPN allowed UE) may move frequently between SNPN and PN which may ruin collected measurements for one network in the other networks.

Observation 4	The current logged MDT framework is wasteful when PN and SNPN co-exist.

Observation 5	RAN2 running CR has implemented changes mirroring RAN3 implementation of PNI-NPN Based MDT and SNPN Based MDT.

Observation 6	In the network provides SNPN cell ID or TAI as part of area configuration, i.e., in “SNPN Cell Based MDT” and “SNPN TAI Based MDT” checking the NID when logging MDT measurement is not needed.

Observation 7	If a UE served by PN, registers to SNPN and later returns to PN, it is unclear what is logged and reported by the UE in the MHI.

Observation 8	It is agreed that the UE performs SNPN ID checking before transmitting the information for corresponding SON and MDT reports, upon the network requests for it.

Observation 9	UE only logs the PCI and the radio measurements of the neighboring cells in the logged MDT report.

Observation 10	From the current logged MDT reports it is impossible for the network to deduce if the provided coverage is by a PN or an NPN cell.

Observation 11	As of now, building coverage map for an area (including both PN and NPN coverage) can be misleading for coverage analysis purpose, as MDT reports does not reflect the cell type (e.g., PN or NPN type), and a coverage of NPN maybe counted as coverage of PN and vice versa. Note that neighbouring cells measurement in MDT reports includes only PCI info of the cell.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Upon logging the RA report for a new SNPN, the UE deletes the existing RA reports belonging to the previous SNPNs/PLMN.

Proposal 2	UE logs NID in the RA report.

Proposal 3	UE does not add CAG information in the RLF/HOF report.

Proposal 4	Upon moving to SNPN (and deregistration in PN) UE stores logged MDT report collected in PN in a separate variable and releases the MDT configuration received in PN.

Proposal 5	RAN2 to agree on the draft LS reply in the Annex.

Proposal 6	RAN2 discuss whether to consider additional memory (e.g., additional 64KB) for storing the PLMN MDT measurements when going to SNPN network.

Proposal 7	RAN2 requests RAN3 to revert their implementation on configuring SNPN Cell Based MDT and SNPN TAI Based MDT configuration.

Proposal 8	RAN2 to agree on the draft LS reply in the Annex.

Proposal 9	RAN2 discuss to log an SNPN out-of-coverage (OoC) indication either in

a.	an RA report, or

b.	a CEF report, or

c.	a new report.

Proposal 10	UE logs time spent in the SNPN network in an entry in the existing PN MHI report.

Proposal 11	RAN2 agree that MHI of PLMN should not be reported to the SNPN i.e., PLMN check is needed before MHI availability indication to the network.

Proposal 12	RAN2 agree that MHI of the SNPN should not be reported to the PLMN i.e., SNPN checking is needed before MHI availability indication to the network.

Proposal 13	RAN2 to enhance the logged MDT report with cell type indication (e.g., NPN cell) as part of the measurement results.

Proposal 14	RAN2 enable MDT measurements in UEs in anyCellSelection state in NPN network.

Proposal 15	UE in anyCellSelection state logs cell identity and measurements of the last suitable cell if the CAG identities broadcasted by the cell that is part of the CAGConfigList received in LoggedMeasurementConfiguration.

Proposal 16	RAN2 to enhance the MDT configuration (interFreqTargetInfo) to enable logging only NPN or PN cells per frequency.

# References

[RP-221825], Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity), CMCC, 3GPP TSG RAN meeting #96, Budapest, Hungary, June 6-9, 2022.

# Annex

## Reply LS to RAN3- R3-232118

3GPP TSG-RAN2 Meeting #123-bis	R2-23xxxx

Xiamen, CN 9th Oct- 13th Oct 2023

Title:	Reply LS on potential override of logged MDT reports upon moving from SNPN to PLMN

Response to:	R2-2304628 (R3-232118)

Release:	Release 18

Work Item:	NR_ENDC_SON_MDT_enh2-Core

Source:	TSG RAN WG2

To:	TSG RAN WG3

Cc:	-

Contact Person:

Name:	 Ali Parichehreh

E-mail Address: ali.parichehreh@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

In the LS R2-2304628/R3-232118, it mentions:

RAN3 noticed that logged MDT reports collected in one type of network, e.g., an SNPN, are lost when UE moves to a network of another type, e.g., a Public Network, due to the UE deregistering from the SNPN network. RAN3 believes that this would lead to a loss of the stored logged MDT reports if the reports are not retrieved before moving to the network of another type.

RAN3 would like to check with RAN2 if there are any solutions to avoid the loss of stored logged MDT reports upon moving from a network of one type to another type, even upon deregistration.

RAN2 reply:

RAN2 thanks RAN3 for the questions. RAN2 has analysed the scenario and agrees on the below solution-

- UE stores logged MDT report in separate variables for PN upon moving to the SNPN. Thus, collecting logged MDT report in SNPN would not delete any potentially collected report in PN.

- To reduce complexity, UE would not store the logged MDT configuration in separate variables. Thus, if a UE is configured with logged MDT configuration in public network (PN), moves to a SNPN and receives a new logged MDT configuration in SNPN, it will overwrite the public network’s logged MDT configuration.

2. Actions:

To RAN WG3 group.

ACTION: 	RAN2 respectfully asks RAN3 to take account of the feedback.

3. Date of Next TSG-RAN WG2 Meeting:

3GPP RAN2#123-bis	from 2023-10-09	to	2023-10-13		Xiamen, CN

3GPP RAN2#124	from 2023-11-13	to	2023-11-17		Chicago, US

## Reply LS to RAN3- R3-234744

3GPP TSG-RAN2 Meeting #123-bis	R2-23xxxx

Xiamen, CN 9th Oct- 13th Oct 2023

Title:	Reply LS on MDT for NPN

Response to:	R2-230xxxx (R3-234744)

Release:	Release 18

Work Item:	NR_ENDC_SON_MDT_enh2-Core

Source:	TSG RAN WG2

To:	TSG RAN WG3

Cc:	-

Contact Person:

Name:	 Ali Parichehreh

E-mail Address: ali.parichehreh@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

In the LS R2-230xxxx/R3-234744, it mentions:

RAN3 would like to inform RAN2 and SA5 about progress achieved so far on MDT for NPN. Such progress is captured in section 9.3.1.169 of the endorsed baseline CR to TS38.413 and in section 5.1.1.1.1 of the endorsed baseline CR to TS37.320, attached.

The work RAN3 has carried out was mainly focussed on updating MDT area scope information to account for NPN networks. RAN3 would like to point out that, in order to cater for support of equivalent SNPNs, a list of up to 16 SNPNs has been added to the MDT Area Scope. Whether any restriction to the usage of such SNPN list will be in place in this release depends on progress on eSNPN support in other WGs.

RAN3 would like to ask RAN2 and SA5 to take the above into account, to apply corresponding modification to their specifications, if needed, and to provide feedback on the progress achieved so far on MDT for NPN, if any

RAN2 reply:

RAN2 thanks RAN3 for the information. RAN2 has analysed the solution and has below observation

- RAN2 has implemented changes corresponding to PNI-NPN Based MDT and SNPN Based MDT configurations.

- RAN2 thinks configuration for SNPN Cell Based MDT and the SNPN TAI based MDT is not required and would like RAN3 to revert the implementation.

2. Actions:

To RAN WG3 group.

ACTION: 	RAN2 respectfully asks RAN3 to take account of the feedback and revert the implementation of SNPN Cell Based MDT.

3. Date of Next TSG-RAN WG2 Meeting:

3GPP RAN2#123-bis	from 2023-10-09	to	2023-10-13		Xiamen, CN

3GPP RAN2#124		from 2023-11-13	to	2023-11-17		Chicago, US