3GPP TSG-RAN WG2 #123bis	R2-2310748

Xiamen, CN, October 9th – October 13th, 2023

Agenda Item:	7.13.6

Source:	Ericsson

Title:	RA report enhancement

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#123 meeting, enhancements of RA report were discussed and some FFSs were identified-

Agreements:

1	At least the NSAG ID that is assigned to the S-NSSAI triggering the RA attempt and belongs to the NSAG ID of the feature combination used to select the RA configuration should be reported.

FFS: Further discuss whether the following NSAG IDs to be included in the RA reports:

a)	NSAG ID(s) that belong to the S-NSSAI(s) triggering the RA attempt and included in SIB1 (even if they were not used to select the RA configuration, e.g., due to belonging to lower priority NSAGs).

b)	NSAG ID(s) that belong to the S-NSSAI(s) triggering the RA attempt (even if they are not included in SIB1).

In this paper, we discuss these FFSs along with the received LS from RAN3 (R3-234643) and some other aspects of the RA report in dual connectivity.

# 2. Discussion

## 2.1 Discussion on the received LS on partitioning info in RA report.

In regard of optimization of the features combinations and the RA partitions allocated to different features and feature combination RAN3 has sent an LS (R3-234643) to RAN2 requesting to verify different solutions in enabling the network to figure out the allocated RACH resource allocation for different partitions. In this section, we discuss possible enhancement of the RA reports collected when UE was configured with feature combination RACH resources and addressing the received LS.

A network might partition and isolate RA resources per feature i.e., a preamble set is configured for a feature. The network that is interested in optimizing the RA parameter configurations, uses the RA report to identify any issues faced by a UE while performing the RA procedure.

However, a gNB may change the RACH partitions over time based on the load in different partitions. A UE logs the RA related information at the execution of RA procedure while the network may change the RA configuration dynamically over time. Therefore, the network may have different RACH resource configuration at the time the network fetches the logged information in RA report.

- A gNB may reconfigure the RACH partitions based on the load over each partition.

Hence, a network can optimize its resources by having information of the concerned configuration of the used feature by the UE that triggered the RA procedure i.e., the set of preambles allocated to the RA partition such as the start preamble index and/or the number of preambles in the partition.

- A network can optimize its RACH partitioning resources by having information of preambles allocated to the RA partition.

In our opinion, RA partitioning related information in RA report is useful for the network in order to optimize the RA performance for each of the feature separately. For example, a gNB can reallocate the RA resources e.g., sub-set of preambles assigned to the group of the features based on the demands coming/initiated from the group of features. On the other side the group of the features bundled together can be shuffled and optimized to evenly distribute the load over different RA partitions.

In light of above we propose:

UE includes start preamble index and the number of preambles in the partition that triggered the RA procedure in the RA report.

A reply LS is proposed to RAN3 in the annex-

RAN2 to agree on the reply LS on RACH enhancement to RAN3.

## NSAG and S-NSSAI info in the RA report

Moreover, in the meeting RAN2#120, RAN2 has agreed to include the NSAG ID in the RA report for the sake of RACH resource optimization when the applicable feature is slicing.

Agreements:

1	For RACH report for RACH partitioning, RAN2 to agree to include NSAG ID when the applicable feature is slicing.

The definition of NSAG is given in the following excerpt from TS 38.300.

-	Network Slice AS Group (NSAG): identifies an association to a slice or a set of slices. An NSAG is defined within a TA, used for slice-based cell reselection and/or slice-based RACH configuration. Values of NSAG IDs associated with different slice or set of slices shall be unique within a TA, also when slice-based cell reselection and slice-based RACH configuration are both supported in the TA.

Thus, an NSAG may identify a list of slices, each of which is identified by an S-NSSAI.

-	Single Network Slice Selection Assistance information (S-NSSAI): identifies a network slice.

On the other hand, NSAGs are mapped to RACH resources through the definitions of FeatureCombinationPreambles and FeatureCombination IEs configured in RACH-ConfigCommon IE for the 4-step RACH. Similar mapping exists for two-step RACH procedure in TS-38.331.

It should be noted that both S-NSSAI to NSAG mapping and mapping between NSAG and RACH resources are performed outside of core network. However, optimizing the RACH partitions allocated to different slice groups is dependent on how the slices (S-NSSAI) are mapped to the slice groups (NSAGs). A plausible scenario is that some S-NSSAI within a NSAG are of high demand which results in imbalanced load over different RACH partitions. A solution to address this is to distribute the high demand S-NSSAIs into different NSAGs, which eventually would result in being distributed over different RACH partitions. Below table illustrates such scenario.

| NSAG | S-NSSAI | RACH partition (combination of start preamble and number of preambles in the partition) |
| --- | --- | --- |
| 1 | 1,2,3,5 | A |
| 2 | 4,6,10 | B |
| 3 | 7,8,9 | C |

In the example, RACH partition-A is highly loaded, partition-B is moderately loaded, and partition-C is normally loaded. From the FeatureCombination IE, it can be deduced that NSAGs 1,2,3 is associated with RACH partitions A, B, C, respectively. Thus, the network would be aware that S-NSSAIs belonging to NSAG-1 are being more frequently accessed by the UEs. However, NSAG-1 has S-NSSAIs 1,2,3,5 mapped to it, and it is not possible for the network to determine which slices are being frequently access by the UEs. In this example, S-NSSAI-1 and S-NSSAI-3 are of high demand and regrouping S-NSSAI-3 to NSAG-3 would result in a balanced network load.

Hence, to address the impact of slice grouping in the RACH performance in each RA partition, we propose the UE includes the S-NSSAI (one or more S- NSSAI) beside the NSAG ID for which the RACH was triggered in the RACH report.

- The slices information (S-NSSAI) beside the NSAG ID is essential to evenly distribute the RACH load between different RACH partitions based on the demands for specific slices.

UE include slice information, i.e., S-NSSAI(s) that triggered the RACH through a given partition in the RA report.

On the other hand, regarding the FFS of which NSAG ID(s) should be included in the RA reports, there are two options under discussion:

a)	NSAG ID(s) that belong to the S-NSSAI(s) triggering the RA attempt and included in SIB1 (even if they were not used to select the RA configuration, e.g., due to belonging to lower priority NSAGs).

b)	NSAG ID(s) that belong to the S-NSSAI(s) triggering the RA attempt (even if they are not included in SIB1).

In our view, option b) provides more information to the network since it allows the network to understand which NSAG ID(s) were demanded by the UE. Equipped with this knowledge, network would be able to optimize SIB1 broadcast.

UE include NSAG ID(s) that belong to the S-NSSAI(s) triggering the RA attempt (even if they are not included in SIB1).

## RACH report logging in case of SDT

One FFS identified in RAN2#123 meeting is whether a single flag is to be added to the RA report to indicate a RA-SDT failure or to add multiple flags to differentiate different types of failures.

2	Addition of an indication in RA report whether RA-SDT procedure is successful or not. Details of the indication and whether it is a single flag or further differentiation of the failure scenarios are needed are FFS.

In our opinion, a SDT failure is defined as a failure to transmit SDT data. Such failures can occur due to multiple reason, for example.

- T319a expiry

- Cell reselection during SDT operation

- RA procedure failure

However, logging such detailed information to the network does not have a clear use case.

- Logging detailed information regarding cause of SDT failure does not have a clear use case for SON purpose.

Hence, we propose that the UE logs information about SDT failure using a single flag without differentiating between different types of failures.

UE include a single flag in the RA report to indicate SDT failure. The gain of further differentiation is unclear.

## 2.4	Enhancement of RA report for DC scenario

In this section we discuss possible enhancement of the RA reports collected when UE was configured with a dual connectivity (DC) operation. In DC operation and based on the current TS 38.331, a UE logs RA report upon successful completion of the RA procedure toward MCG or SCG. In MR-DC, a UE logs the same set of RA related information and measurements, no matter if the RA was performed toward a cell in the SCG or in the MCG. Thus, a RAN node analyzing the RA reports would not be able to determine whether the reported RA information is associated to an RA performed toward a Cell in the MCG or in the SCG. Needless to mention that the performance of a RA performed toward an MCG cell might be different from the performance of an RA procedure performed toward an SCG cell. For instance, an RA procedure may imply suffering poor coverage or too aggressive SN addition/SN change policy by neighbor cell for a node operating as SN in MR-DC. This this information is useful for network optimization.

- Performance of the RA procedure for a cell in the MCG can be significantly different from the performance of RA procedure for the same cell being part of SCG. This is due to the fact that network policies for DC connectivity can be different from single connectivity.

- When a RA report is received and analysed by a RAN node, the RAN node is not aware whether the RA procedure is performed toward a cell in the MCG or in the SCG.

- By knowing whether the RA report is associated to a random-access procedure executed toward a cell belonging to SCG or MCG, the network can differentiate the RACH issues as well as coverage issues for a cell acting as MCG or as SCG.

In light of above discussion, we request RAN2 to include information in the RA report to assist the network differentiating between RA reports associated to a random-access procedure performed toward a cell acting as MN or as SN.

Include information in the RA report on whether the random-access procedure was executed towards an MCG cell or an SCG cell.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	A gNB may reconfigure the RACH partitions based on the load over each partition.

Observation 2	A network can optimize its RACH partitioning resources by having information of preambles allocated to the RA partition.

Observation 3	The slices information (S-NSSAI) beside the NSAG ID is essential to evenly distribute the RACH load between different RACH partitions based on the demands for specific slices.

Observation 4	Logging detailed information regarding cause of SDT failure does not have a clear use case for SON purpose.

Observation 5	Performance of the RA procedure for a cell in the MCG can be significantly different from the performance of RA procedure for the same cell being part of SCG. This is due to the fact that network policies for DC connectivity can be different from single connectivity.

Observation 6	When a RA report is received and analysed by a RAN node, the RAN node is not aware whether the RA procedure is performed toward a cell in the MCG or in the SCG.

Observation 7	By knowing whether the RA report is associated to a random-access procedure executed toward a cell belonging to SCG or MCG, the network can differentiate the RACH issues as well as coverage issues for a cell acting as MCG or as SCG.

Based on the discussion in the previous sections we propose the following:

Proposal 1	UE includes start preamble index and the number of preambles in the partition that triggered the RA procedure in the RA report.

Proposal 2	RAN2 to agree on the reply LS on RACH enhancement to RAN3.

Proposal 3	UE include slice information, i.e., S-NSSAI(s) that triggered the RACH through a given partition in the RA report.

Proposal 4	UE include NSAG ID(s) that belong to the S-NSSAI(s) triggering the RA attempt (even if they are not included in SIB1).

Proposal 5	UE include a single flag in the RA report to indicate SDT failure. The gain of further differentiation is unclear.

Proposal 6	Include information in the RA report on whether the random-access procedure was executed towards an MCG cell or an SCG cell.

# References

R3-226053, Reply LS on SN RACH report status in R17, RAN3, 3GPP TSG-RAN3 Meeting #117bis-e, 10th - 18th October 2022

# Annex

## Reply LS to RAN3- R2-23xxxx/R3-234643

3GPP TSG-RAN2 Meeting #123-bis	R2-23xxxx

Xiamen, CN 9th Oct- 13th Oct 2023

Title:	Reply LS on potential override of logged MDT reports upon moving from SNPN to PLMN

Response to:	R2-230xxxx (R3-234643)

Release:	Release 18

Work Item:	NR_ENDC_SON_MDT_enh2-Core

Source:	TSG RAN WG2

To:	TSG RAN WG3

Cc:	-

Contact Person:

Name:	 Ali Parichehreh

E-mail Address: ali.parichehreh@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

In the LS R2-230xxxx/R3-234643, it mentions:

In the context of RACH optimization, RAN3 discussed how to associate the RA Report(s) retrieved at a gNB with the corresponding RACH configuration, e.g. RACH partitioning configuration, at the time when the UE records the RA Report(s), in case the RACH configuration changed by the time an RA Report is retrieved. Following alternatives were discussed, but there was no consensus in RAN3:

Alt1: Enable the addition in the RA Report of the feature priority of each feature in the feature combination used by the UE at the time RACH access is triggered. This enables the NG-RAN to determine whether any optimizsation is needed with respect to how features with different priorities are combined in the same feature combination associated to a RACH partition.

Alt2: Enable the addition in the RA Report of RACH partition configuration information. This information consists of the start preamble index and the number of preambles in the partition for which the RA Report was generated. This enables the NG-RAN to determine the RACH partition in use.

Alt 3: Enable the addition in the RA Report of the time between RACH access that led to the generation of a RA Report and when RA Report was retrieved. Using this timer, and in case the NG-RAN stores time records of past RA Partitions configurations, feature priorities and feature combinations, the NG-RAN can figure out the RACH configuration, feature priorities and feature combination in use.

RAN3 would like to check with RAN2 on the feasibility and their preference on the above three alternatives

RAN2 reply:

RAN2 thanks RAN3 for the questions. RAN2 has analysed the scenario and agrees to the alt2 proposed by RAN3-

- UE includes start preamble index and the number of preambles in the partition that triggered the RA procedure in the RA report.

2. Actions:

To RAN WG3 group.

ACTION: 	RAN2 respectfully asks RAN3 to take account of the feedback.

3. Date of Next TSG-RAN WG2 Meeting:

3GPP RAN2#123-bis	from 2023-10-09	to	2023-10-13		Xiamen, CN

3GPP RAN2#124	from 2023-11-13	to	2023-11-17		Chicago, US