3GPP TSG-RAN WG2 Meeting #123-bis                                                 R2-2310783

# Xiamen, China, Oct 9 – Oct 13, 2023

Source:	Qualcomm Incorporated

Title:	Open issues to support QoE collection in NR-DC

Agenda Item:	7.14 QoE

Document for:	Discussion and Decision

# Introduction & Background

This contribution discusses RAN2 open issues for QoE collection in NR-DC based on the agreements in RAN2 and RAN3 meetings.

# Discussion

RAN2 agreed UE should not request to activate SCG only for the purpose of QoE reporting via SRB5, but it is still FFS for RVQoE reporting.

UE should not request to activate SCG only for the purpose of QoE reporting via SRB5. FFS for RVQoE reporting.

Rel-17, UE is not required to buffer RVQoE measurement. For Rel-18 NR DC QoE, it is preferred the same principle to be applied, that is, when UE has RVQoE data to be reported when SCG is deactivated, UE should request to activate SCG for RVQoE reporting if there is no activated bearer.

Proposal 1: UE should request to activate SCG for RVQoE reporting if there is no activated bearer for RVQoE reporting.

In the incoming LS from RAN3 [3], RAN3 agreed that for a UE in NR-DC, each legacy QoE configuration can have only one corresponding RVQoE configuration when needed.

In Rel-17, the container QoE configuration and the corresponding RVQoE configuration share the same MeasAppLayerID. In NR-DC, the similar principle should be reused, then the container QoE configuration and the corresponding RVQoE configuration share the same MeasAppLayerID; otherwise, the container QoE configuration has to be reconfigured to the UE along with the RVQoE configuration, which will cost signalling overhead.

Proposal 2: In NR-DC QoE, the container QoE configuration and the corresponding RVQoE configuration share the same MeasAppLayerID.

# Conclusion

Proposal 1: UE should request to activate SCG for RVQoE reporting if there is no activated bearer for RVQoE reporting.

Proposal 2: In NR-DC QoE, the container QoE configuration and the corresponding RVQoE configuration share the same MeasAppLayerID.

# Reference

[1]    RP-221803: Enhancement on NR QoE management and optimizations for diverse services

[2]    RAN2 #123 Chair Notes.

[3]    R3-234750: LS on RAN3 progress on QoE in NR-DC