3GPP TSG-RAN WG2 Meeting #123-bis	R2-2310784

# Xiamen, China, Oct 9 – Oct 13, 2023

Source:                Qualcomm Incorporated

Title:	Discussion on UE QoE capabilities

Agenda Item:	7.14

Document for:	Discussion and Decision

# Introduction & Background

This contribution discuss UE capabilities for Rel-18 QoE features, including QoE collection in IDLE and Inactive state, QoE collection in NR-DC, enhancemnets for Rel-17 leftover issues.

# Discussion

UE capabilities on MBS service QoE collection

Since IDLE and Inactive state QoE collection is for MBS broadcast service, then only those UEs supports MBS broadcast service can report of supporting QoE collection in IDLE/Inactive state

- UE can report capability of supporting IDLE and Inactive state QoE collection only when UE supports MBS broadcast.

Another issue is whether to introduce UE capability for CONNECTE state for MBS service, SA4 replied in [2] that there is no new service type for MBS service and MBS is treated as delivery mode. If multicast delivery mode is introduced for CONNECTED QoE collection in AS layer, considering UE has to support multicast delivery mode, then a new UE capability is needed.

- If multicast delivery mode is introduced for CONNECTED QoE collection, then a new UE capability is needed to be reported to the gNB.

In last RAN2 meeting, it was agreed to introduce a UE capability for the supported buffer size. But is still FFS whether this is shared or additional to the Rel-17 buffer size requirement.

Introduce a UE capability for the supported buffer size. It is conditionally mandatory if UE supports MBS QoE. The range is from 64 kB to 1024 kB (exact values can be discussed in RRC running CR discussion). FFS whether this is shared or additional to the Rel-17 buffer size requirement.

In Rel-17, the AS buffer size is 64KB and used to buffer the QoE data when QoE reporting is paused. When the UE is released to Inactive state, UE should continue to buffer the QoE data. Actually, buffering QoE data in Inactive state is not critical, the proper gNB implementation should configure the UE to report the buffered QoE data before releasing the UE into Inactive state.

In Rel-18, when the UE reports the supported buffer size to the gNB, if the UE reports a low buffer size capability, gNB should resume QoE reporting before releasing the UE into Inactive state. Then the buffer size capability should be shared with the Rel-17 buffer size requirement.

- The buffer size capability should be shared with the Rel-17 buffer size requirement. If the UE reports a low buffer size capability, gNB can resume QoE reporting before releasing the UE into Inactive state.

- Send LS to SA4 on UE capability for IDLE and Inactive state QoE collection.

UE capabilities on NR-DC QoE

NR-DC QoE collection is new feature with introducing QoE configuration, then UE capability should be introduced for QoE collection in NR-DC.

- Introduce UE capability of supporting NR-DC configuration with radio access capability parameter.

SRB5 is agreed to be introduced for UE reporting QoE data to SN, this is new SRB and similar with SRB3, it should be optional for the UE to support SRB5.

- Introduce UE capability of supporting SRB5 for QoE reporting with radio access capability parameters.

# Conclusion

This contribution discusses the MBS broadcast service QoE configuration and report for IDLE and Inactive state.

UE capabilities on IDLE/Inactive state QoE

- UE can report capability of supporting IDLE and Inactive state QoE collection only when UE supports MBS broadcast.

- If multicast delivery mode is introduced for CONNECTED QoE collection, then a new UE capability is needed to be reported to the gNB.

- The buffer size capability should be shared with the Rel-17 buffer size requirement. If the UE reports a low buffer size capability, gNB can resume QoE reporting before releasing the UE into Inactive state.

- Send LS to SA4 on UE capability for IDLE and Inactive state QoE collection.

UE capabilities on NR-DC QoE

- Introduce UE capability of supporting NR-DC configuration with radio access capability parameter.

- Introduce UE capability of supporting SRB5 for QoE reporting with radio access capability parameters.

# Reference

[1]    RP-221803: Enhancement on NR QoE management and optimizations for diverse services

[2]    R3-235025	Reply LS on QoE measurement collection for application sessions delivered via MBS broadcast or multicast	SA4(Huawei)