3GPP TSG-RAN WG2 Meeting #123-bis					  R2-2310870

Xiamen, China, October 9 – October 13, 2023

Agenda item:	7.3.2

Source:			MediaTek Inc.

Title:	Remaining RRC issues of Cell DTX and DRX mechanism

Document for:	Discussion and Decision

- Background

This contribution aims for providing views and discussion materials for resolving the issues of email [Post123][NES] Remaining RRC Open Issues (Huawei) on the reflector.

- Discussion

Issue 1-1: Configuration of cell DTX/DRX per serving cell vs per MAC entity.

In a CA scenario, if the Cell DTX and Cell DRX patterns and/or configurations are different between PCell and SCells, it could lead to several potential issues or increase the network and UE implementation complexity:

- Synchronization and alignment challenges: The network would need to manage different downlink and uplink transmission patterns, which could increase the complexity of the scheduling and resource allocation processes, such as HARQ processes management, the allocation of UE-dedicated SPS, SR, and CG resources. The UE also needs to cope with the complexity of aligning such as SR, BSR and retransmission.

- Potential for interference: If the DTX/DRX patterns for the PCell and SCell are not properly aligned, it could potentially lead to interference issues. For example, if the PCell is in the ON duration while the SCell is in the OFF duration (or vice versa), it could lead to interference and degrade the performance of the network.

- Impact on UE power consumption: Different DTX/DRX patterns for the PCell and SCells could potentially impact the power consumption of the UE. The UE could wake up more frequently for different ON occasions for the PCell and SCells, which would seriously impact the power consumption of the UE.

Moreover, RAN3 had agreed the WA to support Cell DTX/DRX configuration exchange between gNBs, we propose that configuration of Cell DTX/DRX shall be per MAC entity.

- Configuration of Cell DTX/DRX is per MAC entity. (The proposed Option 2 below)

As a result of [POST123][312][NES] Running CR 38.331, the rapporteur proposes the following way forward:

Proposal 1    RAN2 to decide between the two options for Cell DTX/DRX configuration:

Option 1 Cell DTX/DRX configuration is provided per Serving Cell with the following restrictions:

    - The UE will not be required to follow more than 2 active patterns of cell DTX/DRX, regardless of the number of servings cells.

    - The two configured active patterns should be aligned, i.e. one periodicity is an integer multiple of the other.

Option 2 Cell DTX/DRX configuration is provided per MAC entity with the following remarks:

    - The NW can configure the UE with a secondary configuration that can be used with cells configured with a secondary C-DRX group.

First, we would like to clarify the above Option 1 does not mean that the UE is required to handle up to 2 active Cell DTX/DRX patterns in single cell scenario even if the multiple configuration support is agreed in the future. We propose to amend the WF to clarify the restrictions are specific for multiple serving cells scenario.

Secondly, the revised Option 1 will still be a concern to us. Assumed that two periodicities are equal in two serving cells scenario, it represents at least the Cell DTX/DRX patterns shown as below:

Case 1 – Commonly overlapped active period (parts in blue) across serving cell1, cell2 and UE.

Serving cell 1:

Serving cell 2:

Tx and Rx of UE: (Active Time of C-DRX is the parts in green)

For case 1, we think it might alleviate aforementioned concerns of the implementation complexity of the network and the UE to some extent. But it is just an ideal example. It could be worse when the overlapped period is shorter.

Case 2 – Separate overlapped active period (parts in blue) between serving cell(s) and UE (i.e., no commonly overlapped active period across all serving cells and UE).

Serving cell 1:

Serving cell 2:

Tx and Rx of UE: (Active Time of C-DRX is the parts in green)

For case 2, that would be a difficult task for guaranteeing QoS if we still remember the mission goal of C-DRX when striving to craft the Cell DTX/DRX for serving the purpose of Rel-18 NES.

Like RAN2 had confirmed, Cell DTX/DRX is going to operate on top of UE C-DRX. A key of having commercial success of Cell DTX/DRX deployment is to enable the gradual migration to full support/coverage stage meanwhile leveraging existing implementations as much as possible. From downlink implementation standpoint, an approach to accommodate the Rx/Tx requirements in the UE side is to keep monitoring PDCCH even there is no active period across multiple serving cells. Although it is suboptimal and the power consumption is high, at least the UE can reuse most hardware and software implementations. However, it is a different story from uplink implementation standpoint. The uplink transmission is prohibited during non-active periods, so a compliant UE must obey the Tx behavior so that existing software, or even hardware implementation needs to be modified. Although we don’t have too many uplink CA cells nowadays, but it is still rather complicated considering of like PUCCH SCell when a certain number of cells are aggregated, and etc. In that sense, we propose to have at least aligned Cell DRX patterns across multiple serving cells when the configuration of Cell DRX is per cell.

- [If the proposed Option 2 is not agreed, as a compromise:] The Cell DTX/DRX configuration is provided per serving sell with the following restrictions:
- The UE will not be required to follow more than 2 active patterns of cell DTX/DRX when there is more than one serving cell.
- The Cell DRX patterns are common across multiple serving cells.

Furthermore, we think it is essential to have corresponding UE capabilities for basic Cell DTX/DRX function, and the complicated operation in multiple serving cells scenario respectively if the configuration of Cell DTX/DRX is per cell. To ensure the flexibility of different migration stage, IoT bits are not precluded. According to valid configurations in previous RAN2 discussions and above discussions, we propose to have at least following UE capabilities for Rel-18 NES:

      - Support of Cell DTX.

      - Support of Cell DTX and Cell DRX in single serving cell, or with the same Cell DRX pattern across multiple serving cells.

      - Support of Cell DTX and Cell DRX with different Cell DRX patterns across multiple serving cells.

- The UE capability parameters of Cell DTX/DRX support in single and/or multiple serving cells will be defined in stage 3 specifications containing at least following features:
1. Support of Cell DTX.
2. Support of Cell DTX and Cell DRX in single serving cell, or with the same Cell DRX pattern across multiple serving cells.
3. Support of Cell DTX and Cell DRX with different Cell DRX patterns across multiple serving cells.

Issue 1-2: Explicit vs implicit activation based on RRC and relation to L1 activation (how UE derives NW L1 capability and is there a need of explicit RRC indication that the NW will use L1).

New PDCCH parameters for configuring such as search space for new L1 common signaling will be defined. Hence the UE knows the L1 common signaling is going to be used by the gNB when receiving the corresponding L1 parameters. So, we think that is the explicit indication of the use of L1 common signaling from the network.

Issue 1-3: Is deactivation by RRC performed only by deconfiguration, or should we have an explicit RRC deactivation without deconfiguration option (to later activate the same configuration by RRC).

Regardless of whether L1 common signaling is used or not, it would be beneficial for having explicit indication of (de-)activation in RRC signaling in terms of overhead reduction. On the other hand, we think it is more efficient when the temporary deactivation is needed for the UE initiating E911 call.

- The explicit indications for the (de-)activation of Cell DTX/DRX will be defined in RRC signaling.

Issue 1-4: Is the stage 3 alignment description needed.

To eliminate UE implementation ambiguity and to facilitate feature robustness, we think it is crucial to capture alignment requirements in stage 3 specifications, such as the Cell DTX/DRX and UE C-DRX periodicity should be integer multiple of each other. The main point is to capture the relationship between numbers in configuring Cell DTX/DRX. Either we put the requirements in the field description, or we have it built-in in ASN.1 implementation.

- Alignment requirements of Cell DTX/DRX mechanism will be captured in stage 3 specifications.

Issue 1-5: Whether the start offset (and slot offset) configuration should be the same for cell DTX and cell DRX.

So far, we don’t know a strong motivation to differentiate the offsets between Cell DTX and Cell DRX. The downlink and uplink transmission occasions are going to be governed by the Cell DTX and Cell DRX patterns, and scheduling is under network control. In addition, the retransmission is allowed regardless of non-active period of Cell DTX. Hence, the benefit is still unclear. We suggest having the same offsets between Cell DTX and Cell DRX for the sake of feature simplicity.

- On-duration, Cycle and Start offset (and slot offset) parameters are common between Cell DTX and Cell DRX, when both are configured.

Issue 1-6: Whether standalone cell DRX is needed.

Like the concerns we pointed out in Issue 1-1, the Cell DRX with unaligned patterns will impact the Tx design of the UE. It’s been clear that Cell DTX has higher NES gain than Cell DRX according to the outcome of SI phase. Also, the use case is unclear. In short, we see no motivation and justification to have standalone Cell DRX operation.

- The dedicated Cell DRX configuration in RRC signaling is not needed.

Issue 1-7: Whether the fields in cellDTX-config should be optional or mandatory.

We think it is related to the Issue 1-6 and accordingly we think these essential parameters for enabling Cell DTX (and Cell DRX) are mandatory.

- Based on Proposal 7, the essential parameters in cellDTX-config are mandatory.

Issue 1-8: Consider a simplification of cell DTX/DRX RRC configuration IE once issues 1-5, 1-6 and 1-7 are resolved.

In general, we’re supportive of the intention of Issue 1-8.

Issue 1-9: Whether to capture in RRC that the focus was on the case where cell DTX in RRC can only be configured when C-DRX is configured.

According to what we agreed in last meeting:

11. We focus on the case where DTX in RRC can only be configured when C-DRX is configured. We will not optimize for the case where C-DRX is not configured.

Our understanding is that if we don’t capture that in RAN2 specification, then the UE behavior is unspecified whenever Cell DTX and C-DRX is wrongly configured in the field. At least the UE needs to know how to handle the unspecified situation. for example, it is an invalid configuration, and the UE shall reject the configuration then re-establish the connection somehow.

- The valid configuration where Cell DTX can only be configured when C-DRX is configured will be captured in stage 2 and/or stage 3 specifications.

Issue 1-10: Whether we specify impact to the dual C-DRX case (SCell with different onDuration/inactivity timers).

In general, we shall strive to clarify and complete the interworking and alignment requirements between Cell DTX/DRX and UE C-DRX for single serving cell case, then the requirement shall be applicable for the second serving cell with different onDuration/inactivity timers setting given that so far we don’t have parameters or procedures are directly related to the onDuration or inactivity timer. Hence, if we can conclude for the Issue 1-1 and related ones then we might not need to specify the same for the dual C-DRX case.

Issue 1-11: Whether multiple configurations of cell DTX or DRX will be supported.

We observed that there is low interest in discussing some open issues which are related to help guaranteeing QoS, such as per-SR configuration for latency-sensitive traffic, and UE specific inactivity timer to extend active period of Cell DTX/DRX, etc. The patterns of Cell DTX/DRX are determined by the network for maximizing the NES gain. Therefore, the NES UEs need to bear the service performance degradation risks whenever it camps on a NES cell. In that sense, we don’t think there is a significant benefit to signal multiple configurations of Cell DTX/DRX in advance.

- Supporting multiple configurations of Cell DTX/DRX is not pursued.

Issue 1-12: Whether to change cellBarredNES to a single ENUM value.

The legacy UE behavior on determining the cell suitability is captured in TS 38.300 since from Rel-15:

If the UE cannot determine the full contents of the minimum SI of a cell by receiving from that cell, the UE shall consider that cell as barred.

Furthermore, the NES-capable UE still needs to respect the legacy barring bit in MIB except for the case where the new cellBarredNES is present.

|  | cellBarred in MIB (mandatory present) | cellBarredNES in SIB1 (optional) | Scenarios (use cases) | Cell suitability |
| --- | --- | --- | --- | --- |
| Non-NES-capable UE | barred | barred | NES cell is not providing public service. | Cell is barred. |
| NES-capable UE |  |  |  | Cell is barred. |
| Non-NES-capable UE |  | Not present | Non-NES cell is not providing public service. | Cell is barred. |
| NES-capable UE |  |  |  | Cell is barred. |
| Non-NES-capable UE |  | notBarred | NES cell is providing service for NES UE only. | Cell is barred. |
| NES-capable UE |  |  |  | Cell is not barred. |
| Non-NES-capable UE | notBarred | barred | NES cell is providing service for non-NES UE only. (Odd but possible.) | Cell is not barred. |
| NES-capable UE |  |  |  | Cell is barred. |
| Non-NES-capable UE |  | Not present | Non-NES cell is providing public service. | Cell is not barred. |
| NES-capable UE |  |  |  | Cell is not barred. |
| Non-NES-capable UE |  | notBarred | NES cell is providing public service. | Cell is not barred. |
| NES-capable UE |  |  |  | Cell is not barred. |

According to the above table, we think cellBarredNES must be two ENUM values to avoid the ambiguities.

- The parameter cellBarredNES shall be a ENUM containing two values: {barred, notBarred}. It is conditionally present.

- Conclusion

In previous sections we made the following proposals:

Issue 1-1: Configuration of cell DTX/DRX per serving cell vs per MAC entity.

Proposal 1	Configuration of Cell DTX/DRX is per MAC entity. (The proposed Option 2 below)

Proposal 2	[If the proposed Option 2 is not agreed, as a compromise:] The Cell DTX/DRX configuration is provided per serving sell with the following restrictions:
- The UE will not be required to follow more than 2 active patterns of cell DTX/DRX when there is more than one serving cell.
- The Cell DRX patterns are common across multiple serving cells.

Proposal 3	The UE capability parameters of Cell DTX/DRX support in single and/or multiple serving cells will be defined in stage 3 specifications containing at least following features:
1. Support of Cell DTX.
2. Support of Cell DTX and Cell DRX in single serving cell, or with the same Cell DRX pattern across multiple serving cells.
3. Support of Cell DTX and Cell DRX with different Cell DRX patterns across multiple serving cells.

Issue 1-2: Explicit vs implicit activation based on RRC and relation to L1 activation (how UE derives NW L1 capability and is there a need of explicit RRC indication that the NW will use L1).

No proposal, see above discussion.

Issue 1-3: Is deactivation by RRC performed only by deconfiguration, or should we have an explicit RRC deactivation without deconfiguration option (to later activate the same configuration by RRC).

Proposal 4	The explicit indications for the (de-)activation of Cell DTX/DRX will be defined in RRC signaling.

Issue 1-4: Is the stage 3 alignment description needed.

Proposal 5	Alignment requirements of Cell DTX/DRX mechanism will be captured in stage 3 specifications.

Issue 1-5: Whether the start offset (and slot offset) configuration should be the same for cell DTX and cell DRX.

Proposal 6	On-duration, Cycle and Start offset (and slot offset) parameters are common between Cell DTX and Cell DRX, when both are configured.

Issue 1-6: Whether standalone cell DRX is needed.

Proposal 7	The dedicated Cell DRX configuration in RRC signaling is not needed.

Issue 1-7: Whether the fields in cellDTX-config should be optional or mandatory.

Proposal 8	Based on Proposal 7, the essential parameters in cellDTX-config are mandatory.

Issue 1-8: Consider a simplification of cell DTX/DRX RRC configuration IE once issues 1-5, 1-6 and 1-7 are resolved.

No proposal, see above discussion.

Issue 1-9: Whether to capture in RRC that the focus was on the case where cell DTX in RRC can only be configured when C-DRX is configured.

Proposal 9	The valid configuration where Cell DTX can only be configured when C-DRX is configured will be captured in stage 2 and/or stage 3 specifications.

Issue 1-10: Whether we specify impact to the dual C-DRX case (SCell with different onDuration/inactivity timers).

No proposal, see above discussion.

Issue 1-11: Whether multiple configurations of cell DTX or DRX will be supported.

Proposal 10	Supporting multiple configurations of Cell DTX/DRX is not pursued.

Issue 1-12: Whether to change cellBarredNES to a single ENUM value.

Proposal 11	The parameter cellBarredNES shall be a ENUM containing two values: {barred, notBarred}. It is conditionally present.

- References

- [Post123][NES] Remaining 38.304 Open Issues (Apple) on the reflector

- [POST123][312][NES] Running CR 38.331 (Huawei) on the reflector