3GPP TSG-RAN WG2 #123-bis	R2-2310891

Xiamen, China, 9th – 13th October 2023

Agenda Item:	7.4.4

Source:	Ericsson

Title:	CHO with associated CPC or CPA

Document for:	Discussion, Decision

# 1	Introduction

The rel-18 work on further NR mobility enhancements is described in the WID in [1]. The WID includes an objective related to specifying CHO configuration including both a target MCG and target candidate SCGs for CPAC (objective 4):

- To specify CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC [RAN3, RAN2]

  - CHO including target MCG and target SCG is used as the baseline

At the last RAN2 meeting (RAN2#123) the following agreements were reached:

UE does not remove the configuration for CHO including target MCG and candidate SCG configuration automatically when SCG is to be released.

R2 assumes Source MN initiates the preparation of the R18 CHO with candidate SCG(s), e.g., S-MN tells the T-MN whether it is allowed to configure candidate SCG(s). FFS the signalling details.

candidate MN recommends the candidate PSCells to candidate SN (for CHO with MN-initiated CPC).

CHO recovery details to handle the additions brought by this feature is FFS

R2 assumes for this R18 feature that the UE does not need to continue conditional reconfiguration evaluation for CHO with Candidate SCG(s) upon initiating SCG failure information procedure

Recommendation of the candidate PSCells can be based on measurement results.

R2 assumes for this R18 feature that the evaluation of the execution conditions for CHO with Candidate SCG(s) do not need to continue once PSCell change is triggered.

maxNrofCondCells = max number of conditional configurations that the UE can store (is assumed to be a memory limitation), value FFS

selectedCondRRCReconfig-r17 is not reused to indicate the selected target SCG to the target MN, i.e., UE indicates physCellId and ARFCN-ValueNR of the selected PSCell to target MN.

condEventA3 or condEventA5 is not used for the execution conditions for candidate PSCells (can be revisited later if strong justification can be provided)

condEvent A4 to be used for current PSCell (i.e., in case it is configured as candidate PSCell for evaluation) for CHO with candidate SCGs case.

In this contribution different remaining open issues for the objective 4 in the WID on rel-18 work on further NR mobility enhancements ([1]) are discussed.

# 2	Discussion

## 2.1	Configuration of execution conditions

In RAN2#122, RAN2 agreed that the candidate target MN informs the source MN about the prepared candidate PSCells and parameters of the associated execution conditions. The source MN will take the received input into account and prepare the execution conditions for the UE (referring to the source MN measConfig) both for the candidate PCell (based on e.g. measurements from the UE) and for the candidate PSCell (based on input from the candidate target MN).

The execution conditions are defined as measIds related to a measConfig. The easiest way to transfer the execution conditions for the candidate target PSCell (CPA/CPC) would be that the target MN sends a whole measConfig to the source MN, including the configuration of the execution conditions. The measConfig may also contain additional information, but it makes the signalling structure more simple and it doesn’t matter if the source MN would receive more information than it needs.

Since the information about CPA/CPC execution conditions consist of RRC IEs, they should be provided in the inter-node RRC message that is sent from the target MN to the source MN, i.e. within the HandoverCommand message that is used for a CHO with candidate SCG configuration. A Text Proposal for this is included in the Annex.

The target MN sends the whole measConfig and the measId for each candidate PSCell (i.e. the execution conditions) to the source MN.

The target MN sends the CPA/CPC execution condition information to the source MN in the HandoverCommand message that includes the CHO with candidate SCG configuration.

RAN2 to include the Text Proposal in the Annex into 38.331.

RAN2 reached an assumption at RAN2#123 that the source MN initiates the preparation of CHO with candidate SCG(s) to the candidate target MN, e.g. by indicating to the candidate target MN whether it is allowed to also configure candidate SCGs. The signalling details were however left FFS.

It is reasonable that source MN can decide whether to allow multiple CHO configurations from the candidate target MN or not. Regarding the signalling details, RAN3 are already discussing an indication from the source MN to the candidate target MN about the number of conditional reconfigurations that the candidate target MN is allowed to configure. Such indication will then tell the candidate target MN whether it is allowed to configure CHO with candidate SCG(s), and then how many such configurations. No additional indication should then be needed.

The source MN can indicate to the target MN whether it is allowed to configure CHO with candidate SCGs through the indication about allowed number of conditional reconfigurations. RAN3 to handle the signalling details for this.

At RAN2#123 the following agreement was reached for maxNrofCondCells.

maxNrofCondCells = max number of conditional configurations that the UE can store (is assumed to be a memory limitation), value FFS

Since each candidate PCell requires a single conditional reconfiguration per candidate PSCell, and one for the CHO only configuration, the current limit of 8 can be very limiting. The value was left FFS. To be able to configure CHO with candidate SCGs the value should therefore be increased in Rel-18. As indicated in the RAN2 agreement the limit is assumed to be a memory limitation, why it could be considered to introduce a separate UE capability for this.

It should be possible to configure more than 8 conditional reconfigurations in case CHO with candidate SCGs is configured.

A UE capability should be introduced to indicate the support for more than 8 conditional reconfigurations.

## 2.2	CHO recovery at re-establishment procedure

For legacy CHO, the UE may apply a CHO configuration instead of performing an RRC re-establishment procedure, if attemptCondReconfig is configured and the UE has a CHO configuration for the cell that it selected while timer T311 was running. In legacy CHO, there were only conditions for the PCell in CHO configurations. In rel-18 the UE may however be configured with CHO configurations that include execution conditions both for the PCell and for an included PSCell. The UE may also be configured with multiple candidate PSCells, associated with the same PCell. It is not obvious how the UE then would select an appropriate target configuration if it selects a CHO configuration that also includes a candidate PSCell.

It is assumed that the network also configures a CHO-only (or CHO with fixed SCG) configuration for PCells which have CHO with CPC/CPA configured, to ensure that CHO is always executed when needed. That configuration does then not include any CPA/CPC execution condition and thus corresponds to a legacy CHO configuration. For CHO recovery, the simplest solution is to only use that type of CHO configuration, that the UE applies the CHO configuration (for the selected PCell) which does not have any conditions for the SCG.

In case of CHO recovery, if attemptCondReconfig has been configured, the UE applies the CHO configuration (for the selected PCell) which does not have any CPA/CPC execution conditions.

## 2.3	Handling of CHO with candidate SCG configurations at reconfigurations

At RAN2#123 the following agreements and assumptions were reached regarding the handling of the CHO with candidate SCG configurations at SCG release and at PSCell change, respectively:

UE does not remove the configuration for CHO including target MCG and candidate SCG configuration automatically when SCG is to be released.

R2 assumes for this R18 feature that the evaluation of the execution conditions for CHO with Candidate SCG(s) do not need to continue once PSCell change is triggered.

The difference in handling for the two cases is unclear where both concern a change of the serving PSCell. RAN2 has agreed that condEvent A4 is to be used for the candidate PSCells, including for configuration of the current PSCell as a candidate PSCell. There is thus no impact to the configured execution conditions due to neither SCG release nor PSCell change.

In case the target SCG configuration is a delta configuration, it may however no longer be valid. This is however valid for both PSCell changes and for SCG release, and it could be handled through network configuration.

For the RAN2 assumption regarding PSCell change it is unclear whether the UE then should keep the CHO with candidate SCG configurations, but not continue evaluation. How should the evaluation then be restarted again? We think the behaviour for the two cases should be aligned and that the UE should keep the CHO with candidate SCG configurations and continue the evaluation, both at SCG release and at change of serving PSCell, unless explicitly reconfigured by the network.

The handling for the two cases should therefore be aligned, where the UE keeps the CHO with candidate SCG configurations, and continues the evaluation, both at SCG release and at change of serving PSCell, unless explicitly reconfigured by the network. If needed, the network can either reconfigure or release the CHO with candidate SCG configuration(s).

The UE should keep the CHO with candidate SCG configurations and continue the evaluation, both at SCG release and at change of serving PSCell, unless explicitly reconfigured by the network.

# 3	Conclusion

Based on the discussion in the previous sections the following is proposed:

Proposal 1	The target MN sends the whole measConfig and the measId for each candidate PSCell (i.e. the execution conditions) to the source MN.

Proposal 2	The target MN sends the CPA/CPC execution condition information to the source MN in the HandoverCommand message that includes the CHO with candidate SCG configuration.

Proposal 3	RAN2 to include the Text Proposal in the Annex into 38.331.

Proposal 4	The source MN can indicate to the target MN whether it is allowed to configure CHO with candidate SCGs through the indication about allowed number of conditional reconfigurations. RAN3 to handle the signalling details for this.

Proposal 5	It should be possible to configure more than 8 conditional reconfigurations in case CHO with candidate SCGs is configured.

Proposal 6	A UE capability should be introduced to indicate the support for more than 8 conditional reconfigurations.

Proposal 7	In case of CHO recovery, if attemptCondReconfig has been configured, the UE applies the CHO configuration (for the selected PCell) which does not have any CPA/CPC execution conditions.

Proposal 8	The UE should keep the CHO with candidate SCG configurations and continue the evaluation, both at SCG release and at change of serving PSCell, unless explicitly reconfigured by the network.

# 4	References

RP-221799, Revised WID on Further NR mobility enhancements, MediaTek Inc., 3GPP TSG RAN Meeting #96, June 6-9, 2022

# Annex

Below is a Text Proposal for inclusion of the execution condition information for CPA/CPC in the inter-node HandoverCommand message (in 38.331, section 11.2.2) that is sent from the target MN to the source MN. The TP is based on 38.331, v17.5.0.

#### –	HandoverCommand

This message is used to transfer the handover command as generated by the target gNB.

Direction: target gNB to source gNB/source RAN.

HandoverCommand message

-- ASN1START

-- TAG-HANDOVER-COMMAND-START

HandoverCommand ::=                 SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

handoverCommand                     HandoverCommand-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

HandoverCommand-IEs ::=             SEQUENCE {

handoverCommandMessage              OCTET STRING (CONTAINING RRCReconfiguration),

nonCriticalExtension                SEQUENCE {}HandoverCommand-v18xxx-IEs              OPTIONAL

}

HandoverCommand-v18xxx-IEs ::=      SEQUENCE {

cpacExecutionConditionInfo-r18      CPAC-ExecutionConditionInfo-r18                    OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                        OPTIONAL

}

CPAC-ExecutionConditionInfo-r18 ::= SEQUENCE {

cpacExecCondMeasConfig-r18          MeasConfig                                         OPTIONAL,

cpacExecCondMeasId-r18              SEQUENCE (SIZE (1..2)) OF MeasId

}

-- TAG-HANDOVER-COMMAND-STOP

-- ASN1STOP