3GPP TSG-RAN WG2 #123-bis	R2-2310897

Xiamen, China, 9th – 13th October 2023

Agenda Item:	7.12.3

Source:	Ericsson

Title:	Conclusions of CAG feature for mobile IAB

Document for:	Discussion, Decision

# 1	Introduction

In this contribution is to address the LS received by SA2 in [1] about the CAG solution for mobile IAB.

# 2	Discussion

About the CAG concerns RAN2 had for mobile IAB, SA2 has just sent a reply in [1] where the following is stated:

------------------------------------------------------------------------------------------------------------------------------------------------

SA2 has specified the CAG mechanism as included in clause 5.35A.7 of TS 23.501 for MBSR/mobile IAB covering following aspects:

- MBSR/mobile IAB node is configured with CAG identifier(s) and responsible for broadcast the CAG identifier(s) when MBSR is integrated into the system.

- UE access control to MBSR/mobile IAB node is based on the CAG information provided in the subscription data.

- In Rel-18, the CAG information for UE access control is enhanced with “time validity information” which indicates the applicable time duration of CAG identifier(s) as described in clauses 5.30.3.3 and 5.30.3.4 of TS 23.501. No AS layer impact is expected for this enhancement, as the “time validity information” is not provided to NG-RAN by AMF and is only provided to UEs that indicated the support of the validity information in NAS.

SA2 indicates that CAG for MBSR/mobile IAB is applied under the concept of existing PNI-NPN as further clarified in the agreed CR.

------------------------------------------------------------------------------------------------------------------------------------------------

According to the reply, SA2 does not envision any AS layer impact on the agreement they made. Therefore, legacy signalling that is present in RRC for broadcasting the CAG identifiers with the PLMN list and NPN PLMN list already support the agreements made by SA2. Therefore, RAN2 does not expect any impact on current specification for the CAG feature for mobile IAB.

RAN2 to agree that no specification impact is foreseen for the CAG feature for mobile IAB specified by SA2.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to agree that no specification impact is foreseen for the CAG feature for mobile IAB specified by SA2.

# 4	References

R2-2309475, Reply LS CAG solution for mobile IAB, SA2, 3GPP RAN2#123-bis meeting, China, October 2023.