3GPP TSG-RAN WG2 Meeting #123-bis	R2-2310933

Xiamen, China, 9 October – 13 October 2023               (revision of R2-2308704)

Agenda item:	7.8.3

Source: 	Huawei, HiSilicon

Title: 	Further discussion on flight path reporting

Document for:	Discussion and Decision

- Introduction

In RAN2 meeting #123, the following agreements have been reached regarding flight path report for NR UAVs [1]:

Agreements:

Flight path update/reporting

- No delta signaling for updated flight path will be supported

- Confirm that same indication is used for both initial and updated flightpath plan

- If any waypoint/timestamp has changed more than the configured threshold the UE triggers flightpath update indication.

- When both distance and time threshold are configured, the flightpath update can be triggered if either of those conditions are met.   No prohibit timer will be introduced.

This contribution is a revision of R2-2308704. Compared to the previous version we analysed a few more technical aspects due to be resolved in RAN2.

- Discussion

The delta time and/or distance can control the frequency of the UE’s flight path update indication very well. Sometimes, the different gNBs may be interested in different waypoints. For example, we assume gNB1 is interested in waypoints 1, 2, 3 (these waypoints are in the coverage of gNB1) and gNB2 is interested in waypoints 5, 6, 7 (these waypoints are in the coverage of gNB2). It is obvious that gNB1 does not care about the changing of others except for waypoints 1, 2, and 3. Another example is that the gNB may need to configure different delta time and/or distance for the centre of the cell and the edge of the cell when the UAV crosses the cell. Thus, the gNB needs to configure the delta time and/or the delta distance only for certain waypoints for the flight path update indication trigger to the UE.

Besides, the gNB may be more interested in certain waypoints and does not want to put any limitations on these waypoints, i.e., these waypoints need to be reported every time, even for the smallest change. In this case, the gNB can set delta time and/or distance to zero as the triggering condition for flight path update indication.

Proposal 1: The gNB can configure the flight path update indication threshold for a portion of the waypoints in the flight path.

Fig. 1 Illustration of flight path transfer.

RAN2 has agreed that the source gNB forwards the flight path to the target gNB during the handover procedure, which can reduce the time that the target gNB obtains the flight path. However, the source gNB may not have (collected) the new flight path or anyway have an old (out of date) flight path, but it has (received) the flight path (update) indication. This is because the source gNB may not be interested in the flight path (update) when it receives the flight path indication. At this time, the source gNB needs to send the old flight path to the target gNB according to the current RAN2 agreement. Then, the target gNB may make the wrong decision based on this old (out of date) flight path.

This issue was confirmed in the last meeting, and some companies think it can be solved by the current mechanism. The current mechanism is that the UAV can send the flight path indication via RRCReconfigurationComplete to the target gNB.

This mechanism will introduce another issue. When the target gNB receives the flight path available indication via RRCReconfigurationComplete during handover procedure, it will request the flight path from the UAV. However, it has been agreed that the source gNB sends the flight path to the target gNB via handover request message in the previous meeting. In this case, we think the source gNB will carry the flight path information in the handover request message if the source gNB has the flight path information even if the flight path information is optional in handover request. This is a reasonable implementation because the source gNB does not know whether the flight path information is needed for the target gNB.

According to the above analysis, we can see that the target gNB will request the flight path again even though it has received the flight path information during handover procedure. At this time, there is no meaning for that source gNB to forward the flight path information to the target gNB during handover procedure, and message signalling overhead is inevitable.

Thus, the UAV should not send the flight path available indication through the RRCReconfigurationComplete during the handover procedure if it has already sent the flight path available to the source gNB. In this way we can clearly distinguish the two different cases of out of date flight path vs. up to date flight path.

Proposal 2: The UAV should not send the flight path available indication through the RRCReconfigurationComplete during the handover procedure if it has already sent the flight path available to the source gNB.

- Conclusion

Proposal 1: The gNB can configure the flight path update indication threshold for a portion of the waypoints in the flight path.

Proposal 2: The UAV should not send the flight path available indication through the RRCReconfigurationComplete during the handover procedure if it has already sent the flight path available to the source gNB.

- Reference

[1] Draft_R2_123_meeting_report_v1