3GPP TSG-RAN WG2 #123-bis					      		 R2-2311039

Xiamen, China – October 9 - 13, 2023		                   Revision of R2-2307751

Agenda item:	7.9.4

Source: 	Kyocera

Title: 	Considerations for multipath relay operations for Scenario 1

Document for:	Discussion

# Introduction

In this contribution, we consider some remaining issues related to RLF handling in Scenario 1.

# Failure handling in Scenario 1

During the RAN2#122 meeting, the following agreement was reached:

For Scenario-1/2, when reporting direct-path failure via indirect-path, use MCGFailureInformation message. FFS on whether additional IE needs to be introduced.

For multipath relay whereby the cell supporting the direct path is different from the cell supporting the indirect path, when the direct path experiences RLF and only the indirect path remains connected, it is likely that when the remote UE is only PC5 connected to the relay UE that it is already OOC from the PCell on the direct path and no suitable cell can be found; hence, recovery from RLF in the direct path is not very likely; and direct path change (Case E) is not applicable. Since PCell is expected to be used by the remote UE for RLM, and RLF has already been declared by the remote UE, to prevent the remote UE to perform RRC Reestablishment when the indirect path is still good, PCell should be reconfigured on the cell on the indirect path. Since the remote UE only has the indirectly connected path to the gNB, this is essentially the same condition before the direct path was added to form multipaths for the remote UE.  Under this condition, the PCell on the direct path should be switched back to the cell on the indirect path, and the remote UE will be configured with only the indirect path.  In order to facilitate the timely reconfiguration of the PCell change to the indirect path, the remote UE could include in the MCGFailureInformation an OOC indication as an additional IE as a new failureType indication which is used to inform the gNB that it is no longer in-coverage of the PCell on the direct path and no suitable cell can be found, if split SRB1 is configured.

Although the direct path is under RLF, it would also be good for the gNB to know the condition of the indirect path.  Currently the MCGFailureInformation includes measurement results for the SCG, which isn’t available to the remote UE. It would be helpful for the gNB if the remote UE also includes the SL-RSRP measurements in the MCGFailureInformation.

- Proposal 1	In case of RLF on the direct path, the remote UE should inform the gNB of the failure over the indirect path with OOC indication as an additional IE in MCGFailureInformation.

- Proposal 2	The remote UE should have the option to include the SL-RSRP measurements when sending the MCGFailureInformation to the gNB.

During the RAN2#121-bis meeting, the following agreement was reached:

In case of Uu-RLF, at least for split SRB1, if SRB1 is available on indirect path not suspended, trigger report to network via indirect path to report the failure via a RRC message. Otherwise, RRC Re-establishment is initiated. RAN2 is requested to discuss whether the RRC message is the existing message e.g. MCGFailureInformation or a new message.

In our view, if non-split SRB1 is configured on the direct path, we should also consider other alternatives before reaching the conclusion that RRC Re-establishment is initiated.  RRC Re-establishment should only be used when there’s no connection available to the gNB. Furthermore, even if the gNB intends to configure the remote UE with split SRB1 (with or without duplication), it may not be supported by the remote UE.  Therefore, RAN2 should also consider the option for the remote UE to send a PC5-RRC message to its relay UE, whereby the contents of MCGFailureInformation is encapsulated within the PC5-RRC message.  Upon reception of this PC5-RRC message, the relay UE may send an RRC message over its own SRB1, with the encapsulated contents from the remote UE. Since a relay UE may support multiple remote UEs, the encapsulated contents should be properly addressed by the remote UE’s ID, e.g., C-RNTI as well as its serving cell on the failed direct path.

- Proposal 3	The remote UE should have the option to send the direct path failure report encapsulated in a PC5-RRC message and delivered via the relay UE’s SRB1, even when split SRB1 is not configured for the remote UE or if the remote UE does not support split SRB1.Y

According to the rapporteur in [xx], one of the remaining open issues is the choice of the message used to report indirect path failure sent on the direct path.  Based on the previous discussions, the remaining options are as follows:

- Indirect path failure is reported via the MCGFailureInformation message

- Indirect path failure is reported via the SidelinkUEInformationNR message

- Indirect path failure is reported via a new message

Although SidelinkUEInformationNR is the legacy method is used for reporting SL-RLF (e.g., in Rel-17 the relay UE reports the SL-RLF to the gNB using this message).  However, from the remote UE’s perspective in Rel-18, the indirect path contains two links, the PC5 link (between the remote UE and the relay UE) and the Uu link (between the relay UE and the gNB); therefore, the failure report should be able to identify the failed link when reporting the failure to the gNB.

With regards to the MCGFailureInformation, it already includes Uu failure information but not SL failure information, so a new IE will need to be included for this failure case.  Also, if MCGFailureInformation is used for the case of direct path failure and indirect path failure, additionally differentiation need to be introduced.  Therefore, we slightly prefer to introduce a new message for the indirect path failure report.

- Proposal 4	A new message is introduced for the remote UE to report the indirect path failure over the direct path.

Regardless of which message is used for the indirect path failure report, it should also be discussed what contents the remote UE should include in the message.  In our view the SL-RSRP measurements of current and candidate relay UEs should also be included in the message.

- Proposal 5	The remote UE should have the option to include the SL-RSRP measurements when sending the indirect path failure report to the gNB.

During the RAN2#122 meeting, it was also discussed how the indirect path should be suspended or released in case the relay UE handovers to another cell.  It is currently FFS: “whether rely on network to release configuration of relay UE at remote UE before relay UE handover, or rely on remote UE to suspend the indirect path upon reception of notification message indicating relay UE handover”.  Although both options can work in the sense that the remote UE will stop using the indirect path, the UE behavior should be further clarified.  In case the remote UE receives a notification message from the relay UE due to its handover operation, the remote UE suspends its transmission on the indirect path; however, it will still be necessary for the PCell to reconfigure the remote UE with the direct path-only configuration or perhaps change the indirect path with a target relay UE (based on the latest measurement report).  Therefore, it would be simpler for the network to release configuration of relay UE at remote UE before relay UE handover.

- Proposal 6	In case the serving relay UE experiences handover, the network should release configuration of relay UE at remote UE before relay UE handover.

Another issue that should be considered concerns the remote UE’s behavior in case both direct and indirect paths experience RLF.  RAN2 has already agreed (based on the highlighted agreements above) that the remote UE would perform RRC Reestablishment procedure under this condition and it was also agreed that the remote UE will not reestablish directly into multipath configuration.  However, it should be further discussed whether the remote UE is allowed to reestablish only on the direct path or it is also allowed to reestablish on the indirect path.  For Rel-17, it was already specified in [2] that the remote UE would initiate RRC Reestablishment procedure when the following conditions are met:

1> upon detecting sidelink radio link failure by L2 U2N Remote UE in RRC_CONNECTED, in accordance with clause 5.8.9.3; or

- upon reception of NotificationMessageSidelink including indicationType by L2 U2N Remote UE in RRC_CONNECTED, in accordance with clause 5.8.9.10; or

During the discussions at the RAN2#121 meeting, some companies considered that it may be simpler to only allow the remote UE to only reestablish on the direct path; however, this assumes the remote UE is in-coverage of the PCell on the direct path when the path failures occur.  In case the remote UE is OOC, the only option for the remote UE would be to reestablish on the indirect path which would be consistent with the Rel-17 UE behavior.  We think it would be too restrictive to only allow the UE to reestablish on the direct link.  However, RRC reestablishment can be succeeded if the NW can verify the UE context and resume the SRB1, which may be available on the indirect path depending on whether split-bearer for SRB1 is configured.   Therefore, rather than restricting the remote UE to only reestablish on the direct path, we could allow the NW to inform the remote UE whether RRC reestablishment can also be initiated by the remote UE on the indirect path.

- Proposal 7	The remote UE is allowed to initiate RRC reestablishment procedure over either the direct path or indirect path when failure occurs on both paths.

If Proposal 7is agreeable, it should also be considered whether the remote UE would attempt to initiate RRC reestablishment on a direct path or on the indirect path, and whether this should be up to UE implementation as it is for Rel-17.

- Proposal 8	When both direct and indirect path experiences failures, RAN2 should consider whether the choice for RRC Reestablishment between direct path and indirect path should be up to UE implementation.

In case RRC reestablishment is performed, it should be also considered if it is allowed only in failure cases or even if the relay UE were to release the PC5 connection to the remote UE, due to upper layer request rather than due to RLF on the Uu link. In our view, when the remote UE receives the PC5-S message to release the connection, it would perform relay reselection.

- Proposal 9	The remote UE is not allowed to initiate RRC reestablishment in case the failure in the indirect path is due to PC5-S release by the relay UE.

# Conclusion

In this contribution, we addressed some of the remaining issues related to failure handling in Scenario 1.  RAN2 is kindly asked to take into account the proposals below:

- Proposal 1	In case of RLF on the direct path, the remote UE should inform the gNB of the failure over the indirect path with OOC indication as an additional IE in MCGFailureInformation.

- Proposal 2	The remote UE should have the option to include the SL-RSRP measurements when sending the MCGFailureInformation to the gNB.

- Proposal 3	The remote UE should have the option to send the direct path failure report via the relay UE’s SRB1, even when split SRB1 is not configured for the remote UE or if the remote UE does not support split SRB1.

- Proposal 4	A new message is introduced for the remote UE to report the indirect path failure over the direct path.

- Proposal 5	The remote UE should have the option to include the SL-RSRP measurements when sending the indirect path failure report to the gNB.

- Proposal 6	In case the serving relay UE experiences handover, the network should release configuration of relay UE at remote UE before relay UE handover.

- Proposal 7	The remote UE is allowed to initiate RRC reestablishment procedure over either the direct path or indirect path when failure occurs on both paths.

- Proposal 8	When both direct and indirect path experiences failures, RAN2 should consider whether the choice for RRC Reestablishment between direct path and indirect path should be up to UE implementation.

- Proposal 9	The remote UE is not allowed to initiate RRC reestablishment in case the failure in the indirect path is due to PC5-S release by the relay UE.

# References

- [Post123][Relay] Remaining open issues, LG Electronics

- TS 38.331