3GPP TSG-RAN WG2 Meeting #123bis	R2-2311047

Xiamen 9th – 13th Oct, 2023

Source: 		   ZTE Corporation, Sanechips

Title:            Clarification on the Intra-band Non-CA Capabilities Reporting

Agenda item:    5.1.3.2

Document for:   Discussion and Decision

- Introduction

About the Intra-band non-CA capability reporting, a paper [1] was submitted to discuss the Intra-band non-CA capability reporting for the “xDL + 1UL (means UL is single cc or contiguous CA) NR CA” capability reporting, and at last a CR[2] was approved based on RAN4’s confirmation LS in [3].

In which, it clarified that the similar principle as the LTE (as specified in TS 36.306) was also adopted for the NR.

supportedBandCombinationList Defines the supported NR and/or MR-DC band combinations by the UE. For each band combination the UE identifies the associated feature set combination by featureSetCombinations index referring to featureSetCombination. A fallback band combination resulting from the reported CA and MR-DC band combination is not signalled but the UE shall support it. For intra-band non-contiguous CA band combinations, the UE only includes one band combination, and exclude the others for which the presence of uplink CA bandwidth class in the band combination entry is different. One band combination entry can also indicate support of any other possible permutations in the presence of uplink CA bandwidth class where a paired downlink CA bandwidth class is the same or where the number of UL CCs is smaller than the one of paired DL CCs expressed by the CA bandwidth class, as specified in TS 36.306 [15]. For these band combinations not included in the capability, the supported feature set is the same as the ones for the band combination included in the UE capability.

However, during the LTE discussion, only “xDL + 1UL LTE CA (means UL is single cc or contiguous CA)” case was considered as in [4][5]. Now according to the current Intra-band non-CA band combination table, more and more “xDL+x UL NR CA (means UL CA is non-contiguous)” BCs were supported, in which non-contiguous CA was also supported on the uplink. In this paper, we’d like to clarify the capability reporting for the “xDL+ x UL NR CA” BCs case.

- Discussion

In this chapter, for the discussion convenience, we’d like to give a short description on on the “xDL + 1UL NR CA” reporting mechanism first, and then list the current supported “xDL+ xUL NR CA” BCs, and at last we share our understanding on the capability reporting for such kinds of BCs.

2.1 “xDL + 1UL NR CA” Reporting

Since that the similar “xDL + 1UL LTE CA” Reporting mechanism was adopted for the NR, we’d like to give a short description on the “xDL + 1UL CA” Reporting. As described in the 36.306:

4.3.5.21 reducedIntNonContComb-r13 This field defines whether the UE supports receiving requestReducedIntNonContComb. If the UE supports reducedIntNonContComb-r13, the UE only includes one intra-band non-contiguous CA band combination, and exclude the other intra-band non-contiguous CA band combinations for which the presence of uplink CA bandwidth class in the band combination entry is different. One band combination entry can also indicate support of any other possible permutations in the presence of uplink CA bandwidth class where a paired downlink CA bandwidth class is the same or where the number of UL CCs is smaller than the one of paired DL CCs expressed by the CA bandwidth class. For example, if the UE supports reducedIntNonContComb-r13, the UE only needs to report "DL: CA_42C-42A, UL: 42A paired with DL 42C", in order to indicate also support of "DL: CA_42C-42A, UL: 42A paired with DL 42A", "DL: CA_42A-42C, UL: 42A paired with DL 42A" and "DL: CA_42A-42C, UL: 42A paired with DL 42C".

In short, for the below 4 BCs, the UE only need to report one of them.

（1）DL: CA_42C-42A, UL: 42A paired with DL 42C （2）DL: CA_42C-42A, UL: 42A paired with DL 42A （3）DL: CA_42A-42C, UL: 42A paired with DL 42A （4）DL: CA_42A-42C, UL: 42A paired with DL 42C

In the LTE, this mechanism was introduced when the network indicate “requestReducedIntNonContComb-R13” to avoid NBC issue, but in the NR, this mechanism was supported natively. During RAN2 discussion, a LS [6] was sent to RAN4 to confirm “whether the same signalling reduction scheme should be supported for NR as default UE behavior”

1. Intra-band non-contiguous NR CA signalling (LS to Ran4 in [6]) LTE CA supports the mechanism to reduce the capability signalling size of intra-band non-contiguous band combinations, a.k.a. reducedIntNonContComb (see section 5 for reference). By this mechanism the UE can reduce the explicitly listed intra-band contiguous band combinations. Since it was introduced in a later release of LTE, the UE had to indicate that it supports this reduced signalling by a UE capability bit and the eNB had to indicate that it understands the reduced signalling, too. RAN2 discussed whether the same signalling reduction scheme should be supported for NR as default UE behaviour. RAN4 feedback is deemed as necessary, since RAN2 introduced the reducedIntNonContComb for LTE based on RAN4 input.

In the RAN4 reply LS [3], it said RAN2 can determine the signaling optimization.

1. Intra-band non-contiguous NR CA signalling Ran4 Reply LS in[3] RAN4 believes that RAN2 is the best TSG to determine signalling optimization that doesn’t involve any loss of functionality. RAN4 have no concerns and kindly asks RAN2 to introduce the reducedNonContComb for NR as default UE behaviour to reduce the capability signalling size.

Thus, the same signalling reduction scheme was supported for NR as default UE behavior at last.

Observation 1: For the intra-band “xDL + 1UL NR CA” capability reporting, NR takes the same mechanism as LTE, e.g. for the below 4 BCs, the UE only need to report one of them.

（1）DL: CA_42C-42A, UL: 42A paired with DL 42C （2）DL: CA_42C-42A, UL: 42A paired with DL 42A （3）DL: CA_42A-42C, UL: 42A paired with DL 42A （4）DL: CA_42A-42C, UL: 42A paired with DL 42C

2.2 “xDL + xUL NR CA” Reporting

According to the above discussion, we can see that the reporting for the “xDL + 1UL CA” is quite clear for both the NR and the LTE. However, whether the similar signalling reduction scheme can also be adopted for the Intra-band “xDL + xUL CA” is not clear. As summarized in the Fig 1, some Intra-band “xDL + xUL NR CA” BCs have been included in the RAN4 spec.

Table 1 Intra-band xDL + xUL CA

| LT or NR FR | DL | UL |
| --- | --- | --- |
| LTE(From 36101-i20 Table 5.6A.1-3) | CA_4A-4A | CA_4A-4A |
| FR1(From 38101-1-i20 Table 5.5A.2) | CA_n41(2A) | CA_n41(2A) |
|  | CA_n77(2A) | CA_n77(2A) |
|  | CA_n78(2A) | CA_n78(2A) |
| FR2(From 38101-1-i20 Table 5.5A.2) | CA_n257(2G) | CA_n257(2G) |
|  | CA_n260(2A) | CA_n260(2A) |
|  | CA_n260(3A) | CA_n260(3A) |

From the Table 1, we can see that in LTE, only one BC was supported for the Intra-band “xDL + xUL CA” case, but for the NR, more cases are supported and the x can be even 3.

Observation 2: Compared with LTE, more Intra-band “xDL + xUL CA”BCs have been supported.

Then the question is whether the similar signalling reduction scheme has also been adopted for the Intra-band “xDL + xUL CA” in the existing UE implementation. For example, the UE can supports the below BC1, then if the UE has taken the same signaling reduction scheme, the UE only need to report the Feature Set Entry 1, otherwise the UE need to report both the Feature Set Entry 1 and 2.

Table 2: Different Feature Set Entry for the “2DL + 2UL NR CA”

| BC1 | DL n41A (DL feature set 1)_n41A(DL feature set 2) UL n41 A (UL feature set 1)_n41A(UL feature set 2) | Feature Set Entry 1 |
| --- | --- | --- |
|  | DL n41A (DL feature set 1)_n41A(DL feature set 2) UL n41 A (UL feature set 2)_n41A(UL feature set 1) | Feature Set Entry 2 |

Proposal 1: RAN2 to confirm whether the signalling reduction scheme (for the “xDL + 1UL CA” case) has been adopted for the Intra-band “xDL + xUL CA” in the existing UE implementation including both LTE and NR.

Based on the proposal 1, we can further confirm whether the current description on signalling reduction scheme for the “intra-band non-contiguous CA” also includes the Intra-band “xDL + xUL CA” case or only for the “xDL + 1UL CA” case.

supportedBandCombinationList --38306 Defines the supported NR and/or MR-DC band combinations by the UE. For each band combination the UE identifies the associated feature set combination by featureSetCombinations index referring to featureSetCombination. A fallback band combination resulting from the reported CA and MR-DC band combination is not signalled but the UE shall support it. For intra-band non-contiguous CA band combinations, the UE only includes one band combination, and exclude the others for which the presence of uplink CA bandwidth class in the band combination entry is different. One band combination entry can also indicate support of any other possible permutations in the presence of uplink CA bandwidth class where a paired downlink CA bandwidth class is the same or where the number of UL CCs is smaller than the one of paired DL CCs expressed by the CA bandwidth class, as specified in TS 36.306 [15]. For these band combinations not included in the capability, the supported feature set is the same as the ones for the band combination included in the UE capability.

4.3.5.21 reducedIntNonContComb-r13 -36306 This field defines whether the UE supports receiving requestReducedIntNonContComb. If the UE supports reducedIntNonContComb-r13, the UE only includes one intra-band non-contiguous CA band combination, and exclude the other intra-band non-contiguous CA band combinations for which the presence of uplink CA bandwidth class in the band combination entry is different. One band combination entry can also indicate support of any other possible permutations in the presence of uplink CA bandwidth class where a paired downlink CA bandwidth class is the same or where the number of UL CCs is smaller than the one of paired DL CCs expressed by the CA bandwidth class. For example, if the UE supports reducedIntNonContComb-r13, the UE only needs to report "DL: CA_42C-42A, UL: 42A paired with DL 42C", in order to indicate also support of "DL: CA_42C-42A, UL: 42A paired with DL 42A", "DL: CA_42A-42C, UL: 42A paired with DL 42A" and "DL: CA_42A-42C, UL: 42A paired with DL 42C".

Proposal 2: RAN2 to confirm whether the current description on signalling reduction scheme for the “intra-band non-contiguous CA” also includes the Intra-band “xDL + xUL CA” case or only for the “xDL + 1UL CA” case.

If the current description is only for the “xDL + 1UL CA” case, then RAN2 need to further clarify the fallback case for the Intra-band “xDL + xUL” CA. For example, the UE only support the below FeatureSet Entry 1:

| BC1 | DL n41A (DL feature set 1)_n41A(DL feature set 2) UL n41 A (UL feature set 1)_n41A(UL feature set 2) | Feature Set Entry 1 |
| --- | --- | --- |

According to the fallback rule, the UE can support the below fallback BCs with single CC on UL, then according to the “xDL + 1UL CA” signaling reduction rule, it would mean that the UE can support the UL single CC paired with any DL CC.

| FallBack BC1 | DL n41A (DL feature set 1)_n41A(DL feature set 2) UL n41 A (UL feature set 1) | The UL feature set 1 can paired with any DL feature set (1 or 2) |
| --- | --- | --- |
| FallBack BC2 | DL n41A (DL feature set 1)_n41A(DL feature set 2) UL n41A(UL feature set 2) | The UL feature set 2 can paired with any DL feature set (1 or 2) |

Proposal 3: If the current description is only for the Intra-band “xDL + 1UL CA” case, Ran2 confirm that the same rule for the “xDL + 1UL CA” case can still be used for the Intra-band “xDL + 1UL CA” BC that’s fallback from the Intra-band “xDL + xUL CA” BC.

If the current description is only for the “xDL + 1UL CA” case, then RAN2 can further discuss whether and how to introduce the similar signalling reduction scheme for the “xDL + xUL CA” case to avoid NBC issue.

Proposal 4: If the current description is only for the Intra-band “xDL + 1UL CA” case, then RAN2 can further discuss whether and how to introduce the similar signalling reduction scheme for the Intra-band “xDL + xUL CA” case.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: For the intra-band “xDL + 1UL NR CA” capability reporting, NR takes the same mechanism as LTE, e.g. for the below 4 BCs, the UE only need to report one of them.

（1）DL: CA_42C-42A, UL: 42A paired with DL 42C （2）DL: CA_42C-42A, UL: 42A paired with DL 42A （3）DL: CA_42A-42C, UL: 42A paired with DL 42A （4）DL: CA_42A-42C, UL: 42A paired with DL 42C

Observation 2: Compared with LTE, more Intra-band “xDL + xUL CA”BCs have been supported.

Proposal 1: RAN2 to confirm whether the signalling reduction scheme (for the “xDL + 1UL CA” case) has been adopted for the Intra-band “xDL + xUL CA” in the existing UE implementation including both LTE and NR.

Proposal 2: RAN2 to confirm whether the current description on signalling reduction scheme for the “intra-band non-contiguous CA” also includes the Intra-band “xDL + xUL CA” case or only for the “xDL + 1UL CA” case.

Proposal 3: If the current description is only for the Intra-band “xDL + 1UL CA” case, Ran2 confirm that the same rule for the “xDL + 1UL CA” case can still be used for the Intra-band “xDL + 1UL CA” BC that’s fallback from the Intra-band “xDL + xUL CA” BC.

Proposal 4: If the current description is only for the Intra-band “xDL + 1UL CA” case, then RAN2 can further discuss whether and how to introduce the similar signalling reduction scheme for the Intra-band “xDL + xUL CA” case.

- Reference

- R2-1811983    Band combination signalling for xDL + 1UL NR CA	NTT DOCOMO, INC.	discussion	Rel-15	38.306	NR_newRAT-Core

- RP-182813      Update of UE capabilities  15.3.0  CR0063  Intel Corporation

- R4-1813862     Reply LS on intra-band combination for NR CA and MR-DC  Ran4 to: RAN2

- R2-161620        Continuing discussion on intra-band non-contiguous CA capability    NTT DOCOMO,

- R2-161785       Modification of network requested CA band combination retrieval for intra-band non-contiguous CA       NTT DOCOMO, INC.     CR   36.306    13.0.0

- R2-1813442     LS on intra-band combination for NR CA and MR-DC  RAN2  to Ran4