3GPP TSG-RAN WG2 Meeting #123bis          			       R2-2311101

Xiamen, China, 21 - 25 Oct 2023                                   Revision of R2-2308796

Agenda item:	7.16.2.2

Source:	LG Electronics Inc.

Title:	AIML method Data Collection

Document for:	Discussion and Decision

# 1. Introduction

In this contribution, we discuss the data collection method, data collection requirements, and existing procedure analysis focusing on the CSI compression and CSI/beam prediction use case.

# 2. Discussion

## 2.1 Data Collection Exchange Scenario

Based on the RAN1 LS and agreement, Table 1, Table 2 and Table 3 can be starting point for input data related entity for the LCM purpose and specific use cases.

| CSI Compression | NW part | UE part |
| --- | --- | --- |
| training | training data = Can be generated by UE/NW |  |
| inference | inference data = Can be generated by UE and terminated at gNB | inference data = internally available in UE |
| monitoring | Monitoring data = Calculated performance metrics (if needed) or data needed for performance metric calculation (if needed) can be generated by UE and terminated at gNB (RAN1 112) NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side. | Monitoring data = Indicated/Obtained from the network side (RAN1 112) UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side. |

<Table 1. LCM input data related entity for CSI compression use case>

| CSI Prediction | NW sided | UE sided |
| --- | --- | --- |
| training |  | training data = can be generated by UE |
| inference |  | input data = internally available in UE |
| monitoring | Monitoring data = Calculated performance metrics (if needed) or data needed for performance metric calculation (if needed) can be generated by UE and terminated at gNB | Monitoring data = Performance metrics are available inside the UE. UE can independently monitor a model's performance w/o any data input from NW |

<Table 2. LCM input data related entity for CSI Prediction use case>

| Beam Prediction | NW sided | UE sided |
| --- | --- | --- |
| training | training data = can be generated by UE/NW |  |
| inference | input data = can be generated by UE and terminated at gNB | input data = internally available in UE |
| monitoring | Calculated performance metrics (if needed) or data needed for performance metric calculation (if needed) can be generated by UE and terminated at gNB | Performance metrics are available inside the UE UE can independently monitor a model's performance w/o any data input from NW |

<Table 3. LCM input data related entity for Beam prediction use case>

Among these, LCMs that require data collection and information exchange for each specific use case include:

- CSI compression: NW/UE part training, NW part inference, and NW/UE part monitoring

- CSI prediction: NW sided monitoring

- Beam management: NW/UE sided training, NW sided inference, and NW sided monitoring

We propose to use Table 1, Table 2, and Table 3 as baselines for the use cases that require data collection and information exchange.

Proposal 1. Capture the Table1/2/3 that requires data collection and data exchange into the TR as Annex

## 2.2 Existing Procedure Analysis

The following table is intended to analyze existing procedures based on the data collection requirements.

|  | Latency (Measure to Report) | Message size | RRC state for measurement | Data contents | Method of Report Trigger |
| --- | --- | --- | --- | --- | --- |
| Logged MDT | High | Large (<=64kbyte) | Idle/Inactive | Cell measurement-related info | Collective report by NW request |
| RRM (Immediate MDT) | Medium | Medium (<=9kbyte) | Connected | Beam measurement-related info | Periodic, EventTriggered |
| UAI | Medium | Medium (<=9kbyte) | Connected | Up to configuration | Condition |
| CSI report | low | Small (<1706bit in PUCCH, <3840bit in PUSCH) | Connected | Beam measurement-related info | Periodic, Aperiodic, Semi-persistence |

< Table 2. Analysis for each existing procedure in RAN2>

Logged MDT

In the case of logged MDT, a framework already exists for transmitting collected measurement results. Measurements can be performed for a certain period, i.e., T330, and the collected measurement results can be transmitted to the network. Since it takes a long time to perform measurement and report, it can be suitable for non-time critical data collection for offline model training.

Observation 1. Since Logged MDT can take a long time to perform measurement and report, it can be suitable for non-time critical data collection. In addition, a framework already exists for transmitting collected measurement results. Considering this, it may be suitable for data collection for model training

However, the currently logged MDT procedure has the additional specific impacts.

- Specific beam measurement settings and reports: Since the purpose of logged MDT is to perform measurement in RRC idle/inactive state, there are no beam specific configurations/reports. For example, measurement/report related to SSB/CSI-RS is not considered.

- When/How to report the logged measurement results (for training) in connected state: In the current procedure, the UE can notify the network that the logged measurement result is available when entering connected state. The network can bring the logged measurement result through the UE information request/response procedure. In the connected state, additional consideration is needed to inform when it is available and when to report.

- When releasing the logged measurement configuration/results (for training): In the current specification, when the memory reserved for the logged measurement information becomes full, 48 hours after T330 expiry, etc., the UE releases the logged measurement configuration/results. It should be considered whether it is suitable to model training as well.

Above all, for the CSI/beam prediction use cases, it requires the beam measurement results in the connected state for the model training. While logged MDT does not take connected mode and beam measurements into account. Considering motivation/efforts for specifying beam measurement mechanisms in the existing logged MDT, Logged MDT is not suitable for model training as well as model inference/monitoring.

Observation 2. For the logged MDT, it is necessary to consider for model training as following:

- Whether measurement results In Idle/Inactive state are necessary

- How to specify beam measurement and report configuration for beam measurement results

- When/How to report the logged measurement results (for training) in connected state

- When releasing the logged measurement configuration/results (for training)

Proposal 2. Do not consider extending the logged MDT method for LCM purposes for CSI/Beam prediction use cases

Immediate MDT and RRM

Immediate MDT and RRM have different data termination locations, with Immediate MDT relying on OAM for data termination while RRM operates with gNB for data termination. However, Immediate MDT utilizes the RRM procedure to report data. From the RAN2 point of view, RRM analysis is required for the two existing procedures.

Regarding RRM, the L3 measurement results can be reported periodically or triggered by specific events based on the measurement configurations. Additionally, the report generated by RRM can include beam measurement results. If the RRC is involved in the beam measurement configuration and reporting, RRM is considered the simplest extensible procedure. This means that RRM can easily accommodate future expansions or modifications related to beam measurements. Accordingly, immediate MDT and RRM can be easily extended to include beam measurement-related information for model training for CSI/Beam prediction use cases.

When utilizing immediate MDT and/or RRM for model training, it is crucial to consider the size of the RRM messages. To ensure optimal results, UE should transmit measured data in real-time instead of collecting data over extended periods. Also, additional consideration is required on how to separately set the beam measurement/reporting conditions for model training.

Observation 3. Immediate MDT utilizes the RRM procedure to report data. From the RAN2 point of view, RRM analysis is required for the immediate MDT/RRM extension.

Observation 4. RRM is considered the simplest extensible procedure since RRM can easily accommodate future expansions or modifications related to beam measurements. When utilizing immediate MDT and/or RRM for model training, UE can send real-time measured data while connected, taking into account the RRM message size.

Proposal 3. Immediate MDT and RRM can be extended for model training for CSI/Beam prediction use cases

- FFS whether/how to separately set the beam measurement/reporting conditions for model training.

Immediate MDT and RRM can also serve the purpose of model monitoring, taking into account latency and message size. To this end, additional considerations are required on how to separately set the beam measurement/reporting conditions for model monitoring. However, since the RRC message takes more than 10ms to transmit the beam measurement result due to the processing delay, immediate MDT/RRM are not suitable for model inference.

Observation 5. Considering latency and message size, immediate MDT/RRM is suitable for model monitoring but not for model inference

Proposal 4. Immediate MDT and RRM procedures can be extended for model monitoring for CSI/beam prediction use cases

- FFS whether separate beam measurement/report condition configuration is necessary for model monitoring

UAI

In the case of UAI, depending on the network settings, the UE’s preference (e.g., relate to DRX/CC/RRC state) or a specific problem (e.g., related IDC), or a specific status (e.g., fulfilment of RRM/ RLM/BFD relaxation conditions) can be reported to the network via UAI message. When sending RRC messages, it takes more than 10ms considering the processing delay. Therefore, UAI is not suitable for time critical report like model inference. Considering latency and message size, it can be suitable for model monitoring.

However, it should be considered which condition will trigger the report for model monitoring. For example, simply, an L1 indication that contains the performance metric-related information from a lower layer can trigger a report.

Observation 6. Considering latency, message size, and data contents, UAI can be used for model monitoring. However, it should be considered which condition will trigger the report for model monitoring.

Proposal 5. UAI procedure can be extended for model monitoring for CSI/beam prediction use cases

- FFS relevant information useful for model monitoring

L1 measurement (CSI reporting)

Lastly, in the case of CSI reporting, L1 measurement results can be delivered in periodic/aperiodic/semi-persistent methods according to configuration. It can be used for model inference taking latency and message size into account. According to the RAN1 agreement, enhancement in L1 reporting by extending the amount of measurement results is being discussed for model inference. RAN2 impact related to enhanced CSI reporting is dependent on the RAN1 input. Therefore, it is needed to wait for input from RAN1 about CSI reporting.

Proposal 6. CSI reporting extension is RAN1 scope. Wait RAN1 input to consider RAN2 impact

In summary, each existing procedure can be applicable for model management as following table:

|  | Terminated entity | Allowed payload size | Report latency | Supported report type | Security and Privacy | Usage of Model Management | Potential spec impact |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Logged MDT | Between UE and TCE/OAM | <=64kbyte (MDT buffer size limit) | Long (Reported after entering CONNECTED) | Upon gNB request after entering CONNECTED | Security via RRC message, Privacy via user consent | FFS | FFS |
| Immediate MDT | Between UE and TCE/OAM | <=9kbyte or 144kbyte (with 16 segments) | Medium (~20ms RRC signaling latency) | Event triggered report, Periodic reporting | Security via RRC message, Privacy via user consent | Training Monitoring | Separate beam measurement configuration for training/monitoring Separate report condition configuration for training/monitoring |
| L3 measure | Between UE and gNB | <=9kbyte or 144kbyte (with 16 segments) | Medium (~20ms RRC signaling latency) | Event triggered report, Periodic reporting | Security via RRC message | Training Monitoring | Separate beam measurement configuration for training/monitoring Separate report condition configuration for training/monitoring |
| L1 measure | Between UE and gNB | Small (<1706bit in PUCCH, <3840bit in PUSCH) | Short (can be symbol or slot level) | Aperiodic report, Semi-persistent report, Periodic report | No security | Inference FFS Monitoring | Up to RAN1 decision |
| UAI | Between UE and gNB | <=9kbyte or 144kbyte (with 16 segments) | Medium (~20ms RRC signaling latency) | Up to UE implementation when to report | Security via RRC message | Monitoring | Specify relevant information useful for monitoring |

< Table 3. Analysis of existing procedure for usage of model management>

Proposal 7. Add two columns for Usage of Model management and Potential spec impact in “Table 7.3.1.2-1. Existing data collection methods identified” in TR

# 3. Conclusion

Proposal 1. Capture the Table1/2/3 that requires data collection and data exchange into the TR as Annex

Observation 1. Since Logged MDT can take a long time to perform measurement and report, it can be suitable for non-time critical data collection. In addition, a framework already exists for transmitting collected measurement results. Considering this, it may be suitable for data collection for model training

Observation 2. For the logged MDT, it is necessary to consider for model training as following:

- Whether measurement results In Idle/Inactive state are necessary

- How to specify beam measurement and report configuration for beam measurement results

- When/How to report the logged measurement results (for training) in connected state

- When releasing the logged measurement configuration/results (for training)

Proposal 2. Do not consider extending the logged MDT method for LCM purposes for CSI/Beam prediction use cases

Observation 3. Immediate MDT utilizes the RRM procedure to report data. From the RAN2 point of view, RRM analysis is required for the immediate MDT/RRM extension.

Observation 4. RRM is considered the simplest extensible procedure since RRM can easily accommodate future expansions or modifications related to beam measurements. When utilizing immediate MDT and/or RRM for model training, UE can send real-time measured data while connected, taking into account the RRM message size.

Proposal 3. Immediate MDT and RRM can be extended for model training for CSI/Beam prediction use cases

- FFS whether/how to separately set the beam measurement/reporting conditions for model training

Observation 5. Considering latency and message size, immediate MDT/RRM is suitable for model monitoring but not for model inference

Proposal 4. Immediate MDT and RRM procedures can be extended for model monitoring for CSI/beam prediction use cases

- FFS whether separate beam measurement/report condition configuration is necessary for model monitoring

Observation 6. Considering latency, message size, and data contents, UAI can be used for model monitoring. However, it should be considered which condition will trigger the report for model monitoring.

Proposal 5. UAI procedure can be extended for model monitoring for CSI/beam prediction use cases

- FFS relevant information useful for model monitoring

Proposal 6. CSI reporting extension is RAN1 scope. Wait RAN1 input to consider RAN2 impact

Proposal 7. Add two columns for Usage of Model management and Potential spec impact in “Table 7.3.1.2-1. Existing data collection methods identified” in TR

# 4. References

[1] R2-2304541 [AT121bis-e][024][AIML18] Data Collection_v24_rapp

[2] R2-230XXXX RAN2 TP to TR 38.843 (from R1-2306235) v11 Ericsson (Rapp)

# 5. Annex

#### 7.3.1.2	Data collection

Data collection plays a crucial role in enabling the different use cases. Hence, the importance of defining the best approaches for collecting data to support model inference, monitoring, and training.

Table 7.3.1.2-1, Table 7.3.1.2-2, and Table 7.3.1.2-3 can be starting point for input data related entity for the LCM purpose and specific use cases

Table 7.3.1.2-1. LCM input data related entity for CSI compression use case

| CSI Compression | NW part | UE part |
| --- | --- | --- |
| training | training data = Can be generated by UE/NW |  |
| inference | inference data = Can be generated by UE and terminated at gNB | inference data = internally available in UE |
| monitoring | Monitoring data = Calculated performance metrics (if needed) or data needed for performance metric calculation (if needed) can be generated by UE and terminated at gNB | Monitoring data = Indicated/Obtained from the network side |

Table 7.3.1.2-2. LCM input data related entity for CSI prediction use case

| CSI Prediction | NW sided | UE sided |
| --- | --- | --- |
| training |  | training data = can be generated by UE |
| inference |  | input data = internally available in UE |
| monitoring | Monitoring data = Calculated performance metrics (if needed) or data needed for performance metric calculation (if needed) can be generated by UE and terminated at gNB | Monitoring data = Performance metrics are available inside the UE. UE can independently monitor a model's performance w/o any data input from NW |

Table 7.3.1.2-3. LCM input data related entity for Beam prediction use case

| Beam Prediction | NW sided | UE sided |
| --- | --- | --- |
| training | training data = can be generated by UE/NW |  |
| inference | input data = can be generated by UE and terminated at gNB | input data = internally available in UE |
| monitoring | Calculated performance metrics (if needed) or data needed for performance metric calculation (if needed) can be generated by UE and terminated at gNB | Performance metrics are available inside the UE UE can independently monitor a model's performance w/o any data input from NW |

Among these, LCMs that require data collection and information exchange for each specific use case include:

- CSI compression: NW/UE part training, NW part inference, and NW/UE part monitoring

- CSI prediction: NW sided monitoring

- Beam management: NW/UE sided training, NW sided inference, and NW sided monitoring

To provide a comprehensive overview of different available data collection methods, RAN2 have endorsed as starting point Table 7.3.1.2-1 which describes key indicators of each to be considered. An extension to the existing table has been agreed upon. This enhanced table includes three additional columns: Inference, Monitoring, and offline Training. These columns serve as valuable resources, detailing the applicability of data collection techniques to the LCM purpose and specific use cases.

Editor’s note: The development of the Data Collection Table is pending on further RAN1 input concerning RAN2’s assumptions and queries.

Table 7.3.1.2-4. Existing data collection methods identified

| Method | Involved Network entity | RRC state to generate data | Max payload size per reporting* | Contents to be collected | End-to-End report latency** | Report type | Security and Privacy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Logged MDT | TCE/OAM (It can be utilized by gNB) | RRC_IDLE/RRRC_INACTIVE | <9kbyte | L3 cell/beam measurements, location info, sensor info, timing info | - Procedure latency***: - Latency to enter CONNECTED state - Latency to receive gNB request signaling (~20ms) - Air interface signaling latency****: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message, Privacy via user consent |
| Immediate MDT | TCE/OAM (It can be utilized by gNB) | RRC_CONNECTED | <9kbyte | L3 cell/beam measurements, location info, sensor info | - Procedure latency: - Report interval: - l20ms~30min for periodic report - TTT for event triggered report - Air interface signaling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | Event triggered report, Periodic reporting | AS security via RRC message, Privacy via user consent |
| L3 measurements | gNB | RRC_CONNECTED | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Report interval: - l20ms~30min for periodic report - TTT for event triggered report - Air interface signaling latency: - 20ms (RRC) | Event triggered report, Periodic reporting | AS security via RRC message. |
| L1 measurement (CSI reporting) | gNB | RRC_CONNECTED | <1706bit in PUCCH, <3840bit in PUSCH | L1 CSI measurement | - Procedure latency: - Report interval: - 4-320 slot for periodic report and semi-persistent report - 0-32 slot after reception of DCI for aperiodic report - Air interface signaling latency: - 1 TTI (PUCCH) | Aperiodic report, Semi-persistent report, Periodic report | No AS security |
| UAI | gNB | RRC_CONNECTED | <9kbyte | Assistance information to show UE preference | - Procedure latency: - Upon generation of UE's preference - Air interface signaling latency: - ~20ms (RRC) | Up to UE implementation when to report | AS security via RRC message |
| Early measurements | gNB | RRC_IDLE/RRC_INACTIVE | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Latency to enter CONNECTED state - Latency to receive gNB request signaling (~20ms) - Air interface signaling latency: - ~20ms (RRC) | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message |
| LPP | LMF | RRC_CONNECTED | <9kbyte | Location info | - Procedure latency: - Latency to get upper layer trigger (for UE triggered) - Or latency to receive NW request message (~20ms) - Air interface signaling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and LMF | UE-triggered, NW-triggered | AS security via RRC message |

Note:
* The payload size doesn't consider signalling overhead.
** The End-to-End report latency is the latency from availability of the measurement report at the UE side to the availability of the measurement report at the terminated network entity. The time to generate data or perform measurements depends on RAN1/RAN4 specification.
*** Procedure latency is the latency caused by procedures, including procedure to ready for reporting (e.g., entering CONNECTED state, report interval).
****Air interface signalling latency is the latency to transmit one report, e.g., RRC signalling latency or PUCCH signalling latency.