3GPP TSG-RAN2 Meeting #123-bis	R2-2311106

# Xiamen, China, 9th – 13th October 2023

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4375 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancing SCell A2 event reporting [TEI] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI Corporation, Ericsson, NTT Docomo, BT Plc., AT&T, Turkcell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the A2 event definition, the term ‘becomes worse than’ is used. This will create some ambiguity when it comes to measurement reporting from the UE for A2 event. Consider the scenario wherein the NR SpCell configures the UE with a measID associated to A2 event towards an SCell. The event A2 refers to a scenario of serving cell becoming worse than a threshold. In this scenario, if the UE is not in the coverage area of the serving cell on the frequency for which the event A2 is configured, then the UE might not have any L3 filtered measurement to compare against the A2-threshold. In such a scenario, it has been observed in the field that some UEs might not trigger the measurement report. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that the UE shall use the lowest value range of the measurement quantity to compare against the A2-threshold when the L3 filtered measurements are not available for the serving cell associated to A2-event. If the UE is implemented according to the CR and the network is not, there will not be inter-operability problems. Implementation of this CR by a Release 15 UE will not cause compatibility issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behaviour remains unclear when there is no serving cell measurement and an A2 event is configured for the corresponding serving cell MO. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

5.5.4.3	Event A2 (Serving becomes worse than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A2-2, as specified below, is fulfilled;

1>	for this measurement, consider the serving cell indicated by the measObjectNR associated to this event.

NOTE X:	If the L3 filtered measurements for the serving cell indicated by the measObjectNR associated to this event is not available, then the UE shall consider for the value of Ms the lowest value of the value range of the measurement quantity as the serving cell measurement.

Inequality A2-1 (Entering condition)

Ms + Hys < Thresh

Inequality A2-2 (Leaving condition)

Ms – Hys > Thresh

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a2-Threshold as defined within reportConfigNR for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

START OF CHANGES