3GPP TSG-RAN WG2 Meeting #123bis                                                                    R2-2311111

Xiamen, China, 9 –13 October 2023

Agenda Item:	7.5.5

Source: 	Meta

Title:  	UE Capabilities for Rel-18 XR

Document for:	Discussion and Decision

# Introduction

WID of Rel-18 XR [1] has the following scope:

Specify the enhancements related to power saving:

-	DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g. RRC signalling) (RAN2).

Specify the enhancements related to capacity:

-	Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2);

-	Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2);

-	Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2);

-	Delay reporting of buffered data in uplink (RAN2);

-	Discard operation of PDU Sets for DL and UL (RAN2, RAN3);

Specify the enhancements for XR Awareness:

-	Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2);

-	Impact of identifying by UE of PDU Sets, Data bursts and PSI, as needed (RAN2);

-	Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3);

-	Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

The UE features and enhancements can be mapped from the objectives in the WID, as follows:

- DRX enhancements

- Configured grant enhancements

- BSR enhancements

- PDU discard operation

- XR awareness

In this paper, we discuss UE capabilities for supporting Rel-18 XR enhancements.

# Discussion

In the power saving enhancements, RAN2 agrees to support DRX cycle with rational numbers for traffic with non-integer XR periodicity. RAN2 also agrees to use a counter in the DRX formula that increments at every SFN wrap around to address the SFN wrap around issue.

Proposal 1: Separate UE capabilities should be introduced. A new UE capability is defined to identify Rel-18 UEs supporting C-DRX enhancements for traffic with non-integer periodicity. Another new UE capability is defined to identify Rel-18 UEs supporting the DRX formula with a counter to resolve the C-DRX SFN wrap-around issue.

On configured grant enhancements, RAN2 agrees to support retransmission-less CG.

Proposal 2: A new UE capability is defined to identify Rel-18 UEs supporting retransmission-less CG.

In the capacity enhancement, RAN2 agrees to introduce a new BS table. RAN2 also agrees to support delay status reporting to improve the delay aware scheduling at the gNB.

Proposal 3: Separate UE capabilities should be introduced.  A new UE capability is defined to identify Rel-18 UEs supporting BSR enhancement. Another new UE capability is defined to identify Rel-18 UEs supporting delay status reporting.

On the PDCP discard, RAN2 agrees to support the PDU set-based discard operation. RAN2 also agrees to support the PSI-based discard.

Proposal 4: Separate UE capabilities should be introduced.  A new UE capability is defined to identify Rel-18 UEs supporting PDU set-based discard. Another new UE capability is defined to identify Rel-18 UEs supporting PSI-based discard.

On the XR awareness enhancements, RAN2 agrees to support traffic assistance information from the UE to the network to improve the scheduling performance.

Proposal 5: A new UE capability is defined to identify Rel-18 UEs supporting UAI-based XR traffic assistance information.

# Conclusion

Proposal 1: Separate UE capabilities should be introduced. A new UE capability is defined to identify Rel-18 UEs supporting C-DRX enhancements for traffic with non-integer periodicity. Another new UE capability is defined to identify Rel-18 UEs supporting the DRX formula with a counter to resolve the C-DRX SFN wrap-around issue.

Proposal 2: A new UE capability is defined to identify Rel-18 UEs supporting retransmission-less CG.

Proposal 3: Separate UE capabilities should be introduced.  A new UE capability is defined to identify Rel-18 UEs supporting BSR enhancement. Another new UE capability is defined to identify Rel-18 UEs supporting delay status reporting.

Proposal 4: Separate UE capabilities should be introduced.  A new UE capability is defined to identify Rel-18 UEs supporting PDU set-based discard. Another new UE capability is defined to identify Rel-18 UEs supporting PSI-based discard.

Proposal 5: A new UE capability is defined to identify Rel-18 UEs supporting UAI-based XR traffic assistance information.

# References

[1]	RP-230786, Updated WID on XR Enhancements for NR, Nokia, Qualcomm

[2]	R2-2305492, UE Capabilities for Rel-18 XR WI, Intel Corporation