3GPP TSG-RAN WG2 Meeting #123bis                                    	R2-2311112

Xiamen, China: October 9-13, 2023

Agenda item:	7.25.4

Source:	Qualcomm Incorporated

Title:	CP and UP latency evaluation TP

Document for:	Discussion and Decision

# Introduction

In RAN2#123, following agreements were made.

Agreements:

- At the moment, RAN2 assumes the best-case scenario even though RAN2 understands that it might not be a common scenario in some cases. Additional scenarios can be considered during the self evaluation work

- RAN2 assumes that both UE and gNB are located at the satellite’s nadir, i.e., elevation angles are 90 degrees, for the calculation of round trip delay (RTD).

- Given the assumptions of Proposal 1, feeder and service link delays are included in the propagation delay computation (RTD).

- For the mobility interruption evaluation, RAN2 assumes that for now it is sufficient to consider beam-based mobility in NTN.

- From RAN2 perspective, satellite on-board delay can be considered negligible.

- RAN2 assumes the CP procedure defined in Figure 1 as the baseline for the CP evaluation.

- For the best-case scenario, RAN2 assumes a lossless scenario (p=0) for the User plane evaluation / RAN2 will not consider retransmissions.

8.	RAN2 assumes the following for the evaluation of CP and UP latency:

-	NR FDD

-	Only NTN bands are considered (n255, n256).

-	UE capabilities 1 & 2

-	Resource type mapping A &B

-	SCS 15 kHz for the baseline scenario. FFS other supported scenarios (e.g., 30 kHz).

After email discussion, baseline scenarios for control plane and user plane latency have been finalized in [1]. In this document, we provide text proposal for evaluation table for the considered specific scenario.

# Discussion

Control plane latency

In [1], the considered scenario for CP latency is provided in Table 1. Further we suggest adding UE processing delay of L1 encoding before sending MsgA in step 1.1 and MsgA processing delay in step 3.

For CP latency, add UE processing delay of L1 encoding before sending MsgA in step 1.1 and MsgA processing delay in step 3.

Table 1 CP latency evaluation for NR NTN with 2 step RACH (i.e., MsgA and MsgB)

| Step | Description | CP latency for UL data transfer |
| --- | --- | --- |
| 1 | Delay due to RACH scheduling period (1TTI) | 0 |
| 1.1 | UE Processing Delay (L1 encoding of RRC Resume Request) for MsgA | Tproc,2/2 (Tproc,2 is defined in Section 6.4 of TS38.214), with d2 = d2,1= d2,2=0 |
| 2.1 | Transmission of MsgA |  |
| 2.1.1 | Transmission of RACH Preamble | Length of the preamble according to the PRACH format |
| 2.1.2 | Transmission interval | Length of the interval between PRACH and PUSCH transmissions |
| 2.1.3 | Transmission of PUSCH payload (RRCResumeRequest) | Ts (the length of 1 slot / non-slot) |
| 2.2 | Propagation delay UE -> BS | RTD/2 |
| 3 | MsgA detection and processing delay in gNB (preamble, L2, RRC) | Tproc,2 (assuming d2 = d2,1= d2,2=0) + 3 ms |
| 4 | Transmission of MsgB |  |
| 4.1 | Transmission of MsgB (RA response and RRCResume) | Ts (the length of 1 slot / non-slot) |
| 4.2 | Propagation delay BS -> UE | RTD/2 |
| 5 | Processing delay in UE of RRC Resume including RA response | 7 ms |
| 6 | Transmission of RRC Resume Complete and data | 0 |
| Notes: 1. For step 1, the procedure for transition from a most “battery efficient” state has yet not begun, hence this step is not relevant for the latency of the procedure which is illustrated by a '0' in the above. 2. For step 2.1.1, the length of the preamble associated with the PRACH format is specified in TS 38.211 [6]. 3. For step 2.1.2, the length of the interval between the transmission of PRACH and PUSCH is specified in TS 38.213. 4. For step 3, the processing delay in gNB (preamble, L2 and RRC) has been reduced to 3 ms. The delays due to inside-gNB or inter-gNB communication are not included in Step 3. Such delays may exist depending on deployment but are not within the scope of this evaluation. 5. For step 5 for UL data transfer, the processing delay in the UE (L2 and RRC) is considered, i.e., from reception of RRC Connection Resume including the RA response to the reception of UL grant. The transmission of UL grant by gNB and processing delay in the UE (processing of UL grant and preparing for UL tx) are also considered. The RRCConnectionResume message only includes MAC and PHY configuration. No DRX, SPS, or MIMO re-configuration will be triggered by this message. Further, the UL grant for transmission of RRC Connection Resume Complete and the data is transmitted over common search space with DCI format 0. 6. For step 6, the beginning of this subframe is considered to be "the start of continuous data transfer", hence this step is not relevant for the latency of the procedure which is illustrated by a '0' in the above. |  |  |

Assuming best case scenario (UE and gateway collocated and 600km satellite altitude), the UE-gNB RTT is assumed to be 8ms. As specified in Section 6.4 of TS 38.214, the PUSCH processing time Tproc,2 is

.

In this case, the assumption is d2 = d2,1= d2,2=0. For 15 kHz SCS, µ =0. The values of  and are also zero for NTN. The value of N2 does not depend on resource type. For UE capability 1, N2 = 10 symbols and for UE capability 2, N2 = 5 symbols. As specified in TS 38.211, the value of k = 64 and Tc = 0.509 ns. Therefore, Tproc,2 is 0.7135 ms for UE capability 1 and 0.7135 ms for UE capability 2.

Text proposal for the evaluation result is provided in Table 2.

Table 2 CP latency for FDD 1 ms PRACH length, 15kHz SCS

| CP latency in ms for FDD 1 ms PRACH length, 15kHz SCS, error p = 0 |  |  |  |
| --- | --- | --- | --- |
| Resource mapping type | Non-slot duration | UE capability 1 | UE capability 2 |
| Type A | M =14 (14OS slot) | 22.71354167 | 22.35677083 |
| Type B | M =7 (7OS non-slot) | 22.71354167 | 22.35677083 |

DL User plane latency

In [1], the considered scenario for DL UP latency is provided in Table 3.

Table 3 DL user plane procedure for NR

| ID | Component | Notations | Value |
| --- | --- | --- | --- |
| 1.1 | BS processing delay | tBS,tx The time interval between the data is arrived, and packet is generated. | Tproc,2/2, with d2,1= d2,2=0. (Tproc,2 is defined in Section 6.4 of TS38.214) (NOTE1) |
| 1.2 | DL frame alignment (transmission alignment) | tFA,DL The time interval between packet generation and the next Tx opportunity. | TFA TFA is the frame alignment time within the current DL slot. = half symbol duration |
| 1.3 | TTI for DL data packet transmission | tDL_duration | Length of one slot (14 OFDM symbol length) or non-slot (4/7 OFDM symbol length), depending on slot or non-slot selected in evaluation. |
| 1.4 | One-way propagation time BS -> satellite -> UE | tprop | RTD/2 |
| 1.5 | UE processing delay | tUE,tx The time interval between PDSCH reception and decoding of the data. | Tproc,1/2 (Tproc,1 is defined in Section 5.3 of TS38.214), d1,1=0 |
| Note: 1. The value is used for evaluation only; gNB processing delay may vary depending on implementation. |  |  |  |

As specified in Section 5.3 of TS 38.214, the PDSCH processing time Tproc,1 is

.

With additional assumption of N1,0=13, we have the value N1 = 8 for Resource type A, UE capability 1, N1 = 13 for resource type B UE capability 2 and N1 = 3 for UE capability 2. In addition, we assume that TFA = ½ symbol duration.

Text proposal for the evaluation result is provided in Table 2.

Table 4 DL UP latency for FDD, 1 ms PRACH length, 15kHz SCS

| DL UP latency for FDD 1 ms PRACH length, 15kHz SCS, error p = 0 |  |  |  |
| --- | --- | --- | --- |
| Resource mapping type | Non-slot duration | UE capability 1 | UE capability 2 |
| Type A | M =14 (14OS slot) | 5.675520833 | 5.31875 |
| Type B | M =7 (7OS non-slot) | 5.85390625 | 5.31875 |

UL User plane latency

In [1], the considered scenario for UL UP latency is provided in Table 5.

Table 5 UL user plane procedure for NR

| ID | Component | Notations | Value |
| --- | --- | --- | --- |
| 1.1 | UE processing delay | tUE,tx The time interval between data arrival and packet generation. | Tproc,2/2, with d2,1 = d2,2 = 0. Tproc,2 is defined in Section 6.4 of TS 38.214. |
| 1.2 | UL frame alignment (transmission alignment) | tFA,UL The time interval between packet generation and the next Tx opportunity. | TFA Length of one slot, since TFA is bounded by the slot duration. |
| 1.3 | TTI for UL data packet transmission | tUL_duration | Length of one slot (14 OFDM symbol length) or non-slot (4/7 OFDM symbol length), depending on slot or non-slot selected in evaluation. |
| 1.4 | One-way propagation time UE -> satellite -> BS | tprop | RTD/2 |
| 1.5 | BS processing delay | tBS,rx The time interval between PUSCH reception and decoding of the data. | Tproc,1/2, with d1,1 = 0. Tproc,1 is defined in Section 5.3 of TS 38.214. (Note 1) |
| Note: 1. The value is used for evaluation only; gNB processing delay may vary depending on implementation. |  |  |  |

Text proposal for the evaluation result is provided in Table 2.

Table 6 UL UP latency for FDD, 1 ms PRACH length, 15kHz SCS

| UL UP latency for FDD 1 ms PRACH length, 15kHz SCS, error p = 0 |  |  |  |
| --- | --- | --- | --- |
| Resource mapping type | Non-slot duration | UE capability 1 | UE capability 2 |
| Type A | M =14 (14OS slot) | 5.675520833 | 5.31875 |
| Type B | M =14 (14OS slot) | 5.85390625 | 5.31875 |

Therefore, for IMT 2020 satellite, we propose to add a table to show values that latency requirements can be met in the considered best case scenario.

Table 7 CP and UP Latency evaluations

| Latency for FDD, 1 ms PRACH length, 15kHz SCS, error p = 0 Satellite altitude 600km and UE-gNB RTT = 8ms |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | UE capability 1 Resource mapping type A | UE capability 2 Resource mapping type A | UE capability 1 Resource mapping type B | UE capability 2 Resource mapping type B |
| CP latency (ms) | 22.71 | 22.35 | 22.71 | 22.35 |
| DL UP latency (ms) | 5.67 | 5.31 | 5.85 | 5.31 |
| UL UP latency (ms) | 5.67 | 5.31 | 5.85 | 5.31 |

Add a Table 7 for CP and UP latency evaluation result in TR 37.911.

# Conclusion

Following observations and proposals are made.

Proposal 1	For CP latency, add UE processing delay of L1 encoding before sending MsgA in step 1.1 and MsgA processing delay in step 3.

Proposal 2	Add a Table 7 for CP and UP latency evaluation result in TR 37.911.

# References

[1] R2-2309714 - Report of [Post123][102]NTN Self Ev] CPUP latency.